3GPP TSG-RAN WG2 Meeting #120	R2-2212427

# Toulouse, France, 14 – 18 November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3699 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on configuration of SDT with SL relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core, NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to what has been agreed in the SL relay WI, a UE acting as a L2 U2N Remote UE does not perform random access when establishing an RRC connection to the network via a Relay UE. In the context of SDT, this means that a UE should not perform any SDT procedure when in RRC_INACTIVE and connected to the network via a Relay UE. This needs to be clarified in current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.3.13.1b - Clarified that if the UE is acting as a L2 U2N Remote UE the UE should not start the SDT procedure. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: SDT Inter-operability: 1. If the network is implemented according to the CR and the UE is not, a UE that is acting as a L2 U2N Remote UE may erroneously start the SDT procedure. 2. If the UE is implemented according to the CR and the network is not, a UE that is acting as a L2 U2N Remote UE may erroneously start the SDT procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, a UE that is acting as a L2 U2N Remote UE may erroneously start the SDT procedure. This may result in a radio link failure as the random access procedure may fail, in case the remote UE is not physically located in the same serving cell of the relay UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.13.1b	Conditions for initiating SDT

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	the upper layers request resumption of RRC connection; and

1>	SIB1 includes sdt-ConfigCommon; and

1>	sdt-Config is configured; and

1>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

1>	lower layers indicate that conditions for initiating SDT as specified in TS 38.321 [3] are fulfilled; and

1> if the UE is not acting as a L2 U2N Remote UE.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

END OF CHANGES