3GPP TSG-RAN WG1 Meeting #120 	R2-2212480

Toulouse, France, November 14-18, 2022

Agenda item:	    8.19 R18 Other

Source: 		Qualcomm Incorporated

Title: 	Discussion of RAN2 response to LS in R2-2211134/S2-2209876 on Time Synchronization Status notification towards UE.

Document for:	Discussion

# Introduction

In this contribution we discuss the LS by SA2 to RAN2 in S2-2209876 on Time Synchronization Status notification towards UE.

# Problem Description

SA2 has determined there is a need to indicate a “clock quality” field to certain UEs that need accurate timing information. In the legacy procedure, the UE always assumes that timing information for SIB9 carries the correct timing information and uses it to synchronize its clock without information about the source clock quality. However, SA2 has identified the need for some UEs to be notified of clock quality, aside from the timing information. To that end, SA2 has proposed four (or five) alternatives to perform that task as explained in [2]:

- Alt 1: gNB provides a reference report ID within SIB

  - Alt 1a: The UE can actively retrieve the RAN timing synchronization status information from the network by entering RRC_CONNECTED.

  - Alt 1b: Report ID is an index that maps to a pre-defined and/or standardized time synchronization characteristics thus the UE can automatically determine this without having to move to RRC_CONNECTED state.

- Alt 2: Flag and Timestamp is included in SIB9.

- Alt 3: Broadcast the time change status in SIB.

- Alt 4: Ciphered RAN Time Synchronization Status in SIB.

# RAN2 Assessment

The task at hand requires UEs to be informed with the clock quality information at the cell, possibly to allow the UE to decide on internal synchronization when moving between cells or when the gNB clock quality changes. It is also clear that there is a need to update the IDLE and INACTIVE UEs with information about improvement or degradation about the clock quality. Therefore, simple RRC signalling is not sufficient for the task, an indication of clock quality or clock quality change needs to be included in SIB.

Observation 1: There is a need to include information about clock quality or clock quality change in SIB in order to inform IDLE and INACTIVE UEs of the clock status.

All the proposed alternatives suggest different variations of what information to include in SIB. For example,  Alternative 1a and Alternative 2 attach a report ID/flag and timestamp that informs the UEs of a clock quality change event in order to motivate the UE to go into RRC_CONNECTED mode to retrieve that clock report in an RRC unicast message (supposedly to create a space for UE subscription to those updates), while not confusing UEs that have already retrieved that clock report into moving to RRC_CONNECTED to retrieve the same report again. Alternatives 1b and Alternative 3 do not mandate that the UEs go into RRC_CONNECTED mode as they include the report or an index to the report in SIB to be available to all the UEs. Alternative 4 attempts a middle ground by including the report to all UEs but making it decipherable to only some of the UEs.

From a RAN2 standpoint, all the alternatives are valid to perform the task of informing the UE of a clock change, especially, when targeting IDLE and INACTIVE UEs. The assessment of which alternative(s) are better depends on the type of service the network attempts to offer to the UE, which is not in RAN2 scope.

Observation 2: From a RAN2 standpoint, all the alternatives are valid to perform the task of informing the UE of a clock quality change event, especially, when targeting IDLE and INACTIVE UEs.

Observation 3: The exact assessment of the proposals relative to each other depends on the service offered by the network, which is not in RAN2 scope.

Proposal 1: RAN2 to consider the proposed draft LS response.

# Conclusion

Observation 1: There is a need to include information about clock quality or clock quality change in SIB in order to inform IDLE and INACTIVE UEs of the clock status.

Observation 2: From a RAN2 standpoint, all the alternatives are valid to perform the task of informing the UE of a clock quality change event, especially, when targeting IDLE and INACTIVE UEs.

Observation 3: The exact assessment of the proposals relative to each other depends on the service offered by the network, which is not in RAN2 scope.

Proposal 1: RAN2 to consider the proposed draft LS response.

# Annex Draft Response from RAN2 to SA2

RAN2 would like to thank SA2 for the question related to Time synchronization status notification.

RAN2 recognizes the need to inform IDLE and INACTIVE UEs of a change in a clock quality mandates that certain information about event of clock quality change or the new clock quality information would need to be included in SIB. From that standpoint, there is not much difference from a RAN2 standpoint between all the proposed alternatives. The relative assessment of which alternatives are preferrable depend on the type of service offered to the UE (e.g., service intended to specific subscribed UEs vs service made available to all UEs for example) and the information that needs to be known at the UE (e.g., index to clock quality vs more detailed report) which are not in RAN2 scope.

# References

[1] LS S2-2209876 on Time Synchronization Status notification towards UE.

[2] TR 23.700-25 Technical Specification Group Services and System Aspects; Study on timing resiliency and TSC and URLLC enhancements (Release 18)

# Annex LS from SA2: R2-2211134/S2-2209879

1. Overall Description:

3GPP SA WG2 is working on the study item “Study on 5G Timing Resiliency and TSC & URLLC enhancements (FS_5TRS_URLLC)”, documented in TR 23.700-25. SA2 would like to get your feedback regarding the following questions.

A) On 5GS time synchronization status report towards the UE(s) (KI#1):

SA2 has agreed that 5GS shall support informing UE and AF about the 5GS clock quality, including informing the UE and AF about 5GS clock quality degradation/improvement at different levels of degradation and improvement.

Editor's note: 	How the clock quality of a given cell is indicated to UE and AF, e.g., by providing a set of individual metrics (for example UTC traceability, accuracy, frequency stability, etc.) or by providing a clock quality index or whether to support both options is FFS.

In this regard, SA2 has discussed different methods on how the RAN time synchronization status (i.e., the time synchronization status of an NG-RAN node) can be provided to UE(s). These methods are documented in TR 23.700-25 Annex A. Related to these methods SA2 has the following questions:

RAN2:

- Please provide feedback on the methods listed in Annex A from RAN2 perspective.

RAN3 and SA3:

- One method proposes to use the Ciphered SIB approach that is used for broadcast of assistance data for positioning. SA2 would like to additionally get feedback on this from SA3 and RAN3 from security and NGAP impact perspective, respectively.

B) On additional methods to obtain RAN timing synchronization status from NG-RAN via control plane signalling (KI#1):

Another aspect under discussion in SA2 is the method the 5G core may use to receive timing synchronization status from NG-RAN. RAN timing synchronization information status is a report that describes the node’s synchronization status related to its time source (e.g., time source degradation/failure/improvement). SA2 has agreed that 5G core can receive this status information directly from OAM. An alternative method under consideration is the use of control plane signalling at node level (documented in TR 23.700-25 solution #4). In this control plane method, the AMF shall configure the NG-RAN node to report the RAN timing synchronization status report (i.e., node level signalling) when there is an event related to the time source the NG-RAN node is using, and these reports will be forwarded to the subscribed NFs in 5G core (i.e., the TSCTSF).

RAN3:

SA2 would like to seek RAN3 input regarding the potential impacts in RAN3 specifications (e.g., NGAP, F1-AP) to enable RAN time synchronization status report control procedure.

2. Actions:

To 3GPP RAN WG2, SA3, RAN3

ACTION:

SA WG2 kindly requests 3GPP RAN WG2, RAN3 and SA3 to provide feedback.

3. Date of Next TSG SA WG2 Meetings:

3GPP TSG-SA2 Meeting #154	14 - 18 November 2022	Toulouse, France

3GPP TSG-SA2 Meeting Ad Hoc	16 - 20 January 2023	e-meeting