3GPP TSG-RAN WG2 Meeting #120	R2-2212485

Online, 14th – 20th November, 2022

Agenda item:	7.2.2.1

Source:	Samsung

Title:	On neighbouring cell ephemeris for IoT NTN

WID/SID:	LTE_NBIOT_eMTC_NTN

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss issues related to neighbor ephemeris for Rel-17 IoT NTN [1].

# Discussion

## Ephemeris signalling

- The Ephemeris IE is 136 and 144 bits for State Vectors and Orbital parameters respectively [2]. TA parameters are 0 to 23 (ta-Common) + 19 (ta-CommonDrift) + 15 (ta-CommonDriftVariation) = 57 bits. This means that for signalling one cell/satellite the amount of bits not counting ASN1 overhead is between 136 to 201 bits. For serving cell satellite, the network also mandatorily signals the ul-SyncValidityDuration, as well as k-Offset, which are 4 bits and 10 bits each. Optional fields epochTime occupies 10 + 4 bits and k-Mac occupies 9 bits. Serving cell satellite info thus occupies between 150 bits to 238 bits depending on implementation, and this is not considering ASN1 overhead that for instance increases due to optionality of fields etc.

Observation 1: Serving cell ephemeris size comes out at least 150 to 238 bits.

## Neighbour cell ephemeris signalling

- In RAN2#120 the following LS was received from RAN4 [3]:

In LTE_NBIOT_eMTC_NTN_req WI, RAN4 is specifying RRM requirements for NB-IoT and eMTC operation over NTN functionality defined in the Rel-17 LTE_NBIOT_eMTC_NTN work item.

In RAN4#104-e, the following agreement [1] is reached on information for neighbor/target cell in IoT NTN

#### Issue 1-3-2: information for the neighbor/target cell - Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE.

- In RAN4, the agreement traces back to one of the contributions [4], where the reasoning for introducing the neighbouring cell ephemeris is explained:

Connected mode mobility (HO and CHO) and measurement are supported for eMTC. In NR NTN, it is agreed that all the mobility and measurement requirements are based on a side condition that valid information for the neighbor/target cell is made available to the UE. Since the basic mechanism for mobility and measurement is same for LTE and NR, for eMTC UE to correctly perform measurement or HO to the target cell, the necessary information (as in Issue 1-6 in [4]) of target cell are also needed.

Proposal 9: Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbor/target cell is made available to the UE.

- In the LS it is stated that RAN4 is specifying requirements for Rel-17. We would thus interpret that neighbouring cell ephemeris is required at least for LTE-M for connected mode measurements and mobility. However, for NB-IoT it is not clearly stated in the LS, but since the mobility and measurement is only applicable for LTE-M and not NB-IoT, it is not clear for RAN2 whether the neighbouring cell ephemeris is required.

Proposal 1: Neighbouring cell ephemeris is introduced for LTE-M in SIB31.

Proposal 2: Confirm with RAN4 whether neighbouring cell ephemeris is needed for NB-IoT for Rel-17 where connected mode measurements are not expected.

- Introducing neighbouring cell ephemeris have some serious consequences for the signalling. The reason is because the ephemeris signalling requires a lot of bits, which effects the coverage, which is already severely limited by typical IoT devices. Furthermore, there are Transport Block size restrictions for NB-IoT and LTE-M Rel-13 at 680 and 1000 bits respectively.

- In order to reduce the size and not have to specify rules for ignoring fields we believe that a backwards compatible structure with only the necessary information should be introduced for measuring neighbouring cells. In our understanding, the required information for neighbour cell measurements would be ephemeris parameters and ta-Common parameters – which comes out at 136 to 201 bits per satellite. The parameters needed for target NTN cell are already provided in dedicated SIB31 for HO and CHO.

Proposal 3: Needed parameters for neighbor cell measurements are ephemeris and common TA.

- To capture that the needed parameters for neighbor cell measurements are ephemeris and common TA, the below CR can be seen as a baseline:

- --------------------- 36.331 V17.2.0 ---------------------

SystemInformationBlockType31 information element

-- ASN1START

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[

neighbourSatelliteInfoList-v1730	NeighbourSatelliteInfoList-v1730	OPTIONAL

]]

}

ServingSatelliteInfo-r17 ::= 	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512) 				OPTIONAL,	-- Need OP

...

}

NeighbourSatelliteInfoList-v1730 ::= SEQUENCE (SIZE (1..maxNeighbourEphemeris)) OF NeighbourSatelliteInfo

NeighbourSatelliteInfo-r17 ::= 	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

}

}

-- ASN1STOP

- --------------------- 36.331 V17.2.0 ---------------------

Proposal 4: Take the CR as a baseline for signaling required neighbour cell information.

# Conclusion

In this contribution we have the following observations:

Observation 1: Serving cell ephemeris size comes out at least 150 to 238 bits.

In this contribution we have the following proposals:

Proposal 1: Neighbouring cell ephemeris is introduced for LTE-M in SIB31.

Proposal 2: Confirm with RAN4 whether neighbouring cell ephemeris is needed for NB-IoT for Rel-17 where connected mode measurements are not expected.

Proposal 3: Needed parameters for neighbor cell measurements are ephemeris and common TA.

Proposal 4: Take the CR as a baseline for signaling required neighbour cell information.

# Reference

RP-221806 Revised WID on IoT NTN enhancements.

Chair’s Notes RAN1#107-e

R2-2211171, “LS on information for neighbor/target cell in IoT NTN”, RAN2#120, Toulouse, November 2022

R4-2212971, “Discussion RRM requirements for IoT NTN”, Huawei, RAN4#104-e, August 2022