3GPP TSG-RAN WG2 #120	R2-2212490

Toulouse, France, November 14 – 18, 2022

Agenda Item:	6.16

Source:	Ericsson

Title:	Changing the Need Code for gins-PerSNPN-List

Document for:	Discussion, Decision

# 1	Introduction

After no agreement was reached during RAN2#119-e (see summary of offline discussion in R2-2209113), this document further discusses the need to change the Need Code of the gins-PerSNPN-List field.

# 2	Discussion

## 2.1	New SIB for GINs

The new SIB18 which is intended for eNPN’s GINs currently looks like this in TS 38.331 [1]:

#### –	SIB18

SIB18 contains Group IDs for Network selection (GINs) to support access using credentials from a Credentials Holder or to support UE onboarding.

SIB18 information element

-- ASN1START

-- TAG-SIB18-START

SIB18-r17 ::=               SEQUENCE {

gin-ElementList-r17         SEQUENCE (SIZE (1..maxGIN-r17)) OF GIN-Element-r17            OPTIONAL,   -- Need R

gins-PerSNPN-List-r17       SEQUENCE (SIZE (1..maxNPN-r16)) OF GINs-PerSNPN-r17           OPTIONAL,   -- Need R

lateNonCriticalExtension    OCTET STRING                                                  OPTIONAL,

...

}

GIN-Element-r17 ::=         SEQUENCE {

plmn-Identity-r17           PLMN-Identity,

nid-List-r17                SEQUENCE (SIZE (1..maxGIN-r17)) OF NID-r16

}

GINs-PerSNPN-r17 ::=        SEQUENCE {

supportedGINs-r17           BIT STRING (SIZE (1..maxGIN-r17))                             OPTIONAL   -- Need R

}

-- TAG-SIB18-STOP

-- ASN1STOP

SIB18 field descriptions

gin-ElementList The gin-ElementList contains one or more GIN elements. Each GIN element contains either one GIN, which is identified by a PLMN ID and a NID, or multiple GINs that share the same PLMN ID. The total number of GINs indicated does not exceed maxGIN-r17. The GIN index m is defined as d1+d2+…+d(n-1)+i for the GIN included in the n-th entry of the gin-ElementList and the i-th entry of its corresponding GIN-Element, where d(k) is the number of GIN index values used in the k-th gin-ElementList entry.

gins-PerSNPN-List Indicates the supported GINs for each SNPN. The network includes the same number of entries as the number of SNPNs in snpn-AccessInfoList in provided in SIB1, and the n-th entry in this list corresponds to the n-th SNPN listed in snpn-AccessInfoList provided in SIB1. This field is not present if there is only a single SNPN in snpn-AccessInfoList in SIB1, as in that case all GINs in this SIB is associated with that SNPN.

GINs-PerSNPN field descriptions

supportedGINs Indicates the GINs which are supported by the given SNPN. The first/leftmost bit corresponds to the GIN with GIN index 1, the second bit corresponds to the GIN with GIN index 2 and so on. A bit set to 1 indicates that the GIN is supported by the SNPN. If the field is not present, then the corresponding SNPN does not support any GINs.

## 2.2	Changing the Need Code of the gins-PerSNPN-List

During RAN2#118-e email discussion in R2-2206429 [3], there was no consensus on whether the Need Code of ginsPerSNPN-List-r17 should be changed to Need Code S, as proposed by RIL E037 [2].

On this matter, the last sentence of the field description (highlighted text above) was indeed modified during that meeting (resulting in what we observe above in Section 2.1), arguably, to avoid such change.

However, as seen in the spec., Need S and Need R are used for the following scenarios:

| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| --- | --- |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

As captured in the field description, gins-PerSNPN-List indicates the association between GINs and SNPNs. Need R means that the field is released upon absence, i.e., the normal interpretation would be that absence indicates that no association is defined between the GIN(s) and SNPN(s). Thus, in order to indicate that the UE shall associate all GINs with a single SNPN, the gins-PerSNPN-List should normally be present with all bits in supportedGINs being set to the value ‘1’.

However, to save the bits needed to broadcast the Gins-PerSNPN, RAN2 agreed the following optimization at RAN2#117-e:

It is agreed (the optimization) that gins-PerSNPN is absent when the cell only supports a single SNPN.

In this regard, how to interpret absence of the field is already captured in the field description, i.e., the UE shall associate all GINs with the (single) SNPN.

The alternative interpretation would be that none of the GINs are associated with the SNPN.

Hence, according to our understanding, even with the new rewording, ginsPerSNPN-List-r17 should still be marked with Need Code S instead of Need Code R.

Even with RAN2#118-e rewording, there is still a need to change the Need Code of ginsPerSNPN-List-r17, as there is a special UE behaviour linked to the absence of the field.

Furthermore, and when focusing on RAN2#119-e’s email discussion (see R2-2209113 [4]), one can observe that it is in some companies’ understanding that the concerning field should not be ‘Need S’, as for this case, the field is “simply” describing a scenario on which the field may be omitted by the network due to signalling optimization.

On this matter, the field description in the specification does not need to capture network recommendations/implementation. There is only a need to capture what the UE is supposed to do.

The field description should not be used to capture recommendations of implementation for the NW. The field description should only need to capture UE behaviour.

Hence, as also stressed by other companies in R2-2209113, this is special UE behavior upon absence, for which ‘Need R’ is not enough. Therefore, the Need Code for this field should be changed to Need S.

Change the Need Code of ginsPerSNPN-List-r17 from Need R to Need S.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Even with RAN2#118-e rewording, there is still a need to change the Need Code of ginsPerSNPN-List-r17, as there is a special UE behaviour linked to the absence of the field.

Observation 2	The field description should not be used to capture recommendations of implementation for the NW. The field description should only need to capture UE behaviour.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Change the Need Code of ginsPerSNPN-List-r17 from Need R to Need S.

# 4	References

TS 38.331, v17.2.0, RAN WG2, Sept. 2022

R2-2206012, “[E036][E037] Corrections for eNPN”, Ericsson, RAN2#118-e, May, 2022

R2-2206429, “Report from [AT118-e][035][eNPN] Corrections”, Nokia, RAN2#118-e, May, 2022

R2-2209113, “Summary of [ AT119-e][027][NPN] NPN corrections (ZTE)”, ZTE Corporation, Sanechips, RAN#119-e, Aug. 2022