3GPP TSG-RAN WG2 Meeting #120                 	                                          R2-2212629

Toulouse, France, 14th- 18th November, 2022

Agenda Item:	8.11.2

Source:	CMCC

Title:	Discussion on multicast reception in RRC_INACTIVE

Document for:	Discussion

Introduction

Multicast reception for the UE in RRC_INACTIVE state was discussed in the previous RAN2 meetings, following are the agreements made in RAN2#119 bis-e meeting [1]:

- The following general description is taken as baseline for PTM configuration delivery Option 1: - (1-a) PTM configuration(s) (i.e., configurations used for multicast reception in RRC_INACTIVE) of one or more multicast sessions for at least one cell are provided via dedicated RRC signaling to a UE. - (1-b) The RRC message for this includes RRCReconfiguration and/or RRCRelease and/or RRCResume (details FFS) - (1-c) UE stores the received configurations while it is in RRC_INACTIVE, and if there is a need to update some or all the configurations, the UE is notified of such changes and may trigger RRC connection resume to obtain the updated configurations. In case of mobility in RRC_INACTIVE, the UE triggers RRC connection resume if the configuration of the session is not available for the new cell. - The following general description is taken as baseline for PTM configuration delivery Option 2: - (2-a) PTM configurations (i.e., configurations used for multicast reception in RRC_INACTIVE) are provided via an MCCH-like channel (same or different as used for MBS broadcast), and information regarding MCCH scheduling is provided via SIB, FFS dedicated signalling - (2-b) UE can receive such configurations when it is in RRC_INACTIVE, FFS whether it is allowed/needed to also receive when UE is in RRC_CONNECTED - (2-c) If there is a need to update some or all the received configurations, UE does not need to resume RRC connection but is notified of such changes (e.g. via MCCH DCI) and obtains the updated configurations via MCCH. - Dedicated RRC signalling (i.e. RRC release message with suspendConfig) is used for switching a multicast receiving UE from RRC_CONNECTED to RRC_INACTIVE and continue multicast reception (details FFS). - For both option 1 and option 2, as a baseline, group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, and UEs continue the multicast reception in CONNECTED. FFS if there is any potential issue if Rel-17 group paging is reused. FFS if there are other cases when UE triggers resume. FFS if MCCH can also be used in case of option 2. - FFS whether to introduce PTM configuration applicable area, i.e., the mechanism that the PTM configurations, once acquired by a UE, may apply to a certain area (i.e., a set of cells instead of a single cell). - Rel-18 UE in INACTIVE can be informed when the session is activated (Details FFS). - As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification). - If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways). - Rel-17 mechanism (NAS-based indication) is applicable for multicast session release. FFS if any enhancement is needed. - FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated, taking into account the following solutions (can further update the descriptions if needed, and several solutions may be needed, some solutions may apply only for certain configuration options) - 1. When the multicast session is activated, UE can receive the multicast session in RRC_INACTIVE if the PTM configuration used in RRC_INACTIVE for the session is available to the UE and the UE has joined the session already (e.g., configuration provided to UE via dedicated RRC signaling or via MCCH), otherwise it goes back to RRC_CONNECTED to receive the multicast session. - 2. When the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not (detailed signaling FFS). - 3. UE is configured "whether it can receive the multicast session in RRC_INACTIVE" by dedicated signaling before UE is released. When the multicast session is activated, UE stays in RRC_INACTIVE or resumes RRC connection accordingly (detailed signaling FFS). - If option 1 is supported for PTM configuration - As a baseline, group paging may be used to inform the UE when network changes the PTM configurations, and UE upon reception triggers RRC connection resume procedure to obtain the updated configurations (details of group paging can be FFS). - FFS whether and how to solve the issue in signalling/system load when a large number of UEs in the cell need PTM configuration update. - FFS if there is an issue that a UE can obtain all the PTM configurations for a multicast service via Option 2 without/before joining the multicast session on the condition that security is enabled by service layer. And if yes FFS how to solve the issue (e.g., dedicated configuration + MCCH).

In this contribution, we would focus on the PTM configuration delivery solution comparison and discuss the related enhancements.

Discussion

Comparison of different PTM configuration delivery solutions

In RAN2#119 bis-e meeting, the general description of the two PTM configuration delivery options were achieved, for option1, the RRC dedicated signalling is used for PTM configuration delivery, while for option 2, MCCH-like channel is used.

The main issues to be solved/enhanced for the two options are identified separately. For Option 1, it’s about PTM configuration updating, there can be signalling/system load issue due to a large number of UE request for RRC connection resume and acquire the new configuration with dedicated signalling. For Option 2, there can be potential security issue.

From our point of view, Option 1 with dedicated signalling for PTM configuration delivery is preferred, considering the following aspects:

As agreed in RAN2, it’s gNB to decide whether a multicast session can be received in RRC_INACTIVE and whether a specific UE can receive multicast in RRC_INACTIVE. And the state transition from RRC_CONNECTED to RRC_INACTIVE for the UE is under network’s control via RRC dedicated signalling, it’s spontaneous to provide the related configuration in RRC dedicated signalling since it cannot be avoided.

Observation 1: To switch UE’s state from RRC_CONNECTED to RRC_INACTIVE, dedicated RRC signalling cannot be avoided, it’s spontaneous to provide the related configuration in it.

When it comes to the configuration updating, the key point to this is the accompanied UE states transition. From our point of view, PTM configuration may not be frequent, and as it was agreed that group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, while current group paging/notification mechanism, the UE monitor the same PO/PF as unicast paging, which disperse UEs in time domain. Therefore, in this case, there cannot be congestion issues. And there’s similar situation for the scenario of group notification used for session activation indication discussed in Rel-17.

Observation 2: For PTM configuration updating case, there cannot be congestion since UEs are informed via group notification in time domain dispersedly.

What’s more, just as we replied to SA2’s LS [3], the MBS session related QoS parameters are the same for different UEs within the same MBS session. Given that, we can assume that UE’s QoS requirements for the MBS session can be satisfied in RRC_INACTIVE, though there can be some quality and reliability difference due to lack of HARQ feedback and PTP leg, compared with UEs in RRC_CONNECTED. Therefore, if a UE is released to RRC_INACTIVE for multicast reception, the network may not resume its RRC connection even the congestion in the network is relieved, unless the transmission quality cannot be assured and the resume is requested by UE.

Observation 3: For congestion relieving case, UE may stay in RRC_INACTIVE for multicast reception if the transmission quality requirements can be achieved.

In Rel-17 discussion on multicast and broadcast, different delivery ways are design for different reliability requirements, and it applies for both configuration and data. Though with Option 2, UE may acquire the configuration, especially the updated configuration without RRC state transition, we should notice that the broadcast way is not reliability enough, and there can be unsuccessful reception or decoding. And due to MCCH is delivered with periodicity, the latency to get the proper configuration is at least one MCCH repetition period. When the channel quality is not good enough, or a long repetition periodicity is configured, the latency can be quit long.

Observation 4: There can be long latency for the acquisition of updated configuration via MCCH like channel, due to the poor channel quality and MCCH’s periodic repetition.

In the description for Multicast Session Establishment in [2], When the join request of the first UE for the multicast session is accepted, the multicast session is established towards the NG-RAN node and the UE, see clause 7.2.1. Multicast session state transitions from "Start (NULL)" or Configured state to either Inactive or Active state. Based on this we can see, from the network point of view, UE’s join is needed, the first UE’s join triggers the RAN resource allocation, and this helps to resource saving.

Observation 5: UE’s join is needed, since the first UE’s join triggers the RAN resource allocation, which helps to resource saving.

If Option 2 is adopted, it seems MCCH like channel is always provided, UE’s join seems helpless for resource saving. And considering that MBS security is addressed in application layer, UE in RRC_INACTIVE may not perform join procedure, which cause inconsistence for UEs subscribing to the same service while receiving in different RRC states.

Observation 6: Option 2 may cause UE behaviour in consistence due to join procedure is not necessary for a UE receiving multicast in RRC_INACTIVE since MCCH is always provided and MBS service security is addressed in application layer.

Besides, one UE may subscribe to several multicast services, and not all services can be received in RRC_INACTIVE state, and even for the one can be received in RRC_INACTIVE, the UE may receive it with different RRC states in the time domain. Therefore, from the uniformity of handling to difference service and uniformity of handling the same service in different RRC states, common configuration is preferred.

Observation 7: From the uniformity of handling to difference service and uniformity of handling the same service in different RRC states, common configuration is preferred.

As to the mix of the two options, though it may solve the UE RRC states transition, most potential standardization work for Option 1 and Option 2 should be considered in this case. Therefore, it’s better to focus on one solution first. And based on the analysis above, we propose to support Option 1, and join procedure should be performed as agreed in Rel-17, no matter what RRC state UE is in for multicast reception.

Observation 8: The mixed solution may increase the standardization workload.

Proposal 1: Option 1 should be supported for PTM configuration delivery for multicast reception in RRC_INACTIVE.

Proposal 2: Join procedure should be performed by UEs, no matter what RRC state UE is in for multicast reception.

Details of Option 1

It was agreed in RAN2#119 meeting that two scenarios are supported for multicast reception for UEs in INACTIVE:
-	Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception.

-	Scenario 2: a UE has joined a multicast session and has been directed to INACTIVE, the UE starts to receive the multicast session

For both scenario 1 and 2, PTM configurations can be provided when the UE is released to RRC_INACTIVE as discussed in Section 2.1. What’s more, for scenario 1, PTM configuration for RRC_INACTIVE reception can also be provided with PTM configuration for RRC_CONNECTED together, and used when indicated by gNB when it is moved to RRC_INACTIVE.

Proposal 3:  PTM configuration can be provided by RRCRelease message or RRCReconfiguration message, when it is provided in RRCReconfiguraiton message, the time to use the configuration may be indicated by RRCRelease message.

Besides, to avoid UE states transition, enhancements to RRC Resume procedure can be considered, for example PTM configuration is provided in RRCResume message while UE stay in RRC_INACTIVE, when the configuration is updated and requested by UE.

Proposal 4: Enhancement for RRC Resume procedure can be considered to avoid UE states transition and for PTM configuration provision.

For Rel-17 broadcast, UE can prioritize some MBS frequency which it is interested in. Similarly, it can be applied to UEs receiving multicast in RRC_INACTIVE state. And to achieve this, multicast supporting or multicast delivery information should be indicated to UEs, which has RAN3 impact.

Proposal 5: Cell reselection prioritization can be considered for multicast reception in RRC_INACTIVE.

Proposal 6: gNB can indicate multicast supporting or multicast delivery information to UEs for cell reselection prioritization handling.

Whether to support area-specific configuration didn’t achieve consensus. From our point of view, it is useful to service continuity and UE power saving, but whether the area range can be the whole RNA should be further discussed, an may be consulted with RAN3.

Proposal 7: Area-specific configuration can be supported for multicast reception in RRC_INACTIVE.

Proposal 8: RAN2 can consult with RAN3 about the neighbour cell information exchange in Xn interface and the area range for the same PTM configuration.

Conclusions

In this contribution, we compare the two options for PTM configuration delivery for multicast reception in RRC_INACTIVE and provide our preference. Following are the observations and our proposals.

Observations:

Observation 1: To switch UE’s state from RRC_CONNECTED to RRC_INACTIVE, dedicated RRC signalling cannot be avoided, it’s spontaneous to provide the related configuration in it.

Observation 2: For PTM configuration updating case, there cannot be congestion since UEs are informed via group notification in time domain dispersedly.

Observation 3: For congestion relieving case, UE may stay in RRC_INACTIVE for multicast reception if the transmission quality requirements can be achieved.

Observation 4: There can be long latency for the acquisition of updated configuration via MCCH like channel, due to the poor channel quality and MCCH’s periodic repetition.

Observation 5: UE’s join is needed, since the first UE’s join triggers the RAN resource allocation, which helps to resource saving.

Observation 6: Option 2 may cause UE behaviour in consistence due to join procedure is not necessary for a UE receiving multicast in RRC_INACTIVE since MCCH is always provided and MBS service security is addressed in application layer.

Observation 7: From the uniformity of handling to difference service and uniformity of handling the same service in different RRC states, common configuration is preferred.

Observation 8: The mixed solution may increase the standardization workload.

Proposals:

Proposal 1: Option 1 should be supported for PTM configuration delivery for multicast reception in RRC_INACTIVE.

Proposal 2: Join procedure should be performed by UEs, no matter what RRC state UE is in for multicast reception.

Proposal 3:  PTM configuration can be provided by RRCRelease message or RRCReconfiguration message, when it is provided in RRCReconfiguraiton message, the time to use the configuration may be indicated by RRCRelease message.

Proposal 4: Enhancement for RRC Resume procedure can be considered to avoid UE states transition and for PTM configuration provision.

Proposal 5: Cell reselection prioritization can be considered for multicast reception in RRC_INACTIVE.

Proposal 6: gNB can indicate multicast supporting or multicast delivery information to UEs for cell reselection prioritization handling.

Proposal 7: Area-specific configuration can be supported for multicast reception in RRC_INACTIVE.

Proposal 8: RAN2 can consult with RAN3 about the neighbour cell information exchange in Xn interface and the area range for the same PTM configuration.

References

- Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online, 10 - 19 October

- TS 23.247 5G multicast-broadcast services; Stage 2 (Release 17)

- R2-2210882	Reply LS on FS_5MBS_Ph2 progress RAN2 LS out Rel-18 NR_MBS_enh-Core, FS_5MBS_Ph2 To:SA2, RAN3 Cc:RAN1