3GPP TSG-RAN WG2 Meeting #120  	               			    R2-2212823

Toulouse, France, 14 – 18 November, 2022

Agenda item:	8.3.5

Source:	LG Electronics Inc.

Title:	Connected mode mobility

Document for:	Discussion and Decision

# 1. Introduction

The following is the agreement regarding group handover in the RAN2 #119e-bis meeting.

=> Scenario 1: UEs are HO’ed due to switch of SOURCE cell to NES mode is considered for further study. FFS whether any enhancements is needed. => FFS Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected based on its mode of operation => As a first priority, discussion on RAN2 group handover are confined to the CHO framework

In this contribution, we discuss furhter on the Group HO/CHO.

# 2. Discussion

At the last meeting, several companies suggested the introduction of new execution conditions for group handover in CHO.  It can help to perform handover in a timely manner when the serving cell is powered off. However, prior to whether a separate group handover indication is required, RAN2 should first study group handover using legacy HO and legacy CHO.

Group handover with HO command

Cell-Off is desirable when only a few UEs are connected within a cell, so the gain from the group handover is small. Rather, it is simple to use the existing HO command instead of introducing a new method. Also, the dynamic switching mechanism (L1/L2 signaling) between candidate serving cells is discussed in the R18 eMOB WI. We can apply it for fast Pcell change without any significant modification when it completes. Otherwise, the discussion may be duplicated. Therefore, we propose not to introduce a new group handover signalling for group HO.

Observation 1. Cell-Off is desirable when only a few UEs are connected, while the group HO would be beneficial when numerous UEs are handed over simultaneously.

Observation 2. Dynamic switch mechanism among candidate serving cells is discussed in R18 eMob WI. If the discussion is completed in eMob WI, we can apply it for fast Pcell change without any significant modification.

Proposal 1. For group handover with HO command, we propose not to introduce new group handover signalling in NES

Group handover with CHO

Regarding CHO for group handover, without group handover indication as an execution condition, legacy CHO can be used to trigger group HO. It can also be supported by legacy UEs.

The following scenario can be considered:

Scenario 1. A cell gradually reduces the TX power and eventually turns it off

Scenario 2. A cell turns off after a certain time

In the case of scenario 1, power-down of the source cell can trigger conditional handover with A3/A5 event. Then the UE executes the conditional handover based on the signal strength.

In the case of scenario 2, if cell-off is expected within a certain time, the network can configure the start time of evaluation of the target cell and the duration for the execution of CHO taking into account the serving cell’s cell-off time. In this case, the network can configure the CHO with a T1 event. T1 event can also be configured with A3/A4/A5 event. Then the UE executes the conditional handover based on the signal strength and time information.

For scenario 1 and scenario 2, the UE can perform mobility with the legacy conditional handover procedure. Each UE will perform mobility with a slight time difference. However, this can prevent congestion that can be caused by connecting to a specific cell at once. Above all, given that the legacy UE is also connected to the NES cell, separately handling the mobility of the legacy UE and the NES-capable UE rather complicates the mobility operation. That is, group handover mechanisms should be applicable to both the legacy UE and the NES-capable UE.

Therefore, we should study legacy CHO to support group handover.

Observation 3. Given that the legacy UE is also connected to the NES cell, separately handling the mobility of the legacy UE and the NES-capable UE rather complicates the mobility operation. That is, group handover mechanisms should be applicable to both the legacy UE and the NES-capable UE.

Proposal 2. For group handover with CHO, we propose to study handover using legacy CHO considering the following scenarios:

- Scenario 1. A cell gradually reduces the TX power and eventually turns it off

- Scenario 2. A cell turns off after a certain time

CHO candidates may be updated depending on the on/off of the neighbor cells if the neighbor cells support NES. However, the dynamic cell-on/off can make a lot of signalling between the gNBs. For example, when a cell is powered off, it should notify the vicinity network in order for that cell to be deleted in cell (re)selection and handover configuration of the vicinity network. Considering the boot time when the cell is turned on, it cannot operate normally within a short time. That is, it is difficult to change the cell on-off quickly and frequently.

Therefore, for group handover, we should study the semi-static scenario first. In that case, CHO candidates can be updated by the legacy RRC reconfiguration procedure.

Proposal 3. For group handover, we propose to study the semi-static scenario first. (i.e., CHO candidates are updated by the legacy RRC reconfiguration procedure)

Candidate (de)prioritization of CHO

Mobility is performed based on the signal strength of the target cell. Generally, in good coverage, the network can allocate resources for high performance, and data interruption is reduced through reliable connections. However, in NES, it is difficult to determine how much performance the target cell can provide by referring only to the NES state. That is, it is difficult for the UE to directly determine the cell most suitable among several candidates. Therefore, it should be decided by the network based on the NES state of the target cell and the UE’s data traffic. In this case, the UE does not need to know the NES state of the neighbor cell.

Observation 4. In NES, it is difficult to determine how much performance the target cell can provide by referring only to the NES state. That is, it is difficult for the UE to directly determine the cell most suitable among several candidates

Proposal 4. NW decides candidates for CHO based on NES state of target cell and UE’s data traffic. (UE does not decide (de)prioritization of NES cell depending on the NES state of the target cell)

# 3. Conclusion

Our observations and proposals are provided:

Observation 1. Cell-Off is desirable when only a few UEs are connected, while the group HO would be beneficial when numerous UEs are handed over simultaneously.

Observation 2. Dynamic switch mechanism among candidate serving cells is discussed in R18 eMob WI. If the discussion is completed in eMob WI, we can apply it for fast Pcell change without any significant modification.

Proposal 1. For group handover with HO command, we propose not to introduce new group handover signalling in NES

Observation 3. Given that the legacy UE is also connected to the NES cell, separately handling the mobility of the legacy UE and the NES-capable UE rather complicates the mobility operation. That is, group handover mechanisms should be applicable to both the legacy UE and the NES-capable UE.

Proposal 2. For group handover with CHO, we propose to study handover using legacy CHO considering the following scenarios:

- Scenario 1. A cell gradually reduces the TX power and eventually turns it off

- Scenario 2. A cell turns off after a certain time

Proposal 3. For group handover, we propose to study the semi-static scenario first. (i.e., CHO candidates are updated by the legacy RRC reconfiguration procedure)

Observation 4. In NES, it is difficult to determine how much performance the target cell can provide by referring only to the NES state. That is, it is difficult for the UE to directly determine the cell most suitable among several candidates

Proposal 4. NW decides candidates for CHO based on NES state of target cell and UE’s data traffic. (UE does not decide (de)prioritization of NES cell depending on the NES state of the target cell)

# 4. References

[1] 36331-h10.docx

[1] 38331-h20.docx