3GPP TSG RAN WG2 #120                                                                                R2-2212847

Toulouse, November 14th – November 18th, 2022

Agenda item:	8.15.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Discussion on RAN2 Aspects in SL-U

Document for:	Discussion

# Introduction

A work item for NR sidelink evolution in Rel-18 was approved in RAN#94-e and revised in RAN#97-e [1].

2.	Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

-	Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

- Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

  - No specific enhancements for Rel-17 resource allocation mechanisms

  - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

-	Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

- The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

-	No specific enhancements for existing NR SL feature

-	The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

-	Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

For support of sidelink in unlicensed spectrum, the following objective has been defined:

Agreement on CAPC:

1: 	Working assumption: PQI is used to determine the CAPC mapping as in NR-U. FFS whether the same principle is also applied to the UE side.

2:	For SL-DRB the CAPC value is (pre)configurable per-DRB as in NR-U.

3:	For all SL-SRBs, CAPC value is fixed to the highest priority (i.e., lowest CAPC value).

4:	If PQI-based CAPC mapping is agreed, for all SL MAC CEs, CAPC value is fixed to the highest priority (i.e., lowest CAPC value).

5:	If PQI-based CAPC mapping is agreed, at least PDB can be used as the criterion to determine the CAPC mapping. FFS if any other additional criterions needed.

6:	As in NR-U, if SL CAPC is determined based on PQI, as a baseline, for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. FFS if any specific work needed for RRC_INACTIVE/RRC_IDLE/OOC UEs.

7:	If PQI-based CAPC mapping is agreed, as in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used; FFS whether this rule can be extended to the case when SL MAC CE(s) multiplexed with STCH.

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used;

- FFS how to select SL CAPC when SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

The following agreements were made during RAN2#119-bis-e regarding CAPC [2]:

Agreement on consistent LBT failure:

1: 	SL-specific LBT failure indication from PHY is needed for SL-specific consistent LBT failure detection in the MAC. How/whether it is used for other purposes can be further discussed.

2:	Support SL-specific consistent LBT failure detection and recovery procedure in the MAC for SL-U. Details of recovery to be further worked on granularity of (consistent) LBT failure.

3:	Send LS to RAN1 asking “When an SL-specific LBT failure indication is notified for an SL transmission by the PHY, in which resource granularity the SL-specific LBT failure can be considered as being detected (e.g. per Resource Pool, per RB set, per SL BWP, etc.)?

- Detailed wording can be discussed during the email discussion. Some background information (e.g. why/what we (actually) ask) can be also provided.

4:	As the general principle, reuse the consistent LBT failure detection procedure in NR-U as the baseline for SL-specific consistent LBT failure detection in SL-U.

5:	As in NR-U, introduce the following parameters and variables for the SL-specific consistent LBT failure detection in SL-U as the baseline:

- An SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER);

- An SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount);

- An SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer).

6:	Reuse the following MAC behaviors on TIMER/COUNTER handling in NR-U for SL-specific consistent LBT failure detection procedure in SL-U as the baseline:

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is incremented by one.

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, start or restart the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer)

- As in NR-U, if the SL-specific LBT failure indication counter value is equal to or larger than the SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount), consistent LBT failure is triggered/declared by the MAC entity.

- As in NR-U, if the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) expires, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

- As in NR-U, if the maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount) or SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) is reconfigured, SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

7:	Support the mechanism that a mode-1 UE can indicate the SL-specific consistent LBT failure to the gNB. FFS on a mode-2 UE in RRC_CONNECTED.

And the following agreements were made for consistent LBT failure [2]:

This contribution discusses further RAN2 aspects of SL-U design in the Rel-18 NR sidelink evolution.

# Sidelink in Unlicensed

In order to operate on the unlicensed bands, the Sidelink has to abide to regulations and co-exist with other systems which already operate on unlicensed bands. The Channel Access Priority Class (CAPC) defined in [2], which is also used in NR-U, should be used in SL-U too in order to comply with some regional regulations. Nonetheless, such classes need to be understood in the context they are used in practice in order to select the best mapping from sidelink priority or PQI to CACPs. By design, the lowest priority class on unlicensed bands, i.e. CAPC 4, may suffer from starvation, if higher priority classes are continuously transmitting. Therefore CAPC 4 is only suitable to traffic which has a very high tolerance to delay.

CAPC 4 may suffer from starvation if higher priority classes are in use. CAPC 4 is only suitable to traffic which has a very high tolerance to delay.

Furthermore, the CAPC mechanism does not work well for a high density of devices. This is even intensified on higher priority classes, as the shorter backoff may lead to increased number of collisions. CAPC 3 can adapt to large contention windows which may be necessary when the density of devices is large. CAPC 1 or 2 should be considered for traffic with demanding QoS.

CAPC 3 can adapt to large contention windows which may be necessary when the density of devices is large.

CAPC 1 or 2 should be considered for traffic with demanding QoS.

In unlicensed bands the general assumption is that multiple devices compete for resource allocation, but once a device has accessed the channel it will typically be used at full band allocation and for the shortest time possible. In contrast, the resource allocation on mode 2 can randomize resource allocation in frequency and time. Such allocation may be suboptimal or not even supported by unlicensed band regulations (minimum occupied bandwidth). Therefore, RAN 2 shall investigate how to adapt sidelink mode 2 resource allocation to unlicensed band.

RAN 2 shall investigate how to adapt sidelink mode 2 resource allocation to unlicensed band.

# Conclusion

We have made the following observations and proposals in this contribution:

Observation 1:	CAPC 4 may suffer from starvation if higher priority classes are in use. CAPC 4 is only suitable to traffic which has a very high tolerance to delay.

Observation 2:	CAPC 3 can adapt to large contention windows which may be necessary when the density of devices is large.

Observation 3:	CAPC 1 or 2 should be considered for traffic with demanding QoS.

Proposal 1:	RAN 2 shall investigate how to adapt sidelink mode 2 resource allocation to unlicensed band.

# References

- RP-221938, “WID revision: NR sidelink evolution”, OPPO, 3GPP RAN#97-e, September 2022.

- “draft Report of 3GPP TSG RAN WG2 meeting #119bis-e”, October 2022.

- TS 37.213 v17.3.0, “Physical layer procedures for shared spectrum channel access”, September 2022.