3GPP TSG-RAN WG2 Meeting #120	R2-2212899

Toulouse, France, November 14th – 18th, 2022

Agenda item:		8.5.2.2

Source:		Futurewei

Title:	On potential impacts to LCP mechanisms for XR

Document for:		Discussion and Decision

# Introduction

A UE allocates UL granted resource to carry data selected from competing logical channels by using an LCP procedure, which is based on a mix of prioritization and token bucket scheduler [1].

In this contribution, we discuss some issues and potential impacts to LCP mechanism due to a need for handling XR traffic.

# Discussions

## XR traffic characteristics

In XR applications, the UL data traffic may include one or more of an audio stream, a pose/control stream, and a video stream, each traffic stream having different traffic periodicities and different delay requirements. For example, for the audio stream, an audio packet may be generated every 10 msec with a packet delay budget (PDB) of 30 msec, for a pose/control stream, a pose packet may be generated every 2.5 msec with a PDB of 10 msec, and for the video stream, a data burst consisting of one or more packets corresponding to a video frame may be generated every 16.666 msec at a frame refresh rate of 60 fps with a PDB or PDU Set delay budget (PSDB) of 30 msec [2].

For these XR traffic streams, the PDB/PSDB is longer than the corresponding traffic periodicity, which means at a given time, data buffered for a logical channel associated with the XR traffic stream may include multiple packets (e.g., for a pose/control stream) or multiple data bursts (e.g. for a video stream), where each packet or each data burst are generated during a different traffic cycle. Hence, at the given time, different packets or different data bursts buffered for the same logical channel have different amount of time remaining until their respective deadlines for delivery. Therefore, packets that arrived at the logical channel during an earlier traffic cycle (and hence their delivery deadline may be fast approaching) may have an urgent need to be transmitted no matter how bad is the current channel condition or interference condition on the UL, while some other packets that just arrived at the same logical channel may have a longer remaining time until their delivery deadline (and hence the gNB has more time to wait for a more favorable channel condition or interference condition to schedule the transmission of them).

## Limitation of existing LCP mechanism

The existing LCP procedure is performed in such a way that competing logical channels are served sequentially in accordance with the priority levels of the competing logical channels, and for each logical channel j served, up to a number of bits represented by the variable Bj. Considering Bj as the tokens in a token bucket, Bj is replenished at a rate referred to as the Prioritized Bit Rate (PBR) until the Bj has reached a limit referred to as the bucket size. The PBR and a bucket size duration (BSD) are configured per logical channel. The product of PBR and BSD produces the bucket size for that logical channel.

The existing LCP mechanism doesn’t take the delay information into account. For example, after serving the remaining bits of a data burst arrived at a logical channel during an earlier traffic cycle, if there are still tokens left in the bucket (i.e., Bj >0), the MAC entity will start to allocate resource for the next data burst buffered for the same logical channel, despite of the fact that the remaining time of the next data burst buffered for the same logical channel is one traffic period longer than the remaining time of the previous one (which is being finished). Consider a case where there is another packet at the moment being buffered for another logical channel with a lower priority but the remaining time of that packet is much shorter, that packet may be starved out (i.e., missing its delivery deadline), because, based on the existing LCP mechanism, the MAC entity is going to allocate the remaining resource to a less urgent data of a higher priority logical channel even if it is going to starve out a more urgent data of a lower priority logical channel.

## Prioritization with delay information being taken into account

A possible way for enhancing LCP for XR is to prioritize among packets and data bursts of different logical channels with delay information being taken into account. An example of the delay information is the remaining time until a deadline for delivering a packet or a data burst. For example, a packet or a data burst of a lower priority logical channel but with a shorter remaining time may be prioritized over another packet or another data burst of a higher priority logical channel but with a longer remaining time. A MAC entity may choose to do so if and only if it determines that the timely delivery of the less urgent packet or data burst of the higher priority logical channel will not be jeopardized.

Proposal 1. RAN2 consider an LCP enhancement where delay information is taken into account for data prioritization.

Due to small jitters on the UL, it is assumed that all packets belonging to a same data burst from a same UL video stream arrives at the UE at a same time. And because they are all needed at the application layer of the recipient, e.g., for video decoding and decompression, it is also assumed that they have a same deadline for delivery. Hence, they share a same PSDB and a same remaining time. Therefore, if delay information is to be taken into account for prioritization among data of different logical channel, the prioritization should be done on a per PDU set basis, where a PDU Set can include one or more packets.

Proposal 2. If delay information is to be taken into account for prioritization among data of different logical channel, the prioritization should be done on a per PDU set basis, where a PDU Set can include one or more packets.

# Conclusions

The followings are proposed:

Proposal 1. RAN2 consider an LCP enhancement where delay information is taken into account for data prioritization

Proposal 2. If delay information is to be taken into account for prioritization among data of different logical channel, the prioritization should be done on a per PDU set basis, where a PDU Set can include one or more packets.

# References

3GPP TS 38.321 v17.2.0.

3GPP TR 38.838 v17.0.0.