3GPP TSG-RAN WG2 #120	Tdoc R2-2212900

Toulouse, France, 14th – 18th November 2022

Agenda Item:	8.8.2

Source:	Ericsson

Title:	Flight path reporting and UAV measurement reports

Document for:	Discussion, Decision

# Introduction

As described in [1], the WID for supporting UAV in NR includes an objective on enhancing measurement reports, as follows:

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

In RAN2#119e, the following agreements are made related to enhancement for measurement reports in NR:

Agreements - Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold. FFS if further NR enhancements are needed. FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how) FFS how to limit excessive measurements and measurement reporting FFS if user consent is needed for location reporting in CONNECTED FFS study the vertical movement and associated mobility for UAV UEs - Rel-18 NR supports reporting of UAV UE’s height, location and velocity. It is for further study what accuracy and reporting mechanisms are required and if further enhancements are needed. - As in LTE, flight path plan reporting will be introduced. Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report. FFS if timestamp is mandatory or optional for NR. FFS if further enhancements are needed - Introduce similar functionality to LTE (numberofTriggeringCells). FFS whether numberoftriggerbeams for NR is required or other enhancements. FFS study how to avoid sending the measurement reports mainly due to reportOnLeave.

In RAN2#119bis-e, the following agreements were made:

Agreements: - The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). - Allow the flight path to be updated. FFS on the details. - FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting - Continue to study height-depending scaling, triggering and combinations - As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells).

In this paper we discuss these agreements and related FFS.

# Flight path reporting

Rel-15 for UAV supports flight part reporting from the UE upon network polling. The UE will report its flight path, typically received from the UTM system, to the eNB when requested by the eNB. The reason for network-polled flight path reporting is simply interference control. Flight path report may be relatively large, hence if reported to the network which is not intending any use with is purely a waste of resources.

Reporting a flight path information to a network which does not have intention to use it is pure waste of resources and it causes unnecessary UL interference.

LTE Rel-15 supports network-polling flight path reporting from the AUE.

In our view, it is also important to specify a network-controlled mechanism for the reporting to avoid UEs autonomously sending to network large flight path reports. In NR, other types of triggers for flight-path reporting can be discussed.

- Network-controlled flight path reporting is the baseline.

In RAN2#109bis, it was agreed that the flight path information can be updated but details are FFS. In our view, one solution would be allowing the UE to indicate to the network that UE’s flight path has been updated. Another option would be allowing the UE to send a flight path report directly if there is a significant update of the flight path or if the UE detects a significant change in a certain indicator related to or impacted by the flight path, otherwise the UE only indicates that the flight path has been updated or changed. Therefore, in our view, RAN2 should discuss the triggering conditions for flight path reporting in NR.

- To handle a flight path update in the UE, RAN2 to decide whether the UE is allowed to send a new flight path report directly and under what condition.

As for the contents of the flight path report, in LTE Rel-15, the report from the UE includes a list of 3-D waypoints and timestamps. RAN2#119bis-e decided that UE includes time information in the flight path report, if configured by the network and available at the UE, but flight path details are FFS (waypoints and what is time information). In our view, besides the baseline contents similar to Rel-15, other alternatives or additions can be specified to improve the usability of the time and location information included in the flight path report. For example, instead of reporting a single timestamp, the UE can report a time window which indicates a period that the UAV is expected to be in or near a certain location, e.g., for a mission such as inspecting a construction site or part of a power grid. The time window can be expressed using a validity timer to allow for flexibility of signalling. Similarly, instead of using 3-D waypoints, one can consider the notion of area or volume or 3-D zone.

Time stamp as UTC time does not well address the key point of GSMA of MNOs to plan the coverage and service reliability to ensure BVLOS flights.

- To increase the informativeness of flight path reporting, include hysteresis, or time window to the time stamp of a UTC waypoint.

Waypoint is single spot coordinate does not well address the key point of GSMA of MNOs to plan the coverage and service reliability to ensure BVLOS flights.

- To increase the informativeness of a reported waypoint, add radius or similar to express the location..

Additionally, in our view, some other types of information related to the flight path and operating condition of the UAV can also be reported, either in the flight path report or in a separate UE report. For example, we believe that information about default/emergency landing spots as well as information that the UE can deduced from the wind condition or battery status is valuable for the network to configure adequate resources in case an emergency landing is required. Therefore, we propose RAN2 to discuss such enhancements.

- Consider reporting information related to emergency/default landing spots and information related to UAV operating conditions.

# Discussion on AUE measurement reports

## Measurement report triggered by height thresholds

In RAN2#119, the following agreements were reached for the height event:

Agreements - Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold. FFS if further NR enhancements are needed. FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how) FFS how to limit excessive measurements and measurement reporting FFS if user consent is needed for location reporting in CONNECTED FFS study the vertical movement and associated mobility for UAV UEs

As confirmed in RAN2#119, the event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold) in LTE will be the baseline for defining similar events in NR. When it comes to the details, several issues need to be discussed by RAN2.

There is FFS on if further enhancements are needed. In our view, RAN2 should discuss the content of the height report. Regarding the contents of the measurement reports, in Rel-15, information about the height, location, and speed of the UE is included. In addition to the altitude of the UE in the field heighUE, the field horizontalWithVerticalVelocityWithUncertainty, was added to the LocationInfo-r10 information element in the RRC message.

In LTE Rel-15, UE measurement report for H1 and H2 includes information about altitude of the UE, location as well as velocity of the UE (vertical, horizontal)

In NR, the LocationInfo is as follows:

#### –	CommonLocationInfo

The IE CommonLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

CommonLocationInfo information element

-- ASN1START

-- TAG-COMMONLOCATIONINFO-START

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

-- TAG-COMMONLOCATIONINFO-STOP

-- ASN1STOP

CommonLocationInfo field descriptions

gnss-TOD-msec Parameter type gnss-TOD-msec defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationTimeStamp Parameter type DisplacementTimeStamp defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationCoordinate Parameter type LocationCoordinates defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationError Parameter LocationError defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationSource Parameter LocationSource defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

velocityEstimate Parameter type Velocity defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

In addition, in Release-17, a coarse location reporting was specified which has granularity of 2km. It is presumed there is not really a use case for coarse location reporting for UAV. RAN2 should discuss if the CommonLocationInfo provides the needed fields for location reporting in H1 and H2 events, or in drone operation in general.

- RAN2 clarify if the NR CommonLocationInfo provides the needed fields for location reporting in H1 and H2 events.

Eventually, RAN2 needs to decide whether the range and granularity used in the configurations related to the two events in LTE Rel-15 can be reused. While exact granularity can be discussed later, it would be good to confirm if 300m is still the height limit for NR as well.

- Reuse the altitude range defined in LTE Rel-15 for events H1 and H2. FFS the granularity.

## Reporting of height, location and speed in “measurement report”

Additionally, RAN2 needs to clarify the meaning of this WID objective:

- Reporting of height, location and speed in measurement report

Is the intention to discuss the report content of specifically the H1/H2 event for which we have Proposal 1 in this paper, or is the intention was to review the content of RSRP/RSRQ measurement reports for drone operation.

- RAN2 to clarify whether other measurement report content (other than the reports triggered by H1 and H2) need to be updated.

Another aspect worth discussing is the content of UE measurement reports. In NR, it is possible to require a UE to report location in RRM measurement report but in general it is a large report because the UE reports everything that is available. Given that an aerial UE (AUE) may sends report frequently, in our view, it is desirable that a leaner report is supported, e.g., only some parts of the LocationInfo are reported and the network can control what to be reported.

- Support UE measurement report containing selected parts of the location information.FFS further details

## Measurement reporting based on a configured number of cells (i.e., larger than one) fulfilling the triggering criteria simultaneously

In RAN2#119e, RAN2 agreed to introduce similar functionality to LTE (numberofTriggeringCells). There was an FFS on whether numberoftriggerbeams for NR is required. However, in our view, it is too early to discuss such a detail. As widely known, one key difference between NR and LTE is that in NR, the cell quality derivation is based on beams, which could be based on one or multiple beams. Specifically, in NR, a UE in RRC_CONNECTED mode can be configured by the network to perform various measurements and report the measurements to the network. The measurements can be performed on either SS/PSBCH blocks (SSB) or CSI Reference Signal (CSI-RS) resources. In both cases, the measurements can be “beam level” or “cell level”. TS 38.331 specified rules for generating a cell level measurement from beam level measurements. Hence, in our view, RAN2 should discuss whether to consider only cell level measurement or bother cell and beam level measurement criteria as a first step.

- RAN2 to discuss whether to consider only cell-level measurement criterion or both cell-level and beam-level measurement criteria in case of triggering measurement reporting based on a configured number of cells.

In case the reporting based on a configured number of cells fulfilling the triggering criteria simultaneously is to be supported in NR, RAN2 needs to discuss how cell quality is evaluated, e.g., whether using one beam or multiple beams, whether the method is fixed or configurable, etc.

- Use the existing mechanism of deriving ceIl quality from beam measurements as a baseline. FFS enhancements.

When it comes to how many beams to be considered in deriving measurement results, it is beneficial to consider some optimizations, e.g., to limit the number of measurements that the AUE needs to perform. For example, the number of beams to be measured can be dependent on AUE height / speed / location.

- RAN2 to consider optimizations on beam measurements for AUEs, e.g., the number of beams to be used for deriving cell-level measurement is height-dependent or speed-dependent.

Another direction of optimization is switching between different measurement reporting configurations, namely cell-level or beam-level or both beam- and cell-level triggers, based on AUE height / speed / location.

- In case different triggers of measurement report are supported (i.e., beam-level trigger, cell-level trigger, combined beam and cell trigger), RAN2 study conditions to apply a certain trigger (e.g., based on AUE height).

In RAN2#119e, another FFS is about how to avoid sending the measurement reports mainly due to reportOnLeave. In this regard, in our view, RAN2 should first agree on how the list of triggering cells are maintained in NR. For LTE, the mechanism works as follows (TS 36.300 V17.2.0– Clause 23.17.4):

- 23.17.4 Interference detection and mitigation for Aerial UE communication For interference detection, an aerial UE can be configured with RRM event A3, A4 or A5 that triggers measurement report when individual (per cell) RSRP values for a configured number of cells fulfill the configured event. Once such condition is met and a measurement report is sent, the list of triggered cells is updated when subsequent cell(s) fulfil the event, however further measurement reports are not sent while the list of triggered cells remains larger than the configured number of cells. The report contains RRM results and location if configured, as described in 23.17.6. For interference mitigation an aerial UE can be configured with a dedicated UE-specific alpha parameter for PUSCH power control.

As can be seen in the text, in LTE, the list of triggered cells is updated whenever a new cell fulfils the event (although a measurement report may not be sent). For NR, we propose to RAN2 to first discuss whether the same way of updating the list of triggered cells as in LTE is to be reused for NR.

- RAN2 to first decide how the list of triggered cells is updated in NR before addressing issues due to reportOnLeave.

As specified in NR, reportOnLeave is a network configurable parameter, and we propose that RAN2 clarify the scope and purpose of this FFS before going into detailed discussions.

reportOnLeave is a network-configurable parameter.

- RAN2 to clarify the scope and purpose of avoiding sending the measurement reports mainly due to reportOnLeave.

## Controlling the amount of measurement reporting

While a triggering condition based on multiple cells (discussed in Section 2.4) reflects interference conditions while airborne, it is not a suitable measurement event configuration for mobility performance where it is crucial to report as quickly as possible since the serving cell quality may drop fast. In fact, the mobility evaluations in Rel-15 used the shortest TTT of 40ms with RSRP triggering based on single cell as the triggering based on multiple cells would be delayed.

For ensuring mobility performance, an aerial UE should be configured with single cell RSRP reporting with short TTT.

Due to the high mobility nature of airborne UEs, the number of measurements for mobility as discussed above can be large. When the number of airborne UEs in the network increases, the total volume of such measurements can negatively impact the resource utilization and network performance. This issue was discussed in Rel-15 and also during the preparation of the Rel-18 WI [7].

RSRP measurement reporting for mobility can create high load in the network. Therefore, it is beneficial to control the amount of reporting.

It is therefore important to limit the volume of UE measurement reports to a reasonable level. Here we review the existing mechanisms to control the measurement reporting. The TTT and hysteresis are used to control reporting if the value for the metric goes back and forth of the threshold value potentially causing entering/leaving of the condition. This is efficient, e.g., for terrestrial UEs which move around on cell edge to prevent ping pong effect for HO.

TTT and hysteresis control reporting amount when the value for the metric goes back and forth of the threshold value potentially causing entering/leaving of the condition.

For periodical reporting there are parameters reportInterval and reportAmount. The ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e., when reportAmount exceeds 1), for triggerType event as well as for triggerType periodical. These parameters control reporting after event has triggered due to a cell. These parameters do not affect reporting when the event is triggered due to different cells.

Parameters reportInterval and reportAmount do not affect reporting when the event is triggered due to different cells.

When an aerial UE flies or takes off, an event triggers consecutively due to different cells as observed in [4]. For example, cell A, B, C and D may exceed threshold one after another and start their respective TTT follow up. Once TTT expires for A UE sends report, then it expires for B and then for C. The report sent based on B and C are very similar and difference start to show maybe around report D. Thus, it is seen beneficial to be able to skip report based on B and possibly also C.

When an aerial UE flies or takes off, an event triggers consecutively due to different cells. This may lead to redundant UE measurement reports.

Thus, we propose that RAN2 specify mechanism to control the amount of measurement reporting.

- RAN2 to specify mechanism to control the amount of measurement reporting.

The proposed way is to limit the measurement reporting by defining a prohibit timer which prevents sending the measurement reports for a configurable time after the event has been triggered and the first report has been sent. The use of prohibit timer is associated to the reporting configuration. This method can control the amount of measurement reporting, also at the same time it enables timely reporting when the event is triggered first time. As the idea of preventing certain measurement results can result in a worry on decreasing mobility performance, we present simulation results to show that this seem not to be the case for drone UEs. The results were performed under LTE UAV simulation assumptions (TR 36.777) and was first shown in [8], but the observations and conclusions should hold true for NR as well.

### Simulation results

The main issue of the excessive measurement reporting lies in measurement reports that are being triggered close in time, as seen in figure 1a where up to 3 measurement reports are being sent within less than 500ms. In figure 1b, we introduce a prohibit timer to reduce the measurement reports triggered close in time. In this case the amount of measurement reports is reduced while not impacting performance.

Figure 1. A RSRP-trace of a drone at 50m altitude flying with 44m/s. The yellow lines denote an A3 measurement triggering without a prohibit timer and red line with a prohibit timer.

In order to compare the prohibit timer to the baseline, we introduce two measures, double-triggering and triple-triggering 3. The A3 measurement event with a threshold of 0.5dB is used. Double-triggering is defined as two measurement reports that are triggered within x ms and triple-triggering is defined as three measurement reports being triggered within y ms.

As an example, of the flexibility, a prohibit timer of 80ms and 160ms is introduced. In figure 2a, the amount of A3 single-triggering, double-triggering and triple-triggering can be seen for different heights. It is clear that the double-triggering and triple-triggering are highly reduced and in the case of a 160ms prohibit timer the triple-triggering are completely removed.

Figure 2. a) The amount of single triggering, denoted A3_1, b) amount of double-triggering denoted A3_2 and c) triple-triggering denoted by A3_3

In order to make sure that the introduced prohibit timer does not affect performance, we also measure the amount of RLFs declared, handovers initiated and handover failures which can be seen in figure 3a, 3b and 3c.

Figure 3. a) The amount of RadioLinkFailures declared, b) the amount of handover initiated and handover failures.

From figure 3 we can see that the RLFs, handover initiated and handover failures remain almost constant. The reason for this can be seen in figure 1. In cases where many measurement reports are sent consecutively in time, the interference is very large and it is unclear whether the condition reported in the measurements will remain, thus delaying some of them will not hurt the performance, because if the condition is still present, then the report will eventually surface.

We propose to introduce a prohibit timer in NR similar to the CRs RAN2 discussed for LTE in [9][10][11]

- Introduce a prohibit timer in reporting configuration: UE is prevented from trigger the measurement report for a configurable time after the event is first triggered and first report is sent.

# RRM parameter scaling

It is well-known that compared with terrestrial UEs, aerial UEs are likely to receive higher downlink interference and create higher uplink interference. Hence it is beneficial to trigger measurement results for aerial UEs earlier than terrestrial UEs. Hence there is “FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how)”.

In LTE, the UE can adjust the value of the time-to-trigger (TTT) configured by the E-UTRAN depending on the UE speed [2]. Based on detected UE mobility state, UE can scale TTT according to preconfigured parameter. For NR UAVs, similar scaling could be applied to TTT based on UEs height.

- RAN2 to decide if scaling of TTT based on UAV height should be supported in NR.

# User consent on location reporting

Another FFS from RAN2#119e is whether user consent is needed for location reporting in CONNECTED. In our view, as this topic has been discussed in NTN WI and it is a general issue which involves other WGs like SA2 and SA3, RAN2 can wait and consider the decisions from these WGs.

- RAN2 to wait for the decision from SA2 and SA3 on the topic of user consent based location report.

# Other types of location reporting

Besides flight path reporting, in many cases it is beneficial for the network to know that an AUE is entering or leaving a certain area. For example, when a drone is to perform a mission in a certain geographical area in a certain time window according to a plan, it can be useful for the network to know that the drone is actually entering or leaving the area so that the network can optimize the connectivity for the mission of the drones. In another example, in many scenarios, the so-called no-transmit zones (NTZ) are imposed to limit the transmissions from drones, e.g., in order not to cause too much interference to a nearby base station. It is therefore very important for the network to know when a connected drone is entering such a NTZ, for the network to be prepared to maintain the connectivity with the drone (e.g., to switch to another frequency band or carrier that is allowed in the NTZ). Vice versa, knowing when the drone is leaving a NTZ enables the network to recover to a previous mode of communication with the drones.

It is beneficial for the mobile network to know when an AUE is entering or leaving a certain area or zone, e.g., a no-transmit zone.

- Consider UE reports triggered when an AUE enters or leaves a special area or zone, e.g., a no-transmit zone.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Reporting a flight path information to a network which does not have intention to use it is pure waste of resources and it causes unnecessary UL interference.

Observation 2	LTE Rel-15 supports network-polling flight path reporting from the AUE.

Observation 3	Time stamp as UTC time does not well address the key point of GSMA of MNOs to plan the coverage and service reliability to ensure BVLOS flights.

Observation 4	Waypoint is single spot coordinate does not well address the key point of GSMA of MNOs to plan the coverage and service reliability to ensure BVLOS flights.

Observation 5	In LTE Rel-15, UE measurement report for H1 and H2 includes information about altitude of the UE, location as well as velocity of the UE (vertical, horizontal)

Observation 6	reportOnLeave is a network-configurable parameter.

Observation 7	For ensuring mobility performance, an aerial UE should be configured with single cell RSRP reporting with short TTT.

Observation 8	RSRP measurement reporting for mobility can create high load in the network. Therefore, it is beneficial to control the amount of reporting.

Observation 9	TTT and hysteresis control reporting amount when the value for the metric goes back and forth of the threshold value potentially causing entering/leaving of the condition.

Observation 10	Parameters reportInterval and reportAmount do not affect reporting when the event is triggered due to different cells.

Observation 11	When an aerial UE flies or takes off, an event triggers consecutively due to different cells. This may lead to redundant UE measurement reports.

Observation 12	It is beneficial for the mobile network to know when an AUE is entering or leaving a certain area or zone, e.g., a no-transmit zone.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Network-controlled flight path reporting is the baseline.

Proposal 2	To handle a flight path update in the UE, RAN2 to decide whether the UE is allowed to send a new flight path report directly and under what condition.

Proposal 3	To increase the informativeness of flight path reporting, include hysteresis, or time window to the time stamp of a UTC waypoint.

Proposal 4	To increase the informativeness of a reported waypoint, add radius or similar to express the location..

Proposal 5	Consider reporting information related to emergency/default landing spots and information related to UAV operating conditions.

Proposal 6	RAN2 clarify if the NR CommonLocationInfo provides the needed fields for location reporting in H1 and H2 events.

Proposal 7	Reuse the altitude range defined in LTE Rel-15 for events H1 and H2. FFS the granularity.

Proposal 8	RAN2 to clarify whether other measurement report content (other than the reports triggered by H1 and H2) need to be updated.

Proposal 9	Support UE measurement report containing selected parts of the location information.FFS further details

Proposal 10	RAN2 to discuss whether to consider only cell-level measurement criterion or both cell-level and beam-level measurement criteria in case of triggering measurement reporting based on a configured number of cells.

Proposal 11	Use the existing mechanism of deriving ceIl quality from beam measurements as a baseline. FFS enhancements.

Proposal 12	RAN2 to consider optimizations on beam measurements for AUEs, e.g., the number of beams to be used for deriving cell-level measurement is height-dependent or speed-dependent.

Proposal 13	In case different triggers of measurement report are supported (i.e., beam-level trigger, cell-level trigger, combined beam and cell trigger), RAN2 study conditions to apply a certain trigger (e.g., based on AUE height).

Proposal 14	RAN2 to first decide how the list of triggered cells is updated in NR before addressing issues due to reportOnLeave.

Proposal 15	RAN2 to clarify the scope and purpose of avoiding sending the measurement reports mainly due to reportOnLeave.

Proposal 16	RAN2 to specify mechanism to control the amount of measurement reporting.

Proposal 17	Introduce a prohibit timer in reporting configuration: UE is prevented from trigger the measurement report for a configurable time after the event is first triggered and first report is sent.

Proposal 18	RAN2 to decide if scaling of TTT based on UAV height should be supported in NR.

Proposal 19	RAN2 to wait for the decision from SA2 and SA3 on the topic of user consent based location report.

Proposal 20	Consider UE reports triggered when an AUE enters or leaves a special area or zone, e.g., a no-transmit zone.

# References

RP-123600, New WID on NR support for UAV (Uncrewed Aerial Vehicles), RAN#94, Dec 2021

TR 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification

TS 38.331, NR; Radio Resource Control (RRC) protocol specification

TR 36.777, Enhanced LTE support for aerial vehicles

R2-1807257, Controlling the amount of measurement reporting, Ericsson, RAN2#102, Korea, May 2018

TR 23.754, Study on supporting Unmanned Aerial Systems (UAS) connectivity, Identification and tracking

RWS-210189, New WID on Enhanced NR Support for Aerial Vehicles, Ericsson

R2-1807257, Controlling the amount of measurement reporting, Ericsson

R2-1814762, Remaining aspects of measurement reporting, Ericsson, NTT Docomo, Nokia, Nokia Shanghai Bell

R2-1814760, Prohibit timer for measurement reporting, Ericsson, NTT Docomo, Nokia, Nokia Shanghai Bell, RAN2#103bis, Chengdu, China, 8th - 12th October 2018

R2-1814754, Introduction of capability for timer t3xx for Rel-15 Aerial W, Ericsson, RAN2#103bis, Chengdu, China, 8th - 12th October 2018