3GPP TSG-RAN2 Meeting #120	R2-2213150

# Toulouse, France, 14th – 18th October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3573 | rev | 1 | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to on-demand SI request for posSIB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Issue 1: msg1-based SI request can be used by UE to request SI messages configured by either SI-SchedulingInfo IE or posSI-SchedulingInfo IE in SIB1 in Rel-16. The IE SI-RequestConfig can be used for configuration of dedicated RACH resource for SI message for positioning. While in the field description, the mapping between the RACH resource and the pos SI message is not explained. - Note that the R17 version of the change has been submitted to RAN2#119bis considering the introduction of SchedulingInfo2 and agreed in principle in R2-2210997. - Changes made during R2#120 - Issue 2: The current spec says that when there is only one entry for the SI resources for msg1 based SI request, it is possible to request all the SI messages whose si-broadcastStatus is set to notBroadcasting. However, the case for posSI-broadcastStatus when the resource is used for the request of pos SI message is not considered |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change1, clarify in the field description for si-RequestResources the mapping between RACH resource and the SI message configured for positionining Change2, Add the case when the on-demand SI request resource is configured for posSI when the posSI-BroadcastStatus is set to notBroadcasting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Without the change, - For Change1, the mapping between the RACH resource configured for msg1-based SI request for positoning SI message and the SI message is not clear. - For Change2: it is not clear when there is only one entry in the list of resources, whether the UE can use the resource to request all the posSI messages. Impact analysis Impacted 5G architecture options: SA, NE-DC, NR-DC Impacted functionality: On-demand SI request Inter-operability: If the UE is implemented according to the CR while the network is not, there is no inter-operability issue. if the network is implemented according to the CR while the UE is not, the network might misunderstand the UE’s requested SI message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Ver0 in RAN2#120: R2-2211258 |  |  |  |  |  |  |  |  |

================================================== CHANGE BEGIN==============================================================

#### –	SI-RequestConfig

The IE SI-RequestConfig contains configuration for Msg1 based SI request.

SI-RequestConfig information element

-- ASN1START

-- TAG-SI-REQUESTCONFIG-START

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SI-REQUESTCONFIG-STOP

-- ASN1STOP

SI-RequestConfig field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occasions for SI. If the field is absent, the UE uses the corresponding parameters configured in rach-ConfigCommon of the initial uplink BWP.

si-RequestPeriod Periodicity of the SI-Request configuration in number of association periods.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting. Otherwise, when the SI-RequestConfig is used for on-demand SI request in SI-SchedulingInfo or posSI-SchedulingInfo, the 1st entry in the list corresponds to the first SI message in schedulingInfoList/posSI-SchedulingInfoList for which si-BroadcastStatus/posSI-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList/posSI-SchedulingInfoList for which si-BroadcastStatus/posSI-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResources should not result in system information change notification.

SI-RequestResources field descriptions

ra-AssociationPeriodIndex Index of the association period in the si-RequestPeriod in which the UE can send the SI request for SI message(s) corresponding to this SI-RequestResources, using the preambles indicated by ra-PreambleStartIndex and rach occasions indicated by ra-ssb-OccasionMaskIndex.

ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SI request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SI request.

=================================================   END OF CHANGES   =========================================================