3GPP TSG-RAN WG2 #121	R2-xxxx

Athens, Greece, 27th February – 3rd March 2023

Agenda item:	8.17.2

Source:	vivo

Title:	[AT121][202][MUSIM] LS to RAN4 on Rel-18 MUSIM impacts (vivo)

Document for:	Discussion and Agreement

1	Introduction

This document is to kick off the following email discussion:

- [AT121][202][MUSIM] LS to RAN4 on Rel-18 MUSIM impacts (vivo)

Scope: Discuss the topic and aim for consensus.

Intended outcome: Summary in R2-2302008 and agreeable LS (if possible) to RAN4 in R2-2302007.

Deadline: Friday morning (before morning coffee break)

2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |

3	Discussions

During the RAN2 post email discussion R2-2300773, the potential RAN4 impact identified by the companies includes the interruption caused by UE capability switching, and the maximum UE power change.

The interruption caused by UE capability switching:

When the UE receives RRC reconfiguration or carrier deactivation in NW A which is a response to the UE’s capability restriction request, there is an interruption on the active carriers in NW A. This interruption is called interruption 1 in this offline. The interruption 1 follows the current RAN4 requirement.

There could be an additional interruption (called interruption 2) in NW A due to RF retuning for starting NW B activity, e.g., RRC connection setup procedure in NW B. The interruption 2 is different from that of RRC reconfiguration or carrier deactivation, and is not specified in RAN4. The interruption 2 could be close to the interruption 1, and for this case we can discuss whether new interruption requirement in NW A can be defined.

Figure1

Observation 1: The interruption 2 in NW A may happen, and it is different from that of RRC reconfiguration or carrier deactivation, and it is possible/realistic to define the interruption 2.

Q1: Do you agree with the above OB1?

| Company | Comments |
| --- | --- |

Q2: if the answer to Q1 is YES, do you think we should send an LS to RAN4 to ask if the interruption 2 in NW A due to MUSIM capability switching should be defined in RAN4?

| Company | Comments |
| --- | --- |

Whenever there is RRC reconfiguration in NW B, interruption may occur in NW A due to RF retuning at the UE. And this interruption time (called interruption 3 in this offline) is also not specified in RAN4. As the RRC reconfiguration in NW B may happen at any time and the interruption time depends on the configuration parameters involved, it is impossible to specify such interruption.

Figure2

Observation 2: The interruption 3 in NW A may happen, and it is different from that of RRC reconfiguration or carrier deactivation, and it is impossible to define the interruption 3 because the uncertain time of the RRC reconfiguration in NW B and the involved configuration parameters.

Q3: Do you agree with the above OB2?

| Company | Comments |
| --- | --- |

Q4: If the answer to Q3 is YES, do you think we should send an LS to RAN4 to confirm the above understanding?

| Company | Comments |
| --- | --- |

UL maximum power:

Due to dual active transmission of R18 MUSIM, the maximum power may be constrained in NW A due to power sharing between NW A and NW B. And basically, this is RAN4 scope, so we think it would be better to get RAN4 input on this.

Q5: Do you think we should send an LS to RAN4 to evaluate potential RAN4 impact for maximum power constraint for R18 MUSIM?

| Company | Comments |
| --- | --- |