3GPP TSG-RAN WG2 Meeting #121	                                                                  R2-2300180

Athens, Greece, Feb 27- March 3, 2023

Source:	CATT, CBN

Title:	Discussion on Shared processing for MBS broadcast and Unicast reception

Agenda Item:	8.11.3

Document for:	Discussion and Decision

1	Introduction

In previous meeting, the following agreements as in [1] has been reached on shared processing for MBS broadcast and Unicast reception,

For shared processing we adopt the following as a baseline:

- 1) new IE is added in system information to control whether MBSInterestIndication for shared processing can be sent or not;

- 2) MBSInterestIndication message content and related procedure is updated for shared processing.

New IE to control whether MBSInterestIndication for shared processing can be sent or not is added to SIB1.

In MBSInterestIndication, for a broadcast service that the UE is receiving or is interested to receive, at least the following information can be signalled: broadcast frequency, subcarrier spacing, and bandwidth. FFS details/exact parameters and other information. FFS in which scenarios the UE reports this information (e.g. intra-PLMN case, inter-PLMN case)

FFS whether UE capability is needed to enable shared processing.

In this paper, we discuss the remaining issues for this objective. More specifically, the issues to be discussed include,

- Which scenarios to reports the enhanced MII information

- The need of UE capability for the enhanced MII information reporting

- Details/exact parameters and other information

- For which broadcast services to report enhanced MII information

2	Discussion

Which scenarios to reports the enhanced MII information

It is a FFS in which scenarios the UE reports this enhanced MII information (e.g. intra-PLMN case, inter-PLMN case).

Obviously the only issue here is whether UE needs to report this information for intra-PLMN case.

Firstly, it is clear that intra-PLMN case is in scope according to the WID [1],

Therefore, the unicast connection might be impacted by the broadcast reception for this kind of UEs. The optimization for such case is not specifically addressed in Rel-17, and should focus on the case of unicast reception in RRC_CONNECTED and broadcast reception from the same or different operators, including emergency and public safety broadcast.

Then in the real network deployment, it is possible that UE receives broadcast services from one gNB but at the same time UE camps on one another unicast gNB of the same PLMN. The information reported by UE is still useful to the unicast gNB as it may be not able to get the information from the broadcast gNB (e.g no Xn exists) even though they are belonging to the same PLMN.

Proposal 1: UE may report the enhanced MII information for intra-PLMN case or inter-PLMN case.

The need of UE capability for the enhanced MII information reporting

It can be further discussed whether UE capability needs to be defined for this feature. One possible way is not to define any UE capability. Since such enhance MII information is based on the Rel-17 MBS Interest Indication information, there seems to be no strong need to introduce a UE capability as in Rel-17 the MBS Interest Indication itself does not require a separate UE capability. Another possible option is to define an optional capability without signalling (i.e. similar as the capability “MBMS reception using Receive Only Mode” in LTE).  In any case we do not see a strong need to introduce a UE capability bit (i.e., to inform network whether the UE support such enhanced MII information reporting or not).

Therefore, the proposal is as following,

Proposal 2: No new UE capability signalling is needed for the enhanced MII information reporting.

Details/exact parameters of the enhanced MII information

In previously meeting it is generally agreed that the information to be signalled includes: broadcast frequency, subcarrier spacing, and bandwidth. However, it is still FFS on the details/exact parameters and other information.

Since the motivations for LTE MBMS and NR MBS are more or less the same, so for this objective it is sufficient to use similar parameters as LTE MBMS. Of course the NR MBS characteristics should be considered, e.g., NR MBS services are transmitted within CFR instead of the full bandwidth. Hence, CFR should be reported as part of the enhanced MII. It can be discussed whether to also report the full bandwidth of the broadcast cell.

Proposal 3: Confirm that the enhanced MII information includes:

- broadcast frequency

- subcarrier spacing

- CFR, FFS whether full bandwidth is also needed

For which broadcast services to report the enhanced MII information

In the LTE case, there is a predefined set of service(s), for which UE may report the related enhanced MII information. This is based on a TMGI value range that UE obtained previously (which is from CN so out of the RAN scope).

In NR we see a few possible options on for which broadcast services to report enhanced MII information,

Option 1: UE report the information for all the interested MBS broadcast service(s) provided on non-serving cell.

For option 1, only if the interested broadcast services is provided on a cell which is not the current serving cell(PCell or SCell),UE reports the  information for the corresponding broadcast services.

Option 2: UE report the information for all the interested MBS broadcast service(s)

Option 2 is simplest from UE perspective and has least spec impact, UE just report the information for all the interested broadcast service and unicast gNB can further decide whether the information is used or not(e.g. for intra-gNB case, it may be not useful).

Option 3：Broadcast service for which TMGI in a specific range ( i.e. similar mechanism as LTE)

For option 3, it has cross-WG impacts as we might need SA2 involvement to define a range of TMGI value. So it is preferred to rule it out.

Option 4: UE report the information for all the interested MBS broadcast service(s) from another gNB.

Option 4 is the most precise solution as the enhanced MII information for sharing processing makes more sense for inter-gNB case. However, in the current NR system UE is not able to identify whether the cell supporting the interested broadcast service belongs to the same gNB as for unicast cell. Hence, it is preferred to rule option 4 out as well.

Based on above, it can be discussed further on which option to choose between option 1 and option 2.

Therefore, it is proposed as following,

Proposal 4: Discuss the following option regarding which broadcast services to report the enhanced MII information:

- Option 1: All the interested MBS broadcast service(s) except for those provided in the current serving cell of the UE

- Option 2: All the interested MBS broadcast service(s) of the UE

5	Conclusion

Based on the discussions, we have the following proposals for shared processing for MBS broadcast and Unicast reception,

Proposal 1: UE may report the enhanced MII information for intra-PLMN case or inter-PLMN case.

Proposal 2: No new UE capability signalling is needed for the enhanced MII information reporting.

Proposal 3: Confirm that the enhanced MII information includes:

- broadcast frequency

- subcarrier spacing

- CFR, FFS whether full bandwidth is also needed

Proposal 4: Discuss the following option regarding which broadcast services to report the enhanced MII information:

- Option 1: All the interested MBS broadcast service(s) except for those provided in the current serving cell of the UE

- Option 2: All the interested MBS broadcast service(s) of the UE

6	Reference

[1] Draft meeting report for RAN2#119bis-e, in https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_119bis-e/Report