3GPP TSG RAN WG2 #121											        R2-2300275

Athens, Greece, February-March, 2023

Agenda Item:	8.9.3

Source: 	NEC

Title:	Discussion on Service Continuity Enhancements

Document for:	Discussion

- Introduction

In RAN2#119bis-e, the following agreements were made [1].

Proposal 1 (modified) For i2i path switch procedure, introduce a new measurement event based on individual thresholds i.e., Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2. FFS if we also have an event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE, and in this case if/how to compare SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE. Proposal 3 For i2i scenario, re-use the SL-RSRP or SD-RSRP measurement quantities for path switching. Proposal 4 (modified) For i2i scenario, serving/candidate U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity. Wording can be revisited if it is determined that L2IDs for U2U a nd U2N are always different (so that candidate U2N relay UEs would never have SL-RSRP available).

This contribution provides our further considerations on service continuity enhancement.

- Discussion

During the Rel-17 WI stage on NR sidelink relay, the procedures for two scenarios, including intra-gNB direct-to-indirect path switching and intra-gNB indirect-to-direct path switching, are discussed and captured in the corresponding specifications [2] [3] which include:

- the configuration procedures and parameters on the addition and release of relay channels, such as releasing indirect path for intra-gNB indirect-to-direct path switching and adding indirect path for intra-gNB direct-to-indirect path switching, and

- parameters for the measurement and report of the path switching, such as trigger events corresponding to intra-gNB direct-to-indirect path switching and intra-gNB indirect-to-direct path switching.

2.1 Considerations on the Measurement Event of Indirect-to-indirect Path Switching

In previous meeting [1], for the indirect-to-indirect path switching, companies agreed to specify Event Z1 which compares the measurement result of the serving Relay UE and that of candidate Relay UE with two thresholds separately. Some companies also suggest introducing Event Z2 which compares the measurement result of the serving Relay UE and that of the candidate Relay UE directly. However, both SL-RSRP and SD-RSRP are available for the PC5 link. It may happen that the SL-RSRP/SD-RSRP of the serving Relay UE is compared with SL-RSRP/SD-RSRP of candidate Relay UE. Since the transmission power of the relay traffic and the discovery message may be different, separate offsets should be configured to enable the comparison between different types of measurement results and the comparison for the same type of measurement results. So we propose:

RAN2 to define multiple offsets for Event Z2 if Event Z2 is accepted by RAN2. Otherwise, we do not think Event Z2 is needed.

2.2 Basic Procedures of for Inter-gNB I2D Path Switching and Inter-gNB I2I Path Switching

When considering the procedure of inter-gNB I2D path switching including the releasing of indirect path, one issue is when to reconfigure Relay UE to release the configuration of relay channel(s).

For intra-gNB I2D path switching, the gNB is aware of the access of Remote UE for the reason that the gNB is performed as both source gNB and the target gNB. So the reconfiguration of Relay UE can be executed at a proper time based on gNB implementation, such as the RRCReconfiguration message to the L2 U2N Relay UE can be sent anytime after step 3 [2].

However, for inter-gNB I2D path switching, the path switching command is sent via the source Relay UE. Due to the per-hop RLC scheme being adopted, the source gNB is not aware of whether the path switching command is sent to Remote UE successfully.  Furthermore, Remote UE will access to the target gNB while the source gNB is in charge of the reconfiguration of Relay UE. Therefore, the source gNB is not aware of whether the path switching of Remote UE is finished. Thus, it may happen that the source gNB reconfigures the source Relay UE to release the PC5 relay RLC channel/the PC5 connection between Remote UE before the successful delivery of path switching command to Remote UE. For example, the path switching command is waiting for RLC retransmission at the source Relay UE, but the PC5 relay RLC channel of Relay UE is released as requested by the source gNB. Moreover, a similar situation may happen to inter-gNB I2I path switching.

For inter-gNB I2D path switching and inter-gNB I2I path switching, when to reconfigure the source Relay UE should be considered to guarantee the successful delivery of the path switch command.

To solve this problem, one simple method is the target gNB be notified by the source gNB with the access of Remote UE. After that, the source Relay UE can be reconfigured. Details can be discussed further at the next meeting. So we propose:

RAN2 to consider when to reconfigure Relay UE to release the configuration of relay channel(s) for inter-gNB I2D path switching and inter-gNB I2I path switching.

2.3 Failure handling for path switching to an indirect path

During D2I or I2I path switching, the RRC layer of Remote UE may trigger the upper layer to establish the PC5 unicast link with the target Relay UE. The RRCReconfigurationComplete message can only be sent if this link is successfully established. Since extra time is needed for the preparation of the target Relay UE between two gNBs compared to intra-gNB scenarios, the channel condition between Relay UE and Remote UE may be changed during that long time period, leading to the failure of the PC5 unicast link establishment. Therefore, the RRCReconfigurationComplete message will not be sent, let alone sending it successfully to stop timer T420. This failure of path switching can be determined by Remote UE once the PC5 unicast link is not established successfully. Thus Remote UE can solve the failure in time by executing the connection re-establishment procedure, rather than waiting for the expiration of timer T420. So we propose:

RAN2 to consider the failure of PC5 unicast link establishment during path switching to trigger the RRC Reconfiguration failure procedure.

- Conclusion

Observation 1:	For inter-gNB I2D path switching and inter-gNB I2I path switching, when to reconfigure the source Relay UE should be considered to guarantee the successful delivery of the path switch command.

Proposal 1: RAN2 to define multiple offsets for Event Z2 if Event Z2 is accepted by RAN2. Otherwise, we do not think Event Z2 is needed.

Proposal 2: RAN2 to consider when to reconfigure Relay UE to release the configuration of relay channel(s) for inter-gNB I2D path switching and inter-gNB I2I path switching.

Proposal 3: RAN2 to consider the failure of PC5 unicast link establishment during path switching to trigger the RRC Reconfiguration failure procedure.

- References

[1] Chairman’s Notes, RAN2#119bis-e.

[2] TS 38.300 V17.2.0

[3] TS 38.331 V17.2.0