3GPP TSG-RAN WG2 Meeting #121                                              	R2-2300296

Athens, Greece, 27th Feb – 3rd Mar, 2023

Source:	CATT

Title:	RACH enhancement for SON

Agenda Item:	8.13.6

Document for:	Discussion and Decision

Introduction

In last meeting, some agreements are made for RACH partitioning and SgNB RACH report for MR-DC scenarios respectively. The agreements are listed as below [1]:

Agreements:

1	For RACH report for RACH partitioning, RAN2 to agree to include NSAG ID when the applicable feature is slicing.

2	 RACH report enhancements required for NE-DC are de-prioritized.

3	 For EN-DC and NG-EN-DC, the UE collects SN RA report container (for NR) and reports to the LTE MN. FFS on whether and which PSCell identity UE should report outside the RACH report.

4	UE includes RA and SDT information in RA report when an SDT operation fails.

In this contribution, we analyse the detailed solution for SgNB RACH report in MR-DC, and also the issues on RACH report for RACH partitioning.

Discussion

SgNB RACH report for different MR-DC scenarios

RAN2 has reached an agreement that NE-DC is de-prioritized for RACH report enhancement, and only the EN-DC and (NG)EN-DC can be considered, based on the RAN3 LS in [2].

There is an FFS left on whether and which PSCell identity UE should report outside the RACH report. Before clarify this issue, we assume the following cases:

Case 1) MN is always LTE: UE records the SgNB RACH information in EN-DC and (NG)EN-DC scenario, and then, stays in this scenario or enters LTE-DC scenario or (e)LTE SA scenario;

Case 2) MN changes to NR: UE records the SgNB RACH information in EN-DC and (NG)EN-DC scenario, and then enters NR-DC, NE-DC or NR SA scenarios;

For both cases, the UE records the SgNB RACH information in NR format. And then for case 1), the result of NR format should be included in the container and be sent to the LTE MN as mentioned in RAN3 LS [2]; for case 2), if the SgNB RACH information recorded in EN-DC and (NG)EN-DC scenario can have the same format as recorded in NR-DC scenario in R17, the UE could report it to the NR MN, and the NR MN can transfer it to corresponding SN node.

Proposal 1: The SgNB RACH information recorded by UE in EN-DC and (NG)EN-DC scenario can be reported to NR MN in NR format, if UE enters NR-DC, NE-DC or NR SA scenarios.

Additionally, available indicator to the SN node RACH report is suggested by some companies in RAN2. But the RAN3 reply LS [2] has confirmed that both the MN and the SN RACH information are reported to the MN node. From RAN2 point of view, the UE may not always be connected to a SN node, so to report all information to the MN node is reasonable.

For case 2) above we suggest reporting the SgNB RACH information as NR-DC scenario does, so only case 1) is considered in the subsequent discussion in this contribution. It is, how to report the recorded SgNB RACH information to the LTE/ (e)LTE MN.

Observation 1: RAN3 LS has confirmed the RACH information of both MN and SN are reported to the MN node.

A network solution has been assumed in RAN3 that for indicating the RACH occurrence from gNB-DU to gNB-CU or the potential availability of RA report from SN to MN, some indicators may be needed between network nodes, therefore it is not necessary to introduce the available indicator or reporting to the SN node from the UE side.

And to align with the LTE specification, there is also no available indicator to MN is needed. Since each entry align to the RACH occurred PSCell identity, the information can be sent to the right node after the network retrieve all the RACH related information from the UE.

Proposal 2: UE reports the RACH information of both MN and SN to the LTE/(e)LTE MN node for EN-DC/(NG)EN-DC scenarios, and no available indicator to MN or SN is needed.

SgNB RACH content

For SgNB RACH information recorded in EN-DC/(NG)EN-DC case, it is natural to follow the structure recorded in NR-DC case.

Proposal 2: UE records the SgNB RACH information in EN-DC/(NG)EN-DC scenario in NR format, and the structure is the same as the SgNB RACH information recorded in NR-DC scenario.

Current RACH Report in TS36.331 only supports simple parameters report such as the number of preambles and whether the contention is detected (For normal UE), and only one entry is allowed to record the information of the last RACH procedure. And since the UE always has RACH information in CONNECTED mode, there is no need to send available indicator to the network. The network can directly retrieve the RACH information by UEInformationRequest/Response procedure.

RACH-Report-r16 ::=					SEQUENCE {

numberOfPreamblesSent-r16			NumberOfPreamblesSent-r11,

contentionDetected-r16				BOOLEAN

}

RACH-Report-v1610 ::=	SEQUENCE {

initialCEL-r16 					INTEGER (0..3),

edt-Fallback-r16					BOOLEAN

}

So there can be 2 options for recording the SgNB RACH information in EN-DC and (NG)EN-DC scenarios:

Option 1: For simplicity, UE records only one entry, since the recording of LTE MN has only one entry;

Option 2: Similar as NR RACH information recording, UE could record at most 8 entries.

Proposal 3: RAN2 to discuss how many entries the UE could record for SgNB RACH information in EN-DC and (NG)EN-DC scenarios, e.g. 1 or 8.

For option 1, the UE should include the NR SgNB RACH information in LTE specification, the format could be an NR container. It is mentioned in the RAN3 LS [2] that:

RAN3 believes that if RAN2 decides to support SN RA Report for EN-DC and (NG)EN-DC, the UE should report the PSCell identity outside the RACH report to help an eNB forward the report to the correct node without the need to decode the RACH report.

Since the SgNB information recorded in the UE may not the current UE connected SgNB, the UE could send the PSCell ID in LTE format together with the NR container to the MN. The PSCell ID indicates the PSCell in which the RACH occurred. The MN does not need to decode the NR container, and send the container to the correct SgNB based on the PSCell ID.

But for option 2, whether and which PSCell identity UE should report outside the RACH report needs to be discussed. One way is to report only one of the NR PSCell ID in which the RACH occurred in LTE format. And then the NR PSCell ID could decode the container and forward all RACH information to the correct node(s). But this solution will increase the RAN3 complex. So an appropriate solution is the eNB directly forward the RACH information to the respective correct node(s) without requesting a gNB to decode and re-transmit the RACH information as mentioned in the RAN3 LS [2].

Observation 2: The solution of eNB requesting a gNB to decode and re-transmit the RACH information will introduce much RAN3 complexity.

Since the RACH information the UE recorded in multiple entries may belong to different PSCells, it is necessary to include multiple containers in the RACH report related message, and each container should be related to one PSCell ID, as the figure1 below:

Figure 1 Enhanced RACH report to LTE/(e)LTE MN

Proposal 4: UE reports the SgNB information using the format of NR container together with the outer PSCell ID in LTE SgNB RACH report, and the PSCell ID indicates the PSCell in which the RACH occurred.

Proposal 5: If more than one entry is recorded, multiple containers of SgNB RACH information with respective PSCell ID should be included in LTE SgNB RACH report, and each container includes only one NR- format RACH information.

For NR-DC scenario, there is no additional UE capability introduced for the SgNB RACH report since the RACH information of MN and SN use the same encode format and share the same mechanism and the UE variable. But for EN-DC and (NG)EN-DC scenarios, if the NR container is introduced, additional UE capability may be needed to identify whether the inter-RAT record and report of RACH information could be supported.

Proposal 6: Additional capability may be needed for NR RACH Report enhancement in LTE for EN-DC and (NG)EN-DC scenarios.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: The SgNB RACH information recorded by UE in EN-DC and (NG)EN-DC scenario can be reported to NR MN in NR format, if UE enters NR-DC, NE-DC or NR SA scenarios.

Observation 1: RAN3 LS has confirmed the RACH information of both MN and SN are reported to the MN node.

Proposal 2: UE records the SgNB RACH information in EN-DC/(NG)EN-DC scenario in NR format, and the structure is the same as the SgNB RACH information recorded in NR-DC scenario.

Proposal 3: RAN2 to discuss how many entries the UE could record for SgNB RACH information in EN-DC and (NG)EN-DC scenarios, e.g. 1 or 8.

Observation 2: The solution of eNB requesting a gNB to decode and re-transmit the RACH information will introduce much RAN3 complexity.

Proposal 4: UE reports the SgNB information using the format of NR container together with the outer PSCell ID in LTE SgNB RACH report, and the PSCell ID indicates the PSCell in which the RACH occurred.

Proposal 5: If more than one entry is recorded, multiple containers of SgNB RACH information with respective PSCell ID should be included in LTE SgNB RACH report, and each container includes only one NR format RACH information.

Proposal 6: Additional capability may be needed for NR RACH Report enhancement in LTE for EN-DC and (NG)EN-DC scenarios.

Reference

- Draft_R2_120_meeting_report_v1.docx

- R2-2211164/R3-226053 Reply LS on SN RACH report status in R17 RAN3