3GPP TSG-RAN WG2 Meeting #121	R2-2300436

Athens, Greece, 27 Feb – 2 March 2023

Agenda item:	8.17.2

Source:	Intel Corporation

Title:	Signalling to indicate temporary capability reduction for Rel-18 MUSIM

WID/SID:	NR_DualTxRx_MUSIM-Core

Document for:	Discussion and Decision

# 1	Introduction

In RAN#119bis-e, the following is agreed:

- RAN2 aims to prioritize only few solutions and avoid multiple solutions for the same problem (FFS pending on solution details).

On the high level, the potential solutions are listed as follow:

- B1: For UAI based solution, the following steps can be used as a baseline:

- The UE is in Connected Mode on NW A .

- The UE is configured for UE capability update via UAI.

- The UE intends to start or stop connection with NW B or is already in Connected mode in NW B.

- The UE requests a change (restriction or removal of restriction) of the UE capabilities at NW A via UAI.

- NW A reconfigures the UE, if needed, according to its new capabilities (FFS if NW response is mandatory)

- The UE operates in NW A with the updated configuration.

- B2: For delta-signaling of UE capability, the following steps can be used as a baseline:

- The UE is in Connected Mode in NW A.

- The UE is configured for UE capability update.

- The UE starts or stops connection with NW B or is already in Connected mode in NW B.

- The UE signals the changed UE capabilities to NW A via delta-signaling.

- NW A reconfigures, if needed, the UE according to its new capabilities (FFS if NW response is mandatory).

- The UE operates in NW A with the updated configuration.

- B3: The solution for the repetition of UE capability enquiry, the following steps can be used as a baseline:

- The UE is in Connected Mode in NW A.

- The UE is configured for UE capability update.

- The UE starts or stops connection with NW B or is already in Connected mode in NW B.

- The UE requests a UE capabilty update request.

- NW A sends UECapabilityEnquiry to the UE

- UE sends UECapabilityInformation to the NW A gNB.

- NW A reconfigures, if needed, the UE according to its new capabilities (FFS if NW response is mandatory.

- The UE operates in NW A with the updated configuration.

- B4: The solution based on using UE-profiles for capability restriction, the following steps can be used as a baseline:

- The UE signals different temporary UE capability sets during registration (FFS if these profiles can be updated later)

- The UE is in Connected Mode in NW A .

- The UE starts or stops connection with NW B or is already in Connected mode in NW B.

- The UE requests to switch to a different UE capability profile, e.g. by signaling an index of the profile.

- NW A reconfigures the UE according to its new capabilities.

- The UE operates in NW A with the updated configuration.

- B5 (11/15): A baseline procedure for MAC-CE based SCell (de)-activation can be considered as follows:

- The UE is in Connected Mode in NW A .

- The UE is configured for MAC-CE based SCell (de)-activation operation.

- The UE starts or stops connection with NW B or is already in Connected mode in NW B.

- The UE sends a request to deactivate SCells via MAC-CE.

- NW A deactivates, if needed, the requested SCells (FFS if NW response is mandatory).

- The UE operates in NW A with the updated configuration.

In this contribution, the B1-5 solutions are further analysed

# 2	Discussion

## 2.1	UE capability restriction indication mechanisms

The objective of the Rel-18 MUSIM WI is to allow the network to be simultaneously connected to both network A and network B.  UE does this by sharing its capability across network A and network B.  This implies that the UE will not be able to use its full capabilities in both network A and network B simultaneously while UE is connected in both networks.  UE hence must reduce its capability in one or both networks and this reduced capability must be signalled to the network. As discussed in the last meeting, there are a few options to provide such restrictions:

Solution B3 is for UE to request a temporary update of the static UE capabilities by indicating the new (reduced) UE capabilities to the network.  This is the actual UE capability that will temporarily replace the static capability (no delta signalling). As MUSIM will only impact some of the features, to avoid having to send the full updated capability, this replacement can be limited to some IEs of the UE capability.  Such a solution seems to generate a lot of unnecessary overhead.

Observation#1: Solution B3 requires sending the actual UE capability that will temporarily replace the static capability as delta signalling is not used.  This will generate a lot of unnecessary overhead.

Instead of sending the full UE capabilities every time the UE capabilities change, Solution B2 proposed to signal the changed UE capabilities via delta-signalling.  However, it is unclear how such delta-signalling can be performed based on the existing UE capability information message since the absence of a feature/UE capability may mean the feature/UE capability is not supported and not that there is no change. One way is to still provide all the possible band combinations that are still possible in the UE capabilities (i.e. a new UE capabilities are provided using the existing signalling); i.e., a partial update of just the band combinations. Hence this solution is not very efficient from the signalling overhead point of view.

Observation#2: Instead of sending the actual UE capability without delta signalling as in Solution B3, delta signalling is applied on the existing UE capability information. However, it is unclear how this can be done without significant changes to the capability signalling to introduce delta signalling since the absence of a feature/UE capability may mean the feature/UE capability is not supported and not that there is no change.

It is clear that a new UE capability restriction signalling is needed to indicate just the restriction in the capability from the static capability to network A due to the connection to network B. This is referred as Solution B1. Such restriction can be indicated in terms of e.g, bands that are affected by the connection to network B (more of the UE capability restriction is discussed in Section 2.1). For example, if the connection to network B requires Band X which may affect the UE’s SCell in Band Y in network A, the UE indicates the restriction of Band X and Band Y as part of the UE capability restriction signalling indication.  The network A can than infer/compute from this restriction of the possible band combinations (e.g. all band combinations containing Band X and Band Y are restricted). This UE capability restriction can be signalled using the UE assistance information mechanism.  Some extensions to the UE assistance information procedure are needed for MUSIM and these are discussed further below.  It seems simpler to extend that framework also to cover capability restriction due to MUSIM as the overall intention is the same – to indicate to the network not to use a band/CC etc.  As it is more efficient to dynamically signal the capability restriction with minimal size to provide only the current restriction required, this seems a better approach over using a capability update directly.

Observation#3: Solution B1 dynamically signals the capability restriction with minimal size to provide only the current restriction required, it is a more efficient than Solution B2 and B3.

As for Solution B5, it can be used in the case where the UE capability restriction is already in use in network A (i.e. reactive approach). In our companion paper [1], we also discussed the need of proactive approach in which the UE proactively provide the temporary UE capability restriction to network A regardless of whether the network A is or is likely to be affected by the temporary UE capability restriction.  This can help mitigate possible reconfiguration and resume/setup delay.  This solution does not allow for proactively indicating the UE capability restriction. If both proactive and reactive approach needs to be supported, Solution B5 alone is not sufficient.  Solution B1 can cover both reactive and proactive approach.

Observation#4: Solution B5 can be used in the case where the UE capability restriction is already in use in network A (i.e. reactive approach), however it does not allow for proactively indicating the UE capability restriction. (e.g. in the case where NW B is using SCell in Band A, UE can’t inform NW A of this restriction and may configure an SCell in Band A later on).

The table below summarised the pros and cons of the various solutions:

| Solutions | Overhead | Support both reactive and proactive approach | Specification complexity |
| --- | --- | --- | --- |
| B1 (capability restriction using UAI) | Low | Yes | Low |
| B2 (Delta signalling of UE capability) | Medium | Yes | High |
| B3 (Full capability update) | High | Yes | Low |
| B5 (release/deactivate Scell) | Low | No ☹ | Low |

Proposal#1: Support only Solution B1 based on UE Assistance Information to indicate capability restriction for Rel-18 MUSIM.

In the following sections, we first discuss what is in the UE capability restriction information and then discuss the possible signalling overhead.

## 2.2	UE capability restriction information

Overheating mechanism is introduced with UE assistance information provided by the UE to inform the network which related configurations in the UE needs reduction in order to prevent UE from overheating.  The UE assistance for overheating can be related to configurations corresponding to bandwidth or to the number of MIMO layers or to the number of CCs.  This is further extended also to UE power saving with additional UE assistance information related to C-DRX configuration preferences, minimum scheduling offset preference and RRC states preference.

For Rel-18 MUSIM, the UE request that is useful for temporary UE capability restriction in NW A is to indicate the bands or the CCs within a band that are no longer possible due to need to connect to NW B.  Other UE assistance from overheating and UE power saving such as bandwidth reduction and number of MIMO layer reduction on the UE capabilities in NW A may also be useful for Rel-18 MUSIM in order to satisfy simultaneous reception and transmission on NW B. However, it is not sufficient to just indicate the need to reduce the bandwidths and the number of MIMO layers; there is also a need to indicate on which bands and/or CC such reduction is needed. The use of bandwidth and MIMO layers reduction may need more analysis whether such finer granularity is useful for Rel-18 MUSIM.

Observation#5: For Rel-18 MUSIM, the UE request for temporary UE capability restriction in NW A has to indicate the bands or the CCs within a band that are no longer possible due to being in RRC CONNECTED in NW A and B simultaneously. Other parameters need further discussion.

As discussed in our companion paper, the UE capability restriction can depend on whether the capability restriction is i) not currently in use in network A (i.e. proactive case) or ii) currently in use in network A (i.e. reactive case). How to signal the capability restriction for the reactive and proactive are discussed below.

For signalling the capability restriction in network A while the restriction is either in use or not in use, UE request for temporary UE capability restriction on the bands and/or CCs within a band can be provided explicitly for example like the following:

- For bands and/CCs within a band in NW A that has not been configured as a serving cell yet (e.g. SCell):

- The FrequencyInfoDL (SCS-SpecificCarrier, absoluteFrequencyPointA, FreqBandIndicatorNRs) for DL CC(s)/Band which is no longer possible

- The FrequencyInfoUL (SCS-SpecificCarrier, absoluteFrequencyPointA, FreqBandIndicatorNRs) for DL CC(s)/Band which is no longer possible

- Instead of indicating all the bands or CCs within a band that are current restricted, the UE can provide just the bands or CCs within a band that are currently used by NW B to NW A and NW A can infer the band/CC/band combination restrictions based on the existing band combinations for CA and DC in the UE capability.

Approach (i) seems to be quite straightforward. For (ii), as the RF architecture of the UE is not known to the network, it may not be easy for the network to infer all the restricted band and/or CCs within a band that are restricted.

Proposal#2: For Rel-18 MUSIM, the UE capability restriction information explicitly indicates the bands or the CCs within a band that are no longer possible/available in NW A due to connectivity in NW B.

## 2.3 Further Overhead reduction

For RRC UAI, in order to reduce the signalling overhead, prohibit timer is introduced to prevent the UE repeating the same preference indication. However, such timer should not prevent UE from requesting changes in the assistance information as configuration in NW B is asynchronous to procedures in NW A and is unpredictable (i.e., UE has no prior knowledge of what configuration will be used in network B and when).   Hence, the use of prohibit timer may not be directly suitable for Rel-18 MUSIM and some further signalling overhead reduction techniques need to be studied.

One approach is that NW A can configure the UE on the number of restrictions that the UE can request for MUSIM.  For example, the NW A can limit the maximum number of CCs or bands that the UE can request to be restricted by the NW A for use in NW B. By doing so, it will reduce the number of changes possible and thus reduce the amount of overhead and allow UE to update the temporary UE capability whenever the number of restrictions has not been reached instead of being prohibited to update due to the prohibit timer.

Observation#6:  UE should not be prevented from requesting changes in the assistance information as configuration in NW B is asynchronous to procedures in NW A and is unpredictable (i.e., UE has no prior knowledge of what configuration will be used in network B and when), e.g. using prohibit timer.

Proposal#3: RAN2 should discuss overhead reduction of sending the UAI in view of the asynchronous nature in the configuration of NW B, (e.g. limiting the maximum number of reductions that can be sent to NW A over certain time etc.)

# 4	Conclusion

It is requested that RAN2 agree to the observations and proposals below:

Observation#1: Solution B3 requires sending the actual UE capability that will temporarily replace the static capability as delta signalling is not used.  This will generate a lot of unnecessary overhead.

Observation#2: Instead of sending the actual UE capability without delta signalling as in Solution B3, delta signalling is applied on the existing UE capability information. However, it is unclear how this can be done without significant changes to the capability signalling to introduce delta signalling since the absence of a feature/UE capability may mean the feature/UE capability is not supported and not that there is no change.

Observation#3: Solution B1 dynamically signals the capability restriction with minimal size to provide only the current restriction required, it is a more efficient than Solution B2 and B3.

Observation#4: Solution B5 can be used in the case where the UE capability restriction is already in use in network A (i.e. reactive approach), however it does not allow for proactively indicating the UE capability restriction. (e.g. in the case where NW B is using SCell in Band A, UE can’t inform NW A of this restriction and may configure an SCell in Band A later on).

Proposal#1: Support only Solution B1 based on UE Assistance Information to indicate capability restriction for Rel-18 MUSIM.

Observation#5: For Rel-18 MUSIM, the UE request for temporary UE capability restriction in NW A has to indicate the bands or the CCs within a band that are no longer possible due to being in RRC CONNECTED in NW A and B simultaneously. Other parameters need further discussion.

Proposal#2: For Rel-18 MUSIM, the UE capability restriction information explicitly indicates the bands or the CCs within a band that are no longer possible/available in NW A due to connectivity in NW B.

Observation#6:  UE should not be prevented from requesting changes in the assistance information as configuration in NW B is asynchronous to procedures in NW A and is unpredictable (i.e., UE has no prior knowledge of what configuration will be used in network B and when), e.g. using prohibit timer.

Proposal#3: RAN2 should discuss overhead reduction of sending the UAI in view of the asynchronous nature in the configuration of NW B, (e.g. limiting the maximum number of reductions that can be sent to NW A over certain time etc.)

# 5	References

[1] R2-2209637 Considerations on Rel-18 MUSIM, Intel Corporation