3GPP TSG-RAN2#121	R2-2300491

Athens, Greece, February-March, 2023

Agenda item:		8.3.2

Source:		Lenovo

Title:		 Alignment to Cell DRX

Document for:		Discussion and Decision

- Introduction

One of the Techniques to improve network energy savings are the Techniques in time domain e.g., Cell DTX/DRX. Cell DTX/DRX is applied to at least UEs in RRC_CONNECTED state. A periodic Cell DTX/DRX (i.e., active, and non-active periods) can be configured by gNB via UE-specific RRC signalling per serving cell. RAN2 made some further agreements on this in RAN2#120 which are capture in [1]:

From 3GPP TR 38.864 v0.4.0: The Cell DTX/DRX mode can be activated/de-activated via dynamic L1/L2 signalling and UE-specific RRC signaling. Both UE specific and common L1/L2 signalling can be considered for activating/deactivating the Cell DTX/DRX mode. Cell DTX and Cell DRX modes can be configured and operated separately (e.g., one RRC configuration set for DL and another for UL). Cell DTX/DRX can also be configured and operated together. At least the following parameters can be configured per Cell DTX/DRX configuration: periodicity, start slot/offset, on duration. Details related to UE behaviour can be discussed during WI phase. Whether to support multiple Cell DTX/DRX configurations can be discussed later in the WI phase. It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs. The alignment mechanism can be discussed during the WI phase.

Since, Cell DTX and Cell DRX modes can be configured and operated separately, we are making independent proposals for Cell DRX and Cell DTX in [2]. This contribution takes a detailed look at UE alignment behaviour for Cell DRX only.

- Discussion

RAN2 should discuss possible UE behaviour when the cell/ Network is (going to) in DRX i.e., will not receive. The below Figure 1 shows two RRC Connected UEs with their corresponding DRX configurations, as well as their serving cell in energy saving mode utilizing a DRX configuration. A UE configured with a DRX configuration can transmit at any time, but it can receive only during certain limited time periods designated as “active time” and shown in the below Figure 1 as “UE reception ON” period. In uplink, cell is not expected to receive during “Cell Reception OFF” periods. With this background let’s discuss some specific aspects.

Figure 1

## RRC Procedures:

If the network triggers a RRC Procedure while the network is in DRX (or about to enter DRX), the UE after successfully executing the procedure (e.g., applying new configuration) does not immediately transmit the procedure completion message (e.g., RRC Reconfiguration Complete message) but transmits this when the network is in reception again i.e., at the start of the next reception time at the network (end of a network DRX period) and, UE assumes that the procedure is successfully completed. Following RRC procedure related completion message can be sent in this “delayed” manner:

- RRC Reconfiguration Complete,

- RRC Setup Complete,

- RRC Reestablishment Complete,

- RRC Resume Complete,

- Security Mode Complete,

- UE Information Response etc.

Practically, Network may attempt to minimize start of RRC procedure that may not be completed in time until the DRX at the network side is started. If a procedure is started, network may keep receiving for a limited time beyond the start of the intended DRX time to ensure that a possible response from a UE is not missed. But since the UE does not know if the network is receiving in a supposedly “Cell Reception Off” period, the following proposal is being made:

The UE after successfully executing a procedure (e.g., applying new configuration) assumes the procedure is successfully completed if the network triggers a RRC Procedure while the network is in DRX (or about to enter DRX). The procedure Complete message is transmitted when the network is in reception again.

UE initiated procedure/ information cases: The UE initiated procedure(s) or RRC messages e.g., Dedicated SIB Request, Idle mode SIB Request, UE Assistance Information, UL Information Transfer, Location Measurement Indication, UE Capability Information, Counter Check Response, MBS Interest Indication, Measurement Report App Layer, UE Positioning Assistance Info etc. shall be postponed until the network starts reception (DRX time is over). The UE shall always report the latest values applicable at the time of actual transmission of these messages.

UE initiated procedure/ information cases shall be postponed until the network starts reception (DRX time is over). The UE shall always report the latest values applicable at the time of actual transmission of these messages.

Further, Re-establishment procedure can be triggered for a number of reasons including reconfiguration failures, RLF etc., where upon initiation of the procedure, the UE shall stop all running timers among T310, T312, T304, T311, T316, reset MAC etc., and perform cell selection in accordance with the cell selection process as specified in TS 38.304. Upon selecting a suitable NR cell, the UE shall ensure having valid and up to date essential system information and determine if the cell is in DRX, or about to enter DRX (say in next 10 ms. or so) – if so, it shall look for another suitable NR cell that is not in DRX and not going to enter DRX anytime soon. Alternatively, this criterion can be included as part of the suitable cell selection procedure such that a cell is considered suitable only when the cell is not in DRX and not going to enter DRX anytime soon – at least when timer T311 is running.

A cell is considered suitable only when the cell is not in DRX and not going to enter DRX anytime soon, at least when timer T311 is running.

## Measurements

RRC Connected UE may make un-necessary measurements if the same can’t be reported successfully because of serving cell being in DRX. If RRC informs physical layer of the DRX time of the network, Physical layer need not perform measurements when a potential triggered measurement event might fall under the network’s DRX time. Indeed, the physical layer of the UE can start radio measurements for configured/ active measurement identities not before parameter TTT (TimeToTrigger) time before the start of the reception time (end of a DRX period) at the network. To this end, the highest value of the parameter TimeToTrigger among all configured measurement events is used. Further, it might take the UE finite time to start measurement (warmup/ tune RF, scan the radio frequency even do cell search/ synchronization etc.), the UE may indeed need to perform measurements a little earlier than just TTT ms from the next Cell Reception ON period, the exact time depending on UE implementation.

UE does not perform measurement until TimeToTrigger time before the start of the reception time (i.e., end of a DRX period) at the source cell.

## UL Data Transmission

UL data transmission can be dynamic scheduling transmission or transmission without dynamic grant. As we mentioned above, a UE configured with a DRX configuration “can” transmit at any time even if cell is not expected to receive during “Cell Reception OFF” periods. Since the cell DRX configured by the network and the dynamic scheduling is controlled also by the network, the network can control the dynamic UL transmission during the cell reception ON periods.

Observation 1	The UE dynamic UL transmission in the cell DRX OFF duration can be avoided by the network scheduling.

Before Rel-18, it has been captured in the specification that multiple configured grant configurations can be configured and active simultaneously in the same BWP. The CG configuration is to satisfy the requirement of UE traffic. And the cell DRX configuration is from the perspective of power consumption of the whole cell. It may occur that the CG resource overlaps with “Cell Reception OFF” period. Then there could be two potential behaviors, one is the UE initiates the transmission on the CG resource, the other is the UE does not initiate the CG transmission because the cell is not receiving in the cell Reception OFF period. We see that if UE initiates the CG transmission when it is clear that network is not receiving, wastes UE battery and will also increment a transmission counter un-necessarily, having further implications. So, we propose:

An UL transmission on the configured grant is not made when CG resource overlaps with cell Reception OFF duration.

# Conclusion

This document discussed some UE alignment behaviour for Cell DRX only. Following proposals are made:

- The UE after successfully executing a procedure (e.g., applying new configuration) assumes the procedure is successfully completed if the network triggers a RRC Procedure while the network is in DRX (or about to enter DRX). The procedure Complete message is transmitted when the network is in reception again.

- UE initiated procedure/ information cases shall be postponed until the network starts reception (DRX time is over). The UE shall always report the latest values applicable at the time of actual transmission of these messages.

- A cell is considered suitable only when the cell is not in DRX and not going to enter DRX anytime soon, at least when timer T311 is running.

- UE does not perform measurement until TimeToTrigger time before the start of the reception time (i.e., end of a DRX period) at the source cell.

- An UL transmission on the configured grant is not made when CG resource overlaps with cell Reception OFF duration.

# References

[1]: R2-2213040	3GPP TR 38.864 v0.4.0

[2]: R2-2300xxx	Alignment to Cell DRX	Lenovo