3GPP TSG-RAN WG2 #121	R2-2300804

Athens, Greece,  27th Feb – 3rd Mar 2023

Source:		NEC

Agenda item:		8.4.2.3	Cell Switch

Title:		Discussion on L2 handling for LTM

Document for:		Discussion and decision

# 1. Introduction

During last RAN2 meeting, the determination of whether to perform L2 reset for LTM was discussed, the following two options are on the table [1]:

ON the determination of whether to reset L2: two options on the table:

- The UE determines whether the switch is intra DU or inter DU and the follows different rule or configuration for these two cases which controls whether to reset or not reset. Determination could be based on configuration (e.g. of a DU ID, cell group id etc)

- The UE receives command to reset or not reset by Mac CE.

In this paper, we provide our view on which one of the two options to selected, and how does the UE perform PDCP SDU discard for SRBs during LTM.

# 2. Discussion

During last RAN2 meeting, how the UE determines whether to perform L2 reset procedures for LTM has been discussed, however it is not crystal clear what the L2 reset procedures that can be omitted for LTM procedures are.

In case of inter-DU LTM, since the DU has been changed, the RLC layer reset (RLC re-establishment) and MAC layer reset (MAC reset) should be performed. And due to the flush of packet by the lower layers, to avoid data loss, PDCP recovery for AM DRBs should be performed. But in case of intra-DU LTM, since DU is not changed, the above procedure can be omitted to save cell switch latency, and instead a partial MAC reset procedure with detail FFS can be performed.

Observation 1: The following L2 reset procedures are only need to be performed for inter-DU LTM and not needed for intra-DU LTM

- full MAC reset

- RLC re-establishment for DRBs and SRBs

- and PDCP recovery for AM DRBs

In case of reconfigurationwithSync without security key change, PDCP re-establishment for SRBs is not performed, but those PDCP SDUS/PDUs stored in the PDCP entity are actually RRC signalling towards the source PSCell. Those RRC message may cause problems if the UE transmits them to the target SPCell as these messages may be cell specific. Therefore, PDCP SDU discard procedure for SRB is performed instead to discard those PDCP SDU/PDUs in this case to avoid any unnecessary confusion to the network. Currently, UE performs it based on explicitly indication discardOnPDCP received from the network. For both intra-DU LTM and inter-DU LTM cases, the security key is not changed and therefore PDCP SDU discard for SRBs shall always be performed.

Observation 2: PDCP SDU discard for SRBs should always be performed for LTM regardless inter-DU or intra-DU.

Firstly, to decide on whether to perform L2 reset for the UE, so far, there are two options on the table: 1. UE determines whether to reset or not based on pre-configured RRC configuration (e.g. DU ID or cell grouping information associate with the LTM candidate cells) from network, or 2. UE determines whether to reset or not based on one indication in the cell switch MAC CE.

Both options work, and the MAC CE option provides more flexibility compared with the RRC configuration option, i.e. the network can configure the UE to perform L2 reset even for the case of intra-DU LTM. However, such flexibility seems not necessary, and unified rule (e.g. based on intra-DU LTM or inter-DU LTM) to control the L2 reset is sufficient. And to let a lower layer signalling to send indication to control higher layer behaviour (e.g. PDCP recovery), CU-DU coordination may needs to be discussed. So, our preference is to use a pre-configured configuration indicating DU ID or cell grouping information associated with the LTM candidate cells, and to let the UE determine based on the pre-configuration for each LTM procedure. More specifically, the UE decides whether to perform L2 reset procedure by determining if the source cell and target cell are from the cell group, or have the same DU ID. If the are/have, then the L2 reset procedures are not performed; Otherwise, L2 reset procedure should be performed. And this can be applied to both first LTM cell switch procedure and subsequent LTM cell switch procedure.

Proposal 1: For first LTM cell switch and subsequent LTM cell switch, the UE determines whether to perform a set of L2 reset procedures based on pre-configured RRC configuration (e.g. DU ID or cell grouping information associate with the LTM candidate cell).

Proposal 1a: the set of L2 procedures include

- full MAC reset

- RLC re-establishment for DRBs and SRBs

- and PDCP recovery for AM DRBs

Proposal 1b: UE performs the set of L2 reset procedures only if the source cell and target cell are from different cell groups or have different DU IDs.

Secondly, with regarding to how to trigger the UE to perform the always performed PDCP SDU discard for SRBs during LTM, since it was agreed that “The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration”, always configuring discardOnPDCP in the LTM candidate configuration would results in a mandatory supported RadioBearerConfig IE in LTM candidate configuration. To solve this, the UE can just implicitly perform (i.e. without RRC IE) PDCP SDU discard for SRBs during LTM. And this can be applied to both first LTM cell switch procedure and subsequent LTM cell switch procedure.

Proposal 2: For first LTM cell switch and subsequent LTM cell switch, the UE implicitly performs (i.e. without RRC IE) PDCP SDU discard for SRBs during LTM.

# 3. Conclusion

In this paper, we provide our view on how does the UE decides on whether to perform L2 handling related procedures for LTM, including L2 reset and PDCU SDU discard for SRBs, and have the following observation and proposal:

Observation 1: The following L2 reset procedures are only need to be performed for inter-DU LTM and not needed for intra-DU LTM

- full MAC reset

- RLC re-establishment for DRBs and SRBs

- and PDCP recovery for AM DRBs

Observation 2: PDCP SDU discard for SRBs should always be performed for LTM regardless inter-DU or intra-DU.

Proposal 1: For first LTM cell switch and subsequent LTM cell switch, the UE determines whether to perform a set of L2 reset procedures based on pre-configured RRC configuration (e.g. DU ID or cell grouping information associate with the LTM candidate cell).

Proposal 1a: the set of L2 procedures include

- full MAC reset

- RLC re-establishment for DRBs and SRBs

- and PDCP recovery for AM DRBs

Proposal 1b: UE performs the set of L2 reset procedures only if the source cell and target cell are from different cell groups or have different DU IDs.

Proposal 2: For first LTM cell switch and subsequent LTM cell switch, the UE implicitly performs (i.e. without RRC IE) PDCP SDU discard for SRBs during LTM.

# Reference

[1] Meeting report of RAN2 #120.