3GPP TSG RAN WG2 #121												R2-2300854

Athens, Greece, 27 February – 3 March, 2023

Agenda Item:	8.8.5

Source: 	NEC

Title:	Considerations on Enhancements for UAV Identification Broadcast

Document for:	Discussion

- Introduction

In RAN#98e, objectives for UAV identification broadcast has been revised as follows. [1].

3. Study and specify, if needed, enhancements to NR PC5 and LTE PC5 to support UAV identification broadcast [RAN2]. Check at RAN#99 for further elaboration of this objective, based on RAN2 discussions and SA2 development. Note: This objective is not intended to introduce new band(s) with support for the PC5 interface. UAV use of LTE PC5 or NR PC5 is not addressing any operator licensed band.

This contribution provides initial considerations on enhancement for UAV identification broadcast.

- Discussion

Remote identification is the ability of an unmanned aircraft in flight to provide certain identification and location information that people on the ground and other airspace users can receive. As the number of UAS operations in all classes of airspace increases rapidly, remote identification become more and more important for unmanned traffic management.

The ability to identify and locate operating UAVs in the airspace provides situational awareness to manned and unmanned aircraft, which, for example, achieve fight organization, collision avoidance or tracking. In addition, the ability to identify and locate UAS provides critical information to law enforcement and other officials charged with ensuring public safety. Thus, a rule for Remote Identification of Unmanned Aircraft Systems is proposed by Federal Aviation Administration, which can be summed up as follows [2].

|  | Standard remote identification UAS | Limited remote identification UAS |
| --- | --- | --- |
| Connectivity prior to takeoff |  |  |
| Internet and Remote ID USS | YES | YES. |
| Broadcast | YES | NO. |
| If, at takeoff, the UAS cannot connect to the internet | Broadcast | Do not take off. |
| If, at takeoff, the UAS is connected to the internet, but is not transmitting to a Remote ID USS | Do not take off | Do not take off. |
| In-flight loss of remote identification |  |  |
| If, during flight, the UAS loses the connection to the internet or stops transmitting to the Remote ID USS | Broadcast | Land as soon as practicable. |
| If, during flight, the UAS loses its ability to broadcast the message elements | Land as soon as practicable | N/A. |

We can see that from takeoff to landing, aerial UEs need to perform remote identification by broadcasting when losing appropriate connections. And the connections would be considered available if cellular connectivity is available in an operational area with sufficient signal strength to maintain a connection between the aerial UE and the internet. Under the proposed rule, RAN2 need to consider the specific scenarios that aerial UE need to perform remote identification.

FAA proposed a rule for Remote Identification of Unmanned Aircraft Systems

RAN2 to consider the conditions that aerial UE need to perform remote identification broadcasting.

- Conclusion

Observation 1:	FAA proposed a rule for Remote Identification of Unmanned Aircraft Systems

Proposal 1:	RAN2 to consider the conditions that aerial UE need to perform remote identification broadcasting.

- References

[1] RP-223545, Revised WID on NR support for UAV, Nokia.

[2] Remote Identification of Unmanned Aircraft Systems, Federal Aviation Administration