3GPP TSG-RAN WG2 #121	R2-2301512

Athens, Greece, 27 February – 3 March 2023

Agenda Item:	8.21.3

Source:	Ericsson

Title:	LS reply to SA2 on feasibility of Time Synchronization Method

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#120, the LS from SA2 on 5GS time synchronization status report towards the UE(s) was discussed and an answer from RAN2 was provided in [1] which highlighted this:

If dedicated information is required, from RAN2 perspective, it would be good to avoid all UEs going to connected at the same time.

SA2 took the response into account and has now sent out a new LS [2] asking for RAN2 input on a number of questions. The first question is on the scope:

With respect to informing UEs in RRC_Inactive/Idle about a change of the RAN clock quality, SA2 has concluded the following:

-	The gNB includes in SIB9 a reference report ID as a notification for the UEs reading the SIB9 that there is new clock quality information available. The UE compares the reference report ID with locally stored reference report ID to determine if it had retrieved the last available clock quality information already.

-	The reference report ID consists of the scope of the report ID and an Event ID (an integer). Scope may either identify a group of cells within a single gNB or a group of cells across gNBs. The latter would reduce the amount of signalling even further since then UEs that move to another gNB would not need to retrieve the clock quality details.

SA2 question: SA2 would like to kindly request RAN2 and RAN3 to provide feedback whether both scopes (group of cells per gNB, group of cells across gNBs) can be beneficial and supported.

In the second question SA2 propose a method for Time Synchronization status reporting to UE(s) and ask of the feasibility of such solution:

From SA2 perspective, it is not required that the UE always transitions to RRC_CONNECTED immediately to retrieve the latest available clock quality information. In order to reduce RACH access from many UE(s) (to move back to RRC_CONNECTED state) at the same time, the following option has been considered pending RAN2 feedback:

It can be required that RAN randomizes the UE(s) re-connecting back to the network, i.e., to spread the UEs' connection attempts in the time domain, e.g., over the course of (1) few seconds (2) one minute, (3) several minutes. RAN2 is expected to determine whether and how this can be achieved.

SA2 question: SA2 would like to kindly request RAN2 to provide feedback whether this approach is feasible.

This document tries to provide an answer to the questions posed from SA2.

# 2	Discussion

### 2.1	Feasibility of Time Synchronization method

An answer to the second question is that there is already existing functionality with the aim to spread out UEs doing an access procedure. For Unified Access Control (UAC) in the chapter 5.3.14.5 in RRC [3] the Access barring check procedure is described. UE wanting to access must draw a random number and compare it to some threshold. If successful, then the access attempt is allowed otherwise the access attempt is barred and a timer is started before the next access can be attempted. The configuration of the threshold and timer will determine how many UE accesses that will be allowed at the same time and how long it will take before having a valid access attempt. Similar functionality could be applied for the Time Synchronization method when trying to go from idle to connected state. Depending on the number of UEs in the system and configuration of the parameters in the solution the range of access could vary from seconds to minutes.

The SA2 proposed method is feasible from RAN2 side

- While the proposed method is feasible its usefulness is however questionable from multiple perspectives. First it should be noted that to keep always accurate time synchronization information the UE needs to be in connected state. In idle state there will always be possible misalignment in the synchronization information. Because of clock drift in the UE and incorrect information there will be uncertainties in the synchronization status and unnecessary transitions to connected will occur. Further to go from idle to connected to get the updated information will always come with a delay and cost. This problem will be amplified since it will be complicated for the gNB to set good parameter values for the spread out in time solution. If the UE has a high requirement on accurate time synchronization information but aim to stay in idle state there will inevitably be large number of state switching happening and this comes with a high cost in signaling. Thus, UEs that need precise time synchronization information should stay in connected mode.

With high accuracy requirement the UE should stay in CONNECTED mode

RAN2 should answer to SA2 that the proposed method is feasible by introducing a solution similar to already existing functionality in RAN but the method does not solve the problems introduced by moving UEs with high requirements on time synchronisation to idle state.

### 2.1	Whether scope can be beneficial and supported

- Our understanding of the SA2 proposed solution with the “scope” is that a cell can broadcast an index that is associated with the current time information, i.e. the reference report ID. If the UE moves around in the network and the UE ends up seeing an index for which the UE already has the time info, the UE can directly apply that stored time information rather than having to go to CONNECTED to acquire it.

- Consider the example below. The UE is in Cell A and gets some time information which the network has assigned to index 17. The UE then applies but also stores this information. If the UE later moves to cell C the UE sees that the time information with index 17 applies there too, there is no need for the UE to go to CONNECTED. If however, the UE would move to Cell B it would see that it should apply time information 5. But the UE has so far not received time information 5 and hence the UE would have to go to CONNECTED.

Cell A

Index = 17

Cell C

Index = 17

Cell B

Index = 5

- The question asks about the benefit of using different scopes, examples include group of cells per gNB or group of cells across different gNBs. In our view, gNB has the timing knowledge for its own cells, but not for the cells in other gNBs. The different gNBs could have different synchronization sources, different status in the complete path, also radio conditions could be very different. The Event ID if sent across the gNBs may be less beneficial. If one gNB has synchronization issues then all gNBs in the scope would have to signal the indication, even if they are not experiencing problems. In essence the larger the scope, i.e. the more gNBs to cover with the proposed change indication, the more UEs that will be affected by such indication. The status change indication gets less precise and less directive towards specific affected UEs. If the UE has high requirements on time synchronization information and move to a new gNB with other characteristics it will likely impose a need to get updated timing information even when it is not needed since its gNB may not be experiencing status changes. Thus having a status report indicating a potential change covering status of multiple gNBs in the SIB, relevant for multiple UEs, would mean larger risk of even more UEs going to connect mode. Introducing a larger scope may in the end not result in reducing signaling which was stated as the goal in the LS.

It is proposed to reply to SA2 that RAN2 does not see any benefit to have the scope of the report ID specifying groups of cells within a gNB or across gNBs. The “scope” information would not be needed.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	The SA2 proposed method is feasible from RAN2 side

Observation 1	With high accuracy requirement the UE should stay in CONNECTED mode

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should answer to SA2 that the proposed method is feasible by introducing a solution similar to already existing functionality in RAN but the method does not solve the problems introduced by moving UEs with high requirements on time synchronisation to idle state.

Proposal 2	It is proposed to reply to SA2 that RAN2 does not see any benefit to have the scope of the report ID specifying groups of cells within a gNB or across gNBs. The “scope” information would not be needed.

# 5 References

R2-2213048, “LS response on 5GS time synchronization status report towards the UE(s)”, RAN2#121, Toulouse, 14 – 18 November 2022

S2-2301463, “Proposed method for Time Synchronization status reporting to UE(s)”, SA2#154-AH-e, Elbonia, January 16 – 20, 2023

38.331, “Radio Resource Control (RRC) protocol specification”, V17.3.0, 2022-12