3GPP TSG-RAN WG2 #121	R2-2301881

Athens, 27th February – 3rd March 2023

Source:                    	DENSO CORPORATION

Title:  	Discussion on possible solutions for dual Rx/Tx MUSIM devices

Document for:        	Discussion and decision

Agenda Item:         	8.17.2	Temporary capability restriction for MUSIM

- Introduction

In the WID of Dual Tx/Rx Multi-SIM for NR [1], the following is stated as one of objective.

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE 2.Specify MN-SN coordination of R17 MUSIM gaps when network A is NR-DC in Rel-18 [RAN2]

During the MUSIM discussions for Rel-17, applicable UE architecture was mainly focused to Single-Tx/Rx UE. As mentioned in the above WID, for Rel-18 discussion to support Dual Tx/Rx UE architecture, it would be necessary to introduce the mechanism to indicate preference on temporary UE capability restriction and its removal. Additionally, at the RAN Plenary #98, it was decided to include the discussion for specifying MN-SN coordination of R17 MUSIM gaps when network A is NR-DC in Rel-18, in the WI scope.

At the previous RAN2 meeting (#119-bis), the following agreements were made based on the online discussion and email discussion.

- RAN2 aims to prioritize only few solutions and avoid multiple solutions for the same problem (FFS pending on solution details). - A7: The UE can initiate signaling for UE capability restrictions on NW A if NW A allows it. The specification will not capture NW B events which can cause such need. - A4: RAN2 to discuss whether the following UE capabilities (not a complete list) are impacted for dual-active MUSIM: MIMO layers, BC capabilities, Measurement capabilities, Bandwidth, srs-TxSwitch, UL tx power, Power Class. - For proposals A1-A2, the solution details need more discussion. Other solutions are not precluded (requires company input with details). Will discuss further over email on the solutions (after this meeting) and which capabilities can be affected. - For B1-B3, B5, the solution details need more discussion. May prioritize B1, B2 and B5. FFS on signalling details. Other solutions are not precluded (requires company input with details) and none of B1-B5 are agreed as solutions for this WI. - Do not consider solution B4 in Rel-18 (since it may have CN impacts which are precluded in this WI)

This paper attempts to investigate about the scenario and the solution for Dual Tx/Rx Multi-SIM based on the above agreements.

- Discussion

  - Support for SCG release and/or deactivation

In the email discussion summary [2] during RAN2#119-bis, it was proposed to support SCG release and/or de-activation for dual-active MUSIM operation as quoted below:

- Proposal A1: As a baseline, the release and removal of release of SCells/SCG is supported for dual-active MUSIM operation. The signaling details (e.g. whether UE request is explicit or implicit) is FFS. - Proposal A2: As a baseline, the (de)-activation of SCells/SCG is supported for dual-active MUSIM operation. The signaling details (e.g. whether UE request is explicit or implicit) is FFS.

As some companies mentioned during the email discussion, supporting either A1 or A2 could accomplish the temporary UE capability restriction for dual-active MUSIM operation. Additionally, whether UE requests SCG release or deactivation explicitly or implicitly would be one of the discussion point. For this matter, we prefer not to specify whether release or deactivation is used in the specification at all. In other words, UE can just send preferred capability restriction information and the network can decide whether release or deactivation is used based on the UE’s capability restriction information. Therefore, the following is proposed:

Proposal 1: UE can just send preferred capability restriction information, and whether release or deactivation is used could be up to network implementation.

  - Candidate solution for sending preference on UE capability restriction

The following options were proposed for the method to send the preference of the temporary UE capability restriction at the last meeting RAN2#119e-bis.

Option B1: 	UAI based solution

Option B2: 	delta-signaling of UE capability

Option B3: 	repetition of UE capability enquiry

Option B5:	MAC-CE based SCell (de)-activation

Our understanding is that Option B2 is one of extension for B1 or B3. Therefore, we would like to focus to discuss about B1, B3, and B5 here. Refer to the following table to compare the options.

Table 1. Pros. and Cons. for each option

|  | Pros. | Cons. |
| --- | --- | --- |
| Option B1 | - Legacy and new specification could be separated explicitly - Further MUSIM specific enhancements could be possible | - Modification on message definition may be needed - Redundant message definition, conflict of configuration could occur with overheating assistance |
| Option B3 | - No modification on message definition is needed for capability information itself (however, some other signalling may be needed to initiate this procedure from UE side) | - Legacy implementation might not expect UE capability information to be dynamically updated. Therefore, the specification change would cause significant impact in the current implementation and behaviour - The message structure would be inefficient |
| Option B5 | - Latency could be reduced compared to RRC signalling | - Finally RRC signalling would be needed when there is configuration change in RRC layer |

With the above table, it could be observed that Option B1 would be most beneficial totally (even though Option 1 also have some disadvantages, they are not so significant problem). The other options might have bigger disadvantage than its advantage.

Proposal 2: RAN2 to introduce new IE in UAI to indicate UE’s preference on temporary UE capability restriction. FFS detailed message definition and capability information.

  - MN-SN coordination of R17 MUSIM gaps when network A is NR-DC

As mentioned in the introduction of this paper, it was decided to have discussion for specifying MN-SN coordination of R17 MUSIM gaps when network A is NR-DC. According to the discussion during RAN2#119e-bis and related contributions[3], we understand that one of the purpose on this MN-SN coordination is to configure CG specific MUSIM gap (not per-UE like Rel-17) to avoid performing unnecessary gap either in SN or MN. The typical use case for the CG specific gap would be configuring the MUSIM gap on a RF chain which is to be used for the communication with the network B. If that is the case, the network A is not able to judge which RF chain (associated with either of a CG) will be used for the communication with the network B without UE indication. Therefore, the following can be observed and proposed:

Observation 1: To configure CG specific MUSIM gap, UE may need to indicate its preference on which CG to configure the MUSIM gap to the network A, since the network A is not able to know which RF chain will be used for the communication with the network B.

Proposal 3: RAN2 to introduce UE indication for its preference on which CG to configure the MUSIM gap.

With the above preference information, MN could judge which CG to configure the MUSIM gap, then, MN would need to inform the gap information to SN using inter-node RRC message such as CGConfigInfo. We suppose one of the option would be just forwarding the gap preference information from MN to SN, and SN decides if the preferred gap can be configured. However, we prefer another option that MN decides and configures the MUSIM gap not only for MN but also for SN side, and send the gap information as a decided configuration to SN, as we think this is simpler way.

Proposal 4: Allow MN to configure the MUSIM gap for SN side, and send the gap information as a decided configuration to SN.

- Summary and proposal

This paper discussed about the possible solutions for Dual Tx/Rx Milti-SIM devices for NR. In summary, the followings were observed and proposed:

Proposal 1: UE can just send preferred capability restriction information, and whether release or deactivation is used could be up to network implementation.

Proposal 2: RAN2 to introduce new IE in UAI to indicate UE’s preference on temporary UE capability restriction. FFS detailed message definition and capability information.

Observation 1: To configure CG specific MUSIM gap, UE may need to indicate its preference on which CG to configure the MUSIM gap to the network A, since the network A is not able to know which RF chain will be used for the communication with the network B.

Proposal 3: RAN2 to introduce UE indication for its preference on which CG to configure the MUSIM gap.

Proposal 4: Allow MN to configure the MUSIM gap for SN side, and send the gap information as a decided configuration to SN.

- References

[1] RP-223492, “Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR”

[2] R2-2210823, “[AT119bis-e][211][MUSIM] MUSIM solutions for Rel-18 (QC)”

[3] R2-2210738, “Discussion on MN-SN MUSIM gaps coordination in INM”