3GPP TSG-RAN WG2 Meeting #121bis-e                               R2-2304398

Online, April 17-26, 2023

Agenda item:		7.17.4

Source:				Samsung

Title:	Report of [AT121bis-e][231][MUSIM] RAN4 aspects of MUSIM (Samsung)

Document for:		Discussion

# 1	Introduction

This document is the report of the following offline discussion:

[AT121bis-e][231][MUSIM] RAN4 aspects of MUSIM (Samsung)

Scope: Discuss what to do in RAN2 for MUSIM gap priorities (based on RAN4 LS): Can UE indicate gap priority preference? Is the gap priority applicable to aperiodic gaps? What is the network behaviour (i.e. accept/reject/change priority)? Are there any RAN4 impacts on maximum UL power change?

Intended outcome: Discussion report in R2-2304398

Deadline:  Deadline 2 (Friday W1, 0900 UTC)

# 2	Contact information

| Company | Contact person (email address) |
| --- | --- |
| Samsung | Sangyeob Jung (sy0123.jung@samsung.com) |
| vivo | Boubacar, kimba@vivo.com |
| Huawei/HiSilicon | Rama Kumar Mopidevi (rama.kumar@huawei.com) |
| Intel Corporation | Seau Sian Lim (seau.s.lim@intel.com) |
| Nokia | Srinivasan Selvaganapathy(srinivasan.selvaganapathy@nokia.com) |
| Apple | Sethuraman Gurumoorthy (sethu@apple.com) |
| ZTE | Wenting Li (Li.Wenting@zte.com.cn) |
| OPPO | Jiangsheng Fan(fanjiangsheng@oppo.com) |
| Qualcomm | Ozcan Ozturk (oozturk@qti.qualcomm.com) |
| MediaTek | Felix Tsai (chun-fan.tsai@mediatek.com) |
| DENSO | Tomoyuki Yamamoto (tomoyuki.yamamoto.j5c@jp.denso.com) |
| Ericsson | Håka Palm (hakan.l.palm@ericsson.com) |

# 3	Discussion

### 3.1	MUSIM gap priorities

In [1, 4, 5, 10], it mentions that there is a need to introduce new UE capability to indicate whether UE supports providing MUSIM gap priority preference and its related configuration.

Q1: Do you agree to introduce a per-UE capability bit to indicate support of MUSIM gap priority configuration and preference?

| Company | Agree/disagree | Comments (if any) |
| --- | --- | --- |
| vivo | Agree | It’s natural to introduce a per-UE capability bit to indicate support of MUSIM gap priority configuration and preference. |
| Huawei/HiSilicon | Agree |  |
| Intel | Agree | We can even include that there is no need for xDD and FRx differentiation. |
| Nokia | Agree |  |
| Apple | Agree | We think that a per UE capability is a natural requirement for this gap priority feature |
| ZTE | Agree | In detail, we think it should take the musim-GapPreference-r17 as prerequisite |
| OPPO | Agree |  |
| Qualcomm | Agree |  |
| MediaTek | Agree |  |
| DENSO | Agree |  |
| Ericsson | Agree |  |

Summary:

It is mentioned in [4, 9, 10] that MUSIM gap priority preference can be reported if UE is configured to do so i.e. explicit network configuration in the OtherConfig.

Q2: Do you agree to introduce a new indication in the OtherConfig to indicate whether UE is allowed to report MUSIM gap priority preference via UAI?

| Company | Agree/disagree | Comments (if any) |
| --- | --- | --- |
| vivo | Agree | Similar to the legacy UAI mechanism. |
| Huawei/HiSilicon | Agree |  |
| Intel | Agree |  |
| Nokia | Agree |  |
| Apple | Agree |  |
| ZTE | Agree |  |
| OPPO | Agree |  |
| Qualcomm | Agree |  |
| MediaTek | Agree |  |
| DENSO | Agree |  |
| Ericsson | Agree | Yes, this aligns with existing pinciple |

Summary:

According to RAN4’s agreement [13], UE can also provide an assistance information for the periodic MUSIM gap priority selection by indicating its preferred priority for all or a subset MUSIM gaps. The rapporteur thinks that the following feasible options need to be discussed as proposed in [6, 9, 12]:

- Option 1: UE indicates an absolute priority for all or a subset periodic MUSIM gaps by taking into account of the Type-2 MG gap priority

- Option 2: UE indicates a relative priority for all or a subset periodic MUSIM gaps, i.e. the priority is relative just among the MUSIM gaps

Note that it is mentioned in [6] that RAN2 may wait for RAN4 feedback for which option to be supported.

Q3: Which of the following options do you prefer for indicating periodic MUSIM gap priority preference?

- Option 1: UE indicates an absolute priority for all or a subset periodic MUSIM gaps by taking into account of the Type-2 MG gap priority

- Option 2: UE indicates a relative priority for all or a subset periodic MUSIM gaps, i.e. the priority is relative just among the MUSIM gaps

- Option 3: wait RAN4 progress/feedback

| Company | Preferred Option(s) | Comments (if any) |
| --- | --- | --- |
| vivo | Option 2 | Option 2 relative priority is preferred. Firstly, it’s sufficient for the UE to report the relative priority values among the periodic MUSIM gaps and it’s up to the network how to configure the relationship of MUSIM gap priorities and MG gap priorities. The value range of relative priority is smaller than the absolute ones, which helps minimize the ASN.1 size. Secondly, in option 1, if the highest priority has been used for configured measurement gap, e.g. priority 0-3. How does UE report one absolute gap priority for paging usage in value range [0-3]? The priority preference is not clear if priority preference value equals to configured gaps. We are fine with option-1 as well if majority prefers it. |
| Huawei/HiSilicon | Option 2 | Currently, the Type-2 MG gap priority is decided by NW without UE providing any preference, we can follow this principle and the UE does not need to consider the priority between Type-2 MG and MUSIM gap. Furthermore, when the NW reconfigures the Type-2 MG priority for the UE, the UE does not need to re-initiate the UAI procedure to update the MUSIM gap priority. |
| Intel | Option 1 or 3 | Based on the following in the LS: - The priority level of MUSIM gap(s) shall be configured to be comparable to priority level of NW A’s Type-2 MGs - MUSIM gap and Type-2 MG cannot be configured with the same priority Our understanding is leaning more to Option 1. However, we are fine to check with RAN4 or wait for further progress in RAN4. Particularly, it is unclear to us why it is optional for the UE to not indicate its preferred priority for a MUSIM gap in the prefereince indication. Does it mean that it is lowest priority (or highest priority)? |
| Nokia | Option 1 is preferred. | RAN4 LS indicates the NW assigned priority should be unique across MUSIM gaps and Type-2 gaps. If the UE intend to have better priority over Type-2 gaps for some of its gaps, it is good to indicate the same for NW to consider in its assignment. Otherwise NW may allocate lower priority than Type-2 which is not preferred for UE. |
| Apple | Option 1 or 3 | Option 1 in our view results in unambiguous priority setting. At the same time, we are fine to wait for RAN4 input if any on this. |
| ZTE | Option 1 or 3 | The absolute value can also indicate the relative priority, so seems that the option 1 implies more information, then it can be left to the network to determine to comply with the absolute priority or just comply with the relative priority |
| OPPO | Option 1 or Option3 | Option 1 is simpler and straightforward. |
| Qualcomm | Option 1 | This gives a lot more information to the NW compared to Option 2 since it indicates the relative priority among MUSIM gaps as well as the actual priority UE requests compared to other non-MUSIM gaps. |
| MediaTek | Option 2 | It is a little unclear to us on option 1. Does option 1 imply that the UE also suggest gap priority for Type-2 gap? If there is no Type-2 gap configured, does this imply that option 1 and 2 are the same? What happen to option 1 if Type-2 gap is configured after the UE providing the MUSIM gap preference? It is also unclear to us how UE determine the priority between MUSIM gap and Type-2 gap. |
| DENSO | Option 1 | Option 1 seems to be simpler and straightforward. |
| Ericsson | Option 1 | Problem for Nw is that Nw does not know what UE is doing within the Musim gap. Nw can only make a guess based on the UE-indicated preferences. |

Summary:

Regardless of the outcome of Q3, the rapporteur understands that most companies propose that the existing IE GapPriority-r17 can be re-used to configure the priority for periodic MUSIM gap, regardless of the outcome of Q3.

Q4: Do you agree that the existing IE GapPriority-r17 is re-used to configure the priority for periodic MUSIM gap?

| Company | Agree/disagree | Comments (if any) |
| --- | --- | --- |
| vivo | Agree | The existing IE GapPriority-r17 can be re-used by the network to configure the priority for periodic MUSIM gap. However, the value range of gap priority preference relies on the outcome of Q3. |
| Huawei/HiSilicon | Agree |  |
| Intel | Agree |  |
| Nokia | Agree |  |
| Apple | Agree |  |
| ZTE | Agree |  |
| OPPO | Agree |  |
| Qualcomm | Agree |  |
| MediaTek | Agree |  |
| DENSO | Agree |  |
| Ericsson | Agree |  |

Summary:

In Rel-17, RAN2 has agreed that network should always provide at least one of the requested gap pattern or no gaps i.e. network is NOT allowed to provide an alternative gap pattern instead of the one requested by the UE. Thus, it is proposed in [3] that network should accept the MUSIM gap priorities requested by the UE. The rapporteur understands that the intent is for network to assign the priority which is equal to the absolute value provided by the UE (if Option 1 is agreed in Q3) or is aligned with the relative value provided by the UE (if Option 2 is agreed in Q3) [9].

Q5: When network accepts gap priority preference for a periodic MUSIM gap, do you agree that network configures the priority which is equal to the absolute value provided by the UE (if Option 1 is agreed in Q3) or is aligned with the relative value provided by the UE (if Option 2 is agreed in Q3)?

| Company | Agree/disagree | Comments (if any) |
| --- | --- | --- |
| vivo | Agree (aligned with the relative value provided by the UE) | Considering network may configure new measurement gaps, it’s impossible for network to always accept the absolute priority value provided by the UE. No matter Option-1 or Option-2 is agreed in Q3, we think network configures the MUSIM priority which is aligned with the relative priority relationship among MUSIM gaps provided by the UE. |
| Huawei/HiSilicon | Agree (Aligned with the relative value provided by the UE) | Based on existing mechanism, for UE suggested MUSIM gaps, if the gNB decides to configure the MUSIM gaps, the gap patterns configured should be the same as the UE’s preference, to match the activities in SIM B. The same motivation applies for UE suggested priority for MUSIM gaps, since only the UE knows the intention (e.g. for paging reception, or RRM measurements, or SI reception) for different MUSIM gaps. Besides, since only relative priority for MUSIM gap should be aligned, the NW still has flexibility for configure priority between Type-2 MG and MUSIM gap. |
| Intel | Agree (Option 1 in Q3) |  |
| Nokia | Agree (with Option 1 in Q3) |  |
| Apple | Agree (with Option 1 in Q3) |  |
| ZTE | See comments | Same as Q3: The absolute value can also indicate the relative priority, so seems that the option 1 implies more information, then it can be left to the network to determine to comply with the absolute priority or just comply with the relative priority. |
| OPPO | Agree (Option 1 in Q3) |  |
| Qualcomm | See comment | For Option 1, the NW can still change the absolute priorities while keeping the relative priorities among MUSIM gaps. For Option 2, agree that the NW should keep the relative priorities. |
| MediaTek | Agree (Aligned with the relative value provided by the UE) |  |
| DENSO | Agree (with Option 1 in Q3) |  |
| Ericsson | See comments | Priority setting should be left to Nw impl. We expect no restriction on nw impl as imposed by this q-n will be specified. This should be the conclusion. |

Summary:

If network can't accept MUSIM gap priority preference, simplest options may be not to assign any priority for a requested periodic MUSIM gap or not to configure a periodic MUSIM gap at all. But further question is raised in [3] whether a fallback option could be for network to follow the relative priorities among periodic MUSIM gaps i.e. assign different priorities for periodic MUSIM gaps while still following the relative ordering between them. The rapporteur understands that this fallback option is valid if the outcome of Q3 is Option 1.

Q6: When network can't accept MUSIM gap priority preference for a periodic MUSIM gap, which of the following options do you support for network behavior?

- Option A: does not configure a periodic MUSIM gap at all

- Option B: does not assign any priority while configuring a periodic MUSIM gap

- Option C: use fallback option as in [3]

- Others

| Company | Preferred Option(s) | Comments (if any) |
| --- | --- | --- |
| vivo | Option C | RAN4 LS has concluded that “It is up to NW A on how to use this (preference) information.” If NW A cannot accept the priority level indicated by a UE, network configures the MUSIM priority which is aligned with the relative priority relationship among MUSIM gaps provided by the UE. |
| Huawei/HiSilicon | See comment | As commented in Q3, we prefer the UE to indicate a preference for a relative priority for all or a subset periodic MUSIM gaps. In this case, we cannot imagine for what reasons the NW cannot accept such relative priority preference of the UE. |
| Intel | Others | It is unclear to us why network cannot provide a priority when the UE has requested a priority preference. |
| Nokia | See comment | As the NW may decide on relative priority across Type-2 Gaps and MUSIM gaps, we should allow network to change the priority. This is needed at least for the priority across MUSIM and Type-2 priority. Within MUSIM gap priority the NW should attempt to keep the same order. Also RAN2 need to discuss the UE behaviour if priority is not assigned or not accepted to all the requested priority. In such cases the Type 2 gaps of NW-A should be given higher priority by default. The priority handling among MUSIM gaps can be left to UE implementation |
| Apple | See comment | Our understanding is that the NW if it is not able to provide the UE requested priority, should atleast provide a default priority. Otherwise, the UE would be force to retrigger this signalling request again, which should be avoided. |
| ZTE | Option C | We agree with Rapporteur that this fallback option is valid if the outcome of Q3 is Option 1 |
| OPPO | See comment | I don’t understand the issue very clear, even if network can't accept MUSIM gap priority preference for a periodic MUSIM gap, it’s still up to NW implementation to configure the priority for each MUSIM gap, a good NW implementation will consider all the available info on the table including MUSIM preference info, so no need to clarify something in the spec, just leave this to NW implementation. Only one thing that matters is to define the UE behavior when a priority is absent for a specific gap when configured in DL, i.e. to answer Q8. |
| Qualcomm | See comment | Agree with HW that the NW should keep the relative priority, irrespective of Option 1 or 2 is agreed. If NW can not do this, it will not schedule MUSIM gaps (similar to Rel-17 MUSIM gap request). |
| MediaTek | See comment | It could be just up to NW implementation with the understanding that sensible NW should follow the priority suggestion. |
| DENSO | Option C | Agree with rapporteur. NW might not use the exact value provided by the UE as this should be up to network implementation. If Option 1 in Q3 is introduced, this behavior could be called “fallback option”. |
| Ericsson | See comments | Priority setting (and even configuration of MUSIM gaps) is for Nw impl to decide. Of course Nw should take UE prefence into account. Seems simplest Nw always provide a priority. |

Summary:

According to RAN4 LS [13], it is stated that each periodic MUSIM gap can be assigned with a different priority. But two companies in [2, 10] would like to discuss whether network can configure the same priority to the periodic MUSIM gaps.

Q7: Do you agree that network can configure the same priority to more than one periodic MUSIM gap?

| Company | Agree/disagree | Comments (if any) |
| --- | --- | --- |
| vivo | See comment | Whether different periodic MUSIM gaps can have same priority or not, it is under discussion in RAN4 for a few meetings already, we suggest to wait for RAN4 progress. |
| Huawei/HiSilicon | See comment | Wait RAN4 progress In our view, the issue of same priority is not only about NW configured priority, but also UE reported priority. As we mentioned above, we think the NW configured priority should be aligned with UE reported relative priority, if UE can report same priority, then the NW can configure same priority. Whether UE can report same priority is being discussed in RAN4 so we can wait for RAN4 progress. |
| Intel | Check with RAN4 | We can check with RAN4 whether collision handling needs to be performed among the MUSIM gaps. |
| Nokia | No | Same priority across the gaps is not allowed as per RAN4 LS. |
| Apple | See comment | Based on our contribution in [8] we feel RAN4 should clarify this part. Currently it is not clear if different periodic gaps can have the same priority. |
| ZTE | Wait for RAN4’s progress | We think it depends on how to solve the collision, some Gap collision solution methods (e.g. gap merging) are under RAN4 discussion, so we’d like to wait for RAN4’s progress. |
| OPPO | Check with RAN4 |  |
| Qualcomm | Yes | But also fine to wait for RAN4 on collision handling. |
| MediaTek | No | According to current RAN4 LS, we think same priority is not allowed. But could change depending on RAN4 further conclusions. |
| DENSO | Check with RAN4 |  |
| Ericsson | Disagree | Seems simplest Nw configures different priorities. We expect UE ensures the MUSIM gap collitions are avoided. |

Summary:

According to RAN4 agreement [13], it is optional for UE to indicate its preferred priority for a periodic MUSIM and it is up to network to decide how to use such information. Thus, it seems quite natural that network can configure a periodic MUSIM gap without any assigned priority, which is related with the following highlighted agreement is made:

RAN2 will aim to address the RAN4 LS in Rel-18 signalling. Should discuss how to handle Rel-17 gaps without priority (e.g. lowest, highest, network-decided somehow, etc.). Handled in email [231]

Note that the focus here is to handle periodic MUSIM gaps without priority i.e. the applicability of priority for aperiodic MUSIM gap will be discussed later on.

In [1], it is suggested that a default priority level should be used for periodic MUSIM gaps which do not have an assigned priority. In [5], it is proposed that absence of configured priority indicates the lowest priority for the gap when there is conflict with other NW-A Type-2 gaps or other MUSIM gaps.

Q8: How UE supporting to indicate periodic MUSIM gap priority preference handles a configured periodic MUSIM gap without priority?

- Option 1: UE uses a default priority value

- Option 2: UE considers a configured periodic MUSIM gap without priority to be the lowest priority gap i.e. lower than any of the network configured priority values

- Others

| Company | Preferred Option(s) | Comments (if any) |
| --- | --- | --- |
| vivo | others | Based on RAN4 discussion, we may assume that, Network A assigns priority levels to all configured periodic MUSIM gaps even if UE does not indicate preferred priority for one or some periodic MUSIM gaps. Then, the case of this question is impossible. |
| Huawei/HiSilicon | See comment | Wait RAN4 progress. RAN4 is discussing “Solution for collision between MUSIM gap and Type-1 MG or gap configured without priority” hence we can wait for RAN4 progress. |
| Intel | Others | It is unclear to us why it is optional for the UE to not indicate its preferred priority for a MUSIM gap in the preference indication. Does it mean that it is lowest priority (or highest priority)? If we can decide on that, this can also be addressed? |
| Nokia | O1 or O2 | By default MUSIM gaps should have lowest priority compared to Type-2 gaps. So if NW does not assign priority to the MUSIM gap it should be considered as lowest priority than the Type-2 Gap priority values. |
| Apple | See Comment | As stated earlier, if UE supports priority for periodic gap, we exepct the NW to assign atleat a default priority level. We are not clear on what circumstances, NW will NOT assign any priority ? |
| ZTE | O2 is simpler | We think O2 is simpler at least from RAN2 aspect. |
| OPPO | Op2 | Op2 is simpler |
| Qualcomm | Other | Agree with Vivo and others that the NW should assign a priority for UEs which support requesting priority. |
| MediaTek | Other | We prefer to wait RAN4. If default priority (normal highest or lowest or whatever value) is needed, they can tell us. Otherwise, it could be just up to UE implementation. |
| DENSO | Other | Agree with vivo. |
| Ericsson | Others | Seems simplest nw always configures the priority. Should discuss the case when supporting UE (Rel-18 MUSIM UE) interacts with Nw that does not support MUSIM gap priority setting. For this case a default (specified) behaviour is needed (and should be captured in RAN4 spec). “Lower priority than the Type-2 Gap priority values” as discussed by Nokia is probably fine. |

Summary:

It is indicated in [13] that RAN4 is still discussing whether priority for aperiodic MUSIM gap needs to be introduced. In [5], it is proposed that RAN2 can indicate its preference to assign explicit priority for aperiodic gaps in RAN4 LS Response, considering that potential overlap between periodic MUSIM gaps and aperiodic MUSIM gap as well as between aperiodic MUSIM gap and NW-A Type-2 MG may occur. On the contrary, one company [8] mention that aperiodic MUSIM gap can be the highest priority implicitly.

Q9: Which of the following options do you prefer for handling of aperiodic MUSIM gap priority preference/configuration?

- Option 1: wait RAN4 progress

- Option 2: assign explicit priority for aperiodic MUSIM gap

- Option 3: aperiodic MUSIM gap is the highest priority gap implicitly

- Others

| Company | Preferred Option(s) | Comments (if any) |
| --- | --- | --- |
| vivo | Option 1 | We’d better wait RAN4 progress since they are discussing this. |
| Huawei/HiSilicon | Option 1 |  |
| Intel | Option 1 | As mentioned in the RAN4 LS below: RAN4 is still discussing whether priority for aperiodic MUSIM gap needs to be introduced. |
| Nokia | Option 1 with comments | To have UE to have some preference for aperiodic gaps, we prefer to have priority for aperiodic gap. We can indicate the same in RAN2-LS indicating the cases where it will be beneficial. |
| Apple | Option 3 (See comments) | The reason we indicated aperiodic gaps to have the highest priority is because they are one time gaps. If for some reason, this gap gets scheduled out, UE has to re-request this gap again. This is not the case with periodic gaps, as they repeat in time domain. Our preference is to treat them at highest priority. At the same time, we understand this is for RAN4 to discuss and confirm. So in that sense option 1 for now is fine. |
| ZTE | Option 1 | This issue is also under RAN4 discussion, so we’d like to wait for RAN4’s RSP. |
| OPPO | Option1 |  |
| Qualcomm | Option 1 or 3 | It is okay to wait for RAN4 but RAN2 can also indicate a preference. Since aperiodic gap is a one time event that the UE will use for an important event, it is more reasonable to assign the highest priority. |
| MediaTek | Option 1 |  |
| DENSO | Option 1 |  |
| Ericsson | Option 2 | We agree with commets on that the aperiodic gap should likely have high priority, e.g. when the aperiodic gap is used for SIB reading from NW B. But also a gap for measurements could have high prio to trigger handover in Nw A. With the MUSIM gap preferences, full freedom is left to UE impl. It is a safe and simple approach for Nw to set the priority also for aperiodic gap, same as for periodic MUSIM gaps. |

Summary:

### 3.2	RAN4 impacts on Maximum UL power change

In RAN2#121 meeting, The following agreement on maximum UL power change has made:

2: RAN2 considers that there may be RAN4 impact on the maximum UL power change due to R18 MUSIM. However, RAN2 needs to analyze the power issue more before asking RAN4 specifically.

Two companies suggest to study/analyze maximum UL power change due to R18 MUSIM operation from RAN2 perspective first. In [11], it is suggested to study when to trigger PHR given that UE reports PHR to NW A due to events occurred in NW B. In [5], it is proposed to support NW control on the uplink-power sharing for MUSIM operation (e.g. static and dynamic sharing mode) should be supported, which may require RAN4 analysis based on RAN2 conclusion.

On the other hand, three companies express that no RAN4 impact is expected from RAN2 point of view. In [2], it is mentioned that RAN2 does not need to study PHR triggering without any RAN4 input since how to calculate maximum UL power is defined in RAN4. In [9], similar view is stated and UE implementation can also handle the concerned scenario. In [5], the band conflict solution discussed in RAN2 can avoid any potential maximum UL power change issues without RAN4 involvement.

Based on companies’ views above, it is still not clear yet what exact RAN4 impacts (if any) are expected from RAN2 point of view. Thus, the rapporteur would like to ask the following question:

Q10: Do you agree that “RAN2 assumes no RAN4 impact is expected on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2”?

| Company | Agree/disagree | Comments (if any) |
| --- | --- | --- |
| vivo | agree | RAN2 need not request RAN4 handling on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2. |
| Huawei/HiSilicon | Agree |  |
| Intel | Agree |  |
| Nokia | See comments | How the concurrent transmission in NW-A and NW-B can be handled using PHR is not clear. There may be some delay in indicating PHR to NW that will have impact on transmission in other NW. Moreover how the power sharing works for other channels such as PUCCH and RACH in Dual active MUSIM operation also require some analysis. PHR cannot resolve this problem |
| Apple | Agree |  |
| ZTE | Agree |  |
| OPPO | Disagree | Currently, PHR is calculated for the activated Serving Cell, which is not suitable for R18 MUSIM case when USIM A is in RRC_CONNECTED mode while USIM B is willing to enter RRC_CONNECTED mode as USIM B is still in idle or inactive, so USIM A will not consider USIM B situation when reporting PHR, i.e. there is no room to consider the USIM B power requirements in idle or inactive according to the MAC spec, in this sense, legacy PHR procedure still does not solve the power sharing issue between USIMs. ### 5.4.6 Power Headroom Reporting The Power Headroom reporting procedure is used to provide the serving gNB with the following information: - Type 1 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell; - Type 2 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH and PUCCH transmission on SpCell of the other MAC entity (i.e. E-UTRA MAC entity in EN-DC, NE-DC, and NGEN-DC cases); - Type 3 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for SRS transmission per activated Serving Cell; - MPE P-MPR: the power backoff to meet the MPE FR2 requirements for a Serving Cell operating on FR2. |
| Qualcomm | Agree |  |
| MediaTek | Agree |  |
| DENSO | Agree |  |
| Ericsson | Agree |  |

Summary:

### 3.3	Others

For any critical other stage-2 issues not covered above, please feel free to indicate them into the following table.

| Company | Discussion points | Comments |
| --- | --- | --- |

# 4	Conclusion

TBD:

# 5	Reference

[1] R2-2303641		MUSIM gap priorities	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

[2] R2-2303828		Discussion on MUSIM gap priorities and maximum UL power change	Samsung

Electronics Austria	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

[3] R2-2302724		Remaining issues for MUSIM gaps	Qualcomm Incorporated	discussion

[4] R2-2302783		Gap collision handling for Rel-17 gaps	Intel Corporation	discussion	Rel-18					NR_DualTxRx_MUSIM-Core

[5] R2-2303190		On MUSIM gap priority and uplink power sharing aspects of MUSIM operation	Nokia,

Nokia Shanghai Bell	discussion

[6] R2-2303269		Discussion on MUSIM gap priorities	vivo	discussion	Rel-18

[7] R2-2303352		Discussion on MUSIM gap priorities	Xiaomi	discussion	Rel-18

NR_DualTxRx_MUSIM-Core

[8] R2-2303411		Views on RAN4 LS for MUSIM gap priorities	Apple	discussion	Rel-17

LTE_NR_MUSIM-Core

[9] R2-2303471		Discussion on MUSIM gaps and other RAN4 topics	Huawei, HiSilicon	discussion	Rel-

18	NR_DualTxRx_MUSIM-Core

[10] R2-2303875	Consideration on the Scheduling Gap Priority	ZTE Corporation, Sanechips	discussion

Rel-18	NR_DualTxRx_MUSIM-Core

[11] R2-2303937	Discussion on maximum UL power change for Dual Tx/Rx Multi-SIM	ASUSTeK				discussion	Rel-18	NR_DualTxRx_MUSIM-Core

[12] R2-2304028	MUSIM Gap Priority	LG Electronics	discussion	Rel-18	NR_DualTxRx_MUSIM-

Core

[13] R2-2302430	LS on priority for MUSIM gaps (R4-2303249; contact: vivo)	RAN4	LS in	Rel-18				NR_DualTxRx_MUSIM-Core	To:RAN2