3GPP TSG-RAN WG2 #121-bis 	R2-2302951

E-meeting, 17th April – 26 th April 2023

Agenda Item:	7.14.4

Source:	NEC

Title:	Discussion on SRB5 configuration and procedure

Document for:	Discussion, Decision

Introduction

RAN2#121 meeting discussed support of QoE measurements for NR-DC and achieved following agreements:

- RRC configuration determines to which node UE sends the QoE report. It is possible to change the reporting leg via RRC signalling after it has been configured. - Split SRB for QoE reporting is not supported (unless serious problems are identified). - Define new SRB (“SRB5”) for the QoE reporting to SN. SRB4 can only be configured for MCG (as in Rel-17). The priority of “SRB5” is lower than SRB1 or SRB3. - If both MN and SN send the QoE configurations to the UE, MN and SN should not use the same set of identities. - RAN2 thinks it’s possible to have different m-based QoE configurations for UE in MN and SN if RAN3 allows it.

This contribution provides our views on SRB5 configuration and procedure.

Discussion

SRB5 is a direct SRB between the SN and the UE, defined for transmitting QoE report from the UE to the SN. When discussing SRB5 configuration and procedures, the existing legacy SRB3, which is a direct SRB between the SN and the UE as in EN-DC, can be taken as a reference.

Based on SRB3 configuration and procedures defined in [2] clause 7.5, we think it is nature to specify the followings for SRB5

- the decision to establish SRB5 is taken by the SN, which provides the SRB5 configuration using an SN RRC message.

- SRB5 establishment and release can be done at Secondary Node Addition and Secondary Node Change.

- SRB5 reconfiguration can be done at Secondary Node Modification procedure.

RAN2 to confirm the baseline of SRB5 configuration as followings:

- the decision to establish SRB5 is taken by the SN, which provides the SRB5 configuration using an SN RRC message.

- SRB5 establishment and release can be done at Secondary Node Addition and Secondary Node Change.

- SRB5 reconfiguration can be done at Secondary Node Modification procedure.

Then go to details, RadioBearerConfig IE can be enhanced to add, modify and release SRB5 following the way of SRB4 (as shown below)

RadioBearerConfig IE can be enhanced to add, modify and release SRB5 following the way of SRB4.

When a UE in MR-DC is configured with both SRB4 and SRB5, then SRB4 is used for QoE report to MN for QoE configurations configured by MN whereas SRB5 is used for QoE report to SN for QoE configurations configured by SN. It is obviously that the QoE report leg can be indicated by whether the corresponding SRB is added/released or not, i.e., if SRB4 is released, then a UE in MR-DC transmits its QoE report to SN via SRB5, and if SRB5 is released, then a UE in MR-DC transmits its QoE report to MN via SRB4.

QoE report leg can be indicated by whether the corresponding SRB is added/released or not.

Explicit indication of QoE report leg change is not supported.

Summary

This contribution provides our views on SRB5 configuration and procedure.

Observation 1 RadioBearerConfig IE can be enhanced to add, modify and release SRB5 following the way of SRB4.

Observation 2 QoE report leg can be indicated by whether the corresponding SRB is added/released or not.

Proposal 1 RAN2 to confirm the baseline of SRB5 configuration as followings:

- the decision to establish SRB5 is taken by the SN, which provides the SRB5 configuration using an SN RRC message.

- SRB5 establishment and release can be done at Secondary Node Addition and Secondary Node Change.

- SRB5 reconfiguration can be done at Secondary Node Modification procedure.

Proposal 2 Explicit indication of QoE report leg change is not supported.

- References

[1] R2-2301902, Report on LTE legacy, XR, QoE and MUSIM

[2] TS37.340 V17.3.0