3GPP TSG-RAN2 Meeting #121-bis-e	R2-2302985

Online 17th – 26th April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3974 | rev | - | Current version: | 16.12.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SI update for posSIB-r16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  |  |  | Date: |  |  |  | 2023-04-07 |  |
| Category: | F |  |  |  |  |  |  |  |  | Release: |  |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |
| Reason for change: | According to TS 38.331 clause 5.2.2.2, as captured, the system information change indication is not applicable to SI messages containing posSIBs: A modification period is used, i.e. updated SI message (other than SI message for ETWS, CMAS and positioning assistance data) is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m = 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. The UE receives indications about SI modifications and/or PWS notifications using Short Message transmitted with P-RNTI over DCI (see clause 6.5). Repetitions of SI change indication may occur within preceding modification period. SI change indication is not applicable for SI messages containing posSIBs.. However, in Table 6.5-1 of clause 6.5, the indication of a BCCH modification is only not applicable to SI messages for ETWS and CMAS, i.e. SIB6~8. It can be interpretated as applicable for posSIB |
| Summary of change: | Add in the description for short message that the indication of a BCCH modification is not applicable to SI messages containing posSIBs. Impact analysis Impacted functionality: BCCH modification Inter-operability: If the UE is implemented according to the CR while the network is not, the network may send a SI change indication for posSIB updates, but the UE would have different understaning when receiving the indication. If the network is implemented according to the CR while the UE is not, the network would not send the indication when the posSIB changes, but the UE would always attempt to update the stored positioning assistance information for each received indication of a BCCH modification. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | The SI change indication is broadcasted for update in SI messages containing posSIBs, which is not aligned with the description in clause 5.2.2.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  | x | Other core specifications |  |  |  |  | TS/TR ... CR .. |  |  |  |
| affected: |  |  |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Ver0 in RAN2#121bis: R2-2302985 |  |  |  |  |  |  |  |  |  |  |  |  |

============================================== CHANGE BEGIN =======================================================

- 6.5	Short Message

Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1).

Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit.

Table 6.5-1: Short Messages

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7,SIB8 and posSIB. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1. |
| 4 – 8 | Not used in this release of the specification, and shall be ignored by UE if received. |

=============================================== END OF CHANGES =====================================================