3GPP TSG-RAN WG2 Meeting #121bis	R2-2303221

E-meeting, 17th Apr.-26th Apr.,2023

Agenda item:		7.4.4

Source:		Lenovo

Title:			Consideration on CHO with candidate SCG for CPAC

Document for:		Discussion and Decision

- Introduction

In RAN#97-e meeting, the WI of Further NR mobility enhancements was revised [1], with the following objective with regards to including target MCG and candidate SCGs for CPC/CPA in NR-DC. Currently, we have the following agreement so far.

- RAN2#119 meeting: - CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported. - FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential. - RAN2#120 meeting: - Execution order: the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation) - RAN2#121 meeting: - RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18. - The UE should not need to unpack any of the nested conditionalconfiguration containers in order to measure, acc to agreement above

This contribution aims to discuss the potential impacts for the case that CHO includes candidate SCGs for CPC/CPA in NR-DC.

- Discussion

In rel-18 mobility, the following cases will be studied in which CHO can further embed another candidate SCG for CPA/CPC [1].

Case 1: CHO including target MCG and candidate SCGs for CPA.

Case 2: CHO including target MCG and candidate SCGs for CPC.

Regarding case 1, UE is configured with standalone in source side. CHO configuration includes target MCG and candidate SCG for CPA. Regarding case 2, CHO configuration includes target MCG and the candidate SCG for CPC. For both CPA and CPC cases, target MN requests target SN to configure candidate SCG for CPAC after target MN receives the request from source MN. Then, target MN transmits the response including candidate SCG to source MN. It is natural that candidate SCG for CPAC can be embedded in CHO configuration.

Proposal 1: The configuration of candidate SCGs for CPA/CPC is embedded in CHO configuration.

In Rel-17 CHO with target MCG and target SCG, the source MN transmits the handover request to target MN to prepare CHO candidate cell and corresponding target SCG. After target MN receives the request, target MN will transmit SgNB addition request to candidate SN to prepare target SCG configuration. In Rel-18 CHO with candidate SCG, the above similar procedure will be reused. The question is which node to configure execution condition for CPAC together with CHO. In legacy CHO, source gNB configures the execution condition since source gNB decides whether to perform HO in traditional HO without condition. Regarding CHO with candidate SCG for CPAC, target MN will determine which candidate SCG is suitable or not. Therefore, target MN is responsible for generating the execution condition for CPAC.

Proposal 2: Target MN is responsible for generating the execution condition for CPAC.

In Rel-17 CHO with target SCG, target SCG will be added by default once CHO is performed. However, the target SCG may not be suitable when CHO condition is met. To avoid the out-of-date SCG, CHO with candidate SCG is supported in Rel-18. Furthermore, it was agreed in RAN2 that CHO and candidate SCG are intended to be applicable together. Therefore, candidate SCG for CPAC should be performed together with CHO execution. Namely, If the condition for CPAC is not met until CHO has completed, UE should stop evaluating the condition for CPAC or release the candidate SCG configuration.

Proposal 3: UE may release configuration for CPAC if the condition(s) for all candidate SCGs is not met when UE completes RA to MCG.

It is possible that all candidate SCGs for CPAC are not executed when UE completes RA to MCG. The situation should be reported to target MN. Then, MN can reconfigure/add a suitable candidate SCG for UE. Furthermore, MN may indicate to the corresponding SN(s) which SCG is not executed.

Proposal 4: If all candidate SCGs for CPAC are not executed when UE completes RA to MCG, UE need to indicate it to target MN.

In Rel-16 CHO and CPC, the compliance check for an RRCReconfiguration message received as part of ConditionalReconfiguration is performed upon the reception of the message or upon CHO and CPC execution (when the message is required to be applied). If the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message for CHO and CPC, the UE continues to use the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected. Then, the UE initiates re-establishment procedure or SCG failure information procedure.

In rel-18 mobility, CHO can further embed another candidate SCG for CPA/CPC. For the above two cases, according to the legacy, the compliance check for the embedded CPA can be performed upon the reception of the message or upon CHO execution (when the CHO message is required to be applied). The failure behavior defined also applies in case the UE cannot comply with the embedded SCG configuration for CPA/CPC. But UE will perform reestablishment procedure even the compliance check for CHO configuration is successful and the failure only happens on the embedded CPA/CPC. However, it seems unnecessary to perform reestablishment procedure or SCG failure information if the compliance check on CHO is successful but only the compliance check of the embedded CPA/CPC fails.

Proposal 5: UE considers the compliance check successful if the compliance check for CHO configuration is successful, but the compliance check fails for the embedded CPA/CPC.

- Conclusion

In this contribution, the conclusion and proposal are made based on the discussion:

Proposal 1: The configuration of candidate SCGs for CPA/CPC is embedded in CHO configuration.

Proposal 2: Target MN is responsible for generating the execution condition for CPAC.

Proposal 3: UE may release configuration for CPAC if the condition(s) for all candidate SCGs is not met when UE completes RA to MCG.

Proposal 4: If all candidate SCGs for CPAC are not executed when UE completes RA to MCG, UE need to indicate it to target MN.

Proposal 5: UE considers the compliance check successful if the compliance check for CHO configuration is successful, but the compliance check fails for the embedded CPA/CPC.

References

RP-223520 R18 MobEnh WID