3GPP TSG-RAN WG2 Meeting #121-bis-electronic	R2-2303571

Online, 17th – 26th April, 2023

Agenda Item:	7.2.3

Source:	Spreadtrum Communications

Title:	Discussion on solutions for integrity of RAT-dependent positioning techniques

Document for:	Discussion and Decision

Introduction

In RAN plenary #98 meeting, R18 positioning WID has been discussed and the following objective of positioning integrity was agreed [1]:

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

In this contribution, we discuss the question of the methodologies and procedures of RAT-dependent positioning and express our opinions.

Discussion

In RAN2#120 [2] meeting, the following agreement have been achieved:

Use DNU flag for RAT-dependent integrity, with the meaning that the concerned assistance data cannot be used for integrity calculation but may be usable for positioning. Signalling details and relation to error sources can be determined in normative work. FFS which positioning methods are affected based on the progress in RAN1.

In Rel-17 GNSS integrity, Providing Assistance Data without the Integrity Service Alert IE or Real Time Integrity IEs is interpreted as a DNU=FALSE condition. And we can see that DNU flag in A-GNSS positioning integrity is shown in two parts. GNSS-Integrity-ServiceAlert is provided in GNSS-CommonAssistData IE and indicates the ionosphere and troposphere corrections can be used for integrity or not. GNSS-RealTimeIntegrity is provided in GNSS-GenericAssistData IE, which is associated with specific GNSS constellation and indicated the specific satellites can be used for integrity or not. This is specified in TS 37.355.

#### GNSS-Integrity-ServiceAlert The IE GNSS-Integrity-ServiceAlert is used by the location server to indicate whether the corresponding assistance data can be used for integrity related applications. -- ASN1START GNSS-Integrity-ServiceAlert-r17 ::= SEQUENCE { ionosphereDoNotUse-r17 BOOLEAN, troposphereDoNotUse-r17 BOOLEAN, ... } -- ASN1STOP

NOTE 1: If GNSS integrity assistance data are provided (i.e., any of GNSS-Integrity-ServiceParameters, GNSS-Integrity-ServiceAlert, ORBIT-IntegrityParameters, SSR-IntegrityOrbitBounds, CLOCK-IntegrityParameters, SSR-IntegrityClockBounds, SSR-IntegrityCodeBiasBounds, SSR-IntegrityPhaseBiasBounds, STEC-IntegrityParameters, STEC-IntegrityErrorBounds, SSR-GriddedCorrectionIntegrityParameters, TropoDelayIntegrityErrorBounds) the following interpretation of the IE GNSS-RealTimeIntegrity applies: - Absence of the IE GNSS-RealTimeIntegrity indicates DNU=FALSE according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40] for all GNSS satellites for which integrity assistance data are provided. - Presence of the IE GNSS-RealTimeIntegrity for a GNSS satellite and signal combination indicates DNU=TRUE for this GNSS satellite and signal combination according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40].

And for RAT-dependent integrity, DNU can be used in DL assistance data, which means that the concerned assistance data cannot be used for integrity calculation but may be used for positioning. And in SI phase, RAN 1 has agreed that for UE-based integrity, two types’ error sources including TRP location, inter-TRP synchronization will be provided to UE via assistance data from LMF. In order to indicate to UE which assistance data information cannot be used for integrity calculation, a DNU flag can be introduced. As for the configuration, DNU flags can be configured per frequency layer, per TRP, per resource set or per resource. Considering configuration overhead and flexibility, DNU flag should be configured with each TRP for the error sources in the assistance data.

Proposal 1: RAN2 to configure DNU flags per TRP per positioning method in assistance data.

In study phase, RAN 1 has identified error sources for timing-based and angle-based positioning methods that affect the integrity of RAT-dependent positioning, which we summarize in table 1 and analyses the specifications impact.

Table 1: Analysis of RAT-dependent error sources and impacted specifications

| mode | method | Error sources | Category of error sources | Spec (Procedure) Impact |
| --- | --- | --- | --- | --- |
| LMF-based | DL-TDOA | RSTD measurement | Timing related measurement in UE | LPP ProvideLocationInformation IE |
|  |  | inter-TRP synchronization (can be caused in part by errors in SFN initialization time) | TRP-related information in TRP | NRPPa |
|  |  | TRP location | TRP-related information | NRPPa |
|  | UL-TDOA | RTOA measurement | Timing related measurement | NRPPa |
|  |  | inter-TRP synchronization (can be caused in part by errors in SFN initialization time) | TRP-related information | NRPPa |
|  |  | TRP location | TRP-related information | NRPPa |
|  | Multi-RTT | UE Rx-Tx time difference measurement | Timing related measurement | LPP ProvideLocationInformation IE |
|  |  | gNB Rx-Tx time difference measurement | Timing related measurement | NRPPa |
|  |  | TRP location | TRP-related information | NRPPa |
|  | UL-AoA | Angle of arrival measurement | Angle related measurement | NRPPa |
|  |  | TRP location | TRP-related information | NRPPa |
|  |  | ARP location (e.g., ARPLocationInformation in TS 38.455) | TRP-related information | NRPPa |
|  | DL-AoD | DL PRS RSRP/RSRPP measurement | measurement | LPP ProvideLocationInformation IE |
|  |  | TRP location | TRP-related information | NRPPa |
| UE-based | DL-TDOA | TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) | Assistance data | LPP provideAssistanceData IE |
|  |  | Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355) | Assistance data | LPP provideAssistanceData IE |
|  | DL-AoD | TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) | Assistance data | LPP provideAssistanceData IE |

Thus, in order to obtain positioning integrity calculation estimation, error sources related information needs to be delivered between UE, network and LMF as shown in the table 1, which will modify the LPP and NRPPa protocols.

In last RAN 2 meeting, the following agreements has achieved.

Agreements:

RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.

LS to RAN1 to check this view and ask about the parameters for the overbound distributions.

TRP related error source bounds can be provided to UE via dedicated LPP providing assistance message or posSIB.

Any interaction between the LMF and NG-RAN to support determination of error sources is in RAN3 scope.  Other aspects of determining the TRP error sources are left to deployment and implementation.

For UE-based RAT-dependent integrity, the PL and/or its corresponding TIR are provided to LMF as legacy, using the existing common LPP signalling from Rel-17.

For LMF-based integrity, the measurement errors at UE and gNB need to be reported to LMF, which requires modification of the LPP and NRPPa protocol. As study phase agreed, LMF can send the request of results related to integrity for integrity error sources to UE and RAN. And UE can send the measurements errors statistics related information, such as, mean and standard deviation information for RSTD measurement, UE Rx-Tx time difference measurement, DL PRS RSRP/RSRPP measurement to LMF by using LPP providelocationInformation IE. And gNB can send the measurement errors distribution related information, such as, mean and standard deviation information for RTOA measurement, gNB Rx-Tx time difference measurement, Angle of arrival measurement to LMF by using NRPPa. In addition, TRP location error, ARP location error and inter-TRP synchronization errors statistics related information also need to be transmitted to LMF by using NRPPa.

Proposal 2: For LMF-based positioning integrity mode, measurement error source related information such as mean and standard deviation should be provided to LMF via the LPP ProvideLocationInformation message if required by LMF via LPP RequestLocationInformation message.

In addition to error sources related information transmission, UE also need to send capability information to LMF on integrity for UE-based and LMF-based integrity. In Rel-17, we introduced GNSS positioning integrity for UE. And for the RAT-dependent positioning integrity, we can follow the Rel-17 to define the positioning integrity capabilities for each positioning method, which includes error source receiving, error source reporting and positioning integrity result reporting.

Proposal 3: To support RAT-dependent positioning integrity, UE sends positioning integrity capabilities for each RAT-dependent positioning method to LMF, which includes error source receiving, error source reporting and positioning integrity result reporting.

Conclusions

In this contribution, we discuss the positioning enhancements of Rel-18 positioning integrity techniques. And the proposals are given as follows:

Proposal 1: RAN2 to configure DNU flags per TRP per positioning method in assistance data.

Proposal 2: For LMF-based positioning integrity mode, measurement error source related information such as mean and standard deviation should be provided to LMF via the LPP ProvideLocationInformation message if required by LMF via LPP RequestLocationInformation message.

Proposal 3: To support RAT-dependent positioning integrity, UE sends positioning integrity capabilities for each RAT-dependent positioning method to LMF, which includes error source receiving, error source reporting and positioning integrity result reporting.

References

[1] RP-223549, New WID on Expanded and Improved NR Positioning, 3GPP RAN plenary #98e.

- Report of 3GPP TSG RAN WG2 meeting #120, 2022-11.