3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2304056

# Elbonia, 17– 26 April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0442 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LOS-NLOS-Indicator Types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - LOS-NLOS-Indicator IE description and field description for the field ‘indicator-r17’ are not accurate. This indicator provides two pieces of information: i) whether the propagation path between source and receiver is Line-of-Sight (LOS) or Non-Line-of-Sight and ii) the likelihood of a Line-of-Sight (LOS) propagation path from the source to the receiver, but this is not very clear from the current descriptions. - The two IEs, LOS-NLOS-IndicatorType1 and LOS-NLOS-IndicatorType2 have the same description and it is not clear why two similar IEs with same description is defined with different values. With LOS-NLOS-IndicatorType2 value of ‘hardAndsoftvalue, without searching all ASN.1 text, it is not readily obvious how it would be possible to report both hard and soft values when the UE can only report either hard or soft value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarified the descriptions for LOS-NLOS-Indicator IE and indicator-r17 field. - Clarified that LOS-NLOS-IndicatorType1 is the type of information that can be requested by location server and LOS-NLOS-IndicatorType2 is the type of information that can be supported by target device. Impact analysis Impacted functionality: LOS/NLOS reporting for DL-TDOA, DL-AoD and Multi-RTT positioning Inter-operability: No inter-operability issues forseen. No functional changes made. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clarity in the specification on how the different LOS-NLOS indicator types are used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### –	LOS-NLOS-Indicator

The IE LOS-NLOS-Indicator provides information on whether the propagation path between source and receiver is Line-of-Sight (LOS) or Non-Line-of-Sight and the likelihood of a Line-of-Sight (LOS) propagation path from the source to the receiver.

-- ASN1START

LOS-NLOS-Indicator-r17 ::= SEQUENCE {

indicator-r17			CHOICE {

soft-r17				INTEGER (0..10),

hard-r17				BOOLEAN

},

...

}

-- ASN1STOP

LOS-NLOS-Indicator field descriptions

indicator This field provides information on whether the propagation path between source and receiver is LOS or NLOS and the likelihood of a Line-of-Sight propagation path from the source to the receiver and has the following choices: - soft: This field specifies the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value '0' indicates NLOS and values '1' through '10' indicates LOS and provide an estimate of the propability for a LOS propagation path between source and receiver. Scale factor 0.1; range 0 to 1. - hard: This field specifies whether the propagation path between source and receiver is estimated to be LOS (true) or NLOS (false).

Next Modified Subclause

#### –	LOS-NLOS-IndicatorType1

The IE LOS-NLOS-IndicatorType1 provides information on the LOS-NLOS-Indicator type that is requested by the location server.

-- ASN1START

LOS-NLOS-IndicatorType1-r17 ::=	ENUMERATED { hardvalue, softvalue }

-- ASN1STOP

#### –	LOS-NLOS-IndicatorType2

The IE LOS-NLOS-IndicatorType2 provides information on the LOS-NLOS-Indicator type that is supported by the target device.

-- ASN1START

LOS-NLOS-IndicatorType2-r17 ::=	ENUMERATED { hardvalue, hardAndsoftvalue }

-- ASN1STOP

End of Changes