3GPP TSG-RAN WG2 #121bis.e	R2-2304064

Online meeting, 17th – 26th April 2023

Agenda Item:	7.19.3

Source:	Ericsson

Title:	Discussion on cell barring for eRedCap UEs

Document for:	Discussion, Decision

# 1	Introduction

The discussion on enhanced support of reduced capability (eRedCap) devices WI (RP-223544) [1] has started in RAN2.

In RAN2#121, the following was agreed:

The NR MIB “cellBarred” bit applies to all UEs (Normal UEs, Redcap UEs and eRedCap UEs).

# 2	Cell barring for eRedCap UEs

Network controls the access for Rel-17 RedCap UEs with respect to whether there is support for 1Rx, 2Rx, HD-FDD. The IE intraFreqReselectionRedCap-r17 was introduced to provide a higher level of control access to the cell for Rel-17 RedCap devices.

Rel-18 eRedCap devices can be considered as RedCap devices with limitations in baseband processing, i.e., 5 MHz vs 20 MHz, so one can consider using the parameters introduced for Rel-17 RedCap UEs for network access, i.e., with respect to the support for 1Rx, 2Rx, or HD-FDD, for Rel-18 eRedCap UEs. Therefore, we propose the following

Cell barring parameters, i.e., cellBarredRedCap1Rx, cellBarredRedCap2Rx, and halfDuplexRedCapAllowed, introduced for Rel-17 RedCap UEs are used for Rel-18 RedCap UEs.

For cell access parameters it should be up to the NW to decide. So, it should be possible for the NW to support Rel-18 RedCap but not Rel-17 RedCap in the serving cell.

It should be possible for the NW to support Rel-18 RedCap but not Rel-17 RedCap in the serving cell.

One can then discuss whether there is a need to have an indication for neighbor cells on whether Rel-18 eRedCap UEs can access the cell.

Discuss if there is a need to have an indication for neighbor cells on whether Rel-18 eRedCap UEs can access the cell.

# 3	Conclusion

Based on the discussion in the section above, we propose the following:

Proposal 1	Cell barring parameters, i.e., cellBarredRedCap1Rx, cellBarredRedCap2Rx, and halfDuplexRedCapAllowed, introduced for Rel-17 RedCap UEs are used for Rel-18 RedCap UEs.

Proposal 2	It should be possible for the NW to support Rel-18 RedCap but not Rel-17 RedCap in the serving cell.

Proposal 3	Discuss if there is a need to have an indication for neighbor cells on whether Rel-18 eRedCap UEs can access the cell.

# References

RP-223544 “Revised WID on Enhanced support of reduced capability NR devices”, RAN#98e, Online, Dec 2022.