3GPP TSG-RAN WG2 Meeting #122	R2-2306542

Incheon, Korea, May 22-26, 2023

Agenda item:	9.2

Source: 	Vice Chairman (Nokia)

Title:	Report on LTE legacy, XR, QoE and MUSIM

Document for:	Approval

# Organizational

Rel-17 CR

-	From R2 121, Rel-17 CRs are treated as normal (as Rel-16 Rel-15 etc), meaning that submitted CRs are agreed/not agreed individually.

-	Chair Observation: As for Rel-16 Rel-15 rapporteurs may still do Rel-17 “rapporteur CRs” for miscellaneous small corrections. The work on Rapporteur CRs in normal maintenance phase is usually organized by TS rapporteurs (for maintenance in breakout sessions may alternatively be by WI rapporteur or other appointed).

Rel-17 UE capabilities

-	Also for UE capabilities, normal CRs handling is planned, i.e. CRs should be per-WI and no planned merge into mega CRs. However, if it makes sense from some perspective, multi-WI CRs are not precluded (dec case by case).

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	WI rapporteurs input for WI planning etc,

-	TS rapporteur input for TS maintenance

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

List of offline email discussions:

NOTE: the email discussion deadlines are meant to allow at least all regions to have one day to comment (other than weekend) and also give rapporteurs time to update their proposals before the meeting)

Email discussion deadlines

NOTE: No AT-meeting email discussion reports will be handled in sessions happening during  Mon-Wed.

Deadline 1 (discussions for Thu online)

- Comment deadline: Thursday W1, 0900 local time (for collecting views)

- Rapporteur proposed outcome: Thursday, 1200 local time (proposed outcome)

- Document deadline: 1h before session (discussion report)

Deadline 2 (LS/CR approval via email):

- Comment deadline: Friday, 0800 local time (for wording proposals)

- Rapporteur proposed outcome: EOM (approved LS or agreed CR)

Organizational

[AT122][200] Organizational – LTE legacy, XR, QoE and MUSIM (RAN2 VC)

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions

    - Share meetings notes and agreements for review and endorsement

    - Flag LSs and in-principle agreed CRs for discussion

Intended outcome:

    - General information sharing about the sessions

Post-meeting email discussions

AT-meeting offline discussions (started only after online session)

[AT122][201][QoE] LS on area scope QoE measurements (Samsung)

Scope: Provide agreeable LS to SA5/SA4/RAN3 to ask feasibility of area scope checking only in AL based on online decisions.

Intended outcome: Agreeable LS R2-2306561

Deadline: Deadline 1

[AT122][202][LTE] Correction on handover procedure completion (vivo)

Scope: Provide Rel-17 version of R2-2304943 for agreement based on comments.

Intended outcome: Agreeable CR in R2-2306562

Deadline: Deadline 1

[AT122][203][LTE] Correction on QoE configuration release (Google)

Scope: Provide updated version of R2-2306539 for agreement based on comments.

Intended outcome: Agreeable CR in R2-2306563

Deadline: Deadline 1

[AT122][204][XR] Discard (Nokia)

Scope: Clarify the options for PSI-based discard based on online discussion: How does each option work?

Intended outcome: Discussion report in R2-2306565

Deadline: Deadline 1

[AT122][205][XR] LS to RAN1/4 on new DRX cycles in rational numbers (Qualcomm)

Scope: Provide agreeable LS out on RAN2 decision to based DRX cycle used for XR on rational numbers and request if they see any issues with that.

Intended outcome: Agreeable CR in R2-2306564

Deadline: Deadline 1

Dates and deadlines – Technical Meeting (see also RP-230050)

May 12th 1000 UTC	General Tdoc Submission Deadline.

June 2nd 1000 UTC	Deadline Short Post Email Discussions

RAN2-122 Session Schedule

NOTE that this schedule may be modified on short notice. 
Some Expectations: Details may be added every day. The Schedule for CBs on Thursday (and Friday) will be updated on Wednesday, and the schedule for CBs on Friday will be further updated on Thursday.

WEEK 1

|  | Main room | Brk 1 room | Brk 2 room | Brk 3 room |
| --- | --- | --- | --- | --- |
| Monday May 22 |  |  |  |  |
| 09:00 – 10:30 | Opening: [1], [2], [3] 20-25 min NR1516 CP (Johan) - Common [5.1.1] [5.1.3] NR17 (Johan) - Common [6.1.1] [6.1.3.0, 6.1.3.1, 6.1.3.3]: In the order: General, 71GHz, feMIMO, TEI, Slicing, ePowSav, MGE, RedCap, QoE, DCCA, [6.1.3.2] UE cap | Breakout to start after formal opening of meeting and NR CPUP - common items if any in the main room: NR151617 UP (Diana) NR18 MT-SDT [0.5] (Diana) NR18 Network Energy Saving [1] (Diana) - DTX/DRX, - SSB-less (if time allows) | Breakout to start after formal opening of meeting in main room: NR17 Pos (Nathan) NR17 SL Relay (Nathan) - UP items if time permits |  |
| 11:00 – 13:00 |  |  |  |  |
| 14:30 – 16:30 |  | NR18 UAV [1] (Diana) - measurement reporting, - flight path, - BRID (if time allows) | NRLTE1516 (Kyeongin) NR17 (Kyeongin) |  |
| 17:00 – 19:00 | NR17 (Johan) - Common[6.1.1] [6.1.3] SDT, IAB NR18 MIMO evo [0.5] (Erlin) - 7.20.1 (if any) - 7.20.2 | NR18 NTN enh [1] (Sergio) | NR18 SL evolution [1] (Kyeongin) |  |
| Tuesday May 23 |  |  |  |  |
| 08:30 – 10:30 | NR18 LP WUS [0.5] (Johan) NR18 fCovEnh [0.5] (Eswar) - Start with Stage-2 CP issues (7.21.2) after organizational - CBRA open issues, CFRA support, RSRP thresholds etc - UP issues (7.21.3) - fallbacks Stage-3 CP issues if time left (how to signal partitions, priorities and capability etc) | NR17 MBS (Dawid) 8:30-10:00 - 6.2.0, 6.2.1 (CP), 6.2.2 (UP) NR18 eQoE [0.5] (Tero) 10:00-10:30 - 7.14.2: Area scope (e.g. R2-2306396), release of QoE data (e.g. R2-2305809) | NR17 (Nathan) - Pos SL Relay TBD |  |
| 11:00 – 13:00 | NR17 NTN Maint (Sergio) | NR18 eQoE [0.5] (Tero) - 7.14.2: RRC state applicability, SIB information and UE buffer sizes (e.g. R2-2305076, R2-2305310), UE capabilities (R2-2305606) - 7.14.3: Need for RVQoE events in RRC (e.g. R2-2306109, R2-2305384), LS replies to RAN3/SA4 (e.g. R2-2305077) EUTRA16+ (Tero) 12:30-13:00 - 4.1: HO completion in Stage-2 (R2-2304943), QoE configuration release (R2-2306539, R2-2305132) | NR18 Pos [2] (Nathan) |  |
| 14:30 – 16:30 | NR18 Other [2] (Johan) | NR18 XR [2] (Tero) - 7.5.1: LSs, rapporteur input, running CR(s) - 7.5.4.1: Delay reporting (e.g. R2-2304955), BSR tables for XR (R2-2305149, R2-2304711), support of piecewise linear BSR table (R2-2305604) - 7.5.4.2: Discard operation for XR (e.g. R2-2305019, R2-2305191, R2-2305784) | NR18 SL relay [1.5] (Nathan)- |  |
| 17:00 – 19:00 | NR18 feMob [2] (Johan) | NR18 XR [2] (Tero) - 7.5.3: Non-integer periodicity for DRX (e.g. R2-2304709, R2-2304808), SFN wrap-around (R2-2304710, R2-2305898) - 7.5.2: UL jitter signaling (e.g. R2-2304708, R2-2305634, R2-2305827), UL EoDB detection at gNB (e.g. R2-2305190, R2-2305897) - 7.5.4.3: Retransmissionless, CG (e.g. R2-2304809, R2-2305654), RAN2 aspects of RAN1 CG enhancements (R2-2304713, R2-2306185) | NR17 (Nathan) NR18 SL relay [1.5] (Nathan) |  |
| Wednesday May 24 |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob [2] (Johan) | NR18 NCR [0.5] (Sasha) | NR18 Pos [2] (Nathan) |  |
| 11:00 – 13:00 | NR18 Mobile IAB [0.5] (Johan) NR18 Other [2] (Johan) | NR18 RedCap [1] (Mattias) | NR17 SONMDT (HuNan) |  |
| 14:30 – 16:30 | NR18 URLLC [0.5] (Diana) NR18 Network Energy Saving [1] (Diana) - Mobility, - Cell reselection | NR17 MBS (Dawid) - Continuation, if needed NR 18 MBS [0.75] (Dawid) - 7.11.1 (Organizational) - 7.11.3 (Shared processing) - 7.11.2.1 (CP issues for INACTIVE) | NR18 SONMDT [1] (HuNan) |  |
| 17:00 – 19:00 | NR18 AIML [1] (Johan) | R18 IoT-NTN [1] (Sergio) | NR18 IDC [1] (Yi) |  |
| Thursday May 25 |  |  |  |  |
| 08:30 – 10:30 | CB NR151617 (Johan) | CB Diana - maintenance CRs, - NES CBs . | CB Kyeongin |  |
| 11:00 – 13:00 | NR18 TEI [1] (Nathan, Johan) No SL Relay proposals. | CB Diana - NES CBs - UAV CBs and subscription based AI. | CB Kyeongin |  |
| 14:30 – 16:30 | CB NR17 (Johan) | CB Sergio (14:30-15:30) (including AI 7.25.4, in case there is any corresponding progress in RAN1) CB Tero (15:30 – 16:30) - 7.14.2: Outcome of [201] - 7.14.4: SRB configuration (e.g. R2-2306477, R2-2305810), MN RRC message used for QoE reporting to SN (e.g. R2-2305383) | CB Nathan |  |
| 17:00 – 19:00 | CB NR17 (Johan) CB NR18 (Johan) | CB Tero - 4.1: Outcome of [202] and [203] - 7.5.3: Outcome of [205] - 7.5.4.2: Outcome of [204] - 7.5.2: UL EoDB detection at gNB (e.g. R2-2305190, R2-2305897), LS reply to SA4 (e.g. R2-2305493) - 7.5.4.3: Retransmissionless, CG (e.g. R2-2304809, R2-2305654), RAN2 aspects of RAN1 CG enhancements (R2-2304713, R2-2306185) | CB Nathan |  |
| Friday May 26 |  |  |  |  |
| 08:30 – 10:30 | NR18 MIMO evo [0.5] (Erlin) - CBs from 7.20.2 (if any), - 7.20.3. CB Dawid | CB Mattias TBD | CB Nathan, Kyeongin TBD |  |
| 11:00 – 13:00 | CB Johan, Eswar TBD | CB Sergio | CB Yi |  |
| 14:30 – 16:00 | CB Johan | CB Sasha | CB HuNan |  |
| 16:00 – 17:00 | CB and conclusion (Johan) |  |  |  |

Breaks

Morning coffee: 		10:30 to 11:00

Lunch: 			13:00 to 14:30

Afternoon coffee:	16:30 to 17:00

Offline Web Conference Schedule

Number		Title					Day/Time 	Place			Coordinator

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP192875;), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

REL-15 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except V2X and Sidelink WIs and Positioning WIs, which are adressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the EUTRA Breakout session

Online (Tuesday) (1)

LTE Rel-8 Stage-2 description HO completion is inaccurate?

R2-2304943	Correction on handover procedure completion	vivo, Nokia (rapporteur)	CR	Rel-8	36.300	8.12.0	1385	-	F	LTE-L23

-	Lenovo thinks cover page should state Stage-2 is not aligned with Stage-3. Issue is not critical.

-	Ericsson agrees that this is not critical but should be corrected. QC thinks we should correct this.

-	Samsung prefers to correct the error. Thinks cover sheet is not fully correct. QC agrees.

-	Huawei agrees there is no inter-operability issues. Thinks it’s not required to update this from Rel-8. Could just update in e.g. Rel-17. Lenovo thinks we could just correct this from Rel-17 since this is not critical to implementation.

We will fix the issue in Rel-17. Should explain in cover page why this has no issues with earlier releases, correct any mistakes in current explanations, etc.

Offline [202] (vivo) to provide the CR in R2-2306562

Offline discussion (Thursday) (1)

[AT122][202][LTE] Correction on handover procedure completion (vivo)

Scope: Provide Rel-17 version of R2-2304943 for agreement based on comments.

Intended outcome: Agreeable CR in R2-2306562

Deadline: Deadline 1

R2-2306562	Correction on handover procedure completion	vivo, Nokia (rapporteur)	CR	Rel-17	36.300	17.4.0	1385	1	F	LTE-L23	R2-2304943

-	QC thinks we should add TEI17 to the WI code.

-	Ericsson thinks the other comments we shoul duse “incorrect text” instead od “impacted text”

RAN2 concludes that the issue resolved by this CR exists in the specification from Rel-8, but it was agreed to make the change from Rel-17 only, considering the incorrect text in clause 10.1.2.1.1 has not been implemented.

Add TEI17 into the list of WI codes

Use “incorrect text” in the other comments instead of “impacted text”

With the above changes, the CR is agreed (unseen) in R2-2306570

R2-2306570	Correction on handover procedure completion	vivo, Nokia (rapporteur)	CR	Rel-17	36.300	17.4.0	1385	2	F	LTE-L23, TEI17		R2-2306562

Agreed (unseen)

Online (Tuesday) (1+1)

CR for release of QoE configuration/reporting at upper layers when UE moves to IDLE/INACTIVE (RAN2#121bis-e continuation):

R2-2306273	Correction on QoE configuration release	Google	CR	Rel-17	36.331	17.4.0	4935	-	F	LTE_QMC_Streaming-Core

Revised in R2-2306539

R2-2306539	Correction on QoE configuration release	Google, Qualcomm, Ericsson	CR	Rel-17	36.331	17.4.0	4935	1	F	LTE_QMC_Streaming-Core

-	Samsung thinks this applies also to INACTIVE now since its on 1>-level. Should restrict it to IDLE only.

Restrict the correction to RRC_IDLE only.

Offline [203] (Google) to revise the CR according to discussion. Final CR can be provided in R2-2306563

Offline discussion (Thursday) (1)

[AT122][203][LTE] Correction on QoE configuration release (Google)

Scope: Provide updated version of R2-2306539 for agreement based on comments.

Intended outcome: Agreeable CR in R2-2306563

Deadline: Deadline 1

R2-2306563	Correction on QoE configuration release	Google, Qualcomm, Ericsson	CR	Rel-17	36.331	17.4.0	4935	2	F	LTE_QMC_Streaming-Core	R2-2306539

Add TEI17 into the list of WI codes

Change “However, acc. to” to “However, according to”

With the above changes, the CR is agreed (unseen) in R2-2306571

R2-2306571	Correction on QoE configuration release	Google, Qualcomm, Ericsson	CR	Rel-17	36.331	17.4.0	4935	3	F	LTE_QMC_Streaming-Core	R2-2306563

Agreed (unseen)

Additional discussion:

R2-2305132	Discussion on QoE configuration release in LTE	Lenovo	discussion	Rel-15	LTE_QMC_Streaming-Core	Late

Proposal 1: RAN2 to confirm that any active QoE measurement configuration in RRC_CONNECTED is expected to be explicity released by the network prior to moving the UE to RRC_INACTIVE or RRC_IDLE via RRC connection release message.

-	Google is fine with P1 for normal case but not for error cases like RLF. Thinks we should align with NR. Ericsson thinks there are cases where network doesnt’ explicitly release this.

-	Samsung agrees with intent but this doesn’t have specification impact. But should consider the other cases as well. Nokia acknowledges that something could be corrected but it’s hard to misunderstand.

-	Huawei agrees with have a CR to fix the UE behaviour. Lenovo is fine to have the CR.

Proposal 2: RAN2 to leave to UE implementation how to handle an active QoE measurement configuration and unsent QoE reports when the UE moves to RRC_IDLE.

Noted

# 7	Rel-18

## 7.5	XR Enhancements for NR

(NR_XR_enh-Core; leading WG: RAN2; REL-18; WID: RP-230786)

Time budget: 2 TU

Tdoc Limitation: 5 Tdocs

### 7.5.1	Organizational

Including LSs and any rapporteur inputs (e.g. work plan, SA2/SA4 progress reports)

Online (Tuesday) (1) - LSs

LSs from other groups:

R2-2304659	LS out on the N6 PDU Set Identification (S4-230739; contact: Intel)	SA4	LS in	Rel-18	5G_RTP, XRM, NR_XR_enh	To:SA2, RAN2

SA4 thanks SA2 for confirming the progress of the normative work timeline. As indicated in S4-230419, the new RTP header extension under SA4 5G_RTP will signal the PDU set information, including PDU set sequence number, PDU set boundary indication, PDU sequence number within a PDU set, PDU set size, and PDU set importance.

During SA4#123-e, it was agreed to add a 3-bit End of Data Burst indication in the new header extension. SA4 has committed to progressing the semantics of the fields and developing normative guidelines for the Application Server on how to populate the fields of the RTP header extension for the supported media codecs. Upon completing such an effort, SA4 will continue to provide guidelines on how the UPF may extract some of the supported PDU set information from existing RTP/SRTP headers, header extensions, and payloads in case the newly defined RTP header extension is absent.

In addition to marking the last PDU of the data burst, SA4 sees the benefit of using additional bits to indicate inter-burst time, which may change dynamically due to various reasons, including application-layer rate control. SA4 believes that this can enable the RAN to switch to the most appropriate power state. SA4 kindly requests feedback from SA2 and RAN2 on the value and feasibility of such solution and if that can be supported within Rel. 18 timeframe.

SA4 is also defining the SDP signaling of the usage of the RTP header extension based on RFC8285. This allows the AF to receive certain PDU set information and pass it along to the PCF/NEF using the N5/N33 interface procedures. The header extension configuration should be shared with the UPF, and SA4 will provide the relevant configuration information to SA2/CT3 for this purpose.

To RAN2:

ACTION:

- SA4 would like to kindly ask RAN2 to provide feedback on the feasibility and value of having additional signaling bits related to End of Burst and inter-burst time within Rel-18.

-	OPPO wonders if this is for UL or DL direction? We can only ask about DL. Intel clarifies this is mainly about DL information RAN uses. So the question is whether the informaiton is useful for RAN.

-	LGE wonders if this requires packet inspection? LGE also wonders why do we need 3 bits for EoDB? Intel clarifies that this was not clear in the LS.

-	ZTE thinks we will nto specify anything for packet inspection but it is feasible. For UL it’s a different question. Huawei agrees with Intel that this is for DL. 3 bits are used for something additional but that is in SA domain. Intel clarifies that the LS went to SA2 as well so they can also reply. RAN2 could reply from UL side.

-	CATT thinks the LS is more for SA2 on RTP headers and for RAN2 on the need of the information. DL  EoDB was already agreed. Thinks we need DL jitter.

Reply LS discussed with contributions under 7.5.2

Online (Tuesday) (1) – Work plan

Work plan:

R2-2305186	Work Plan for Rel-18 WI on XR Enhancements for NR	Nokia, Qualcomm (Rapporteurs); Ericsson (RAN1 FL)	Work Plan	Rel-18	NR_XR_enh-Core

Endorsed

Online (Tuesday) (2) – SA2/SA4 status

SA2 and SA4 work status:

R2-2305187	SA2 Status for XR	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-18	NR_XR_enh-Core

-	Lenovo wonders if there is some RAN2 impacts on the PDU set-less PDUs? Nokia thinks this depends on how SA2 agrees on importance handling. That could affect discard operation.

Noted

R2-2305188	SA4 Status for XR	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-18	NR_XR_enh-Core

Noted

Online (Tuesday) (1) – Running CRs

Running CRs:

R2-2305189	Stage 2 Overview of XR Enhancements	Nokia, Qualcomm (Rapporteurs)	draftCR	Rel-18	38.300	17.4.0	B	NR_XR_enh-Core

-	OPPO wonders what PDU set based handling means.

Companies are encouraged to provide comments on the CR to rapporteur(s) already during the meeting (i.e. before any post-meeting email discussion)

For MAC, RRC, PDCP and RLC (if needed) running CRs, the CR rapporteurs are requested to submit first running CRs as rapporteur input (which are not counted against the Tdoc limits).

Post-meeting email discussions (XR) (1) – Running CR(s)

[Post122][211][XR] Stage-2 running CR for XR (Nokia)

Scope: Update 38.300 running CR based on this meeting’s agreements.

Intended outcome: Endorsed running CR

Deadline:  Short

IF time allows: Online (Thursday) (1)

What kind of UE capabilities are needed for XR?

R2-2305492	UE Capabilities for Rel-18 XR WI	Intel Corporation	discussion	Rel-18	NR_XR_enh-Core

Observation 1.	During Rel-18 XR SI phase, RAN2 informed SA2 and SA4 multiple times the assumption that PDU set concept is applicable to UL side and UE is able to identify the corresponding PDU set related information. By not responding to this, RAN2 understands that there is no concern/issue identified by SA2 and SA4 on this regard.

Observation 2.	RAN2 agreed for UE to provide UL jitter information to RAN and it is FFS whether UE might also be defined other ones such as end of data burst or inter-burst time.

Proposal 1.	To confirm that UE AS layer has visibility to the same PDU set concept/information for UL as it is currently defined by SA2 for DL traffic.

Proposal 2.	A new optional radio capability (e.g., supportOfPDU-Set) is defined to identify UE supporting PDU set concept/information. FFS whether a new radio capability is required for UE’s knowledge of the Data Burst related concept/information,

Proposal 3.	At least one new optional UE capability signaling is defined for the new UE assistance information related to XR traffic requires.

Proposal 3.1.	If RAN2 agrees to provide multiple kind of UE assistance information (so far UL jitter is agreed, and it is FFS at least End of data burst and inter-burst time), to discuss whether a single capability is sufficient or multiple ones is preferable (i.e., one per each kind of information that UE can provide).

Proposal 4.	A new optional UE capability signaling (e.g., supportOfDiscardPDU-Set) is defined to identify Rel-18 UEs supporting discard operation associated with the PDU Set concept. This supportOfDiscardPDU-Set is optional capability only if UE also supports supportOfPDU-Set.

Proposal 5.	A new optional UE capability signaling (e.g., supportOfNewBS-Table) is defined to identify Rel-18 UEs supporting BSR enhancements associated with the new BS tables. FFS whether this capability may also include other BSR related enhancements (e.g., new BSR trigger conditions) and whether it has any pre-requirement (e.g., to the support of supportOfPDU-Set).

Proposal 6.	To discuss whether a new optional UE capability signaling (e.g., supportOfDelayReporting) is defined to identify Rel-18 UEs supporting enhancements associated with the delay reporting of the buffered data or whether this is included as part of Proposal 4, i.e., new UE capability on BSR related enhancements.

Proposal 7.	Wait for RAN1 input on new UE capabilities for RAN1 lead objectives, i.e., Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration and Dynamic indication of unused CG PUSCH occasion(s) based on UCI.

Proposal 8.	A new optional UE capability signaling (e.g., supportOfRationalDRX) is defined to identify Rel-18 UEs supporting C-DRX enhancements targeting any traffic with non-integer periodicity.

### 7.5.2 XR awareness

Including discussion on XR traffic assistance information from UE to networkIncluding discussion on how UL jitter information is reported from UE to network: what exactly is reported and via which signalling, what are the value ranges, how does network detect UL EoDB (e.g. can padding BSR be used for that?), etc.

Online (Tuesday) (1) – UL jitter information

Jitter signalling:

R2-2304708	Discussion on XR awareness	Qualcomm Incorporated	discussion	Rel-18	NR_XR_enh-Core

UL jitter information

Proposal 1. 	UE reports to RAN the range of jitter in its UL traffic, defined in the same way as the one for N6 jitter.

Proposal 2. 	Range of UL jitter is reported per logical channel. Network can configure for which logical channel UE should report jitter information.

Proposal 3. 	Network can configure whether UE reports UL jitter information periodically or only when the range exceeds a threshold.

End of data burst indication

Observation 1. 	End of burst indication by UE can help network determine whether to terminate DRX active time early and thus save UE more power.

Observation 2. 	Padding BSR may not be a reliable way to indicate EoDB when there is jitter in UL traffic. It may cause premature termination of DRX active time and interruption in scheduling.

Observation 3.	An explicit EoDB indication can help network schedule UE with more certainty.

Proposal 4.	Introduce EoDB indication explicitly signaled by UE. FFS the type of signal used to send the indication.

Proposal 5. 	Specify a method for UE to terminate its DRX active time faster than the legacy ones after it sends a UL EoDB indication. FFS details of this method.

Other UL traffic information

Proposal 6.	UE can report UL traffic periodicity of a logical channel to RAN. This information is a complement, not a replacement, to the traffic periodicity provided by CN to RAN.

Proposal 7. 	UE can report its preferred start offset for a CG to RAN.

Proposal 8. 	UL traffic information can be signaled via the RRC message UE Assistance Information.

Focus on P1-3, P6-8

R2-2305634	Remaining Issues on UL Traffic assistance information for XR	CMCC	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: the UL Jitter information can be defined associated with the Periodicity in uplink, which can be defined as BAT offset, i.e., variation of burst arrival time for UL traffic resulting from UL jitter which value can be positive or negative.

Proposal 2: the range of UL Jitter information is better to be set as [-4, +4].

Proposal 3: the UL Jitter information can be optional, in case of the UL pose/control traffic.

Proposal 4: it is proposed gNB can acquire the anticipated UL jitter information from UE via UEAssistanceInformation message as in LTE V2X.

Proposal 5:	It is proposed to define an approach to enable the gNB be aware of EoDB timely, e.g. in regular BSR.

Focus on P1-2

R2-2305827	Discussion on XR-awareness	Ericsson	discussion	Rel-18	NR_XR_enh

Observation 1	BSR will indicate when a UE has empty buffer (padding BSR), such can be considered as EoDB information

Proposal 1	RRC UAI framework is updated for Rel-18 to support signalling UL jitter and associated periodicity information.

Proposal 2	UE report statistical UL jitter range X and associated periodicity Y on a per QoS flow

Proposal 3	RAN2 discuss the value range of UL jitter X and associated periodicity Y

Figure 1 Illustration of two options representing jitter information

Proposal 4	If RRC UAI is enhanced with UL jitter and associated periodicity information, RAN2 should discuss the configuration of report triggers, e.g. prohibition timer or change threshold that regulate when UE is allowed to send UL jitter and periodicity information.

Proposal 5	Explicit EoDB signalling in UL is not needed.

Proposal 6	RAN2 responds to SA4, with SA2 cc, that RAN2 does not see the usefulness of the EoDB. Nonetheless, if SA4 wants to define it, one single bit would be sufficient.

Focus on P1-4

Proposal 1. 	UE reports to RAN the range of jitter in its UL traffic, defined in the same way as the one for N6 jitter.

-	Intel wonders if the range is clear. CMCC thinks we should allow both negative and positive values for jitter. Huawei wonders who defines the N6 jitter? Will that be RAN3 to define the signalling?  Thinks there should be some reference time for the jitter.

-	vivo agrees with Huawei on reference time.

-	Intel wonders if the information from UL is the same as in that provided by CN, different, or complementary? Thinks periodicity would come from CN, UE just provides the jitter information. CATT thinks periodicity from CN is optional. Intel thinks that the DL signalling provides periodicity already. Huawei thinks BAT is only for TSCAI, not for XR. Samsung thinks TSCAI does include jitter.

1. 	UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter.

2: Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side.

Periodicity in uplink, which can be defined as BAT offset, i.e., variation of burst arrival time for UL traffic resulting from UL jitter which value can be positive or negative.

Proposal 2. 	Range of UL jitter is reported per logical channel. Network can configure for which logical channel UE should report jitter information.

-	Samsung thinks DL is reported per QoS flow. Chair wonder how this impacts scheduling?

-	Huawei agrees with Ericsson that QoS flows can be mapped to single DRB. The information could come from AL where QoS flows are more visible. Apple thinks the reporting from CN is for UL and DL, and RAN does the mapping anyway. This would align with the QoS flow-based mapping.

-	KDDI wonders if we should consider PDU set which has multiple QoS flows.

-	CMCC thinks different QoS flows are more likely to be used.

-	Intel thinks in AS we normally configure everything based on LCH/LCG. How is this used? QC thinks the whole point is to configure resources. If per QoS flow is used, both UE and NW need to map the information to AS layer. Lenovo agrees with QC, but if different QoS flows have different periodicities, the reporting will not be so useful. CATT thinks we could associate UL jitter with different periodicity in LCH. KDDI thinks one QoS flow can have multiple PDU sets.

-	QC wonders how we handle the reference time now? OPPO thinks URLLC already considered QoS flow. Intel thinks the reference time is no longer needed.

-	QC thinks we should limit this to UL jitter. NEC thinks N6 interface is associated with periodicity. What is the association with QoS flow and UL jitter? CATT thinks the periodicity applies to both UL and DL of a QoS flow.

-	LGE wonders if arrival time and burst arrival time are the same?

2. 	UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information.

Show of hands on UL jitter reporting:

Per LCH: Intel, QC, Futurewei (3)

Per QoS flow: Vodafone, Huawei, Samsung, CATT, LGE, vivo, Apple, Nokia, Ericsson, TCL, OPPO, Xiaomi, CMCC, Sony (15)

3	RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

Proposal 3. 	Network can configure whether UE reports UL jitter information periodically or only when the range exceeds a threshold.

Proposal 1	RRC UAI framework is updated for Rel-18 to support signalling UL jitter and associated periodicity information.

Proposal 2	UE report statistical UL jitter range X and associated periodicity Y on a per QoS flow

Proposal 3	RAN2 discuss the value range of UL jitter X and associated periodicity Y

Proposal 4	If RRC UAI is enhanced with UL jitter and associated periodicity information, RAN2 should discuss the configuration of report triggers, e.g. prohibition timer or change threshold that regulate when UE is allowed to send UL jitter and periodicity information.

R2-2305158	XR awareness	InterDigital	discussion	Rel-18	NR_XR_enh-Core

Observation 1: 	Enhancements to the TSCAI framework are needed to provide UL jitter statistics.

Observation 2: 	There are latency considerations associated with providing jitter information to the RAN from the CN.

Proposal 1: 	UE transmits UL jitter statistics to the RAN.

Proposal 2: 	Enhance UAI framework to enable UE reporting of UL jitter to the RAN.

Proposal 3: 	Reporting of UL jitter statistics from UE to RAN via UAI is configurable on a per flow or per radio bearer basis.

Proposal 4: 	RAN2 to discuss the applicability of the DL jitter ranges for UL traffic by also taking into account the tethering use case. Discussion with SA4 may be needed.

Proposal 5: 	UE reports EoDB to the RAN.

Proposal 6: 	EoDB may additionally convey the inter-burst time between consecutive data bursts, e.g., based on traffic periodicity.

Proposal 7: 	RAN2 to discuss if reporting of statistics on inter-burst time between consecutive data bursts (e.g., via UAI) is also required.

R2-2305740	Discussion on UL jitter information	Samsung	discussion	Rel-18	NR_XR_enh-Core

Proposal 1. UL jitter information and the associated UL periodicity for a QoS Flow can be reported from UE to network.

Proposal 2. UEAssistanceInformation message can be used for indicating the assistance information of UL jitter.

Online (Thursday) (2) – EoDB determination for UL

EoDB determination: Is padding BSR sufficient for UL EoDB or is something else needed?

R2-2305190	Jitter and End of Data Burst Signalling	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_XR_enh-Core

Proposal 1:  Do not use PIN delay budget request for jitter reporting for XR services.

1:  Do not use PIN delay budget request for jitter reporting for XR services.

Proposal 2: Adopt both alternatives 1 and 2 for signalling and measuring uplink jitter.

Proposal 3: Adopt Padding BSR with BS value equal to zero as implicit End of Data Burst (EoDB) indicator for the RAN.

-	Nokia clarifies there might be data on other LCGs when burst ends, so we need another trigger for padding BSR in that case but no new indication about end of burst.

-	CATT agrees we dont need explicit EoDB but is not sure if it’s padding SR. Enhanced CGs need indication of unused CGs, which could be used.

-	QC agrees there could be power saving gains with EoDB indication. Could still use data over CG. But thinks padding BSR could be cancelled or not fit. thinks BSR cancellation could happen more often than in legacy.

-	Apple agrees with QC and thinks explicit EoDB is useful. Thinks some protocol header could be used. Huawei thinks EoDB indication is not needed and padding BSR is fine. If it’s not sent, there is more data and give another grant to UE. Can discuss BSR triggering enhancements. Sony agrees. Ericsson agrees.

-	MTK thinks if UE has UL data, we should send it out as soon as possible. So having padding BSR can be harmful. Ue should send data when it has it not go to sleep. Lenovo agrees and thinks we could use padding BSR as it is and use the UCI indication in addition.

-	LGE thinks padding BSR can be used but its only gain is power saving, but the gains there are not clear. vivo thinks EoDB is useful for power saving but padding BSR is not always useful. Could have separate explicit indication. Samsung thinks existing padding BSR can be used for this without any modifications. Google thinks EoDB is not needed.

-	Ericsson thinks we should say without “specification modifications”.

-	vivo thinks we can still discuss new BSR triggers. MTK thinks we should discuss how this enables power saving e.g. in the next meeting. Intel thinks this is close to what we discussed in the last meeting. Futurewei thinks zero value is the mechanism to use.

3: Reuse existing mechanisms (e.g. (Padding) BSR with BS value equal to zero) as implicit End of Data Burst (EoDB) indicator for the RAN.

R2-2305897	RAN awareness of XR characteristics	MediaTek Inc.	discussion	Rel-18	NR_XR_enh	R2-2303301

Observation 1: Assistance information on UL traffic is not needed for the gNB to configure DRX correctly.

Observation 2: Assistance information on UL traffic can be useful to the gNB to ensure CG is appropriately configured.

Proposal 1: On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation.

-	CMCC thinks there is some static information like QoS. Futurewei wonders if gNB knows what PSI value from UE means.

1: On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how.

Proposal 2: XR traffic assistance consists of periodicity, latest arrival time, and min/max size of data expected per period.

Proposal 3: XR traffic assistance is signalled using the UE assistance framework.

R2-2305301	Discussion on periodicity, jitter, and end of burst indication	KDDI Corporation	discussion

Observation 1: UL/DL periodicity in TSCAI is defined per QoS Flow not per PDU Set.

Observation2: Configured grant/Semi persistent scheduling can have multiple configurations.

Observation3: If gNB knows UL/DL periodicity per PDU set, then gNB can optimize Configured grant/Semi persistent scheduling based on each UL/DL periodicity of each PDU set.

Observation4: gNB can acquire DL periodicity and jitter per PDU set by counting PDU packets with PDU Set Information.

Observation5: For UL periodicity and jitter per PDU Set, gNB needs to be indicated from UEs.

Proposal1: RAN2 agree that UE can report UL jitter information per QoS Flow.

Proposal2: RAN2 discuss how UE reports UL jitter information per QoS Flow to gNB.

Proposal3: RAN2 agree to introduce UL/DL periodicity and jitter per PDU set.

Proposal4: RAN2 discuss how UE reports UL periodicity and jitter per PDU set to gNB.

Proposal5: RAN2 agree not to introduce UL end of burst indication from UE.

Online (Thursday) (1) – LS reply to SA4/2 on EoDB

R2-2305493	XR Awareness in UE and RAN	Intel Corporation	discussion	Rel-18	NR_XR_enh-Core

Observation 1.	SA4 LS on N6 PDU Set identification asks for RAN2 input on the feasibility and value on defining new signaling information about End of Burst and Inter-Burst Time for DL XR traffic as part of the RTP header.

Proposal 1.	To reply SA4/SA2 that RAN can benefit from getting XR related information about End of Burst.

Proposal 2.	To discuss how Inter-Burst Time could be used by RAN as a dynamic kind of information.

Proposal 3.	UE can provide End of Data Burst indication associated to the UL XR traffic via MAC CE.

Proposal 4.	UAI is used to convey Inter-Burst Time if its changes are not frequent.

Proposal 5.	UE can provide Jitter associated to UL traffic as part of UEAssistanceInformation message.

Focus on P1

-	Nokia thinks we should be more constructive and indicate what information could be useful to us. Could indicate that EoDB tells that UE can sleep. SA2/4 should focus on timing.

-	Lenovo is not clear on what inter-burst arrival time means. If application knows the timing, it can enforce the application behaviour. IS not sure that is useful to RAN. OPPO thinks inter-burst timing is not clear and majority in SA2 do not want it. Can reply RAN can benefit from EoDB. CATT agrees with Lenovo and this is just about DL jitter.

-	Nokia clarifies that within one period there may be only one burst.

-	Intel thinks if we say nothing, it can be misinterpreted. Could consider details from Huawei contribution. Huawei agrees and thinks the EoDB is sufficient. Could say one bit is enough.

Reply LS based on (only) Intel contribution:

R2-2306568	LS response to N6 PDU Set Identification	RAN2	LS out	Rel-18	NR_XR_enh-Core	To: SA4, SA2	Cc: RAN1

Use the text: “RAN2 would like to thank SA4 for their LS on N6 PDU Set Identification and inform that RAN can benefit from getting XR related information about End of Burst (1 bit). RAN2 have not identified any other dynamic information that is useful.”

With the above changes, the LS is approved (unseen) in

R2-2306572	LS response to N6 PDU Set Identification	RAN2	LS out	Rel-18	NR_XR_enh-Core	To: SA4, SA2	Cc: RAN1

Approved (unseen)

R2-2304865	Further discussions on XR awareness	Futurewei	discussion	Rel-18	NR_XR_enh-Core

R2-2304915	Discussion on XR awareness	vivo	discussion	Rel-18	NR_XR_enh-Core

R2-2304967	Enhancements for XR awareness	CATT, Dell Technologies	discussion	Rel-18	NR_XR_enh-Core

R2-2305005	Discussion on XR awareness	Xiaomi Communications	discussion

R2-2305016	XR Awareness in RAN	ZTE Corporation, Sanechips	discussion

R2-2305071	Views on XR-Awareness	Apple	discussion	Rel-18	NR_XR_enh-Core

R2-2305361	XR awareness	NEC	discussion	Rel-18	NR_XR_enh-Core

R2-2305513	Considerations on XR PDU prioritization	Sony	discussion	Rel-18	NR_XR_enh-Core

R2-2305532	Discussion on XR awareness	OPPO	discussion	Rel-18	NR_XR_enh-Core

R2-2305536	On XR awareness	Google Inc.	discussion

R2-2305565	Discussion on XR awareness	Spreadtrum Communications	discussion	Rel-18

R2-2305684	Discussion on PDU sets and data burst awareness in RAN	Lenovo	discussion	Rel-18

R2-2305808	Discussion on PDU set and data burst information	Huawei, HiSilicon	discussion	Rel-18	NR_XR_enh-Core

R2-2306205	Further discussion on XR awareness	TCL Communication	discussion	Rel-18

R2-2306333	Discussion on XR awareness	LG Electronics Inc.	discussion	NR_XR_enh-Core

R2-2306463	Discussion on XR-awareness	NTT DOCOMO, INC.	discussion

R2-2306481	Discussion on XR awareness	China Unicom	discussion	NR_XR_enh-Core

### 7.5.3	XR-specific power saving

Including discussion and details of solutions for DRX cycles with XR: do we use rational numbers for DRX cycle or do integer adjustments? How does each solution work in details?

Including discussion on solutions for SFN wrap-around, e.g. what is the reference SFN: H-SFN, E-SFN or some generic counter?

Online (Tuesday) (2) – DRX handling for non-integer periodicity

DRX mismatch:

R2-2304709	Discussion on DRX mismatch problem for XR	Qualcomm Incorporated, MediaTek, CATT, vivo, NEC, Meta	discussion	Rel-18	NR_XR_enh-Core

Observation 1. 	Options that necessitate non-uniform DRX cycles or multiple on durations within a DRX cycle have non-trivial impacts on RAN1/4 specs but do not offer better performance (e.g. delay, power savings) than others.

Observation 2.	For the option with uniform DRX cycle expressed as a rational number, there are methods to implement modulo operation on rational numbers without rounding errors.

Observation 3.	The option with uniform DRX cycle expressed as a rational number consistently introduces less amount of mismatch between the start of traffic and DRX cycles across various frame rates than the option with periodic adjustment of drx-StartOffset.

Observation 4.	The option with uniform DRX cycle expressed as a rational number has much less impact on the legacy DRX formula than the option with periodic adjustments of drx-StartOffset.

Observation 5. 	There is no forward compatibility issue with the option with uniform DRX cycle expressed as a rational number, if the ASN.1 signaling for new DRX cycles is properly designed.

Proposal 1.	Deprioritize options that require non-uniform DRX cycles or multiple on durations within a DRX cycle.

Proposal 2. 	Adopt the option with uniform DRX cycle expressed as a rational number.

R2-2304808	Discussion on C-DRX enhancements for XR	Huawei, HiSilicon	discussion	Rel-18	NR_XR_enh-Core

Non-Integer periodicitry

Proposal1: To address the issue of non-integer periodicities, single DRX configuration with multiple start offsets should be supported.

SFN wrap-around

Proposal2: Introduce E-SFN, which increments at every SFN wrap around and has a pre-defined range such as 0~99, to address the SFN wrap around issue.

Proposal3: Reference SFN is used to help align the E-SFN between the gNB and the UE.

R2-2305652	DRX enhancements for XR	Nokia, Nokia Shanghai Bell, Continental Automotive	discussion	Rel-18	NR_XR_enh-Core

Observation: no difference from the start time point of view for all the (sub)options when the UE wakes up the on Duration. Which option to adopt is more of a modelling issue.

Proposal 1: DRX cycle is defined as hyper cycle / number of cycles per hyper cycle. Possible combinations of hyper cycle and number of cycles per hyper cycle are: (100, 3), (200, 9), (50, 3), (125, 9), (100, 9), (25, 3) for the frame rate of 30fps, 45fps, 60fps, 72fps, 90fps, 120fps.

Proposal 2: for DRX cycle alignment, introduce an integer hyper cycle and number of cycles per hyper cycle as RRC parameters and make the MAC formula change as follows:

1>  if the Long DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset:

Proposal 3: discuss whether it is enough to only support Long Cycle or Short Cycle should also be supported.

Proposal 4: option 3 with drxReferenceSFN is used as the reference for SFN wrap around.

Proposal 5: The formula is changed as follows:

1>	if the Long DRX cycle is used for a DRX group, and [(SFN × 10 + N × 10240 + drxReferenceSFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

Proposal 6: When both hyper cycle and drxReferenceSFN are configured, the formula for Long DRX Cycle would be as follows:

1>  if the Long DRX cycle is used for a DRX group, and floor ([(SFN × 10 + N × 10240 + drxReferenceSFN × 10) + subframe number] modulo ((drx-LongCycle))) = drx-StartOffset :

Proposal 7: the legacy formula should be left untouched and which new one is used depends on whether either hyper cycle or drxReferenceSFN or both are configured.

-	Samsung thinks for rounding errors it’s possible to detect and fix them. Should avoid floor to avoid different implementations. Also thinks the QC solution delays the start time of the DRX period. MTK thinks floor avoids rounding errors.

-	ZTE thinks multiple cycles avoids additional signalling.

-	Huawei thinks for the rational numbers, what is the specification impact of restricting them to integers? Still thinks RAN1/4 impact is there. QC thinks RAN1/4 only refer to the DRX cycle, not integer values.

-	Sony wonders what granularity we need? Thinks there could be dynamic adjustments to make this easier. Samsung thinks we should evaluate whether there is delay for DRX starting point of OnDuration.

Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications.

Offline [205] (QC, Thursday CB): LS to RAN1/4 about this. Final LS in R2-2306564

Offline discussion (Thursday) (1)

[AT122][205][XR] LS to RAN1/4 on new DRX cycles in rational numbers (Qualcomm)

Scope: Provide agreeable LS out on RAN2 decision to based DRX cycle used for XR on rational numbers and request if they see any issues with that.

Intended outcome: Agreeable LS in R2-2306564

Deadline: Deadline 1

R2-2306564	LS on on new DRX cycles in rational numbers	RAN2	LS out	Rel-18	NR_XR_enh-Core	To: RAN1, RAN4

-	CMCC thinks TSN service has the same issues.

LS is approved

Online (Tuesday) (2) – SFN wrap-around

SFN wrap-around:

R2-2304710	Discussion on SFN wrap around problem for XR	Qualcomm Incorporated, Huawei, HiSilicon, Meta	discussion	Rel-19	NR_XR_enh-Core

Observation 1. 	If maximum value of the counter is not specified, there can be inter-operability issue between different spec implementations.

Proposal 1. 	The counter in DRX formula for addressing SFN wrap around is not obtained from system information.

Proposal 2. 	The maximum value of the counter is RRC configured by network during DRX re-/configuration.

Proposal 3. 	Network sets DRX reference SFN drx-ReferenceSFN to either 0 or 512, in the same way as in Rel-16 IIoT.

Proposal 4.	RAN2 discuss and select one of the following options:

•	Option A: both the counter NSFN and the DRX reference SFN drx-ReferenceSFN are added to DRX formula. The initial value of NSFN =0;

•	Option B: only NSFN is added to DRX formula. However, the initial value of NSFN is set according to drx-ReferenceSFN as follows:

◦	If UE successfully receives RRC configuration in SFNUE, UE initializes the counter NSFN to 1 if 0 ≤ SFNUE < 512 and drx-ReferenceSFN = 512 (i.e. there may be ambiguity);

◦	Otherwise (i.e. there is no ambiguity), UE initializes NSFN to 0.

R2-2305898	Considerations for SFN wrap around solution	MediaTek Inc., LGE	discussion	Rel-18	NR_XR_enh

Observation 1: The option of broadcasting E-SFN in system information has small MAC specification impact and would not require a reference SFN but has a small overhead impact in system information (e.g., extra 10 bits in SIB1).

Observation 2: The option of maintaining the E-SFN independently on the UE and the network has slightly more MAC specification impact and would require a reference SFN but it has less signalling overhead.

Observation 3: Shared knowledge of the maximum value of E-SFN by the UE and the network can improve the reliability and inter-operability of different spec implementations.

Proposal 1: Adopt the following enhanced long and short DRX formula by introducing E-SFN:

•	Long DRX: [(E-SFN × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset

•	Short DRX: [(E-SFN × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle)

Proposal 2: RAN2 to discuss and select one of the following options:

•	Option 1: E-SFN is broadcast by the network

•	Option 2: E-SFN is maintained independently by the UE and the network

Proposal 3a: If option 1 in P2 is selected:

•	Network broadcasts the E-SFN in system information (FFS which SIB, e.g., SIB1).

•	FFS the size of the E-SFN field in system information (e.g., 10 bits).

Proposal 3b: If option 2 in P2 is selected:

•	Network provides the reference SFN, SFN_ref, in binary value (i.e., either 0 or 512), as in Rel-16 IIoT via dedicated RRC signalling (in RRC configuration for DRX).

•	If the UE successfully receives RRC configuration for DRX at SFN_UE:

o	If SFN_ref=0 (or not present):

	UE initializes the counter E-SFN to 0.

o	If SFN_ref=512:

	If 0 ≤ SFN_UE < 512, UE initializes the counter E-SFN to 1

	Otherwise, UE initializes the counter E-SFN to 0.

•	UE increments the counter E-SFN by 1 when SFN wraps around 1023 to 0.

Proposal 4: RAN2 to discuss and select one of the following options:

•	Option A: The maximum value of the E-SFN counter is decided by the network and indicated to the UE via RRC signalling (e.g., in DRX configuration)

•	Option B: The maximum value of the E-SFN counter is fixed in the specifications (FFS what the value is, e.g., 999)

•	Option C: There is no maximum value specified for the E-SFN counter (the counter runs to infinity)

Focus on P2, P3a, P3b

R2-2304916	Analysis on introducing H-SFN for DRX formulas	vivo	discussion	Rel-18	NR_XR_enh-Core

Observation 1: Mismatch happens when SFN wrap-around if the DRX cycle is set to a value when 10240ms is not integer times of DRX cycle.

Observation 2: The approach of introducing H-SFN in the C-DRX formulas does not require a reference SFN and no new parameters need to be introduced in the RRC specification.

Observation 3: H-SFN takes nearly 3 hours to wrap around, so not much signaling overhead will be introduced for DRX reconfiguration.

Observation 4: The approach of introducing E-SFN in the C-DRX formulas brings more MAC specification and RRC specification impact than the approach of introducing H-SFN in the C-DRX formulas.

Proposal 1: To handle the SFN wrap-around issue, introduce hyper frame number already existing in SIB1  in the C-DRX formulas. The C-DRX formulas can be enhanced as below:

‐	[(SFN + 1024* H-SFN) × 10+ subframe number] modulo (drx-LongCycle) = drx-StartOffset

‐	[(SFN + 1024* H-SFN) × 10+ subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle)

-	MTK wonders if we could choose between broadcast and non-broadcast. Bcast has NW overhead but there are initialization problems. Unicast has no such problem but has some overhead. Ericsson thinks unicast is better. Nokia agrees.

-	vivo thinks H-SFN has no extra overhead since it uses existing signalling. Huawei thinks we cannot reuse H-SFN since there are still SFN wraparound issues. vivo thinks HSFN is much longer so the problem is minimized. LGE agrees with Huawei. Thinks UE-specific counter may have issues with other mismatch issues. It’s also easier if all UEs get the same information.

Not use broadcast signalling for counter and reference SFN in XR.

R2-2304954	Discussions on DRX enhancements for XR	Fujitsu	discussion	Rel-18	NR_XR_enh-Core

R2-2304968	Enhancements for SFN wrap-around	CATT	discussion	Rel-18	NR_XR_enh-Core

R2-2305006	Discussing on XR-specific C-DRX enhancement	Xiaomi Communications	discussion

R2-2305007	Discussion on power saving scheme for XR	Samsung	discussion	Rel-18	NR_XR_enh

R2-2305017	XR-specific power saving	ZTE Corporation, Sanechips	discussion

R2-2305072	C-DRX enhancements for XR	Apple	discussion	Rel-18	NR_XR_enh-Core

R2-2305159	XR-specific power saving	InterDigital	discussion	Rel-18	NR_XR_enh-Core

R2-2305367	Discussion on DRX enhancements for XR	FGI	discussion

R2-2305456	Discussion on C-DRX enhancement for XR	NEC Corporation	discussion	Rel-18	NR_XR_enh-Core

R2-2305458	Discussion on DRX cycle alignment for XR	ITRI	discussion	NR_XR_enh-Core

R2-2305494	C-DRX Enhancements for XR Traffic	Intel Corporation	discussion	Rel-18	NR_XR_enh-Core

R2-2305543	XR-specific power saving enhancement	Google Inc.	discussion

R2-2305593	Discussion on power saving aspects for XR	Continental Automotive	discussion	Rel-18

R2-2305626	Discussion on the DRX enhancement	CMCC	discussion	Rel-18	NR_XR_enh-Core

R2-2305685	Discussion of DRX enhancement	Lenovo	discussion	Rel-18

R2-2305830	Discussion on XR-specific power saving	Ericsson	discussion	Rel-18	NR_XR_enh

R2-2306143	DRX enhancement for power saving in XR	LG Electronics Inc.	discussion	Rel-18	NR_XR_enh-Core

R2-2306203	Discussion on various frame rates supported for XR-specific power	III	discussion

### 7.5.4	XR-specific capacity improvements

No documents should be submitted to 7.5.4. Please submit to 7.5.4.x

#### 7.5.4.1 BSR enhancements for XR

Including discussion on delay status reporting: What does UE report for the remaining time and how is the reporting triggered? How does UE calculate the remaining time and what is the granularity of the reporting?

Including discussion on how to decide whether to use static or configured BSR tables for XR, explaining the details of the solutions, e.g. selection of BSR table, amount of needed new tables and how they are created (e.g. based on which distributions/parameters), analysis of quantization errors with the proposed solution, BSR MAC CE structure (e.g. extend/reuse current MAC CE format), etc.

Online (Tuesday) (1) – Delay reporting

Delay reporting:

R2-2304955	Discussions on delay information reporting	Fujitsu	discussion	Rel-18	NR_XR_enh-Core

-	For P6, CMCC wonders how the remaining time information is derived based on P1? Fujitsu clarifies that P1 is about the calculation, while P6 is how the reference point of time is determined.

-	Google thinks P1 uses the reference time when the BSR is generated. UE and network should have common understanding of that.

Proposal 1: Remaining time is the PSDB of a PDU set minus the buffered time of the PDU set in PDCP and RLC.

Proposal 6: The remaining time indicates the PSDB of a PDU set minus the buffered time of the PDU set in PDCP and RLC calculated at the initial transmission of the delay information report/BSR MAC CE.

Figure 2 Example of delay information generation and reporting

-	Nokia thinks this requires UE is aware of the PSDB. Thinks UE is only aware of the discard timer, so UE can use that instead of the PSDB. Discard timer could be different than the exact PSDB. Lenovo agrees and UE only cares about the discard timer. QC also agrees and thinks only the total time to schedule matters. Shoudl use discard timer.

-	Intel also agrees but since this is calcualted at the UE, it will be aggregated when reported to network.

-	Huawei thinks we should use PSDB instead of discard timer. This is because some frames may have longer discard values than others (I vs. P). Thinks we could have different discard timers within PDU set. vivo thinks we could just report the buffered time. MTK thinks the current definition is correct. Thinks SA2 indicated there is only one value for discard timer.

-	Samsung thinks we should use “remaining PDCP discard timer”. LGE thinks PDCP doesn’t know the PSDB and only works based on the discard timer. IDT wonders if this is for only one PDU or for all PDUs? LGE thinks this depends on how we design the reporting. Ericson thinks we should have the shortest time. Intel thinks we could have PDUs arriving at different times, which can impact what UE reports. Should address all scenarios.

-	LGE thinks we haven’t agreed this is sent using MAC CE. ZTE wonders why the proposal says this? LGE thinks UE may not need to calculate anything. Samsung thinks “transmitted” implies unspecified time. Apple thinks we need to consider CG cases as well.

1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

Proposal 2: The data volume corresponding to the remaining time is the total amount of PDCP data and RLC data corresponding to the remaining time.

Proposal 3: When the remaining time of a PDU set is lower than a threshold, a delay information report or a BSR (if extension of BSR is agreed for delay information report) is triggered.

Proposal 4: The index indicating a range of the remaining time can be included in the delay information report.

Proposal 5: A table for including the index of the remaining time and the corresponding range of the remaining time is defined in MAC specification.

Online (Tuesday) (2) – Static vs. configured BSR tables

BSR tables:

R2-2305149	New BS table(s) and BSR trigger(s)	NEC	discussion	Rel-18	FS_NR_XR_enh

BSR table generation

Proposal 1	New BS table is generated based on gNB’s configuration

For BS configuration:

Proposal 2	In addition to legacy BS table(s), Network configure at most one additional BS table a LCG is eligible to use. Different BS table can be configured for different LCG

Proposal 3	Network configure Bmin and step size X for each new BS table. (Bmax is equal to Bmin+ X*number of codepoints)

For BSR MAC CE:

Proposal 4	Per LCG BS report is taken as baseline. Higher granularity than per LCG BS report can be discussed together with remaining delay report.

Observation 1: it should be possible not to apply new BS table for SRB(s) and DRB not for XR traffic

Observation 2: UE should be able to fall back to legacy BS table(s) for a LCG which is configured/enabled with new BS table (e.g., in case the buffer size becomes small after being scheduled)

Proposal 5	BS table indication per LCG is introduced in BSR MAC CE, to enable UE to switch BS table depending on the buffer size

Proposal 6	For a long or long truncated BSR MAC CE, BS field has fixed length of 8 bits.

Proposal 7	if new BS table is configured/enabled for a LCG, for a short or short truncated BSR MAC CE, BS field has fixed length of 8 bits too, either legacy or new 8bit BS table could be used.

Regarding new BSR trigger(s):

Proposal 8	Introduce new BSR trigger based on unknown/unexpected buffer size change(not due to being scheduled, but due to discard or additional data arrival)

Proposal 9	Introduce new BSR trigger based on UL grant size is bigger than a threshold

Focus on P1-3

R2-2304711	BSR and delay status report for XR	Qualcomm Incorporated	discussion	Rel-18	NR_XR_enh-Core

New BSR table

Observation 1. 	It is possible to define a single new BSR table that achieves an average quantization error of ~1% as follows:

•	It covers all common bit rates and frame rates;

•	Use linear distribution and 8-bit BS field length;

•	Choose maximum BS based on maximum bit rate and minimum frame rate;

•	Choose minimum BS such that no BS in the new table has a quantization error worse than the legacy one.

Observation 2. 	The new BSR table generated by the method in Observation 1 is forward compatible because the entire table scales linearly with maximum bit rate.

Observation 3.	If new BSR table(s) is RRC configured, then parameters and formula used to generate a new BSR table should be defined in a way that different UE/NW implementations can produce the same table.

Proposal 1. 	Introduce a single pre-defined BSR table, e.g. derived based on the method described in Observation 1.

Proposal 2.	If new BSR table(s) is RRC configured,

•	Network configures UE with minimum buffer size Bmin, distribution type (linear or exponential), and step size factor p.

•	Buffer size Bk can be generated according to the following formula: B1 = Bmin, and Bk = Bk-1 + floor(BS x p), for k=2, …, N, where BS = Bmin linear distribution and BS = Bk-1 for exponential distribution.

Proposal 3.	The length of BS field is 8 bits, regardless of whether new BSR table(s) is pre-defined or RRC configured.

Proposal 4.	If there are multiple BSR tables (including the legacy one) for UE to use, then UE should always choose the one which has the least quantization error for the buffer size to be reported.

Proposal 5. 	The new BSR MAC CE should include an indication by UE that which BSR table it used for reporting.

Focus on P1-5

- 	QC thinks the linear calculation can still be shifted or scaled using different signalling in the future.

-	Ericsson thinks that we should allow different codepoints. Configurability provides this more easily.

-	ZTE agrees with QC analysis. Range could still be FFS. Linear and fixed table can still be useful. Thinks reconfiguration can be a problem so could require reconfiguration with sync. Nokia thinks this is not specific to BSR configuration. Futurewei thinks it’s unlikely we get different distributions in the near future. MTK supports NEC proposal and thinks reconfiguration with sync is not a problem. Should be future-proof and not require new WI for new BSR tables.

-	LGE thinks any manipulation of existing tables is also complex.

Show of hands (support)

Static BSR table: LGE, QC, vivo, Lenovo, Apple, Futurewei, Google, ZTE, Xiaomi, OPPO, Sony, Spreadtrum, Fujitsu, Huawei, ETRI, TCL (16)

Configurable BSR table: NEC, CATT, CMCC, Samsung, Nokia, Intel, Interdigital, Ericsson, MTK, Verizon, Vodafone, KDDI (12)

-	Vodafone wonders if we go for majority, then we will introduce configurability in the next release?

Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

Delay status reporting

Observation 4.	It is not necessary for UE to report delay status of every QoS flow, e.g. those without stringent delay requirements.

Proposal 6.	Network can configure which LCG(s) should report its delay status.

Proposal 7.	UE triggers a DSR when an LCG configured for reporting and its associated L2 buffer has data whose remaining time drops below a configured triggering threshold.

Proposal 8. 	The remaining time of a PDU is defined as the residual value of its associated PDCP discard timer.

Proposal 9.	Network can also configure an LCG to periodically report its delay status.

Proposal 10.	Network can configure one or more reporting thresholds for an LCG. For each reporting threshold, UE reports the amount of data whose remaining time is below that threshold.

Proposal 11.	The remaining time reported in a DSR is the duration between the time when the DSR is transmitted and the delay deadline of the corresponding PDU.

R2-2306130	Discussion on MAC enhancements for XR-specific capacity improvement	Huawei, HiSilicon	discussion	Rel-18	NR_XR_enh-Core

For the design of BSR MAC CE:

Observation 1:	Reporting only a single value of remaining time per LCG when there are multiple data bursts with different remaining times in a LCG is not sufficient for the gNB to perform efficient traffic scheduling.

Proposal 1:	Define a new BSR MAC CE format indicating the data volume together with its associated remaining time.

Proposal 2:	UE should be able to report multiple BS values and their associated remaining times in a single BSR MAC CE when there is data with different remaining times within the same LCG.

Proposal 3:	UE should have at most one new BSR table applicable per LCG. The UE determines whether to use the new table or the legacy table for each LCG based on the buffer size and the new table’s range, i.e. new BSR table is used in case BS value falls within its range, otherwise UE uses legacy BSR table.

Proposal 4:	The length of index in the new BSR tables should be the same as Long BSR format, i.e. 8 bits.

Proposal 5:	The current value of the PDCP discard timer of a data burst is used as the remaining PSDB (i.e. “remaining time”) of the data burst.

Proposal 6:	The remaining time is signalled with an index which corresponds to a certain remaining time threshold pre-configured by the network

For the design of new BS tables:

Observation 2:	Ideally, the target BSR quantization error shall be less than 100 bytes, to minimize the resource wastage.

Observation 3:	New BS tables are limited to 256 rows which limits the range of the new BSR table if the target quantization error is to be achieved.

Observation 4:	The quantization error may still exist since the new BS table cannot match with all XR services well.

Observation 5:	Even if the traffic pattern is known, the buffer can contain traffic from multiple LCHs and from different data bursts, making it impossible to come up with the proper BSR table

Proposal 7:	New BSR tables should be designed with at least: linear distribution of code points and step size of 100 bytes.

Proposal 8:	In order to alleviate the quantization error, RAN2 can consider the following approach:

1.	Pre-define multiple buffer size tables in combination with network indicating which buffer size table the UE can use.

2.	If the buffer size is not within a new table’s range, the UE uses an index referring to the legacy BSR table.

3.	For the large buffer size/large quantization error, the UE uses an additional index to indicate a more precise range of buffer size (within the “roughly” range indicated in step 2).

For the trigger of BSR:

Proposal 9:	The following BSR triggering enhancements should be introduced:

-	trigger BSR when a new data burst arrives (alternatively: enhance the Periodic BSR by allowing the periodicBSR-Timer not to be restarted by other transmitted BSRs);

-	trigger BSR when the data volume of discarded packets exceeds a threshold.

Proposal 10:	UE should trigger the BSR with the remaining time when the remaining time is smaller than a pre-configured threshold.

For LCP restriction:

Observation 6:	In case the UE is not able to prioritize urgent data during UL transmission, the usefulness and purpose of the remaining time reporting in the BSR is unclear.

Proposal 11:	Introduce a new LCP restriction related to the remaining time of the data buffered in a certain LCH.

R2-2305828	Discussion on BSR enhancements for XR	Ericsson	discussion	Rel-18	NR_XR_enh

Proposal 1	Introduce RRC-based BS tables i.e. the NW provides a configuration for the UE to build additional BS tables

Proposal 2	NW may configure 1 table for the newly defined Short BSR.

Proposal 3	It is preferable that the NW can configure more than one table per LCG for the newly defined long BSR. Value to be selected depending on the BSR format (see Section 2.3)

Proposal 4	BS tables are defined by: an index, min value, max value, and stepSize.

Proposal 5	If stepSize is not provided, the UE calculates the step size as the (max value – min value) / (nr of indexes in the BS table)

Proposal 6	Tables are built as exemplified in Figure 2: For BS index 0, BS value is defined by: [ ≥ min value & ≤ min value x (stepSize x (BS index + 1) For next BS indexes, BS value is defined by [≤ min value x (stepSize x (BS index + 1)] Until reaching the max value.

Proposal 7	The NW may associate a LCG with up to “aa” (aa depends on Proposal 2) tables for a new long BSR

Proposal 8	Adopt the ASN.1 outlined above for the RRC-based BS table configuration.

Proposal 9	Only linear distribution is allowed for BS table creation.

Proposal 10	One new 5-bit BS table can be configured per LCG (as in Proposal 2).

Proposal 11	The new BSR has the same format as the legacy short BSR.

Proposal 12	The LCID index in the MAC subheader to identify this new short BSR should be taken from the eLCID field

Proposal 13	RAN2 to discuss if more tables can be configured.

Proposal 14	BS format can indicate up to 3 additional 8-bit BS tables (in addition to legacy table)

Proposal 15	Select option 4 for the table/BS format.

Proposal 16	The eLCID (1 octet) is used to for this new long BSR MAC CE.

Proposal 17	When multiple tables contain an index which can represent the UE buffer size, the UE shall use the BS table/index that minimizes the index error (smallest different between the minimum and maximum value represented by the index) and represents the UE buffer size.

Proposal 18	When only one table (including legacy BS tables) contains an index which represents the UE buffer size, the UE shall use the corresponding BS table and format.

Proposal 19	Current BSR triggering conditions are the baseline conditions for the new BSR introduced in Section 2.1.

Proposal 20	Delay reporting should also provide buffer information utilizing new defined BS tables.

Proposal 21	Delay reporting is done by indicating bucket indexes similar as for the buffer status, per LCG.

Proposal 22	Two delay tables per LCG can be configured: one for short delay reporting, another table for long delay reporting.

Proposal 23	A delay table is defined by: - min value, - max value, and - stepSize.

Proposal 24	Up to 8 buckets can be configured for long delay reporting. 1 bucket is enough for short delay reporting (see 2.2.3)

Proposal 25	For long delay reporting, if stepSize is not provided, the UE calculates the step size as the (max value – min value) / (nr of buckets e.g., 8)

Proposal 26	Delay table is built as: For index 0, BS value is defined by: [ ≥ min value & ≤ min value x (stepSize x (BS index + 1) Second and third index, BS value is defined by [≤ min value x (stepSize x (BS index + 1)] Last bucket index is defined by ≥ min value x (stepSize x (BS index + 1)] or ≥ max value (if provided)

Proposal 27	For short delay reporting, min and max value, or min and step size needs to be provided.

Proposal 28	Adopt the ASN.1 outlined above to configure the delay table.

Proposal 29	Delay reporting is triggered when new data enters an empty delay bucket. The buckets which trigger the delay reporting are configured by the network

Proposal 30	Delay reporting represents the waiting time for the PDU set since the first packet of the PDU set arrived to the UE buffer.

Proposal 31	The UE reports the buffer status in each of the delay/latency buckets.

Proposal 32	A short delay reporting is introduced. Its format is the same as the legacy BSR

Proposal 33	A short delay reporting indicates the highest priority LCG configured with delay reporting which has data in a bucket configured by the network.

Proposal 34	One bit is used to indicate the presence of data in a bucket.

Proposal 35	Adopt option 1:

a.	1 byte is introduced to indicate 8 buckets.

b.	BS is reported using Option 4 as in Proposal 15

Proposal 36	The eLCID (1 octet) is used to for this new long delay reporting MAC CE.

R2-2306176	BSR enhancements for XR	MediaTek Inc.	discussion	Rel-18

Observation 1: In XR Traffic model, two factors 1) data rate (R) and 2) frames per second (FPS) determines the XR traffic characteristic, including mean, maximum and minimum packet sizes.

Observation 2: XR bitrate adaptation mechanism leads to different mean, maximum and minimum packet sizes.

Observation 3: The RRC configured BSR table can avoid quantisation errors of any form and can achieve better target level of quantization error than static BSR tables.

Proposal 1: RAN2 to adopt RRC configured BSR tables mechanism for new BSR table(s)

Proposal 2: gNB to control which BSR table is used by UE via presence of RRC BSR IE(s).

Proposal 3: BSR MAC CE format can be the same as legacy BSR MAC CE.

Proposal 4: BSR entry is generated using the formula Bmin + (k - 1) * step-size, where Bmin and step size and configured by the NW.

	If Bmin and step size are not configured, use legacy BSR table.

	If configured, use new linear BSR table.

Proposal 5: UE chooses minimum delay value of buffered data as reported information.

Proposal 6: UE reports delay information with BSR.

Proposal 7: The reported delay information could be a range.

Proposal 8: RAN2 to study new trigger condition for BSR for XR services.

Online (Tuesday) (1) – Support of piecewise linear BSR table

Support of piecewise linear BSR tables:

R2-2305604	Consideration on Piecewise Linear BS Table	CMCC, Huawei, HiSilicon, China Unicom	discussion	Rel-18	NR_XR_enh-Core

Observation 1: Although there are some traffics of pose information, video traffics still occupy a large proportion in the UL XR traffics, which issues desire to be addressed as well.

Observation 2: Piecewise linear distribution has the least quantization error compared to exponential and linear distribution.

Based on the above observation, we propose that:

Proposal: RAN2 to agree that piecewise linear distribution can be used for new BS table(s) for XR traffic.

We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

R2-2304826	Discussion on BSR and DSR for XR	TCL Communication Ltd.	discussion	Rel-18

R2-2306242	Discussion on delay information for XR	Google Inc.	discussion	Rel-18	NR_XR_enh-Core

R2-2305364	Generate new buffer status report table	FGI	discussion

R2-2306346	Discussion on new BSR table and delay information report	LG Electronics Inc.	discussion	Rel-18	NR_XR_enh-Core

R2-2306393	XR BSR and Delay Information Enhancements	Meta USA	discussion	Rel-18	NR_XR_enh-Core

R2-2304861	BSR Enhancements For XR	Dell Technologies	discussion	Rel-18

R2-2304864	Further discussions on BSR enhancements for XR	Futurewei	discussion	Rel-18	NR_XR_enh-Core

R2-2304917	Discussion on BSR enhancements for XR	vivo	discussion	Rel-18	NR_XR_enh-Core

R2-2304969	On BSR Enhancements	CATT	discussion	Rel-18	NR_XR_enh-Core

R2-2305002	Discussing on BSR enhancements for XR capacity	Xiaomi Communications	discussion

R2-2305018	BSR enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2305073	Views on BSR Enhancements for XR	Apple	discussion	Rel-18	NR_XR_enh-Core

R2-2305388	Discussion on BSR enhancements for XR	Honor	discussion

R2-2305454	Discussion on BSR enhancement for delay information report	NEC Corporation	discussion	Rel-18	NR_XR_enh-Core

R2-2305495	BSR Enhancements for XR Traffic	Intel Corporation	discussion	Rel-18	NR_XR_enh-Core

R2-2305514	Considerations on XR UL PDU set information	Sony	discussion	Rel-18	NR_XR_enh-Core

R2-2305515	Some considerations on BSR enhancements for XR	Sony	discussion	Rel-18	NR_XR_enh-Core

R2-2305533	Discussion on BSR enhancement for XR	OPPO	discussion	Rel-18	NR_XR_enh-Core

R2-2305571	Consideration on BSR enhancements for XR	Spreadtrum Communications	discussion	Rel-18

R2-2305653	BSR enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_XR_enh-Core

R2-2305723	Discussion on BSR enhancements for XR	Lenovo	discussion	Rel-18	NR_XR_enh-Core

R2-2305816	BSR enhancements for XR	Interdigital Inc.	discussion	Rel-18	NR_XR_enh-Core

R2-2306243	Discussion on BSR enhancements for XR	Google Inc.	discussion	Rel-18	NR_XR_enh-Core

R2-2306252	Discussion on residual resource allocation for XR 	Google Inc.	discussion	Rel-18	NR_XR_enh-Core

R2-2306275	Discussion on BSR enhancements for XR	III	discussion	NR_XR_enh-Core

R2-2306353	Discussion on BSR enhancements for XR	Samsung	discussion	Rel-18	FS_NR_XR_enh

#### 7.5.4.2 Discard operation for XR

Including discussion how the achieve PDU-set based discard in PDCP layer works for UL and DL  and how is that specified (e.g. is there need for any PDCP CEs).

Including discussion on whether PDU set discard at PDCP impacts RLC layer (e.g. does discarding at PDCP also trigger discarding at buffered RLC PDUs).

Online (Tuesday) (1) – Is PSIHI configured for UL?

R2-2305019	PDU discard for XR	ZTE Corporation, Sanechips	discussion

Observation 1: When PSIHI indication is configured, if one of the PDUs within the PDU Set is lost/discarded, then, the entire PDU Set is discarded.

Proposal 1: PDCP entity can notify RLC layer to discard PDUs within a PDU set.

Proposal 2: PSIHI indication for UL is configured using RRC (in the PDCP-Config) to handle the PDU Set based discard functionality.

Proposal 3: The PDCP layer can take into account the PDU Set related information to enable PDU Set based discard functionality and no new fields are added in SDAP or PDCP header to enable this.

Proposal 4: When the PDCP entity discards the PDUs belonging to a PDU Set, this should be indicated to the receiving PDCP entity.

Proposal 5: The PDCP Control PDU format for PDCP status report is used as the baseline for indicating discarded PDUs.

Proposal 6: RAN2 discuss whether PDCP Control PDU format for PDCP status report should be optimized to indicate the discarded PDUs in one PDU set.

Proposal 7: When PSIHI indication is configured, if the PDCP transmitter entity discards the PDU Set, any PDUs belonging to the discarded PDU Set that are already received at the receiver should also be discarded at the receiving PDCP entity.

Focus on P2

-	Nokia thinks this is fine and the granularity is per DRB. Ericsson thinks this is optional information. Thinks this is network information. Intel agrees with Nokia on per DRB and for PSIHI, SA4 already agreed that it’s always set for Rel-18. MTK thinks we should make it simple. vivo thinks the PSIHI is per QoS flow. OPPO wonders if this means UE doesn’t need to know PSIHI information?

Proposal 2: PSIHI indication for UL is configured using RRC (in the PDCP-Config) to handle the PDU Set based discard functionality.

2: PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity.

Online (Tuesday) (1) – PDU set discard operation impacts to PDCP and RLC

Discard operation details:

R2-2305191	Discard operation for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: when PSIHI is configured for a DRB, upon timer-based discarding of a PDCP SDU, all other SDUs belonging to the same PDU set are discarded (if still stored) and indicated to lower layers to be discarded (regardless of if still stored at PDCP).

Proposal 2: an additional early-discard timer is introduced at PDCP whose expiry time can depend on PSI (as opposed to the current discard timer that should depend on P(S)DB).

Proposal 3: upon expiry of the early-discard timer for a PDCP SDU, the SDU is discarded only if there are more important SDUs (in terms of PSI) associated with higher Count value stored for transmission.

-	Lenovo thinks PSI should be taken into account. But not necessarily with two discard timers. ZTE thinks congestion is about radio link quality. Different levels of congestion doesn’t seem needed. Huawei support considering PSI but not two discard timers. vivo also supports considering PSI. MTK thinks we shoudl consider PSI. Details can be considered later. QC agrees. Ericsson agrees using different timers. LGE also thinks PSI should be considered.

Coffee break offline on PSI usage

-	Nokia reports that the discussion converged somewhat:

PSI should be taken into account in discard.

- Network signals congestion indication. UE immediately discards packets up to given PSI level based on receiving the indication.

- Rely on UE to discard with two timer( value)s. Which timer to use depends on PSI. Only used when UE detects congestion.

- Rely on UE to discard with two timer( value)s. Which timer to use depends on PSI. Network indicates to UE which timer value is used.

Offline discussion [204] (Nokia, Thu CB): Clarify the options for PSI-based discard based on online discussion: How does each option work?

Offline discussion (Thursday) (1)

[AT122][204][XR] Discard (Nokia)

Scope: Clarify the options for PSI-based discard based on online discussion: How does each option work?

Intended outcome: Discussion report in R2-2306565

Deadline: Deadline 1

R2-2306565	Report of [AT122][204][XR] Discard (Nokia)	Nokia	discussion	Rel-18	NR_XR_enh-Core

-	Intel thinks we need to discuss the proposals in the next meeting. Should focus on P5.

-	LGE thinks O1 is not fully correct.

-	Nokia clarifies that O2 was because it was not clear how congestion is handled. ZTE thinks DL signalling could even cause UL signalling.

-	MTK thinks O3 is not needed.

-	Lenovo thinks we could just have agreement that UE just discards only based on discard timer regardless of any NW indication used or not.

-	Interdigital thinks we have two things to consider: What happens to data in the buffer and what happens after this? Is this a one-shot thing or also subsequent packets?

-	ZTE thinks there should not be ACKs in the UL for this. MTK is not sure this is possible since we have HARQ.

-	KDDI wonders what happens if UE doesn’t receive the indication?

-	CATT thinks majority thinks this is dedicated signalling.

Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling.

FFS how/whether to minimize additional UL signalling after this indication.

FFS if the NW indication is a one-shot or also subsequent packets

Proposal 4: PSI-based discard mechanism in uplink for XR can assume that DL signalling can be used.

-	Samsung thinks P4 is about network congestion. Apple thinks we should discuss how UE determines there is congestion. Nokia thinks it was not obvious that network tells to UE there is discard due to congestion, that’s why it talks about signalling. P4 does not exclude solution via DL signalling.

Proposal 5: if network signalling for PSI-based discard is introduced, it should be dedicated.

-	Intel thinks the mechanism might be based on PSI but not only. Ericsson agrees and thinks this is only about NW configuration about different discard mechanism. It  could have different timers per PSI level, but this depends on the details. Huawei thinks we should use “in case of congestion”. Apple thinks “NW signalling” is ambiguous. QC thinks this is changing the proposal and this introduces NW signalling of congestion. Nokia thinks this is about activation.

Proposal 5: if network signalling for PSI-based discard is introduced, it should be dedicated.

Proposal 5: if PDU set discard considering PSI in case of congestion is introduced, it is indicated via dedicated signalling.

Does UE or network detect congestion?

When congestion is detected, how does UE discard the packet?

-	Intel thinks SA4 already agreed on PSI levels. Might provide 16 different timer values (one per PSI level). Leaving this up to UE will mean NW doesn’t know when the timer would expire. Nokia thinks this doesn’t matter since anyway UE can discard. We just need to be able to handle gaps in the received PDUs.

-	Huawei wonders if the two timers are per PDU set, or per PDCP entity? Nokia clarifies this would be two timers per PDU. Apple thinks this means two timers per PDCP entity. Apple thinks for option 1 would not be congestion level, just network indication to UE on PSI discard. CMCC thinks anyway network would signal something for the UE.

-	LGE wonders if the second timer would be shorter than normal discard timer, so when does the UE perform discard based on that? Would that be network indication?

-	ZTE thinks PDCP entity woul dhav etow timer values. UE would look at the PSI and start one of the timers. Huawei thinks network could tell UE which value to use in option 2. Ericsson thinks this is just about signalling different mode from network: Ue then uses different discard timer based on that. MTK thinks discarding immediately can become very costly for UL traffic. CATT calrifies that above PSI level, it’s regular PDCP timer.

Proposal 4: in both PDCP and RLC AM, introduce an indication from the transmitting entity to the receiving entity that reception of PDU(s) with given SN(s) is not to be expected.

Proposal 5: when indicated from upper layer (i.e. PDCP) to discard a particular RLC SDU, the transmitting UM RLC entity shall discard the indicated RLC SDU even if a segment thereof has been submitted to the lower layers.

-	Google supports P5 but also for AM mode. LGE thinks P4-5 may work but it’s an optimization. Thinks that will complicate RLC operation. RLC PDU transmission also depends on implementation. Also reception of discarded PDUs causes no problems. Ericsson is OK for RLC UM but not for RLC AM. Lenovo supports P5 since XR is expected to do more discarding to increase capacity. Intel agrees. QC thinks for UM mode there is no storing of SDUs. For AM this depends on PSIHI. Nokia thinks there are restrictions in specification.

-	ZTE wonders if this is for UL or also for DL? If it’s for DL, this could have RAN3 impact. Nokia only considered UL.

-	KDDI wonders if this is used under congestion or in normal cases? Nokia thinks this is the abnormal cases. KDDI wonders who judges if this is an abnormal case? Nokia thinks gNB sets the discard timer accordingly. That will handle the case. Doesn’t want additional signalling since that only worsens congestion.

-	Apple thinks for AM this is not needed. For UM it could be but is not sure.

R2-2304918	Discussion on discard operation for XR	vivo	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: It is up to gNB to configure the value of discardTimer for each PDCP SDU of PDU set based on PSDB.  FFS values of discardTimer.

Proposal 2: If PSIHI is indicated, when one SDU associated with a PDU set has not been successfully transmitted, the rest of this PDU set should be discarded.

Proposal 3: Whether need to keep the integrity of the PDU set (i.e. PSIHI) is per QoS flow indicated from CN to RAN via the control plane, e.g. from AMF by NAS signaling.

Proposal 4: In case congestion occurs, PDU set with lower importance (PSI) could be discarded. FFS how to determine the congestion.

Proposal 5: For UL, PDU set importance (i.e. PSI) is indicated from upper layer at UE side (e.g. by UE implementation)

Proposal 6: PDCP needs to inform RLC in case PDU set is discarded due to PSIHI.

Proposal 7: PDCP needs to inform RLC in case PDU set is discarded based on PSI in case of congestion.

Proposal 8: Upon discard is indicated from PDCP, RLC discards the indicated RLC SDU regardless of whether the RLC SDU nor a segment has been submitted to the lower layers.

Proposal 10: If in-sequence delivery is configured, the transmitter should inform the receiver of the DL PDU set discard information.

Proposal 11: If UE is configured to report PDU set discard information, the UE will inform gNB UL PDU set discard information.

Proposal 12: Transmitter informs the receiver of discard information when there is PDU set discard.

Online (Tuesday) (1) – How does PSI impact UE operation for discard?

R2-2305784	Discard Operation for XR	Samsung R&D Institute India	discussion	Rel-18

Observation 1: It is in the gNB remit to use PSIHI information when configuring PDCP discard per PDU Set basis.

Observation 2: The approach of using different discard timer values for PSIs may not be effective when congestion status changes dynamically, as there will be time delays from the congestion indication to actual packet discarding. While PSI threshold based approach can tackle congestion situation with early discard of less important PDU Sets, regardless of expiry of discard timer.

Observation 3: Discard operation at RLC layer is not always achievable. Discard enhancements on RLC may introduce RLC SN gap, and there may be undesired complexity for Tx and Rx RLC entities. It seems reasonable to avoid such large specification impact and efforts involved.

Observation 4: It is possible that receiver side PDCP may receive incomplete PDU Set, when PDU Set based PDCP discard is carried out at the transmitter side PDCP.

Observation 5: Different application media layer mappings and receiver implementations can be addressed by the PDU Set concept and the media/application layer should be able to configure the appropriate handling.

Observation 6: With the PDU Set identification information signalling, PDCP operation can be facilitated e.g. receiver side PDCP can easily identify whether the PDU Set is completely received, or is incompletely received. Existing PDCP SN can be reused and be further complimented with additional embedded signalling information to indicate start PDU, in-between PDU and end PDU of the PDU Set as part of the PDCP header.

Proposal 1: gNB configures the UE with PSIHI based configuration per XR DRB for performing PDCP discard per PDU Set basis in the uplink.

Proposal 2: gNB configures the UE per XR DRB, either to use the expiry of the PDCP discard timer to discard respective PDCP SDU (legacy behavior) or to use the expiry of the PDCP discard timer pertaining to the first SDU of the PDU Set to discard the PDCP PDU Set (new behavior).

Proposal 3: RAN2 is kindly asked to agree:

•	Upon a NW congestion, PSI based PDCP discarding can be activated.

•	NW congestion can be detected by the gNB. When the gNB indicates the congestion to the UE, the UE starts PSI based PDCP discarding.

Proposal 4: RAN2 is asked to discuss and select PSI based approach among:

•	Option 1: Timer based discarding with different timer value for different PSI

•	Option 2: PSI threshold based discarding (regardless of expiry of discard timer)

Proposal 5: Discard enhancements for PDU set should be limited to the PDCP layer and no enhancements are pursued for RLC layer.

Proposal 6: RAN2 to discuss potential enhancement on receiver side PDCP to handle and deliver received PDUs to application layer considering following:

a)	If all PDUs are needed for the usage of PDU Set by application layer, the PDCP does not deliver the received PDUs of the incompletely received PDU Set to the application.

b)	If all PDUs are not needed for the usage of PDU Set by application layer, PDCP delivers the received PDUs of the incompletely received PDU Set to the application.

c)	It is configurable to the receiver PDCP entity whether a) or b) is required.

Proposal 7: RAN2 is to discuss the PDU Set identification information signalling to facilitate the receiver PDCP operation.

Focus on P3-4

R2-2304712	Discussion on discard operation	Qualcomm Incorporated	discussion	Rel-18	NR_XR_enh-Core

R2-2304827	Discussion on discard indication for XR	TCL Communication Ltd.	discussion	Rel-18

R2-2304956	Discussions on PDU discard based on PDU Set Importance	Fujitsu	discussion	Rel-18	NR_XR_enh-Core

R2-2304970	Discard Operation for XR	CATT	discussion	Rel-18	NR_XR_enh-Core

R2-2305001	Discussing on PDU discarding of XR traffic	Xiaomi Communications	discussion

R2-2305012	PDU discard	CANON Research Centre France	discussion	Rel-18	NR_XR_enh-Core

R2-2305074	Views on PDU Discard Operation for XR	Apple	discussion	Rel-18	NR_XR_enh-Core

R2-2305150	PDU discard	NEC	discussion	Rel-18	FS_NR_XR_enh

R2-2305160	Discard operation for XR	InterDigital	discussion	Rel-18	NR_XR_enh-Core

R2-2305457	Discussion on the issues of PDU-Set discard	ITRI	discussion	NR_XR_enh-Core

R2-2305496	Discard Enhancements for XR Traffic	Intel Corporation	discussion	Rel-18	NR_XR_enh-Core

R2-2305534	Discussion on discard operation for XR	OPPO	discussion	Rel-18	NR_XR_enh-Core

R2-2305566	Discussion on XR discard	Spreadtrum Communications	discussion	Rel-18

R2-2305635	PDU-Set Discard operation for XR	CMCC	discussion	Rel-18	NR_XR_enh-Core

R2-2305724	Discard operation for XR communications	Lenovo	discussion	Rel-18	NR_XR_enh-Core

R2-2305829	Discussion on PDU Discard	Ericsson	discussion	Rel-18	NR_XR_enh

R2-2305899	Further aspects of PDU discard	MediaTek Inc.	discussion	Rel-18	NR_XR_enh	R2-2303303

R2-2306106	Further discussions on discard operation for XR	Futurewei	discussion	Rel-18	NR_XR_enh-Core

R2-2306121	Discussion on PDU Set discard in PDCP layer for DL and UL	ASUSTeK	discussion	Rel-18	NR_XR_enh-Core

R2-2306137	Discussion on PDU set discarding for XR traffic	Huawei, HiSilicon	discussion	Rel-18	NR_XR_enh-Core

R2-2306331	Discussion on the discard for XR	LG Electronics Inc.	discussion	NR_XR_enh-Core

R2-2306402	Discussion on PDU Discard Operation for XR	Meta USA	discussion	Rel-18	NR_XR_enh-Core

#### 7.5.4.3 Configured Grant enhancements for XR

Including RAN2-specific aspects of Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration.

Including RAN2-specific aspects of dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE.

Including discussion on retransmission-less CG, e.g. how does the solution discussed in RAN2#121bis-e ensure consistent HARQ operation?

Online (Thursday) (2) – Retransmission-less CG for XR

Retransmission-less CG (already discussed in RAN2#121bis-e):

R2-2304809	Discussion on retransmission-less CG for XR	Huawei, Apple, Futurewei, Google, HiSilicon, Intel,  Lenovo, MediaTek, Meta, Qualcomm	discussion	Rel-18	NR_XR_enh-Core

Review of NTN solution

Observation1: Retransmission-less NTN solution is applied for certain HARQ processes carrying services that do not require HARQ retransmission for both CG and DG.

Why HARQ RTT Timer should not be disabled for DG

Observation2: For pose control information, retransmission by PDCCH addressed to CS-RNTI should be allowed that the DRX RTT/ReTx timer should not be disabled per HARQ process for both CG/DG.

Observation3: Retransmission for UL grant scheduled by PDCCH addressed to C-RNTI that share the same HARQ process as CG carrying pose control information should not be disabled.

Proposed Way-forward

Observation4: The legacy logical channel restriction allowedCG-List can be reused for retransmission-less CG

Proposal1: For retransmission less CG enhancement in XR, adapt the NTN solution by disabling the HARQ RTT timer per CG configuration. Specifically, the following modifications shall be introduced:

•	A new RRC parameter for disabling drx-HARQ-RTT-TimerUL for a CG configuration;

•	Changes in the procedural text of DRX operations for CG in the MAC specification;

•	A new UE capability for supporting disabling drx-HARQ-RTT-TimerUL for a CG configuration.

For retransmission less CG enhancement in XR, adapt the NTN solution by disabling the HARQ RTT timer per CG configuration. Specifically, the following modifications shall be introduced:

- •	A new RRC parameter for disabling drx-HARQ-RTT-TimerUL for a CG configuration;

- •	Changes in the procedural text of DRX operations for CG in the MAC specification;

- •	A new UE capability for supporting disabling drx-HARQ-RTT-TimerUL for a CG configuration.

R2-2305654	Retransmission-less operation	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: reuse the mechanism from NTN with retransmission-less operation supported for both dynamic grant and configured grant, thus no new RRC parameter and capability is needed.

Proposal 2: update the DRX section in MAC to start drx-HARQ-RTT-TimerUL only for HARQ mode A and remove the restriction of limiting the uplink-Harq-ModeB capability to NTN, as TP provided in the annex.

-	Ericsson thinks the Huawei proposal is not needed since network can just not apply this for DG.

-	Interdigital thinks that to ensure consistent behaviour, the Huawei proposal is better. CATT thinks this feature is not really needed so prefers to use existing solution. NEC agrees and support the Nokia proposal.

-	Apple supports Huawei proposal and points out that per-HARQ process approach will require additional per-LCH rules. Lenovo, Sony and MTK agrees. vivo supports Nokia proposal.

-	CMCC supports Huawei proposal since configuration is per symbol level. The limitations with HARQ process are too restrictive. OPPO thinks Huawei proposal is better but does not agree with O2 from Huawei.

-	LGE has concerns with both options. Thinks we should decide whether to decide same HARQ mode and then whether we allow retransmission for retransmission-less.

-	Ericsson thinks reTx-less allowing ReTxs is because UE always follows the grant. Anything else is error case which was not specified to avoid error cases. We also have 16 HARQ processes in Rel-15 already and don’t need more than 6 of them normally, so 10 can be used for CGs. Then Rel-17 added up to 32 HARQ processes so it’s possible to allocate some HARQs for CG and some for DG only. MTK thinks 32 HARQ is only for NTN and more than 60 GHz. Not for FR1.

-	Nokia thinks 32 HARQ was because of known long RTT time. In this case we might only need one HARQ process so that’s why there is no problem of running out of HARQ processes. Interdigital explains NTN discussion only brought CG at late phase, and the current mechanism was the simplest proposal. Here CG is the main use case, not DG. There also was a dummy timer for NTN solution which is needed at gNB for this. Intel agrees and support Huawei solution.

-	Nokia thinks the changes to MAC are the same, it’s all about RRC parameters.

Round 1

Concerns with existing retransmission-less: Huawei, MTK, Sony, Apple, IDT, Lenovo, QC, LGE

Concerns with Huawei proposal: Ericsson, ZTE, Nokia, CATT, vivo, LGE

Round 2

Concerns with Huawei proposal: -

Concerns with existing retransmission-less: QC

R2-2305741	Discussion on retransmission-less CG	Samsung	discussion	Rel-18	NR_XR_enh-Core

R2-2305517	Options for Retransmission-less CG for XR traffic	Sony	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: Proposal 1: RAN2 should consider Retransmission-less indicator included in every transmitted CG-PUSCH via UCI (Option 3). RAN2 should send an LS to RAN1 to do the final specification.

-	Ericsson thinks this will never happen since a good network avoids the problems.

Not considered

Online (Thursday) (1) – RAN2 aspects of RAN1 work on UTO-UCI

RAN2 aspects of configured grant enhancements specified in RAN1:

R2-2304713	Configured grant enhancements for XR	Qualcomm Incorporated	discussion	Rel-18	NR_XR_enh-Core

Proposal 1. 	Whether/when to send skipping indication for a CG occasion is a decision made by MAC entity based on availability of L2 data. The related UE behaviors should be discussed by RAN2.

Proposal 2.	Even if UE has previously indicated that it will not skip a CG occasion, UE is still allowed to skip that CG occasion if no eligible data is available in that slot.

Proposal 3.	UE is allowed to change its indication for a future CG occasion in any UTO-UCI transmitted up to that occasion.

Proposal 4.	If UE indicates in CG occasion #n that it will skip occasion #n+1, then UE is not allowed to transmit over occasion #n+1, even if there is data eligible for the CG in occasion #n+1.

Proposal 5.	If UTO-UCI is absent in a CG PUSCH transmission (e.g. due to intra-UE prioritization), network should consider that the UE may use the corresponding CG occasion(s).

Proposal 6.	If UE has a UCI overlapping with a CG occasion and does not have any UL data eligible for the CG occasion, UE can send skipping indication before the CG occasion and transmit the UCI over PUCCH. UE is not required to transmit over PUSCH in this case.

R2-2306206	Discussion on multiple-PUSCHs CG for XR	TCL Communication	discussion	Rel-18

Proposal 1: the solution for multiple-PUSCHs CG configuration should be similar as that for DRX configuration which is being discussed

Proposal 2: the number of CG PUSCH transmission occasions and/or the interval between two adjacent CG PUSCH transmission occasions in a period could be configured by high layer differently for different Types of CG and TDRA alternatives

Proposal 3: a special BSR or other MAC CE could be generated in MAC to trigger the first CG PUSCH transmission with the UTO-UCI when no XR traffic data for the first CG PUSCH transmission occasion

Proposal 4: a BSR could be used to request supplementary DG in the case where CG resources are insufficient and the potential additional delay for the DG could be FFS

R2-2305605	Consideration on Retransmission less CG on XR	CMCC	discussion	Rel-18	NR_XR_enh-Core

Retransmission-less CG aspect:

Observation 1: uplink-Harq-ModeB-r17 and uplinkHARQ-mode can be reused for retransmission-less CG for XR in TN.

Observation 2: Reusing uplink-Harq-ModeB-r17 and uplinkHARQ-mode can support retransmission-less transmission for both CG and DG and offer a flexible compromise between power consumption and reliability.

Proposal 1: RAN2 to support retransmission-less CG by reusing uplinkHARQ-mode and uplink-Harq-ModeB in TN.

Proposal 2: RAN2 to introduce reliability assurance mechanism for retransmission-less CG, e.g., blind retransmission, using LowSE MCS table.

Multiple-TOs aspect:

Proposal 3: NW (RRC) can configure the number of transmission occasions (TOs) within one CG period in multi-PUSCHs CG configuration.

Observation 3: Uneven interval between TOs is more appreciated since jitter obeys Poisson distribution other than Uniform distribution.

Observation 4: Too many multi-PUSCHs CG TOs can cause overhead and difficulty in indicating unused CG PUSCH TOs.

Proposal 4: NW (RRC) can configure the interval between TO within one CG period, and the intervals can be uneven. FFS configure all intervals or the step of interval (scale factor).

Proposal 5: UE can be allocated multiple TOs within one CG period for better adaptation the jitter caused by jitter in data volume for data burst.

Observation 5: When the TBS of data burst exceeds that of single TO within one CG period, using multiple TOs within one CG period can reduce overall latency but may have impacts on RAN1, while using single TO+DG has no specification impact but could increase latency.

Proposal 6: RAN2 is kindly asked to discuss how to transfer data burst in uplink when its TBS exceeds single TO in Multi-PUSCHs CG:

Opt1: using multiple TOs within one CG period firstly, if all TOs are still not enough, use DG.

Opt2: using single TO+DG. FFS whether to send UTO indication for less latency.

Indication of UTO aspect:

Proposal 7: MAC entity is responsible for recognize unused CG TO(s) and signaling PHY.

Proposal 8: UTO-UCI should be sent as soon as possible for UTO recycling, while if there left little time for NW recycling, UE can omit to send UTO-UCI.

Proposal 9: RAN2 is kindly asked to discuss whether to allow UTO indication be multiplexed in MAC PDU, if exists.

Online (Thursday) (1) – RAN2 aspects of RAN1 work on HARQ ID determination

R2-2306185	HARQ ID determination formula for CG	MediaTek Inc.	discussion	Rel-18

Observation 1: Modifications to HARQ ID determination formula is expected to be defined mostly in the MAC layer specification with zero or very minor L1 specification impact.

Proposal 1: RAN2 shall continue discussions on the modifications to the HARQ ID determination formula for CG due to multiple PUSCH occasions.

R2-2304971	Enhancements for configured grant	CATT	discussion	Rel-18	NR_XR_enh-Core

Observation 1: The NTN specification of retransmission-less CG can easily be extended to terrestrial networks with only editorial revisions.

Proposal 1: For retransmission-less CG, the NTN solution is reused as is, i.e. network disables the HARQ RTT timer per HARQ process associated with the target CG configuration.

Proposal 2: RAN2 leaves to RAN1 the design of the determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH.

Observation 2: MAC will need to feed L1 with UTO contents.

Proposal 3: RAN2 study the MAC impacts from providing L1 with the expected usage of CG PUSCH transmission occasions (aka UTO-UCI).

R2-2304919	Discussion on CG enhancements for XR	vivo	discussion	Rel-18	NR_XR_enh-Core

R2-2305020	Configured Grant enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2305075	Views on Configured Grant Enhancements for XR	Apple	discussion	Rel-18	NR_XR_enh-Core

R2-2305161	Configured Grant enhancements for XR	InterDigital	discussion	Rel-18	NR_XR_enh-Core

R2-2305516	Configured Grant enhancements for XR	Sony	discussion	Rel-18	NR_XR_enh-Core

R2-2305535	Discussion on configured grant enhancement for XR	OPPO	discussion	Rel-18	NR_XR_enh-Core

R2-2305538	On Configured Grant enhancements for XR	Google Inc.	discussion

R2-2305725	Details of CG enhancements for XR communications	Lenovo	discussion	Rel-18	NR_XR_enh-Core

R2-2306266	Configured Grant enhancements for XR	Ericsson	discussion	Rel-18	NR_XR_enh-Core

R2-2306272	Discussion on Configured Grant enhancements for XR	III	discussion	NR_XR_enh-Core

R2-2306347	Discussion on CG enhancement for XR	LG Electronics Inc.	discussion	Rel-18	NR_XR_enh-Core

## 7.14	Enhancement on NR QoE management and optimizations for diverse services

(NR_QoE_enh-Core; leading WG: RAN3; REL-18; WID: RP-223488)

Time budget: 0.5 TU

Tdoc Limitation: 3 tdocs

Prioritization of topics TBD based on input tdocs.

### 7.14.1	Organizational

Including LSs and any rapporteur inputs (e.g. work plan)

Online (Tuesday) (1) – Work plan

Work plan:

R2-2306476	Revised Work Plan for Rel-18 NR QoE Enhancement	China Unicom	Work Plan	NR_QoE_enh-Core

Endorsed (will attempt to provide initial version of 37.340 during the meeting)

Online (Tuesday) (1+1+2) - LSs

RAN3 LS on assistance information during RAN overload:

R2-2304625	LS on the feasibility of introducing assistance information for handling of QoE reporting during RAN overload (R3-232047; contact: ZTE)	RAN3	LS in	Rel-18	NR_QoE_enh-Core	To:SA5	Cc:RAN2

-	Lenovo thinks we can wait for SA5 conclusion before RAN2 discussion.

Noted (RAN2 in CC, no actions until SA5 reply)

SA5 LS on MBS service area:

R2-2304626	LS on collecting QoE measurements per MBS service area and MBS session ID (R3-232079; contact: Huawei)	RAN3	LS in	Rel-18	NR_QoE_enh-Core	To:SA5	Cc:RAN2

Noted (RAN2 in CC, no actions until SA5 reply)

SA4 LS on buffer-level threshold-based RVQoE reporting (received late in RAN2#121bis-e)

R2-2304658	Reply LS on buffer level threshold-based RVQoE reporting (S4-230684; contact: Apple)	SA4	LS in	Rel-18	NR_QoE_enh-Core	To:RAN2	Cc:RAN3

SA4 thanks RAN2 on their LS on buffer level threshold-based RVQoE reporting. Given that

•	There already exists mechanism before Rel-18 for application layer to be configured for QoE reporting, and that this mechanism can be reused by the application layer to do RVQoE reporting based on the trigger of the buffer level threshold, and

•	The application layer can make a buffer-threshold based decision in a more timely fashion compared to the AS layer, since the corresponding application layer reporting, based strictly on reporting periodicity, may be unable to submit QoE reports at the exact time that buffer level threshold is reached. As result, and depending on the reporting periodicity, the delay between a threshold occurrence and the next scheduled QoE report may precluding a more timely remedial response by the gNB.

Hence SA4 can confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer.

-	Apple thinks RAN3 is still discussing the mechanisms of the reporting so we should wait.

-	Huawei agrees.

Noted

Online (Tuesday) (1) – Running CRs

R2-2305381	Running CR for QoE measurements	Ericsson	draftCR	Rel-18	38.331	17.4.0	NR_QoE_enh-Core

Companies are encouraged to provide comments on the CR to rapporteur(s) already during the meeting (i.e. before any post-meeting email discussion)

For running CRs, the CR rapporteurs are requested to submit first running CRs as rapporteur input to RAN2#123 (which are not counted against the Tdoc limits).

### 7.14.2	QoE measurements in RRC_IDLE INACTIVE

Including discussion on handling area scope for MBS QoE (i.e. is it done by AS or AL, whether the same mechanism applies for all RRC states, etc.)

Including discussion on AS layer buffer size (e.g. how many values, what is the minimum value).

Including discussion on what AS layer stores in IDLE/INACTIVE and what exactly is sent to AL.

Online (Tuesday) (2) – Area scope, delta signalling and QoE configuration release

R2-2306396	Discussion on QoE measurements in RRC IDLE and INACTIVE state	CATT	discussion	Rel-18	NR_QoE_enh-Core

Observation 1: In Rel-17, the network will always handle the area scope checking and no LocationFilter will be specified in QoE configuration when UE is in RRC_CONNECTED state.

Proposal 1: When UE is in RRC_CONNECTED state, we can reuse the Rel-17 mechanism to check area scope, i.e. the network is responsible to check area scope of QoE measurement for broadcast.

Proposal 2: When UE is in RRC_IDLE or RRC_INACTIVE state, UE AS layer is responsible to check area scope of QoE measurement for broadcast.

Proposal 3: When UE enters RRC IDLE/INACTIVE state, UE AS layer should store the content of QoE configuration for broadcast service including:

-	MeasConfigAppLayerId

-	Service type

-	Pause reporting

-	Session start/stop

-	Area socpe information

-	MBS session ID(FFS)

-	RV QoE parameters(FFS)

Proposal 4: If RAN3 choose the UE-based solution, the UE AS layer may also need to store the following content:

-	QoE reference

-	MCE information

-	QoE measurement type

-	MDT alignment information (FFS)

Proposal 5: We can assume that the minimal memory size for QoE report generated in IDLE/INACTIVE states should be larger than 64KB, such as 256KB and send LS to SA4/SA5 to confirm.

Focus on P1-2

R2-2305809	Discussion on QoE measurements for MBS broadcast services	Huawei, HiSilicon	discussion	Rel-18	NR_QoE_enh-Core

Area scope checking

Observation 1: SA4 specifications already provide a readily available solution for handling QoE measurement area scope for MBS broadcast services. No specifications changes are required to support it, neither in SA4 nor in RAN2.

Observation 2: There are no technical issues with using application layer area scope checking for QoE of MBS broadcast services.

Observation 3: When LocationFilter is configured, the application layer always considers it, as per the current SA4 specifications. There is no need for the NW to perform area scope checking for the UE in RRC CONNECTED state in that case, but there are also no issues in case the network wants to do that.

Observation 4: It is inefficient and unusual to specify two different solutions to address exactly the same issue.

Proposal 1:	RAN2 to discuss whether there are any benefits of specifying AS layer area scope handling compared to the existing application layer area scope handling which would justify the additional specifications efforts.

Proposal 1: When UE is in RRC_CONNECTED state, we can reuse the Rel-17 mechanism to check area scope, i.e. the network is responsible to check area scope of QoE measurement for broadcast.

Proposal 2: When UE is in RRC_IDLE or RRC_INACTIVE state, UE AS layer is responsible to check area scope of QoE measurement for broadcast.

Coffee break offline (Samsung): Discuss area scope open questions and whether there is need to send LS to SA5/SA4/RAN3

-	Samsung reports that there is consensus to send LS to SA5 with 3 questions:

Send LS to SA5, SA4, RAN3 about the feasibility of area scope checking only in AL. Ask questions on:

- 1) So far SA5 has restriction that area scope and LocationFilter cannot be provided simultaneously. Ask if this can be removed so gNB can select the UEs?

- 2) Ask RAN3 if provision of area scope to gNB can be optional?

- 3) If PLMN/TA information is needed, is it feasible to include those in LocationFilter?

- 4) SA4: Is there a problem with redundant checks (ie. both NW and UE AL)?

- FFS if something can be asked about polygons.

Offline [201] (Samsung): LS to SA5

-	QC thinks AL is already aware of the geographic area so could do the checking, but not sure RAN2 can decide on AL. Ericsson thinks AL solution requires addition of PLMN/TA to the information. Also thinks switching between UE and NW handling is difficult. if UE does the area checking in CONNECTED, NW is not aware of the UE selection. Huawei thinks there is no need to switch if UE does it always. OAM will configure it for both UE and NW. That is now NW is aware of the area scope. In CONNECTED NW knows where the UE is. Agrees that adding PLMN/TA scope would be something that might be needed but is not sure and needs to be asked.

-	Ericsson wonders what the benefit of doing this in AL is?

-	China Unicom thinks AL solution has some disadvantages since AL needs to send information to AS. using different procedures for different service types is not easy.

-	Nokia thinks in CONNECTED should be done by network since that’s the legacy. SA4 indicated the checking is only done at the start of the session. Thinks it’s easier if only one entity does.

-	QC thinks it’s possible to allow both AL and AS but let OAM choose which is used.

-	Huawei thinks we have a different mechanism now compared to before. Sees no problem for UE to manage the area scope also in CONNECTED even of UE has LocationFilter. With AS layer solution there is some information exchance when area scope validity changes. Samsung also prefers a unified solution but is concerned about PLMN/TA in LocationFilter.

QoE buffer handling

Observation 5: The memory requirements for storing QoE reports generated for MBS broadcast in RRC_IDLE/INACTIVE states will be much higher than in case of pause due to RAN overload.

Proposal 2:	RAN2 will introduce UE capability signaling for support of QoE reports buffer size(s) larger than 64kB. Exact values to be supported FFS.

Proposal 3:	RAN2 agrees that assistance information for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full is useful. RAN2 should wait for RAN3 conclusion on the contents of assistance information provided from OAM to RAN before working on the details.

QoE configuration storage

Observation 6: RAN3 conclusion on whether the required QoE information (e.g. QoE reference, MCE Information) should be provided to the new gNB from the UE or from the CN is needed to decide what exactly needs to be stored at the UE AS layer when the UE moves to RRC IDLE state.

Proposal 4:	RAN2 to discuss whether delta configuration of the QoE configuration applied in RRC IDLE needs to be supported when the UE moves to RRC CONNECTED state.

Proposal 5:	Timer based QoE configuration release is not supported, i.e. the UE stores the IDLE/INACTIVE QoE configuration until it is released by the network.

-	Samsung thinks RAN3 is discussing UE and CN-based solutions already. With UE-based solution delta is not possible. Ericsson thinks or IDLE delta is not needed but INACTIVE it is. Should avoid changes. Huawei agrees for INACTIVE.

-	Qualcomm thinks delta configuration is beneficial. Ful configuration can make the QoE to stop or restart. Huawei thinks this is not the case: In R17 gNB had to provide full config already, and RAN2 agreed to allow AS layer not to reconfigure AL.

4:	Do not support delta configuration of the QoE configuration applied in RRC IDLE when the UE moves to RRC CONNECTED state unless it causes issues for QoE AL continuity in state transition.

-	Lenovo thinks that for IDLE, UE has to keep the configuration until it powers off or goes to CONNECTED. Also wonders what happens to the stored reports. Could also have some issues with inter-RAT. For logged MDT we had a 48h timer for the report.

-	China Unicom thinks we discussed this already. Qualcomm has concern for P5 with m-based QoE where it’s not possible to release the configuration.

-	Ericsson is fine with the proposal but understands the concern. Could be fine with MDT-like timer. ZTE thinks we sent LS to SA4 on time requirement and they said there was none. Lenovo thinks SA5 replied that latest one was more important.

-	Nokia is fine for with P5 and thinks it should be in NW control to release. CATT agrees with Lenovo that this increases power consumption.

5: UE is allowed to release stored reports and configuration after 48h (similar to logged MDT). No timer is configured by the network.

Selection of UEs for MBS QoE configuration

Observation 7: Forcing the gNB to utilize blind configuration of MBS broadcast QoE to all MBS capable UEs is sub-optimal for both the UE and the network in terms of signaling overhead, memory/storage requirements, predictability of receiving QoE measurements etc.

Proposal 6:	RAN2 should investigate the means for the gNB to identify which UEs should be provided with MBS broadcast QoE configuration for a specific MBS session via, e.g.:

1.	Allowing the network to indicate to the UE the IDs of MBS broadcast sessions for which it is interested in receiving QoE measurements.

2.	The UE indicating to the network when the UE is configured with or receiving/starting to receive the indicated MBS sessions.

QoE configuration details

Observation 8: Considering SA4 input, MBS cannot be treated as a separate QoE service type as MBS is a transmission mean which is used to deliver existing service types.

Proposal 7:	RAN2 should wait with the decision on whether to introduce explicit indication about the QoE applicability to RRC IDLE/INACTIVE until it is clear whether MBS session IDs need to be included in the QoE configuration (pending SA5 input and RAN3 decision).

QoE reporting details

Observation 9: Resuming/setting up an RRC connection just for the sake of reporting QoE brings no benefits while it causes MBS broadcast service performance deterioration, increases signaling overhead, impacts UE battery life and brings additional complexity.

Proposal 8:	The UE does not setup/resume RRC connection just for QoE reporting, i.e. the QoE reports are sent to the network when the UE moves to RRC_CONNECTED state due to other reasons.

Focus on P1 (area scope), P4-5 (delta signalling and QoE release timer)

Offline discussion (Thursday) (1)

[AT122][201][QoE] LS on area scope QoE measurements (Samsung)

Scope: Provide agreeable LS to SA5/SA4/RAN3 to ask feasibility of area scope checking only in AL based on online decisions.

Intended outcome: Agreeable LS R2-2306561

Deadline: Deadline 1

R2-2306561	[DRAFT] LS on area scope QoE measurements	Samsung	LS out	Rel-18	NR_QoE_enh-Core	To: SA5, SA4, RAN3

-	Samsung clarifies that question about polling were not included since it was not clear. Also questions were raised about the usefulness of PLMN/TA information.

-	Ericsson is OK not to ask about the polygon aspects.

-	Lenovo thinks Q3 goes too far. Should ask in general what SA4/5 intends and is not sure it’s usefulness to SA4/5. Samsung thinks this is SA4/5 discussion. Huawei had proposed rewording for this question. China Unicom thinks the motivation is already clear and we can just ask about feasibility. Might need to ask about RAN3 first.

-	ZTE thinks the feasibility question is important for RAN2 to know if this can be done.

RAN2 assumes PLMN/TA information is needed in area scope (in one way or another). FFS how this is expressed, e.g. as list of cells.

Replace the sentence “RAN2 strives for a unified solution that is applicable for all RRC states but identified a number of issues which need to be clarified with other working groups.” with “RAN2 has identified a number of issues which need to be clarified with other working groups.” since it hasn’t been agreed yet what solution RAN2 adopts for each state

Replace “Besides, RAN2 has discussed whether PLMN/TA information is needed as area scope for Rel-18 QoE measurement for MBS broadcast, but could not reach consensus. Regarding this issue, RAN2 has the following question. with “RAN2 has discussed PLMN/TA information for area scope and assumed it is needed for Rel-18 QoE measurement for MBS broadcast. Regarding this issue, RAN2 has the following question.”

Replace Q3 with “Q3) RAN2 would like to ask SA4/SA5 if it is feasible to include PLMN/TA information in LocationFilter?”

Remove Draft and use “RAN2” as source

With the above changes, the LS is approved (unseen) in R2-2306569

R2-2306569	LS on area scope QoE measurements	RAN2	LS out	Rel-18	NR_QoE_enh-Core	To: SA5, SA4, RAN3

Approved (unseen)

Online (Tuesday) (2) – MBS QoE applicability to RRC states, SIB information and UE buffer sizes

R2-2305076	QoE Measurements in IDLE/INACTIVE States	Apple	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1: If a new service type of MBS is to be introduced, whether a QoE configuration is also applicable in RRC-IDLE/INACTIVE states can be implicitly indicated by the service type. Otherwise, explicit indication can be used.

Proposal 2: Priority level per QoE configuration should be introduced for the UE to decide which QoE report can be discarded first when the buffer becomes full in RRC-IDLE/INACTIVE states.

Proposal 3: Send a reply LS to RAN3 to notify that the assistance information for handling of QoE reporting upon RAN overload may also be useful for UE to handle QoE report discarding in RRC-IDLE/INACTIVE states.

Proposal 4: Area scope checking should be handled by UE APP in all RRC states.

Focus on P1

-	Ericsson thinks we could have explicit indicator.

-	Lenovo wonders if this indicator is sent to AL? Thinks it could be kept in AS layer.

1: As working assumption, RAN2 will use explicit indicator in AS-layer on whether a QoE configuration is also applicable in RRC-IDLE/INACTIVE states. Can be revisited if RAN3 decides to introduce a service type.

R2-2305310	Discussion on QoE measurement in RRC_IDLE and RRC_INACTIVE	Samsung	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1. NW indicates via SIB1 whether it supports Rel-18 QoE measurement. Only if this indicator is received, UE is allowed to report availability indicator.

-	Lenovo is fine with SIB1 indicator but thinks it’s only whether UE is allowed to send the indicator. Huawei thinks UE can just send the indicator and if NW doesn’t understand it, it will ignore it (i.e. not configured SRB4). Nokia agrees. CMCC agrees. CATT, ZTE, Ericsson agrees.

-	Lenovo thinks network may not retrieve the reports even if UE sends the indicator. Does it then repeat the indicator?

-	Samsung thinks that this is about UE side and not network. But there is no critical issue.

1. Do not introduce SIB1 indicator on whether UE is allowed to indicate presence of QoE measurements. UE always indicates if it has stored QoE report(s), and it’s up to network whether/when to retrieve them.

Proposal 2. Introduce 1 bit indicator (separate from serviceType) in QoE configuration to indicate whether it is for MBS broadcast service or not.

Proposal 3. If QoE configuration indicates MBS broadcast, UE performs corresponding QoE measurement in all RRC states. Otherwise (i.e., if QoE configuration does not indicate MBS broadcast), UE performs corresponding QoE measurement only in RRC_CONNECTED.

Proposal 4. Introduce AS layer minimum memory requirement for storing QoE reports measured in RRC_IDLE/RRC_INACTIVE, which is separate from the Rel-17 memory requirement (i.e., 64KB) for storing paused QoE reports.

Proposal 5. Define an UE capability with values {64KB, 128KB, 256KB, 512KB} for separate Rel-18 AS layer minimum memory requirement. UE supporting QoE measurement for MBS broadcast shall indicate one of those values as UE capability. The exact range of the values can be discussed or checked with other groups.

-	Lenovo thinks 64 kB is anyway too low and is not sure how the proposed values were derived. Do we assume UE can be configured with up to 16 reports?

-	Ericsson agrees the number of configurations affects the memory size. Thinks 64 kB is too low.

-	QC is fine with separate capability for larger size. China Unicom thinks 64 kB as minimum size is fine.

Introduce AS layer minimum memory requirement for storing Rel-18 QoE reports measured in RRC_IDLE/RRC_INACTIVE. Could have larger values than in Rel-17. FFS what is the minimum size requirement capability. FFS what is the value range of the capability.

Proposal 6. As baseline, LocationFilter can be used for area scope of QoE configuration for MBS broadcast.

Proposal 7. Discuss whether to define AS layer area scope:

- Option 1. AS layer area scope is not defined. LocationFilter is mandatory in QoE configuration for MBS broadcast.

- Option 2. AS layer area scope can be configured if LocationFilter is absent. (i.e., UE APP performs QoE measurement anywhere, but UE AS discards QoE reports received from UE APP based on configured AS area scope)

Proposal 8. Area scope is checked by UE in all RRC states.

Proposal 9. UE can send QoE report or availability indicator outside (or regardless) of the area scope (i.e., LocationFilter or AS layer area scope).

Proposal 10. Support QoE measurement per MBS broadcast session.

Proposal 11: RAN2 discusses the signalling-based QoE override protection, and makes a way-forward.

Focus on P1-5

Online (Tuesday) (1) – UE capabilities

R2-2305606	Consideration on QoE measurement in RRC_IDLE/INACTIVE	CMCC	discussion	Rel-18	NR_QoE_enh-Core

QMC configuration and release aspect:

Observation 1: Paging mechanism can be reused for QoE configuration release in RRC_IDLE/RRC_INACTIVE

Proposal 1: QoE measurement configuration release via broadcast signaling is not support in R18.

Observation 2: As a service type, MBS/MBS BC can be served as an implicit indication for QoE configuration in RRC_IDLE/RRC_INACTIVE, but RAN3 has not decided that.

Proposal 2: Potstone the discussion on indicator for QoE configuration in RRC_IDLE/RRC_INACTIVE until RAN3 has conclusion on whether MBS/MBS BC is a service type, FFS send LS to RAN3.

Proposal 3: gNB should select UE(s) both are capable for MBS QoE in RRC_IDLE/RRC_INACTIVE and configured with MBS.

Proposal 4: RAN2 is kindly asked to discuss which message can determine whether UE shall perform MBS in RRC_IDLE/RRC_INACTIVE to match and select UE for MBS QoE configuration,

Proposal 5: RAN2 waits for RAN3's conclusion on whether to support RVQoE in RRC_IDLE/RRC_INACTIVE.

QoE configuration storage aspect:

Observation 3: Based on previous conclusion from RAN2 and RAN3, UE should store service type and QoE reference, RRC level ID, MCE info and QoE configuration container. RAN3 is discussing more parameters in RRC_IDLE/RRC_INACTIVE.

Proposal 6: AS layer stores QoE reference, RRC level ID (measConfigAppLayerId), MCE info in RRC_IDLE.

Proposal 7: For per-slice QoE, RVQoE, Alignment with MDT, RAN2 waits for RAN3's conclusion.

UE capability aspect:

Proposal 8: Introduce an independent UE capability for QoE in RRC_IDLE/RRC_INACTIVE regardless of service type.

Proposal 9: Reuse 64KiB AS buffer size for paused QoE is the baseline for QoE report storage in RRC_IDLE/RRC_INACTIVE.

Proposal 10: Introduce a UE capability for QoE in RRC_IDLE/RRC_INACTIVE indicates whether UE support additional AS buffer size for QoE in RRC_IDLE/RRC_INACTIVE, FFS extra AS buffer size is fixed or not.

Area scope handling aspect:

Observation 4: LocationFilter in QoE container (APP layer) and area scope (AS layer) can be configured simultaneously.

Proposal 11: NW can provide UE with area scope information (e.g., Cell ID, TAC, Area scope configuration) for RRC_IDLE/RRC_INACTIVE, if necessary.

Focus on P8-10

R2-2305138	Discussion on support of QoE measurements in RRC_IDLE and RRC_INACTIVE	Lenovo	discussion	Rel-18	NR_QoE_enh-Core

R2-2305382	QoE measurements in RRC_INACTIVE and RRC_IDLE	Ericsson	discussion	Rel-18	NR_QoE_enh-Core

R2-2305755	QoE measurements support in RRC IDLE and INACTIVE	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_QoE_enh-Core

R2-2305766	Open issues on QoE collection for IDLE and Inactive state	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2306107	Considerations on QoE measurement in IDLE and INACTIVE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_QoE_enh-Core

R2-2306478	Discussion on QoE measurements in RRC_IDLE and INACTIVE states	China Unicom	discussion	Rel-18	NR_QoE_enh-Core

### 7.14.3	Rel-17 leftover topics for QoE

Including discussion on Rel-17 leftover topics as agreed in previous meetings.

Online (Tuesday) (1) – Need to reply to RAN3 on overload handling?

R2-2305077	[DRAFT] Reply LS on assistance information for handling of QoE reporting upon RAN overload	Apple	LS out	Rel-18	NR_QoE_enh-Core	To:RAN3

(moved from 7.14.2)

RAN2 thanks RAN3 for the LS on assistance information for handling of QoE reporting upon RAN overload. RAN2 would like to inform that, such information may also be useful if it is provided to the UE. For instance, the UE may use the priority levels associating to QoE configurations to decide which QoE measurement report should be discarded when the AS buffer becomes full in RRC-IDLE/INACTIVE states.

Noted

Online (Tuesday) (2) – Are RRC events needed for buffer level threshold - based RVQoE reporting?

R2-2306109	Considerations on Rel-17 leftover issues for QoE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1: RAN2 doesn’t discuss further enhancements (i.e. UE-based solution other than pause/resume) to deal with RAN overload

Proposal 2: Confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by  application layer.

Proposal 3: RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting.

Proposal 4: Introduce a buffer level threshold information in the RVQoE configuration from the gNB.

R2-2305384	Event based RVQoE reporting	Ericsson	discussion	Rel-18	NR_QoE_enh-Core

Observation 1	In the RAN3#118 meeting the following was agreed: Turn the WA to agreement: Introduce buffer level as a threshold-based trigger for RVQoE reporting. This agreement is like option 1 from RAN2119bis, suggesting introduction of a new event for RVQOE reporting, where the trigger for the event is the BufferLevel below a configured threshold.

Observation 2	Option 4, i.e., using RVQoE metrics fulfilling certain conditions as triggers for RVQoE reporting, encompasses options 1 to 3 as well and provides easier extensions of the list of trigger events in the future.

Observation 3	Option 6 may result in limited usefulness of RVQoE to RAN for real-time radio resources’ optimization.

Proposal 1	Consider using RVQoE metrics fulfilling certain conditions, as triggers for RVQoE reporting.

Proposal 2	The UE application layer evaluates whether conditions for the event- or threshold- triggered RVQoE reporting are met before the RVQoE report is sent to RAN.

Proposal 3	Events in UE AS layer can trigger event-based RVQoE reporting. FFS on the type of events.

-	Huawei thinks we should go with AL events only. NEC thinks AS layer events are beneficial. QC agrees with ZTE. Apple thinks Ericsson proposals are a bit different than ZTE proposals. Thinks ZTE proposals are fine and radio-related events are not supported by RAN3.

-	CATT thinks priority information is still beneficial for UE but that is undecided in RAN3. Apple thinks assistance information could be useful for IDLE but not for RAN overload. Huawei thinks introducing assistance information for managing stored reports needs not be restricted to RAN overload. So if we introduce it for other reasons, we can do it and need not wait for RAN3.

-	ZTE wonders if UE knows about RAN overload. Thinks assistance information may be beneficial.

-	Nokia thinks assistance information can be useful for buffer management. Thinks RAN3 has the same view as well. QC thinks we should still wait for RAN3. Without the assistance information UE doesn’t know. Lenovo thinks this also links to the UE buffer size discussion. We also had default behaviour for storing only latest reports. Thinks specifying different rules is more complicated.

2: RAN2 confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by application layer.

3: RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting.

R2-2305756	Discussion on Rel-17 leftovers	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_QoE_enh-Core

Observation 1: Given the above-mentioned details, it seems that both RAN2 and RAN3 agree that there is no need for radio-related event triggers for RVQoE reporting.

Observation 2: The UE cannot have a clear view on which QoE configuration/reporting has greater importance if no assistance information is provided by network.

Observation 3: In case of RAN overload, the released QoE configuration and temporarily paused QoE reporting may have an adverse effect on QoE management and outcome.

Observation 4: In case of RAN overload, QoE Pause mechanism enables pausing all the QoE reports simultaneously.

Proposal 1: Since SA4 LS reply confirmed that for buffer level threshold-based RVQoE reporting the UE APP layer will be responsible, any discussion with respect to the buffer level threshold should be initially discussed in SA4.

Proposal 2: RAN2 to wait RAN3 and SA4 progress before making any decision with respect to how the UE sends the RVQoE reports after the threshold is met.

Proposal 3: RAN2 to confirm and align with RAN3’s agreement of not supporting radio-related event triggers for RVQoE reporting in Rel-18.

Proposal 4: RAN2 to wait for SA5’s reply on QoE assistance information before proceeding further on solutions for RAN overload scenarios.

3: RAN2 will align with RAN3’s agreement on support of radio-related event triggers for RVQoE reporting in Rel-18.

IF time allows: Online (Thursday) (1) – Do we support QoE for shared spectrum?

R2-2305015	Application Layer Measurement Reporting for unlicensed spectrum	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1: Application layer measurement configuration and reporting for shared spectrum channel access is supported in R18.

Proposal 2: RAN2 to discuss and agree on one of the following for CAPC of SRB4:

Option 1: Fix the CAPC of SRB 4 to highest priority (similar to SRB 0, SRB 1 and SRB 3)

Option 2: CAPC of SRB4 can be signaled by gNB in RRCReconfiguration message (similar to SRB2)

Proposal 3: Introduce a new UE capability indicating that UE supports application layer measurement configuration and reporting for shared spectrum channel access.

-	QC thinks this is out of Rel-18 scope. China Unicom confirms this is not in Rel-18 scope.

1: Application layer measurement configuration and reporting for shared spectrum channel access is not supported in R18.

R2-2305139	Discussion on Rel-17 leftover topics for QoE	Lenovo	discussion	Rel-18	NR_QoE_enh-Core

R2-2305362	Discussion on Rel-17 leftover issues for QoE	NEC	discussion	Rel-18	NR_QoE_enh-Core

R2-2305811	Discussion on Rel-17 left-over issues	Huawei, HiSilicon	discussion	Rel-18	NR_QoE_enh-Core

R2-2306397	Discussion on Rel-17 leftover topics for QoE	CATT	discussion	Rel-18	NR_QoE_enh-Core

### 7.14.4	Support of QoE measurements for NR-DC

Including discussion on granularity of QoE reporting (e.g. per QoE config or something else)

Including disucssion on how MN knows to corrrectly forward SN-associated QoE reports received via SRB4

Including discussion on how to achieve splitting of QoE configuration identities between MN and SN.

Including discussion on different m-based QoE configurations for MN/SN (pending RAN3 decisions).

Online (Tuesday) (2) – Configuration of SRB usage

R2-2306477	Discussion on QoE configuration and reporting for NR-DC	China Unicom	discussion	NR_QoE_enh-Core

Observation 1: If RAN3 agrees that MN and SN shares a same MCE, UE can transmit the SN-associated QoE reports via SRB4 directly in the case of that SCG is not activated.

Proposal 1: The network can use one bit indication per QoE config to indicate the SRB for the QoE reporting if both SRB4 and SRB5 are configured.

Proposal 5: In the case only one of SRB4 or SRB5 is configured for the UE, the UE transmits all the QoE reports directly to the node where SRB4 or SRB5 is configured without any explicit indication from the network.

Proposal 6: In the case both SRB4 and SRB5 are configured, the network can explicitly indicate SRB switching.

-	Qualcomm thinks RAN3 already concluded. Can just say we use explicit indication. China Unicom clarifies one bit is enough for the explicit indication. Ericsson thinks RAN3 assumed there would be explicit indication but no need to say one bit.

-	China Unicom thinks that if we have only one SRB and leg switch occurs, UE ay have to wait for explicit indication from network.

1: The network can use explicit indication per QoE config to indicate which SRB is used for the QoE reporting. Details can be discussed in Stage-3.

Example: Assume UE has only SRB4 configured.  QoE config1 is for SRB4, QoE config2 is for SRB5.

- UE has to be reconfigured to transmit report for QoE config2 since it is not explicitly using SRB4.

- UE is allowed to transmit report for QoE config2 any SRB4.

-	Huawei thinks having such configuration is not suitable. Network should make QoE configs usable in all cases. Samsung thinks it’s necessary to check the RAN3 agreements on this.

FFS how to handle the QoE report transmission if there is only one SRB and the QoE report is not (explicitly) configured for that SRB (to be checked if this is possible according to latest RAN3 agreements)

FFS how the above case works if SCG is deactivated or released.

Proposal 4: For NR-DC, MN splits RRC IDs and assigns them to the SN for QoE configuring to the UE.

-	Ericsson thinks this is not clear, should say MN assigns but RAN3 already agreed this is done dynamically per QoE config.

RAN2 will follow RAN3 agreement on QoE config RRC IDs.

2: RAN2 will follow RAN3 agreements (for NR-DC when SRB5 is not configured) on forwarding the received encapsulated QoE reports to the correct recipient (i.e. MCE or SN)

Proposal 3: For NR-DC, if SCG is deactivated, whether UE can transmit the SN-associated QoE reports via SRB4 directly depends on RAN3’s decision.

R2-2305810	Discussion on QoE measurements in NR-DC	Huawei, HiSilicon	discussion	Rel-18	NR_QoE_enh-Core

Observation 1: RAN3 has made some progress regarding “How MN forwards SN-associated QoE reports received via SRB4”, and RAN2 can rely on RAN3 on solving the issue.

Observation 2: MN knows whether the measurement report needs to be forwarded to SN based on MN-SN coordination and by knowing the measConfigAppLayerId of the report.

Observation 3: Using MN ULInformationTransferMRDC message for sending the SN MeasurementReportAppLayer may increase the delay and signaling overhead of RVQoE transfer.

Observation 4: If RAN3 has more progress on “WA: QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over different legs.”, RAN2 may need to discuss possible impacts.

Observation 5: There is already a possibility for the RAN to change the reporting leg of the QoE configuration and it is unclear why pause indication should be used for that.

Observation 6: For the UE selection of reporting SRB:

-	For a single QoE SRB (i.e. one of SRB4 or SRB5 is configured for the UE), implicit indication can work

-	For two QoE SRBs (i.e. both SRB4 and SRB5 are configured for the UE), explicit indication can work

No extra impacts are foreseen.

Proposal 1: The explicit indication of SRB for the QoE reporting is configured per RRC ID (i.e. measConfigAppLayerId-r17).

Proposal 2: If SRB5 is not configured, UE sends the SN QoE results in MeasurementReportAppLayer message via SRB4 and MN forwards them to SN.

Proposal 3: RAN visible QoE configuration is generated by the same node which generates the configuration for container based QoE. The other node will not send the RRC message to update/modify the RAN visible QoE configuration which was not configured by this node.

Proposal 4: Rel-17 defined pause/resume mechanism is kept, i.e. pause/resume reporting of one or multiple QoE measurement configurations in a UE.

Online (Tuesday) (1) – MN RRC message used for QoE reporting to SN

R2-2305383	QoE measurements in NR-DC	Ericsson	discussion	Rel-18	NR_QoE_enh-Core

Observation 1	Reusing existing DC procedures for QoE guarantees support in multi-vendor and multi-RAT scenarios and decreases the complexity of the feature.

Proposal 1	A MeasurementReportAppLayer message to the SN can be sent embedded in ULInformationTransferMRDC to the MN.

Proposal 2	ULInformationTransferMRDC can be sent using SRB4.

Proposal 3	The following are the default options for QoE reporting, used unless the UE is instructed otherwise:  - If the QoE configuration was included in RRCReconfiguration from the MN, the QoE report is included in MeasurementReportAppLayer to the MN.  - If the QoE configuration was included in RRCReconfiguration from the SN, the SN RRCReconfiguration embedded in an MN RRCReconfiguration, the QoE report is included in MeasurementReportAppLayer to the SN, where the SN-bound MeasurementReportAppLayer is embedded in an ULInformationTransferMRDC to the MN.   - If the QoE configuration was included in RRCReconfiguration from the SN, sent via SRB3, the QoE report is included in MeasurementReportAppLayer to the SN, sent via SRB5.

Proposal 4	Network configuration is needed for the UE to transmit reports to a different node than the default option for reporting.

Proposal 5	The RRC reporting instruction for QoE reporting can include the options:  - report to MN via SRB4,  - report to SN transparently via SRB4, and  - report to SN directly via SRB5.

Proposal 6	The reporting instruction is configured at least per QoE configuration, i.e. per measConfigAppLayerId. FFS on separate indications for QoE and RVQoE reports.

Proposal 7	If the default reporting option is not available and the network didn’t indicate any other reporting option, the UE may discard the QoE reports.

Proposal 8	QoE configurations configured by the SN are released in the UE when the SCG is released.

Proposal 9	The UE sends a UEAssistanceInformation message indicating that it has UL data to send (according to existing procedures), if the SCG is deactivated when the UE has a QoE report to send.

Proposal 10	Discuss if the UE should indicate in UEAssistanceInformation that it is a QoE report that the UE has to transmit.

Focus on P1-2

R2-2305078	Discussions on QoE Reporting for NR-DC	Apple	discussion	Rel-18	NR_QoE_enh-Core

R2-2305311	Discussion on QoE measurement for NR-DC	Samsung	discussion	Rel-18	NR_QoE_enh-Core

R2-2305479	Support of QoE measurements for NR-DC	LG Electronics Inc.	discussion	Rel-18

R2-2305607	Consideration on QoE measurement for NR-DC	CMCC	discussion	Rel-18	NR_QoE_enh-Core

R2-2305757	Detailed handling of QoE configuration and reporting in NR-DC	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_QoE_enh-Core

R2-2305767	Open issues to support QoE collection in NR-DC	Qualcomm Incorporated	discussion	NR_QoE_enh-Core

R2-2306108	Considerations on QoE measurement for NR-DC	ZTE Corporation, Sanechips	discussion	Rel-18	NR_QoE_enh-Core

R2-2306398	Discussion on support of QoE measurement for NR-DC	CATT	discussion	Rel-18	NR_QoE_enh-Core

### 7.14.5	Other topics

Including discussion on the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process.

Including any other QoE enhancement discussion (e.g. service type aspects).

This agenda item is not treated in this meeting (except for LSs received from other WGs).

## 7.17	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

(NR_DualTxRx_MUSIM-Core; leading WG: RAN2; REL-18; WID: RP-230751)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

This topic is not planned to be treated in this meeting (except for urgent LSs received from other WGs).

# Summary

Comebacks:

Agreed documents (2)

4.1: LTE legacy (2)

R2-2306570	Correction on handover procedure completion	vivo, Nokia (rapporteur)	CR	Rel-17	36.300	17.4.0	1385	2	F	LTE-L23, TEI17		R2-2306562

R2-2306571	Correction on QoE configuration release	Google, Qualcomm, Ericsson	CR	Rel-17	36.331	17.4.0	4935	3	F	LTE_QMC_Streaming-Core	R2-2306563

Endorsed (2)

7.5: Rel-18 XR enhancements (1)

R2-2305186	Work Plan for Rel-18 WI on XR Enhancements for NR	Nokia, Qualcomm (Rapporteurs); Ericsson (RAN1 FL)	Work Plan	Rel-18	NR_XR_enh-Core

7.14: Rel-18 QoE enhancements (1)

R2-2306476	Revised Work Plan for Rel-18 NR QoE Enhancement	China Unicom	Work Plan	NR_QoE_enh-Core

Approved LS out (3)

7.5: Rel-18 XR enhancements (2)

R2-2306572	LS response to N6 PDU Set Identification	RAN2	LS out	Rel-18	NR_XR_enh-Core	To: SA4, SA2	Cc: RAN1

R2-2306564	LS on on new DRX cycles in rational numbers	RAN2	LS out	Rel-18	NR_XR_enh-Core	To: RAN1, RAN4

7.14: Rel-18 QoE enhancements (1)

R2-2306569	LS on area scope QoE measurements	RAN2	LS out	Rel-18	NR_QoE_enh-Core	To: SA5, SA4, RAN3

Post-meeting email discussions (short, CR/LS finalization) (1)

[Post122][211][XR] Stage-2 running CR for XR (Nokia)

Scope: Update 38.300 running CR based on this meeting’s agreements.

Intended outcome: Endorsed running CR

Deadline:  Short

Post-meeting email discussions (long) (0)

Post-meeting email discussions (long, from RAN2#121bis) (2)

[Post122][232][MUSIM] Running Stage-2 CR for NR MUSIM enhancements (China Telecom)

Scope: Update running Stage-2 CR based on agreements in RAN2#121bis-e meeting for NR Rel-18 MUSIM

Intended outcome: Endorsed running CR

Deadline:  Long

[Post122][233][MUSIM] Running RRC CR for NR MUSIM enhancements (vivo)

Scope: Update running RRC CR based on agreements in RAN2#121bis-e meeting for NR Rel-18 MUSIM

Intended outcome: Endorsed running CR

Deadline:  Long