3GPP TSG-RAN2 Meeting #122	R2-2305912

Incheon, South Korea, 22 - 26 May 2023

Agenda Item:	6.1.3.1 NR RRC

Source: 	Ericsson

Title:  	RLM and BFD relaxation and no DRX

Document for:	Discussion and Decision

# Introduction

The meaning of “no DRX” specified in 38.133 for relaxed RLM/BFD measurements is discussed further in this contribution.

# Discussion

Meaning of “no DRX”

In section 8.1.1 (relaxed RLM) and 8.5.1.1 (relaxed BFD) in 38.133 it is specified that the UE is not allowed to relax RLM/BFD measurements when “no DRX is used”:

The UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met:

-	The UE sends out-of sync indications to the higher layers,

-	The timer T310 is running.

-	No DRX is used or DRX cycle is longer than 80ms

The UE is no longer allowed to relax BFD measurements and apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

-	The timer beamFailureDetectionTimer is running.

-	No DRX is used or DRX cycle is longer than 80ms

During RAN2#121bis-e it was discussed whether “no DRX is used” refers to the definition in section 3.6.1 in 38.133:

3.6.1	RRC connected state requirements in DRX

For the requirements in RRC connected state specified in this version of the specification, the UE shall assume that no DRX is used provided the following conditions are met:

-	DRX parameters are not configured or

-	DRX parameters are configured and

-	drx-InactivityTimer is running or

-	drx-RetransmissionTimerDL is running or

-	drx-RetransmissionTimerUL is running or

-	ra-ContentionResolutionTimer is running or

-	a Scheduling Request sent on PUCCH is pending or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the preamble not selected by the MAC entity

Otherwise the UE shall assume that DRX is used.

The definition of “no DRX is used” in 3.6.1 in 38.133 is the same as the definition of Active Time in section 5.7 in 38.321, except when drx-OnDurationTimer is running (and assuming that the UE is continuously in Active Time when DRX is not configured):

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

The “no DRX is used” as defined in 3.6.1 can be viewed as “when there is data activity”:

The definition of “no DRX is used” in 3.6.1 does not only refer to the DRX configuration, but also to the DRX state/condition.

When the definition in 3.6.1 is used in 38.133, then there is an explicit reference to section 3.6.1 and the term “condition” it used, e.g.:

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(8 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent gaps, the MRGP above is the MRGP of the measurement gap associated with the target frequency layer to be measured if concurrent measurement gaps are configured. |  |

For the relaxed RLM/BFD measurements there is no reference to 3.6.1 and the term “configuration” is used. For example see section 8.1.2.4 for relaxed RLM (SSB-based) and section 8.5.2.4 for relaxed BFD (SSB-based):

Table 8.1.2.4-1: Evaluation period TEvaluate_out_SSB_Relax for FR1

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80ms | Max(200 K3 NOTE3, Ceil(15 K1NOTE2 P) Max(TDRX,TSSB)) |
| 80ms < Max(TDRX,TSSB) ≤160ms | Ceil(15 P) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80ms. NOTE 2: K1 = 4 for Max(TDRX,TSSB) ≤40ms and K1 = 2 for 40ms<Max(TDRX,TSSB) ≤80ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

Table 8.5.2.4-1: Evaluation period TEvaluate_BFD_SSB_Relax for FR1

| Configuration | TEvaluate_BFD_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX, TSSB) ≤ 80 ms | Max(50 K3, Ceil(7.5 K1 P) Max(TDRX,TSSB)) |
| 80ms＜Max(TDRX, TSSB) ≤ 160 ms | Max(50, Ceil(7.5 P) Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K1 is the relaxation factor. K1 = 2 for 40ms＜Max(TDRX, TSSB) ≤ 80 ms, K1 = 4 for Max(TDRX, TSSB) ≤ 40 ms Note 3: K3 is the relaxation factor for the lower bound. K3 = K1, if 1 < K1 ≤ 2; K3 = 1 otherwise. |  |

Thus “no DRX” for relaxed RLM/BFD does not refer to “no DRX” in section 3.6.1, but to the DRX configuration:

-	DRX is not configured or the configured DRX cycle is longer than 80ms

It is left to RAN4 to clarify 38.133, if needed.

Observation 1: “no DRX” for relaxed RLM/BFD measurements refers to the DRX configuration and does not refer to “no DRX” as defined in section 3.6.1 in 38.133.

The WID specified that the relaxed RLM/BFD is applicable for short DRX [3]:

  - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4]

- NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

Normally DRX is configured. If DRX is not configured, the DRX can be considered very short. But relaxation is not allowed/supported when DRX is not configured, because the relaxed measurement requirements are defined in terms of the configured DRX.

Relaxed RLM/BFD and long/short DRX

The UE is not allowed to relax RLM/BFD measurements when the configured DRX > 80 ms:

Observation 2: The UE is not allowed to relax RLM/BFD measurements when long DRX > 80 ms is configured and short DRX is not configured or short DRX > 80 ms is configured.

Observation 2: The UE is allowed to relax RLM/BFD measurements when long DRX < 80 ms is configured and short DRX is not configured or short DRX < 80 ms is configured.

In case long DRX > 80 ms and short DRX < 80 ms are configured, it is not clear whether RLM/BFD relaxation is allowed:

Observation 3: It is not clear whether RLM/BFD relaxation is allowed when long DRX > 80 ms and short DRX < 80 ms are configured, i.e. whether RLM/BFD relaxation depends on the DRX state in such case.

An example of the DRX states including Active Time, short DRX and long DRX:

When the UE is in

- Long DRX (> 80 ms) the UE is not allowed to relax RLM/BFD (independent whether the relaxation criterion is fulfilled)

- Active Time or short DRX (< 80 ms) and the relaxation criterion is fulfilled the UE is allowed to relaxed RLM/BFD

But in such case there is unwanted behavior. If the UE in short DRX is allowed to relax, then transition into long DRX due to inactivity will trigger the UE to send relaxation report and exit long DRX again.

RAN2 should send an LS to RAN4 to clarify the issues discussed in this contribution:

Proposal: Send LS to RAN4 to confirm:

- Section 3.6.1 does not apply to RLM/BFD relaxation?

- RLM/BFD relaxation is not allowed when DRX is not configured?

- RLM/BFD relaxation is not allowed when the configured DRX is longer than 80 ms?

- RLM/BFD relaxation is not allowed when long DRX > 80 ms and short DRX is < 80 ms?

# Summary

RAN2 is kindly asked to discuss RLM/BFD relaxation and DRX:

Observation 1: “no DRX” for relaxed RLM/BFD measurements refers to the DRX configuration and does not refer to “no DRX” as defined in section 3.6.1 in 38.133.

Observation 2: The UE is not allowed to relax RLM/BFD measurements when long DRX > 80 ms is configured and short DRX is not configured or short DRX > 80 ms is configured.

Observation 2: The UE is allowed to relax RLM/BFD measurements when long DRX < 80 ms is configured and short DRX is not configured or short DRX < 80 ms is configured.

Observation 3: It is not clear whether RLM/BFD relaxation is allowed when long DRX > 80 ms and short DRX < 80 ms are configured, i.e. whether RLM/BFD relaxation depends on the DRX state in such case.

Proposal: Send LS to RAN4 to confirm:

- Section 3.6.1 does not apply to RLM/BFD relaxation?

- RLM/BFD relaxation is not allowed when DRX is not configured?

- RLM/BFD relaxation is not allowed when the configured DRX is longer than 80 ms?

- RLM/BFD relaxation is not allowed when long DRX > 80 ms and short DRX is < 80 ms?

# References

- R2-2304538, Report of [AT121bis-e][006][NR17] CP PowSav and DCCA Corrections – 2nd round, CATT, Report, RAN2#121bis-e

- R2-2302800, Correction on RLM/BFD relaxation state reporting, Nokia, CR 38.331, RAN2#121bis-e

- RP-220748, Revised WID UE Power Saving Enhancements for NR, MediaTek, WID, RAN#95