3GPP TSG-RAN WG2 Meeting #122	R2-2306017

Incheon, South Korea, 22nd – 26th May 2023

Agenda Item:	7.25.1

Source:	Ericsson

Title:	Discussion on FR2 unknown SCell activation enhancement

Document for:	Discussion, Decision

# 1	Introduction

RAN2 just received an LS in [1] from RAN4 where there is a task to design a new signalling in order to accommodate some enhancements agreed in RAN4 about the FR2 unknown SCell activation.

In this contribution we discuss how to address the aspect mentioned by RAN4 in their LS.

# 2	Discussion

## 2.1	How to configure the L3 measurement report

According to the LS in [1], RAN2 is asked to design the corresponding signalling according to the following:

- How to configure the report of L3 measurement result for unknown FR2 SCell activation enhancement - Technical enhancement goal: - network configures UE to report L3 measurement result when triggering conditions in (2) are met. - It was RAN4 understanding that a one-time report may be configured. The type of report configuration may be RAN2 decision. - Action request to RAN2: - RAN4 requests RAN2 to design corresponding signalling for the technical enhancement goal above

Therefore, the scenario is that the network configures the UE with L3 measurements and, when certain triggering conditions are met, the UE shall also report L3 measurements that are available but that has not been previously reported. Here the triggering condition mentioned in (2) refer to the reception of the SCell activation command.

Based on this, the understanding is that the legacy way of configuring L3 measurements can be re-used, but the only missing piece is to indicate to the UE that all the L3 measurements shall be reported upon the reception of the SCell activation command.

Network explicitly indicates to the UE whether all available L3 measurements shall be reported upon the reception of the SCell activation command.

How the network can indicate to the UE to trigger a measurement report with all available measurements (that have not been sent before), a straightforward solution would be to introduce a flag in the measurement reporting configuration. An example is given in the following:

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportUponScellActivation                   ReportUponScellActivation

}

}

ReportCGI ::=                     SEQUENCE {

reportAvailableMeas              ENUMERATED {true},

...

}

A new flag in ReportConfigNR IE is introduced to indicate to the UE that it shall report all available L3 measurements (that has not been sent before) when receiving an SCell activation command.

## 2.1	Triggering of L3 measurement report for unknown SCell activation

Based on the text in the LS, it seems that the main triggering condition for reporting the L3 measurement for the unknown SCell activation enhancement is the reception of the SCell activation command.

- How to trigger the report of L3 measurement result for unknown FR2 SCell activation enhancement - Technical enhancement goal: RAN4 had following agreements in RAN4 #106-bis-e meeting, - Agreements (GTW, Monday Apr 17, 2023) - The L3 measurement report for unknown FR2 Scell activation after Scell activation command is triggered by Scell activation command. - FFS whether to have additional events for report triggering. - Action request to RAN2: - RAN4 requests RAN2 to decide: - How the measurement report gets triggered by SCell activation command. RAN4 request RAN2 to design the corresponding signaling and decide whether a new event is needed to achieve above technical enhancement goal.

Even if there could be other triggering conditions for which the UE should report the available L3 measurement results, having as triggering only the reception of the SCell activation command is a quite simple solution and there is no need to specify further complicated options. Therefore, we propose:

The Reception of the SCell activation command is the only triggering condition for reporting the L3 measurements for unknown FR2 SCell activation. No additional triggers are specified.

## 2.1	Content of the L3 measurement report for unknown SCell activation

The last aspect to be addressed based on the LS received by RAN4 is what to include in the L3 measurement report for the unknown SCell activation. Based on the RAN4 agreements, the following is mentioned:

- Contents of the report of L3 measurement result for unknown FR2 SCell activation enhancement and how to send the report - Technical enhancement goal: RAN4 had following agreements in RAN4 #106-bis-e meeting, - Agreements (GTW, Monday Apr 17, 2023) - UE needs to report the L3 measurement result after SCell activation command - Agreement (in 2nd round issue 1-1-5 of R4-2306315) - UE reports L3 measurement result with SSB index for unknown FR2 SCell activation enhancement. - Action request to RAN2: - RAN4 requests RAN2 to design corresponding signaling for “report of L3 measurement result” after SCell activation command for unknown FR2 SCell activation enhancement. - It’s up to RAN2’s decision on which layer’s signaling shall be used to send the report to NW

About the RAN4 agreement that the UE shall report the L3 measurement result with SSB index, the understanding that current RAN2 signalling already support this, but of course it would need to be clarified in the procedural text that if the L3 measurement report is triggered because the reception of an SCell activation command then the UE should include SSB-index of the measured beams.

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                                  OPTIONAL

]] ,

[[

choCandidate-r17                        ENUMERATED {true}                                                           OPTIONAL,

choConfig-r17                           SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16                             OPTIONAL,

triggeredEvent-r17                      SEQUENCE {

timeBetweenEvents-r17                   TimeBetweenEvent-r17                                                    OPTIONAL,

firstTriggeredEvent                     ENUMERATED {condFirstEvent, condSecondEvent}                            OPTIONAL

}                                                                                                               OPTIONAL

]]

}

Therefore, we propose:

Current signalling already supports the reporting of L3 measurement result with SSB index and only an update on the procedural text is needed.

The last issue in on which layer’s signalling this L3 measurement report should be send. So far L3 measurement reports have been always reported via RRC and, it is worth noticing, there is a strong security requirement that measurements shall never be reported without security been activated. According to this, we believe that is quite clear that the reporting has to happen over RRC.

The L3 measurement report for FR2 unknown SCell activation enhancement is reported over RRC signalling.

RAN2 to send a Reply LS to RAN4 with the RAN2 agreements about the FR2 unknown SCell activation enhancements.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Network explicitly indicates to the UE whether all available L3 measurements shall be reported upon the reception of the SCell activation command.

Proposal 2	A new flag in ReportConfigNR IE is introduced to indicate to the UE that it shall report all available L3 measurements (that has not been sent before) when receiving an SCell activation command.

Proposal 3	The Reception of the SCell activation command is the only triggering condition for reporting the L3 measurements for unknown FR2 SCell activation. No additional triggers are specified.

Proposal 4	Current signalling already supports the reporting of L3 measurement result with SSB index and only an update on the procedural text is needed.

Proposal 5	The L3 measurement report for FR2 unknown SCell activation enhancement is reported over RRC signalling.

Proposal 6	RAN2 to send a Reply LS to RAN4 with the RAN2 agreements about the FR2 unknown SCell activation enhancements.

# 4	References

R2-2304636, LS on FR2 unknown SCell activation enhancement, RAN4, 3GPP TSG-RAN WG2#122, Incheon, South Korea, 22 – 26 May 2023