3GPP TSG-RAN WG2 Meeting #123                                        R2-2308135

Toulouse, France, Aug 21th – 25th, 2023

Source:         ZTE Corporation

Title:            Discussion on LPHAP

Agenda item:    7.2.4

Document for:	Discussion and Decision

- Introduction

The WID[1] of Rel-18 positioning for LPHAP is specified as below:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between (e)DRX in RRC_INACTIVE and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

Based on the WID, this contribution provides our thoughts on the each item of the WID, including SRS in RRC_INACTIVE, DL positioning in RRC_IDLE, eDRX cycle extension in RRC_INACTIVE, PRS&DRX alignment, and other enhancement that facilitates power saving positioning, i.e. relaxed PRS measurement for LPHAP.

- Discussion

  - SRS enhancement for LPHAP

In the Rel-18 WID, several solutions towards the SRS mobility are specified as below:

- For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1].

Detailed analysis about the 3 bullets are given in the following subsections.

    - SRS configuration with validity area

Similar as DL-PRS configuration with validity area in Rel-17, the SRS is configured with a cell list, i.e. validity area by the last serving gNB that releases the UE into RRC_INACTIVE. UE will utilize the SRS configuration as long as the UE does not move out of the range of the cell list as RAN2 agreed in RAN2#121[2]:

Agreements: When configured with SRS configuration along with SRS validity area, if the UE reselects to another cell within the SRS validity area during SRS transmission, the UE continues the SRS transmission, subject to validation for SRS transmission. Wait for RAN1 progress for the validation of SRS transmission with issues such as interference, timing advance and spatial relation information, etc.

Based on RAN1’s agreements in RAN1#112[3], the SRS configuration for a validity area is divided into common part and dedicated part. The common part contains SRS time and frequency resource which does not change among cells, the dedicated part contains pathloss and spatial relation information which may change regarding to different cells. Furthermore in RAN#113[4] meeting, RAN1 has agreed that UE just adopts a pathloss/spatial relation information based on a pre-defined rule or a default configuration. Therefore, UE is not required to initiate RA procedure in the new cell requiring/acquiring new configuration.

There is an opinion that let UE notify the new camping gNB when the UE camps on a new cell within the validity area, then LMF can schedule the neighbor gNBs/TRPs of the camping gNB to monitor the SRS, which can avoid the case that multiple gNBs always keep monitoring the SRS configuration. This is more suitable when the validity area is configured very large. When the validity area is configured small, it is not necessary to let all TRPs(small number of them since the area is small) frequently switch the SRS monitoring status. In addition, if UE re-selects frequently, the UE’s power consumption will be higher as UE needs to frequently request/receive new configuration of the new cell, which may also cause frequent NRPPa signaling update between gNB and LMF.

A better way is that UE does not notify the new camping gNB when UE camps on a new cell within the validity area, LMF just schedules all gNBs/TRPs in the validity area to monitor the SRS. This is more power saving at UE side especially when UE reselects frequently, and it saves much NRPPa signaling between LMF and camping gNB/neighbor gNBs, which is suitable for the case that the validity area is configured small. A brief figure is present as figure 1, where the validity area is configured with 3 cells and LMF notify all the 3 TRPs of these cells to monitor the SRS configuration.

Figure 1. Example of LMF notifying all TRPs within validity area to receive SRS

Proposal 1: For the UE within the SRS validity area, support UE keeps on transmitting SRS without notifying the camping gNB. LMF schedules all gNBs/TRPs in the validity area to monitor the SRS.

When UE moves out of the SRS validity area, RAN2#121 has made the following agreement:

Agreement: RAN2 assume when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request. LS to RAN3 to confirm this.

If UE moves out of the validity area and UE still needs to perform positioning using SRS, UE will send a RRC message to the camping cell requesting for new SRS configuration. To ensure the UE is still in RRC_INACTIVE state, UE can initiate RA procedure to the camping cell, then the new cell should interact with the old cell for SRS configuration, finally the new cell releases the UE into RRC_INACTIVE state with the new SRS configuration.

Since the LS has been sent to RAN3 to trigger the procedure discussion, RAN2 should wait for RAN3’s final decision on the procedure when UE reselects out of the validity area.

Proposal 2: Support RAN2 to wait for RAN3’s decision on the procedure when UE reselects out of SRS validity area.

    - Pre-configure multiple SRS configurations

- The motivation of pre-configured multiple SRS configurations and SRS in validity area is the same, i.e., reducing the reception of SRS configuration and reducing state transaction from RRC_INACTIVE to RRC_CONNECTED to save power. The two features also have similar use case that the SRS configuration (or multiple pre-configured SRS configurations) is used when UE performs cell reselection in RRC_INACTIVE, i.e., the SRS configuration (or multiple pre-configured SRS configurations) is applied across multiple cells.

Since the two features have motivation and use case in common, in order to reduce the duplicate specification work, we think a better way is to make the two features (pre-configured multiple SRS configurations and SRS in validity area) as an unified approach. To be specific, each pre-configured SRS configuration can be associated with a cell ID or a validity area:

- If cell ID is associated, multiple pre-configured SRS configurations are associated with multiple cell IDs, which can also formulate a ‘cell list’.

- If validity area is associated, it means UE is configured with multiple validity areas and each of them is pre-configured with a SRS configuration. Within each validity area, it is the the feature of SRS in validity area; among validity areas, it is the feature of pre-configured multiple SRS with SRS activation request.

- If only one pre-configuration with cell ID or validity area is configured, it is the feature of SRS in validity area; if more than one pre-configuration with cell ID or validity area is configured, it is the feature of pre-configured multiple SRS.

Proposal 3: Support to pre-configure one or more SRS configurations, each of the SRS configuration is associated with a cell ID or a validity area.

    - SRS activation/configuration request

RAN2#122[5] has agreed a SRS activation request:

Agreement: RAN2 will introduce an activation indication and/or request for preconfigured SRS using at least Msg3/MsgA; FFS if Msg1 would be supported also. FFS RRC signalling or MAC CE for the Msg3/MsgA case, as for the configuration request. This agreement does not imply that the UE will be allowed to transmit autonomously.

This SRS activation request is used for UE to activate one of the pre-configured SRS when some condition happens. Such as figure 2, if each pre-configured SRS is associated with a cell or a validity area, when UE switches the cell or validity area, UE can send this SRS activation request to the camping gNB.

Figure 2. Example of UE sends SRS activation request upon switching the validity area

- If the SRS activation request is via Msg3/MsgA, that means camping gNB knows the UE ID at the same time. gNB can response UE using Msg4 RRCRelease with suspend config to indicate activated SRS transmission.

- if pre-configured SRS is associated with cell identification and told to all neighboring gNBs in advance, when the camping gNB receives the Msg3, the camping gNB will also know which SRS the UE want to send since camping gNB is aware of the cell ID. In this case, Msg3 does not need to indicate SRS ID that the UE want to transmit, UE can reuse RRCResumeRequest message with additional cause.

- If pre-configured SRS is not associated with cell identification, UE has to include the requested SRS ID to the gNB in Msg3. In this case, the RRCResumeRequest can not be reused. So a new Msg3 UL RRC message can be introduced for the SRS activation request.

- If the SRS activation request is via Msg1 preamble, gNB needs to know the UE identity. So the Msg1 preamble should be UE specific dedicate preamble. The problem is that there may not be enough dedicate preamble resources if the cell owns plenty of LPHAP UEs. And the dedicate preamble of the UE should be reserved by all the neighbor gNBs. Furthermore, the agreement does not imply that the UE will be allowed to transmit autonomously, that means gNB should provide a response to UE on whether the SRS can be permitted. That means the dedicate preamble should further associated with SRS ID. That will make the preamble resource even more scarce.

- Proposal 4: Support UE sends SRS activation request to request pre-configured SRS upon switching the validity area. The requested pre-configured SRS is the pre-configured SRS that is associated with the validity area.

- Proposal 5: Support UE to reuse RRCResumeRequest with additional cause as SRS activation request.

- Proposal 6: Do not support Msg1 based SRS activation request.

RAN2#121-bis-e[6] meeting has agreed a SRS configuration request:

Agreement: SRS configuration request can be indicated via Msg3/MsgA transmission. FFS if the request is in the RRC message or an accompanying MAC CE.

When the UE moves out of a validity area and the UE does not have the SRS configuration for the new validity area, UE should send the SRS configuration request to request one. Since this request only contains a information that UE wants a SRS configuration, a RRCResumeRequest with a new cause would be enough.

- The additional cause can be the same with that in SRS activation request. If UE do the request with the cause, and if the gNB has the pre-configured SRS associated with the cell/area, gNB will assume the UE is asking for sending the SRS according to the pre-configured SRS; If the gNB does not have the pre-configured SRS associated with the cell/area, gNB will assume the UE is asking for the new SRS configuration.

- Proposal 7: SRS activation request and SRS configuration request both use RRCResumeRequest with same additional resume cause.

    - Common aspects of the two features

      - Supported service type

During Rel-18 SI phase in RAN2#119e[7] meeting, the following agreement is achieved:

Agreement RAN2 to consider at least the ‘Low Power Periodic and Triggered 5GC-MT-LR Procedures’ in TS 23.273. Other procedures are not excluded from discussion.

We agreed at least deferred MT-LR based solution is applied to Rel-18 SRS enhancement. In implementation, there exists scenario that MT-LR is useful in LPHAP UE adopting use case 6., i.e., network-triggered real-time inventory/positioning of goods in the factory. In this way LPHAP UE does not need to wait for the event trigger. Therefore, MT-LR should also be supported for Rel-18 SRS enhancement of LPHAP UE.

Proposal 8: Support MT-LR for Rel-18 SRS enhancement of LPHAP UE.

      - SRS type

Both Rel-18 SRS features share the cell reselection procedure in RRC_INACTIVE. In Rel-17 SRS transmission in RRC_INACTIVE, periodic SRS and semi-persistent SRS are both supported. In Rel-18, the first issue is to confirm whether the two kinds of SRS are still valid during cell reselection. For periodic SRS, if the RRCRelease configures UE with periodic SRS for a validity area, UE can start to transmit the periodic SRS in the last serving cell, and cell reselection will not interrupt the sending; if semi-persistent SRS is configured for a validity area, UE is feasible to receive the DL MAC CE activation command in a cell if SDT is supported in that cell without transiting to RRC_CONNECTED state. Therefore, periodic SRS and semi-persistent SRS can both be supported in the SRS (pre-)configuration across multiple cells.

For semi-persistent SRS, if DL MAC CE activation command is received in the last serving cell via SDT, UE starts transmitting the SRS until the TA timer expires, cell reselection does not break the SRS sending. If UE camps on another cell rather than the last serving cell, under the condition that LMF requests last serving gNB to activate the semi-persistent SRS transmission, since gNB does not know which cell UE is currently in, all gNBs should first perform UE paging, then if the cell that UE camps on supports SDT function, UE will send RRC Resume request on the camp cell using RA SDT resource configured via broadcast signaling. When SDT is activated, gNB can send the DL MAC CE to activate/deactive the semi-persistent SRS transmission in that cell.

Observation 1: If semi-persistent SRS is (pre-)configured across multiple cells, UE is feasible to receive the DL MAC CE activation command in a cell if SDT is supported in that cell.

Proposal 9:For positioning SRS (pre-)configured across multiple cells, support to (pre-)configure both periodic SRS and semi-persistent SRS.

      - TA timer validation

The SRS validation is also determined by TA timer and RSRP threshold according to the agreement of RAN1#112:

Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold

The validity area-specific TA timer is agreed to be introduced for SRS in validity area, and the agreement indicates the details can be up to RAN2. To ensure the smooth switch between cells while the SRS keeps transmitting, RAN2 should further consider and define how this TA timer works (start, restart, stop) across multiple cells. RAN2#122 has achieved the following agreement about the TA validation:

Agreement Define an SRS for positioning validity-area specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. - The UE starts/restarts the area-specific TA timer when it receives the TA command. - The UE stops the SRS transmission when the area-specific TA timer expires. - The UE stops the area-specific TA timer when it reselects to a cell out of the SRS validity area. - Other stop/restart conditions can be discussed.

In Rel-17, according to TS 38.331, except for the cell reselection case, the TA timer will be stopped (if running) at the following conditions:

- If the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1, and srs-PosRRC-InactiveConfig is configured;

- Upon reception of the RRCResume, and srs-PosRRC-InactiveConfig is configured.

- So the new TA timer should better reuse the Rel-17 timer behaviour on stop action, i.e., if RRCSetup or RRCResume is received, UE stops the TA timer. That means UE enters the RRC_CONNECTED when move around.

- Proposal 10: Support additional TA timer stop condition: if RRCSetup or RRCResume is received, UE stops the Rel-18 SRS TA timer.

The agreement says that the UE stops the SRS transmission when the area-specific TA timer expires. if UE just stops the SRS transmission and still keeps the SRS configuration in the memory, there seems no use case for this. Therefore we believe the Rel-17 mechanism can be reused in Rel-18 SRS, i.e., UE should release the SRS configuration when the area-specific TA timer expires.

- When UE switches validity area and use pre-configured SRSs, UE will stop transmitting the SRS but UE does not need to release the SRS configuration, since UE may switch back later. When UE switches to a cell/validity area that is not pre-configured to UE before, UE should release the multiple pre-confiogured SRS configuration.

- Proposal 11: Support UE to release the Rel-18 SRS configuration when the Rel-18 TA timer is expired.

- Proposal 12: Support UE to release the Rel-18 SRS configuration when UE switches to a cell that is not pre-configured to UE before.

      - NRPPa enhancement

In the legacy procedure, LMF firstly request the serving gNB of the UE to configure SRS according to the NRPPa IE positioning information request. Since in RRC_CONNECTED state, LMF can track every serving gNB that UE currently in, it is enough to only send the positioning information request to the serving gNB. However in RRC_INACTIVE state, the SRS configuration adopted by UE should guarantee that the SRS is allowed to send(TRP is available to receive) in each camping cell. Note that LMF is not aware of each cell’s available SRS characteristic which best suits the cell, how can LMF make the right decision on the request of multiple SRS characteristics? That is to say, letting LMF to request multiple SRS configuration characteristics or letting serving gNB itself configure multiple SRS configuration in isolation to suit the cell reselection case can not guarantee the valid SRS transmission when cell reselection happens.

Therefore, it is necessary to let LMF coordinate with multiple TRPs on the available SRS configuration in advance. In addition, LMF know the LPHAP UE indication and can assume LPHAP UE as always in RRC_INACTIVE/RRC_IDLE. As analysed in section 2.1.1 and 2.1.2, this enhancement can be applied to both features of SRS in validity area and pre-configure multiple SRSs. The procedure is as figure 3 below:

Figure 3. NRPPa procedure of coordination Rel-18 SRS configurations between LMF and multiple TRPs

When LMF gathers the available SRS configuration of each cell, LMF can tell the serving gNB. The last serving gNB can make the final decision on the size of the validity area and the corresponding SRS configuration according to the RNA area.

Since RAN2 is the leading group of this feature, it is reasonable for RAN2 to discuss NRPPa details and notify RAN3. so if this is agreed, an LS to RAN3 is also needed.

Proposal 13: Support LMF to coordinate with multiple TRPs on the available SRS configuration before the SRS configuration is provided to the LPHAP UE. The available SRS configuration includes both SRS for validity area and Send LS to RAN3 to notify them RAN2’s agreement.

      - Unified RRC configuration

In RAN2#121-b, the following agreement is achieved for SRS with validity area:

Agreements: RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.

We prefer to pre-configure the multiple SRS configuration in the RRC Release message too. From specification perspective, SRS with validity and pre-configured multiple SRS share the same principle and the same structure of configuration, i.e., one or multiple SRS configurations, in each SRS configuration, one or multiple SRS resource sets/SRS resources can be included. Therefore, a unified ASN.1 design of them is desired to simplify the specification impact. Like figure 4 indicates, TA timer and RSRP threshold are associated with a SRS (pre-)configuration list, and each SRS configuration in the SRS (pre-)configuration list should be associated with a physical cell ID or a cell list. If there is only one SRS config element in the SRS config list, that is the feature of SRS with validity; if there are multiple SRS config elements in the SRS config list, and each SRS config element associates with a cell ID or a cell ID list, that is the feature of pre-configured multiple SRS.

Figure 4. an example of a unified ASN.1 design of 1st bullet and 2nd bullet

For dedicate signaling, the Rel-18 SRS configuration for multiple cells can be configured in RRC Release with Suspend Config, and an example ASN.1 design is provided in the Annex.

Proposal 14: Support to configure the Rel-18 SRS(both SRS in validity area and pre-configurd multiple SRS) in RRC Release with Suspend Config.

Proposal 15: Support a unified ASN.1 design of SRS within validity area and pre-configured multiple SRSs,  and take the example configuration in Annex as baseline.

  - Extend eDRX cycle in RRC-INACTIVE

- In Rel-18 discussion, RAN2#121 RedCap AI has made the following agreements regarding to the larger eDRX cycle:

- Agreements： - RAN2 confirms the R17 agreements made at RAN2#114 for enhanced INACTIVE eDRX: - - It is up to RAN to configure the length for PTW for RAN paging, the RAN PTW length can be different from the CN PTW length. - - When RAN and CN paging coincide in the same PH, the actually used PTW starting location is the same for RAN and CN paging. FFS how to calculate the PTW starting location so that it is the same for RAN and CN PTW. - PTW length value range of enhanced INACTIVE eDRX is same as IDLE eDRX, i.e. from 1.28s to 40.96s in the step of 1.28s. - Long eDRX cycle (>10.24 s) value range of enhanced INACTIVE eDRX is same as IDLE eDRX from 20.48s to 10485.76s, i.e. hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024 - Add the configuration of eDRX cycle (>10.24 s) and PTW length for enhanced INACTIVE eDRX in the RRCRelease message

It is observed that the PTW for eDRX of RAN initiated paging in RRC_INACTIVE will be introduced, the RAN initiated PTW and CN initiated PTW are individually configured. So the new RAN initiated PTW should be take into consideration for LPHAP UE. In addition, the value range of eDRX cycle for RRC_INACTIVE is the same as that in RRC_IDLE, which means the hyperframe will also be used in RRC_INACTIVE. However the current value range of PRS periodicity is {4,5,8,10,16,20.32.40.64.80.160.320.640.1280.2560.5120,10240} in the unit of ms. Thus, extending PRS periodicity is benefit for making better alignment between PRS and eDRX in RRC_INACTIVE and/or RRC IDLE for the power saving purpose.

Proposal 16: Support to extend the PRS periodicity larger than 10240ms to suit the eDRX cycle value for eDRX paging cycle in RRC_INACTIVE and/or RRC_IDLE.

  - Alignment between DL-PRS and DRX/eDRX

RAN2#120[8] positioning AI has reached the following agreement on alignment between DL-PRS and DRX:

Proposal2: Alignment between DRX and PRS is beneficial from power saving point of view for LPHAP and is recommended to normative work. (14/15)  Two directions of solutions for DRX/PRS alignments are considered: (a) PRS alignment with fixed DRX (b) DRX alignment with fixed PRS  Solutions for the PRS/DRX alignment, e.g., LMF-based/UE-based solution, is to be discussed  Impacts to different RRC states (RRC_INACTIVE and RRC_IDLE) is to be discussed

And the WID is updated as “specify solutions on alignment between (e)DRX in RRC_INACTIVE and PRS configurations”, so DRX and eDRX in RRC_INAVTIVE are both supported.

According to TS38.304[9], for the case that DRX is configured for UE in RRC_INACTIVE or RRC_IDLE, but eDRX for CN paging and RAN paging is not configured(or it is configured but not on according to the switch in SIB1), UE monitors CN paging or RAN paging according to the minimum paging cycle value (among RAN initiated paging cycle, CN initiated paging cycle and default paging cycle in the broadcast message) and the paging location parameter in the RRC SIB.

For the case that eDRX for CN paging(and/or eDRX for RAN paging) is configured and is on, and eDRX paging cycle value is longer than 1024 radio frames, there will be PTW(paging time window). Currently UE is configured with one PTW within one eDRX cycle for CN paging.

- In RRC_IDLE, since there is no RAN paging, UE can only receive CN initiated paging during the PTW.

- In RRC_INACTIVE, there is a case that eDRX cycle for CN paging and eDRX cycle for RAN paging coexists, at this case if eDRX cycle for CN paging is larger than 1024 radio frames, UE should maintain 2 paging cycles/periodicities for indise-PTW and outside-PTW, respectively.

- Additionally in RAN2#121-bis RedCap AI has made the following agreements towards RAN PTW configuration:

Agreements: For the overlapping PH, RAN PTW starting location is determined based on CN eDRX cycle. For the non-overlapping PH, PTW starting location for RAN PTW is determined based on the CN eDRX cycle. In an overlapped or non-overlapped PH: Within RAN PTW and outside CN PTW, T = RAN configured DRX cycle If this is even a valid case (we will decide later): In an overlapped PH: Within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}. In an overlapped PH: Within both CN PTW and RAN PTW, T = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}.

- It is observed that RAN PTW is also introduced in RRC_INACTIVE. Therefore PRS alignment should consider the RAN PTW in eDRX cycle >10.24s, and consider the case that DRX cycle is configured and eDRX cycle is configured but PTW is not enabled.

- Observation 2: In RRC_INACTIVE and eDRX cycle is larger than 10.24s, RAN PTW and CN PTW may be coexist.

- After determining the paging cycle value, UE will have to determine the exact PF location, PO location and MO location for monitoring paging. It can be seen that the final paging location(PF/PO/MO location) that UE adopts in the current cell is determined by many network configuration and pre-defined criteria. In addition, in RRC_INACTIVE, UE may adopt different paging location(PF/PO/MO location) for different cells when cell-reselection happens. Therefore, it is harder to adjust DRX(especially for multiple-cell cases) than adjust PRS.

- Observation 3: The final paging location(PF/PO/MO location) that UE adopts in the current cell is determined by many network configuration and pre-defined criteria. In addition, in RRC_INACTIVE, UE may adopt different paging location(PF/PO/MO location) for different cells when cell-reselection happens. So it is harder to adjust (e)DRX than adjust PRS.

On-demand PRS procedure can be reused here for PRS to align with fixed DRX. Note that on-demand PRS procedure contains UE-initiated on-demand PRS procedure and LMF-initiated on-demand PRS procedure. LMF needs to acquire the UE’s current paging location to determine the PRS transmission characteristics. For LMF-initiated on-demand PRS procedure, this can be enabled by serving NG-RAN node telling LMF. However UE’s paging location in different cell may be different, and gNB has no idea where the UE is currently camping on in RRC_INACTIVE/RRC_IDLE, that means in LMF initiate on-demand PRS procedure, it is hard for gNB to update paging information to the LMF when UE cell reselects. On the contrary, UE can trigger on-demand PRS request if the paging location changed during cell-reselection. Therefore, we support to use UE-initiated on-demand PRS procedure for PRS to align with fixed DRX.

Proposal 17: Support to use UE-initiated on-demand PRS procedure for PRS to align with fixed DRX.

The percentage of PF/PO time occupation within one DRX/eDRX cycle is rather small. For example, normally one PO can last 4ms, while the smallest DRX paging cycle is even 320ms. Therefore if UE wants to request some PRS configuration/parameter to suit the current PO location, only requesting PRS periodicity may not be enough, UE should also request the PRS time offset to LMF. The PRS time offset is associated with the requested PRS periodicity, with the unit of subframe or slot. The PRS time offset can indicate the start time of the PF, PO or the first MO in a paging cycle.

Note that UE’s requested PRS periodicity is not necessarily the same with the DRX/eDRX cycle. For power saving purpose, UE should request a PRS periodicity larger or equal to the DRX/eDRX cycle value. If UE requests a PRS periodicity larger than the DRX cycle value, since one PRS period may contain multiple DRX cycles, UE should also request more than one PRS time offset values regarding to the requested PRS periodicity, where each PRS time offset corresponds to a paging location in one DRX/eDRX cycle. If UE requests a PRS periodicity same as the paging cycle value, since UE only monitor one PO in a DRX cycle, if requested PRS periodicity is the same as DRX value, requesting one PRS time offset is enough. Figure 5 gives an example of requested PRS periodicity is larger than DRX cycle. In this case, 2 PRS time offset values should be associated with a requested PRS periodicity.

- Figure 5. Example of UE’s PRS request when PRS periodicity is larger than DRX cycle

Proposal 18: In LPP on-demand PRS request message, support UE to request the PRS time offset associated with each requested PRS periodicity to better align the actual paging location.

As analysed above, UE may have different paging cycle value inside PTW and outside the PTW, resulting in different periodicities of PF/PO/MO location. However in Rel-17 on-demand PRS procedure, UE is only able to request one PRS resource set periodicity value per frequency layer, indicating the UE wants the PRS to have a certain single PRS periodicity. If PTW is enabled by UE, the UE should request more than 1 PRS periodicity values in a on-demand PRS request for the alignment of paging cycle inside the PTW and outside the PTW, respectively. An example is provided in figure 6 where UE is enabled with a PTW.

Figure 6. Example of UE’s PRS request when having a PTW

Proposal 19: In LPP on-demand PRS request message, support UE to request separate PRS periodicities per PFL in order to align with the paging cycle inside-PTW and outside-PTW, respectively.

Current PTW length configuration value is {1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21,22,23,24,25,26,27,28,29,30,31,32} with the unit of 1.28 seconds, which means a PTW can last from 1280ms to 40960ms. According to the 37.355 QoS requirement of response time, a positioning procedure is desired with a time duration between 10ms to 1280s. It can be seen that the time duration of positioning procedure is different from the time duration of a PTW and they have overlap part. It is feasible to let UE perform positioning only during the PTW. In addition, since UE usually monitors more frequently in the PTW, UE is better to perform positioning as well as receiving PRS during the PTW.

This can be achieved by UE with the IE dl-prs-StartTime-and-Duration in the on-demand PRS request. For example, UE firstly determines the PTW location in time domain according to the formula in TS38.304, then UE can set the requested PRS start time as the PTW_start and set the requested PRS end time as the PTW_end, as specified in TS38.304, section 7.4.

Proposal 20: In LPP on-demand PRS request message, support UE to use dl-prs-StartTime-and-Duration to request the PRS reception within the PTW(if existed).

For NRPPa design, LMF requests the PRS transmission characteristics to the serving gNB and neighbor gNBs in the NRPPa PRS CONFIGURATION REQUEST message. As analysed above, UE may request more than one PRS periodicities or PRS time offset values. These should also be included in the NRPPa PRS CONFIGURATION REQUEST, too.

Proposal 21: Support LMF to include UE’s requested PRS periodicities or PRS time offsets in the NRPPa PRS CONFIGURATION REQUEST message.

  - Measurement in RRC_IDLE and report in RRC_CONNECTED

According to RAN2#116[10] meeting, when UE is to perform positioning in RRC_INACTIVE, UE can utilize the positioning assistance data by several ways:

Agreement: Proposal 4 (modified): For positioning in RRC_INACTIVE state, the positioning assistance data can be delivered to UE through the following ways: - positioning system information, i.e. posSIB;(12/13) - pre-configure assistance data when UE in RRC_CONNECTED state;(11/13) - send to UE in RRC_INACTIVE during ongoing SDT procedure. (9/13)

Since UE does not have SDT in RRC_IDLE, when UE is to perform positioning in RRC_IDLE, UE can utilize the positioning assistance data through posSIB or pre-configured AD when UE in RRC_CONNECTED.

Proposal 22: Support UE to utilize the positioning assistance data through posSIB or pre-configured assistance data in RRC_CONNECTED when UE is to perform positioning in RRC_IDLE.

  - Relaxed PRS measurement for LPHAP

The main purpose of LPHAP is to perform low power positioning for work pieces tracking in a assembly area and warehouse of a factory, as use case 6 in TS22.104[11] indicates. Under the scenario, most of the time the work pieces remains almost the same position, or the moving speed and track range are largely lower than the normal UE. Therefore, reducing the PRS reception chance of these low mobility LPHAP UE will not have huge impact on the positioning accuracy.

Observation 4: Regarding use case 6, reducing the PRS reception chance of these low mobility LPHAP UE will not have huge impact on the positioning accuracy.

There are two directions to reduce the PRS reception chance:

- NW configures less PRS reception chance for LPHAP UE. This is inflexible since LPHAP UE may have unstable movement speed or track range, e.g., move fast at one time, move slow at another time. It is hard and complex for network to schedule UE with PRS reception suitable for UE’s current situation.

- NW does not change the PRS configuration, instead, UE determines when to reduce the PRS reception chance according to some criteria. The critical issue is how to set this appropriate criteria while lightening the impact on the positioning accuracy. We think the UE not at cell edge criteria or low mobility criteria for relaxed RRM measurement (see TS 38.304, section 5.2.4.9) can be reused here, i.e., if UE satisfies the not at cell edge criteria or low mobility criteria by measuring the SSB, which means UE is in a relatively stable movement situation, thus it is not necessary to have frequent PRS reception or measurement as the network may configure to track the UE’s real-time position. Then, UE can decide to perform relaxed PRS measurement by reducing some PRS reception chance. There are some specific methods for relaxed PRS measurement. For example, larger the PRS reception period at UE side, or a pre-defined PRS configuration for UE to apply when UE satisfies the not at cell edge criteria or low mobility criteria.

Proposal 23: Support relaxed PRS measurement for LPHAP by reducing the PRS reception chance according to some criteria, e.g., UE not at cell edge criteria or low mobility criteria for relaxed RRM measurement can be reused.

- Conclusion

- In this section we provide the following observations and proposals:

- SRS enhancement

Proposal 1: For the UE within the SRS validity area, support UE keeps on transmitting SRS without notifying the camping gNB. LMF schedules all gNBs/TRPs in the validity area to monitor the SRS.

Proposal 2: Support RAN2 to wait for RAN3’s decision on the procedure when UE reselects out of SRS validity area.

Proposal 3: Support to pre-configure one or more SRS configurations, each of the SRS configuration is associated with a cell ID or a validity area.

- Proposal 4: Support UE sends SRS activation request to request pre-configured SRS upon switching the validity area.

- Proposal 5: Support UE to reuse RRCResumeRequest with additional cause as SRS activation request.

- Proposal 6: Do not support Msg1 based SRS activation request.

- Proposal 7: SRS activation request and SRS configuration request both use RRCResumeRequest with same additional resume cause.

Proposal 8: Support MT-LR for Rel-18 SRS enhancement of LPHAP UE.

Observation 1: If semi-persistent SRS is (pre-)configured across multiple cells, UE is feasible to receive the DL MAC CE activation command in a cell if SDT is supported in that cell.

Proposal 9:For positioning SRS (pre-)configured across multiple cells, support to (pre-)configure both periodic SRS and semi-persistent SRS.

- Proposal 10: Support additional TA timer stop condition: if RRCSetup or RRCResume is received, UE stops the Rel-18 SRS TA timer.

- Proposal 11: Support UE to release the Rel-18 SRS configuration when the Rel-18 TA timer is expired.

- Proposal 12: Support UE to release the Rel-18 SRS configuration when UE switches to a cell that is not pre-configured to UE before.

Proposal 13: Support LMF to coordinate with multiple TRPs on the available SRS configuration before the SRS configuration is provided to the LPHAP UE. The available SRS configuration includes both SRS for validity area and Send LS to RAN3 to notify them RAN2’s agreement.

Proposal 14: Support to configure the Rel-18 SRS(both SRS in validity area and pre-configurd multiple SRS) in RRC Release with Suspend Config.

Proposal 15: Support a unified ASN.1 design of SRS within validity area and pre-configured multiple SRSs,  and take the example configuration in Annex as baseline.

- Extend eDRX cycle in RRC_INACTIVE

Proposal 16: Support to extend the PRS periodicity larger than 10240ms to suit the eDRX cycle value for eDRX paging cycle in RRC_INACTIVE and/or RRC_IDLE.

- Alignment between PRS and DRX

- Observation 2: In RRC_INACTIVE and eDRX cycle is larger than 10.24s, RAN PTW and CN PTW may be coexist.

- Observation 3: The final paging location(PF/PO/MO location) that UE adopts in the current cell is determined by many network configuration and pre-defined criteria. In addition, in RRC_INACTIVE or RRC_IDLE, UE may adopt different paging location(PF/PO/MO location) for different cells when cell-reselection happens. So it is harder to adjust (e)DRX than adjust PRS.

Proposal 17: Support to use UE-initiated on-demand PRS procedure for PRS to align with fixed DRX.

Proposal 18: In LPP on-demand PRS request message, support UE to request the PRS time offset associated with each requested PRS periodicity to better align the actual paging location.

Proposal 19: In LPP on-demand PRS request message, support UE to request separate PRS periodicities per PFL in order to align with the paging cycle inside-PTW and outside-PTW, respectively.

Proposal 20: In LPP on-demand PRS request message, support UE to use dl-prs-StartTime-and-Duration to request the PRS reception within the PTW(if existed).

Proposal 21: Support LMF to include UE’s requested PRS periodicities or PRS time offsets in the NRPPa PRS CONFIGURATION REQUEST message.

- Measurement in RRC_IDLE

Proposal 22: Support UE to utilize the positioning assistance data through posSIB or pre-configured assistance data in RRC_CONNECTED when UE is to perform positioning in RRC_IDLE.

Relaxed PRS measurement for LPHAP

Observation 4: Regarding use case 6, reducing the PRS reception chance of these low mobility LPHAP UE will not have huge impact on the positioning accuracy.

Proposal 23: Support relaxed PRS measurement for LPHAP by reducing the PRS reception chance according to some criteria, e.g., UE not at cell edge criteria or low mobility criteria for relaxed RRM measurement can be reused.

- References

- RP-2301460, Revised WID on Expanded and Improved NR Positioning, 2023-06.

- Report of 3GPP TSG RAN WG2 meeting #121, 2023-03.

- Report of 3GPP TSG RAN WG1 meeting #112, 2023-03.

- Report of 3GPP TSG RAN WG1 meeting #113, 2023-05.

- Report of 3GPP TSG RAN WG2 meeting #122, 2023-05.

- Report of 3GPP TSG RAN WG2 meeting #121-bis-e, 2023-04.

- Report of 3GPP TSG RAN WG2 meeting #119-e, 2022-08.

- Report of 3GPP TSG RAN WG2 meeting #120, 2022-11.

- TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state, 2022-03.

- Report of 3GPP TSG RAN WG2 meeting #116, 2021-11.

- TS 22.104, Service requirements for cyber-physical control applications in vertical domains, 2021-03.

- Annex: unified Rel-18 SRS configuration

This section provides an example of SRS (pre-)configuration across multiple cells.

SuspendConfig ::=                   SEQUENCE {

...

srs-PosRRC-Inactive-Pre-r18             SetupRelease { SRS-PosRRC-Inactive-Pre-r18 }                            OPTIONAL,   -- Need M

...}

SRS-PosRRC-Inactive-Pre-r18 ::= OCTET STRING (CONTAINING SRS-PosRRC-Inactive-PreConfig-r18)

SRS-PosRRC-Inactive-PreConfig-r18 ::=      SEQUENCE {

SRS-PosRRC-Inactive-PreConfigList-r18        SEQUENCE (SIZE(1..maxNrofPre-SRS-configId-r18)) OF SRS-PosRRC-Inactive-PreConfig-r18                                          OPTIONAL,    -- Need M

inactivePrePosSRS-TimeAlignmentTimer-r18   TimeAlignmentTimer           OPTIONAL,    -- Need M

inactivePrePosSRS-RSRP-changeThreshold-r18 RSRP-ChangeThreshold-r17   OPTIONAL     -- Need M

}

SRS-PosRRC-Inactive-PreConfig-r18 ::=       SEQUENCE {

Pre-srs-configId-r18                         Pre-SRS-configId-r18

PreSRS-AssociatedCell-r18	                PreSRS-CellList-r18

Pre-srs-PosConfigNUL-r18                SRS-PosConfig-r17               OPTIONAL,    -- Need R

Pre-srs-PosConfigSUL-r18                SRS-PosConfig-r17             OPTIONAL,    -- Need R

Pre-srs-bwp-NUL-r18                        BWP                       OPTIONAL,    -- Need S

Pre-srs-bwp-SUL-r18                        BWP                        OPTIONAL,    -- Need S

}

Pre-SRS-configId-r18 ::=                   INTEGER (0..maxNrofPre-SRS-configId-r18-1)

PreSRS-CellList-r18 ::= SEQUENCE (SIZE(1..maxPreSRS-CellIDs-r18)) OF NR-Cell-IDs-r18

NR-Cell-IDs-r18 ::= SEQUENCE {

nr-CellGlobalID-r18				NCGI-r15					OPTIONAL,	-- Need M

nr-PhysCellID-r18				NR-PhysCellID-r16			OPTIONAL,	-- Need M

nr-ARFCN-r18					ARFCN-ValueNR-r15			OPTIONAL,	-- Need M

...

}