3GPP TSG-RAN WG2 Meeting #123                                        R2-2308138

Toulouse, France, Aug 21th – Aug 25th, 2023

Source:         ZTE Corporation

Title:            Discussion on sidelink positioning

Agenda item:    7.2.2

Document for:	Discussion and Decision

- Introduction

The Rel-18 WID[1] of sidelink positioning has been achieved as follows:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

RAN2 is mainly responsible for the protocol design between UEs, or between UE and LMF. Based on above, this contribution provides discussion and proposals for SLPP protocol and procedure design.

- Discussion

  - Anchor/server UE selection

In RAN2#122[2] meeting, agreement on anchor UE selection has been made as follows:

Agreements: Anchor UE selection is supported by information about the candidate anchor UEs. At least the following list can be discussed for use in anchor UE selection: 1. UE roles 2. Supported positioning method 3. In coverage or not 4. RSRP 5. LOS/NLOS 6. Location 7. PLMN A normative requirement on which anchor UEs to select (e.g., ranking) will not be specified. RAN2 impact of this information to be determined. FFS which information would be determined statically/dynamically.

In TS23.586[2], the located UE selection is described in section 5.2.2 as:

A Target UE may discover and select one or more Located UEs to be used in the Ranging/SL positioning procedures as specified in clauses 5.3 to 5.5. Multiple candidate Located UEs may be discovered, in that case, the Located UE(s) is selected from the candidate Located UE list. The Located UE(s) is selected based on: - Candidate list of Located UE(s), if available - Capabilities of the candidate Located UE(s), e.g. the supported Sidelink Positioning methods - The required positioning QoS - Whether the serving PLMN of candidate Located UE(s) is same with serving PLMN of Target UE

It can be seen that the criteria that RAN2 and SA2 defined has some overlap. To be specific, the 1st 2nd 7th in RAN2 agreement is also specified in SA2’s spec. For each of the criteria that RAN2 and SA2 have raised, we provide our views on whether it applies to anchor UE selection and/or server UE selection, and whether the criteria should be in dynamic signaling, meta data field in discovery message or no signaling:

- UE roles

  - It can be applied to both anchor UE selection and server UE selection.

  - the SA2’s LS S2-2305735[4] clearly indicates it should be in meta data field in discovery message.

- Capabilities of supported positioning method

- It will be efficient if target UE can find some anchor UEs that supports specific positioning method. It can be applied to both anchor UE selection and server UE selection.

- This restriction can be conveyed in SLPP capability transfer message, via broadcast/groupcast.

- Capabilities other than supported positioning method

- For anchor UE selection, it is better for target UE to take anchor UE’s other UE capabilities into consideration when selecting an anchor UE. This enables a more accurate searching&selection for different positioning QoS requirements. For server UE selection, target UE should find the UE with server UE functionalities, i.e., method determination, assistant data distribution and/or location calculation. Only a UE that has one of these functionalities can be declared as a server UE.

- This restriction can be conveyed in SLPP capability transfer message, via broadcast/groupcast.

- PLMN

- It can be applied to both anchor UE selection and server UE selection.

- TS23.586 has restricted that the anchor UE and target UE should be in the same PLMN in Rel-18, furthermore, this indication has less burden from signaling payload perspective. So it is also ok to convey this in the meta data field in discovery message.

- QoS

- According to TS23.596 and TS23.273, the current Ranging/SL Positioning QoS information includes the following:

- Accuracy: i.e. Horizontal Accuracy and Vertical Accuracy. - the relative horizontal accuracy, and the relative vertical accuracy for relative positioning; - the distance accuracy and direction accuracy for Ranging. - Range, which indicates the applicability of the QoS attributes in the Ranging/SL Positioning operation over PC5. - Response Time (e.g. no delay, low delay or delay tolerant as described in clause 4.3.3 of TS 22.071 [2]).

- It is not clear how target UE can select anchor UE with specific QoS requirement, since it is the first step of a positioning session, the target UE or anchor UE will not know whether the service can satisfy the QoS at this moment when the positioning has not started yet. Therefore, RAN2 should wait for SA2’s further clarification on this QoS criteria.

- In coverage or not

- Although current SL positioning can support all kinds of coverage scenarios, it is better for target UE to know whether the discovered candidate anchor UEs are in coverage or not, more accurately, target UE should have the knowledge that the other anchor UEs and itself can be assisted by the LMF/NG-RAN or not. If target UE and anchor UEs are both non-network-assisted and none of the UEs can perform SL Positioning Server UE functionalities, a SL Positioning Server UE should be discovered and selected. When the target UE selects a server UE, it does not matter that the server UE is in coverage or not.

- 23.586 has the following editor’s note about server UE selection:

SL Positioning Server UE Discovery & Selection is performed by the Target UE, when it meets one or more of the following criteria: - The Target UE and the discovered SL Reference UEs are currently not served by a network supporting Ranging/SL Positioning (e.g. because they are out-of-coverage or the serving network does not support Ranging/SL Positioning). The Ranging/SL Positioning Support is configured in the AMF according to network capabilities. If AMF receives Ranging/SL Positioning Capability from UE, AMF may include Ranging/SL Positioning Support indicator in the Registration Accept message to indicate that the serving network supports Ranging/SL Positioning. SL Reference UE and Target UE may indicate whether its serving network supports Ranging/SL Positioning or not to each other. The indication of serving network not supporting Ranging/SL Positioning can be reused to notify each other when UE has no NAS connection available. Editor's note: Need to determine if such indication is provided during discovery or capability exchange, and whether or not it requires RAN WG2 involvement.

The indication ‘whether its serving network supports Ranging/SL Positioning or not’ implies whether the node is in coverage or not. We slightly prefer to include this indication in discovery message meta data field since it may only contain 1 bit.

- RSRP

- Regarding to the issue how the anchor UE is considered good enough to be selected as anchor UE or server UE, at least the AS layer criteria of relay UE selection is suitable to be reused, since SL positioning also needs good channel condition at PC5 interface. To be specific, the criteria that target UE determines whether the UE can be an anchor UE or a server UE is: whether the candidate UE’s transmission RSRP is larger than a threshold. The transmission RSRP can be SL signaling RSRP or SL-PRS RSRP, depending on the case of first time selection or reselection. It can be applied to both anchor UE selection and server UE selection.

- As for the threshold, target UE can determine a threshold by implementation, and if the received RSRP of the candidate UE is good enough to beyond the threshold, the candidate UE can be selected as anchor UE or server UE. Therefore, neither dynamic signaling nor the meta data field in discovery message will be needed.

- LOS/NLOS

- Target UE should take LOS/NLOS conditions of a anchor UE or server UE when performing selection, the more LOS links it have, the better positioning accuracy it will get. It can be applied to both anchor UE selection and server UE selection.

- RAN2 does not need to define the dynamic signaling for this criteria or to convey it in discovery message, it is enough to only describe it in stage-2.

- Location

- Sometimes target UE would like to find some anchor UEs with known location, especially when the target UE is not provided by LMF with a list of anchor UEs whose location are known. However, it does not matter if server UE has a known location or not, as long as the server UE has good channel linkage quality between itself and target UE. Therefore, it only applies to anchor UE selection, not server UE selection.

- With known location or without known location is a 1 bit signaling. So it can be conveyed in discovery message.

- In summary, for anchor UE selection, target UE should consider UE capabilities, in coverage or not, RSRP, LOS/NLOS and known location or not as RAN2 criteria; for server UE selection, target UE should consider UE capabilities, in coverage or not, RSRP and LOS/NLOS as RAN2 criteria. The criteria of UE roles, PLMN, in coverage or not, known location or not should be in meta data field in discovery message, and the criteria of UE capability should be in the SLPP capability transfer message.

According to SA2’s specification TS23.586, both mode A and mode B discovery are supported. Mode A discovery is for one UE to broadcast ‘I am an anchor/server UE. I have some capabilities/functionalities. I can be discovered’ to the neighbor UEs. Mode B discovery is for target UE to request other UE’s discovery response, so the solicitation message should be as ‘I am a target UE, and I would like to find the anchor/server UE with some specific positioning requirements’, and if anchor/server UE is interested at the service, it can make the discovery message response as ‘I am an anchor/server UE. I satisfies such specific positioning requirements’.

Proposal 1: For anchor UE selection, support target UE to consider RSRP, LOS/NLOS, UE capability and known location as RAN2 criteria. For server UE selection, support target UE to consider RSRP, LOS/NLOS and UE capability as RAN2 criteria. FFS on QoS.

Proposal 2: The selection criteria of UE roles, PLMN, in coverage or not, known location or not can be contained in discovery message(both Mode A announcement and Mode B Solicitation and Response), and selection criteria of UE capability can be contained in SLPP capability transfer message. FFS on QoS.

- SA2’s LS also asks ‘when does the selection take place’. Since some of the criteria are included in discovery message and some of the criteria are included in SLPP capability transfer message, it is reasonable that the anchor/server UE selection takes place when target UE and anchor/server UE finish the discovery message transfer and SLPP capability transfer.

Here gives example procedures for mode A and mode B discovery and anchor UE selection in figure 1 and figure 2, respectively:

Figure 1. Procedure of mode A, anchor/server UE discovery and selection

- Step 1: Anchor UE(s) broadcast discovery announce message, which contains some of the selection criteria.

- Step 2: target UE sends request capability message via SLPP broadcast/groupcast.

- Step 3: candidate anchor/server UEs send their SL positioning capability to the target UE via broadcast/groupcast/unicast.

- Step 4: target UE determines final anchor/server UE which satisfies the criteria that carried in discovery message and the criteria in UE capability.

- Step 5: target UE may send the acknowledgement to the selected anchor/server UEs in step 4, e.g., to set up SL positioning session.

- Step 6: Target UE and selected anchor/server UEs begin to perfrom SL positioning (further capability if any/assistance data/SL-PRS transfer/location information transfer).

Figure 2. Procedure of mode B, anchor/server UE discovery and selection

- Step 1: Target UE sends the discovery solicitation message which contain its UE role as target UE, and contains some of the selection criteria.

- Step 2: other UE monitors the discovery request message to see if it satisfies the criteria that contained in the discovery request message.

- Step 3: Target UE receives the discovery response of several candidate UEs which satisfy the criteria that contained in the discovery request message.

- Step 4: target UE sends request capability message via SLPP broadcast/groupcast.

- Step 5: several candidate UEs send their SL positioning capability to the target UE via broadcast/groupcast/unicast.

- Step 6: Target UE selects final anchor/server UEs based on the criteria in UE capability from the several candidate UEs.

- Step 7: target UE may send the acknowledgement to the selected anchor/server UE, e.g., to set up SL positioning session.

- Step 8: Target UE and selected anchor/server UEs begins to perfom SL positioning (capability if any/assistance data/SL-PRS transfer).

Proposal 3: Reply LS to SA2 that anchor/server UE selection takes place when finishing discovery message and SLPP capability transfer message.

  - Assistance information to UE

SA2 has sent a LS S2-2307553[5] to RAN2 as following:

SA2 has defined procedures of SL-MO-LR and SL-MO-LR, which include - UE based Ranging/SL Positioning, for which result calculation is performed at UE - Network based Ranging/SL Positioning, for which result calculation is performed at LMF For UE based Ranging/SL Positioning, SA2 believes that LMF may provide some assistance information to UE to facilitate UE to perform result calculation. SA2 would check with RAN2 about the following questions: Q1: Whether it is needed to provide assistance information to UE for result calculation? Q2: If answer to Q1 is yes, whether the assistance information needs to be exposed to SA2 and is relevant to any SA2 procedure? Q3: If answer to Q2 is yes, what are the assistance information and how SA2 is expected to update the related procedure?

In legacy Uu positioning, LMF provides NR-PositionCalculationAssistance to UE for UE-based positioning, which contains TRP location information, spatial directions and corresponding power of DL-PRS Resources for TRPs for DL-AoD method, time synchronization information between TRPs for DL-TDOA method, expected likelihood of a LOS propagation path and TRP Tx TEG information.

For UE based Ranging/SL Positioning, and for absolute positioning, the UE needs to know the SL-PRS measurements and anchor UE’s location for all the positioning methods. If SL-AoA or SL-RTT is used, no additional assistance information is needed.If SL-TDOA is used, RTD between anchor UEs is also needed at the UE side where the location is calculated, as also indicated by RAN1#113 agreement:

Agreement Support at least the following mechanism to mitigate the impact of synchronization errors between anchor UEs for SL-TDoA based measurement Exchange of synchronization information of anchor UEs between a UE and LMF or another UE. FFS detailed synchronization information. E.g: synchronization source, relative time difference (RTD), synchronization quality information FFS other mechanisms

For relative positioning and ranging, the UE only needs to know the SL-PRS measurements. So the UE does not need to be provided with any assistance information for location calculation for relation positioning/ranging.

Proposal 4: reply SA2 with a LS indicating that, for UE based Ranging/SL Positioning, when absolute positioning and SL-TDOA is used, the UE should be provided with RTD between anchor UEs as assistance information. For other cases in Rel-18, no assistance information is needed.

  - Stage-2 procedure

SA2 has developed the following procedures in TS23.586:

- 6.5 Procedure for UE Positioning assisted by Sidelink Positioning and involving 5GC

  - 6.5.1 Procedures for Network based SL Positioning for UE with NAS connection

    - 6.5.1.1 MO-LR Procedure for Network based SL Positioning for UE with NAS connection(reference: 6.20.1, 6.20.2 in TS23.273)

    - 6.5.1.2 MT-LR Procedure for Network based SL Positioning for UE with NAS connection(reference: 6.20.5 in TS23.273)

  - 6.5.2 Procedures for Network based SL positioning for UE without NAS connection(reference: 6.20.1, 6.20.2 in TS23.273+ modification in TS23.586)

  - 6.5.3 Procedures for UE based SL Positioning for UE with NAS connection

    - 6.5.3.1 MO-LR Procedure for UE based SL Positioning for UE with NAS connection(reference: 6.20.1, 6.20.2 in TS23.273)

    - 6.5.3.2 MT-LR Procedure for UE based SL Positioning for UE with NAS connection (reference: 6.20.5 in TS23.273)

  - 6.5.4 Procedures for UE based SL Positioning for UE without NAS connection(reference: 6.5.2 in TS23.586)

- 6.6 Procedure of UE only Sidelink Positioning for Target UE using Located UE (reference: 6.8 in TS23.586)

- In TS23.273[7] section 6.20.1, the procedure is as follows, in which step 10~20 are RAN2 procedures for the scenario that either UE(target UE or anchor UEs) has a NAS connection:

Figure 6.20.1-1: SL-MO-LR Procedure 10. The LMF sends a request to UE1 for the capabilities of UEs 1 to n. NOTE 3: UE2/.../UEn is not assumed to be served by the same LMF serving UE1. Editor's note: It is FFS the update when the LMF also serves UE2/.../UEn with alignment with RAN. 11. UE 1 returns its capabilities to the LMF. UE1 may additionally return the capabilities of the UEs obtained at step 5 if requested by the LMF at step 10. 12. UE1 may send a request for specific assistance data to the LMF. 13. LMF sends the requested assistance data to UE1, and UE1 forwards the assistance data received from LMF to UE2/.../UEn. The assistance data may assist UEs 1 to n to obtain Sidelink location measurements at step 15 and/or may assist UE1 to calculate Sidelink positioning/ranging location results at step 16. NOTE 4: Steps 10 and 11 can be omitted if UE1 includes a message containing the capabilities of UEs 1 to n in the SL-MO-LR request at step 8. Step 12 can be omitted if UE1 includes a message containing the request for specific assistance data in the SL-MO-LR request at step 8. Editor's note: Whether step 10-11 are needed will be aligned with RAN WGs. 14. If the SL-MO-LR request at step 8 indicated location calculation assistance is needed and/or indicated transfer of Sidelink positioning/ranging location results to an LCS Client or AF, the LMF sends a request for location information to UE1 and may also send a request for location information to UE2/.../UEn if it is served by the LMF. If LMF determines to apply UE based SL Positioning, LMF includes in the request the indication of UE based SL Positioning. LMF may also provide the list of candidate Located UE(s), if absolute location is requested at step 8. If scheduled location time is received at step 14. LMF may include a scheduled location time. Editor's note: It needs to be aligned with RAN WG2 if the list of candidate Located UE(s) is provided in step 13 or step 14. 15. UE1 instigates a Sidelink positioning/ranging procedure among UEs 1 to n in which UEs 1 to n obtain Sidelink location measurements and UEs 2 to n transfer their Sidelink location measurements to UE 1 and/or to the LMF (depending on the assistance requested). If scheduled location time is received at step 14, Sidelink positioning/ranging is performed at the scheduled location time. 16. If Target UE's absolute location information is required at step 8 and if absolute location of Located UE(s) is not available, the Target UE sends a request to the Located UE(s) to trigger 5GC-MO-LR procedure to let the Located UE(s) acquire their own absolute location. The QoS requirement received at step 8 is included in the request, which is used to derive the QoS for Located UE(s) positioning. 17. If LMF determines to use UE based calculation, at least one of UE1/.../UEn calculates Sidelink positioning/ranging location results based on the Sidelink location measurements obtained at step 15 and possibly using assistance data received at step 13. The Sidelink positioning/ranging location results can include absolute locations, relative locations or ranges and directions related to the UEs 1 to n. 18. If UE1 received a request for location information at step 14, UE1 sends a response to the LMF and includes the Sidelink location measurements obtained at step 15, the Sidelink positioning/ranging location results obtained at step 17 if step 17 was performed, or Located UE's absolute location obtained at step 16. 19. If Target UE's absolute location information is required at step 8 and if absolute location of Located UE(s) is not received at step 18, LMF can either retrieved the location of the Located UE(s) locally or triggers 5GC-MT-LR procedure to the GMLC to acquire the absolute location of the Located UE(s) using Application Layer ID or GPSI of the Located UE(s). LMF includes the QoS requirement received at step 8 in the request, which is used to derive the QoS for Located UE(s) positioning. If scheduled location time is used, LMF includes the scheduled location time in the request to GMLC. 20. The LMF calculates Sidelink positioning/ranging location results for UEs 1 to n from the Sidelink location measurements received at step 18 and absolute location of Located UE(s) at step 19. The Sidelink positioning/ranging location results can include absolute locations, relative locations or ranges and directions related to the UEs 1 to n, depending on the location request received in step 8.

In TS23.586 section 6.8, the procedure is as follows, in which step 4 ~8 are RAN2 procedures and should be same among the scenarios that all the UEs without NAS connection:

Figure 6.8.1-1 Procedures for Ranging/Sidelink Positioning control (UE-only operation) 4. UE1 and UE2/.../UEn perform capability exchange. Step 4 may be performed during step 5 and step 6 with coordination of SL Positioning Server UE. 5. If UE1 does not support SL Positioning Server functionalities, a SL Positioning Server UE (either co-located with a SL Reference UE/Located UE, or operated by a separate UE) is discovered (if not yet discovered in step 2) and selected. If a SL Positioning Server UE that is co-located with a SL Reference UE/Located UE or operated by a separate UE, UE1 discovers and selects the SL Positioning Server UE as described in clause 6.4 and requests SL Positioning Server UE to participate in the Ranging/Sidelink positioning. NOTE 2: Details of security and privacy related procedures during SL Positioning Server UE discovery and operation are developed by SA WG3. 6. Sidelink Positioning assistant data is transferred among UE1/ .../UEn and the SL Positioning Server UE. 7. SL-PRS measurement is performed between UE1 and UE2/.../UEn and possibly also amongst UE2/.../UEn. 8. SL-PRS measurement data is transferred to the SL Positioning Server UE or is transferred to UE1 if it supports SL Positioning Server functionalities, in order to perform result calculation. Based on the type of the result received in step 1, absolute location, relative location or ranging information is calculated at the UE. NOTE 3: Details of step 4-8 are developed by RAN WGs.

Therefore, there are 3 kinds of scenarios that will impact stage 2 procedure:

- Scenario 1: at least one of a UE has NAS connection

  - Scenario 1-1: target UE has NAS connection;

  - Scenario 1-2: target UE does not have NAS connection, while one of the anchor UE has NAS connection;

- Scenario 2: none of the UE has NAS connection

Scenario 1 can be used for the procedure with LMF involvement, and can take TS23.273 section 6.20.1 as baseline; scenario 2 can be used for the procedure without LMF involvement, and can take TS23.586 section 6.8 as baseline. From specification writing perspective, we think at least the above two scenarios should be captured in the stage-2 specification.

Proposal 5: Capture at least the following scenarios in the RAN2 stage-2 specification:

- Scenario 1: at least one of a UE has NAS connection(take TS23.273 section 6.20.1, step 10~20 as baseline)

  - Scenario 1-1: target UE has NAS connection;

  - Scenario 1-2: target UE does not have NAS connection, while one of the anchor UE has NAS connection;

- Scenario 2: none of the UE has NAS connection(take TS23.586 section 6.8, step 4~8 as baseline)

  - SLPP specification design

In RAN2#121-bis-e meeting, SLPP specification design has been discussed. In which, all the need code(need M, need S, need N, need R) and toaddmodlist/toreleaselist are called delta signaling and they are not sure whether to be introduced in the SLPP specification. Delta signaling is useful for forward compatibility. Note that SLPP signaling can be broadcasted/groupcasted/unicasted, they are different from PC5-RRC message which can only be unicasted. For example, need M and toaddmodlist/toreleaselist can not be applied to broadcast or groupcast since UE may not able to receive the corresponding field before. To decide the delta signaling use, RAN2 should first decide whether to separate broadcast/groupcast ASN.1 signaling and unicast ASN.1 signaling as current RRC (SIBx and dedicate signaling) in SLPP specification, or to have joint ASN.1 signaling that can be applied to all broadcast/groupcast/unicast. If they are present separately, unicast dedicate signaling can have all the need code and toaddmodlist/toreleaselist, broadcast/groupcast signaling can only have need S, need R and need N; if they are present jointly, all the need code can be applied, need M should be clarified that only applied to unicast mode. Further, toaddmodlist/toreleaselist should not be introduced in the joint signaling.

To reduce the work of specification design, we slightly prefer to have joint signaling since there are almost same IEs of unicast and broadcast/groupcast.

Another issue is that, SLPP can be transmitted between UEs, or between UE to LMF. So in 38.355, term ‘sender’ and ‘receiver’ can be used to indicate signaling direction, and distinguish UE/LMF when it is necessary, e.g., when the ASN.1 signaling contains different IEs for UE-UE and UE-LMF.

Observation 1: Delta signaling (including all the need code and toaddmodlist/toreleaselist) is useful for forward compatibility.

Proposal 6: For SLPP specification design, support joint ASN.1 signaling of broadcast/groupcast and unicast, all the need code(need M, need R, need S, need N) can be applied, need M should be clarified that only applied to unicast mode.

Proposal 7: ToAddModList/ToReleaseList should not be introduced in the joint signaling.

Proposal 8: The term ‘sender’ and ‘receiver’ can be used to indicate signaling direction, and signali8ngs should distinguish UE/LMF when it is necessary.

  - RAN1-related design for SL positioning

- RAN1#113 has agreed the following that RAN2 should pay attention to:

- Agreement - In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot: - The PSSCH is used for 2nd SCI and SL-SCH - Note: the UE may not have data available for transmission. Up to RAN2 how to define the specification support for this case.

- The MAC PDU format is specified as follows:

- 6.1.6 MAC PDU (SL-SCH) A MAC PDU consists of one SL-SCH subheader and one or more MAC subPDUs. Each MAC subPDU consists of one of the following: - A MAC subheader only (including padding); - A MAC subheader and a MAC SDU; - A MAC subheader and a MAC CE; - A MAC subheader and padding. The MAC SDUs are of variable sizes.

- The empty PSSCH can be achieved by - a MAC subheader only, or - a MAC subheader and padding. That means, it is feasible from the format design perspective to have this empty PSSCH. However the MAC entity should know the empty PSSCH is for the SL-PRS transmission in shared resource pool, otherwise the MAC entity shall not generate such MAC PDU, which is specified by the following MAC specification:

- 5.22.1.4.1.3 Allocation of sidelink resources The MAC entity shall not generate a MAC PDU for the HARQ entity if the following conditions are satisfied: - there is no Sidelink CSI Reporting MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.7; and - there is no Sidelink DRX Command MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.8; and - there is no Sidelink Inter-UE Coordination Request MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.9; and - there is no Sidelink Inter-UE Coordination Information MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.10; and - the MAC PDU includes zero MAC SDUs.

- So, the MAC specification should add an additional bullet to indicate when a SL-PRS is available, even if there is zero MAC SDUs and zero MAC CEs, the MAC entity can also generate a MAC PDU. The example spec impact is as follows:

- 5.22.1.4.1.3 Allocation of sidelink resources The MAC entity shall not generate a MAC PDU for the HARQ entity if the following conditions are satisfied: - there is no Sidelink CSI Reporting MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.7; and - there is no Sidelink DRX Command MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.8; and - there is no Sidelink Inter-UE Coordination Request MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.9; and - there is no Sidelink Inter-UE Coordination Information MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.10; and - the MAC PDU includes zero MAC SDUs; and - there is no SL-PRS transmission in shared resource pool.

- Observation 2: It is feasible from the format design perspective to have this empty PSSCH.

- Proposal 9: The MAC specification should have another bullet to indicate when a SL-PRS is available, even if there is zero MAC SDUs and zero MAC CEs, the MAC entity can also generate a MAC PDU.

- Further, RAN1#113 has agreed the following:

Agreement Location information of the target UE based on sidelink positioning measurements can be reported at least to LMF. - FFS: on whether quality information of location is included, e.g., uncertainty etc - Up to other WGs to determine whether location information of the target UE can be reported to another UE - Up to RAN2 for signaling details - FFS: whether and how to report per ARP location information.

- Based on current investigation, UE can report measurement results of SL-PRS to another UE or LMF. Further, RAN1 agreed that the calculated location information can be reported to LMF, similar like UE-based positioning(DL-TDOA, DL-AoD) in Uu interface. SA2 and RAN2 already agreed that the server UE has the location calculation functionality. If server UE is not the same UE as the target UE, it should be allowed that the server UE gathers all UEs SL-PRS measurement, calculates the location information of the target UE, and sends the calculated location information to the target UE. For privacy consideration, the server UE should not send the location information of the target UE to the UE(s) which is not target UE.

- Proposal 10: Server UE can send the calculated location information of the target UE to the target UE. Other cases are not allowed.

  - Joint SL and Uu positioning methods and measurements

According to RAN2#119[7] in SI phase, both SL-only-based positioning and joint SL & Uu positioning are studied and recommended for normative work:

- Agreement - Proposal 2: Study the architecture and signaling procedures to enable at least the following two operation scenarios: - - Operation Scenario 1: PC5-only-based positioning. - - Operation Scenario 2: Combination of Uu- and PC5-based positioning.

DL-TDOA-like method is a typical technique for joint positioning where both DL-PRS from multiple TRPs and SL-PRS from multiple anchor UEs are transmitted to target UE. Take Figure 3 as an example, target UE’s position cannot be calculated by either Uu positioning or SL positioning since DL-TDOA-like positioning requires at least 3 anchor nodes wherein one of them is a reference node. In such case, we have to merge both Uu measurements (i.e. measured between target UE and TRP 1/2) and SL measurements (i.e. measured between target UE and anchor UE 1/2).

Figure 3. DL-TDOA-like joint SL & Uu positioning

However, usually anchor UEs and TRPs are not accurately synchronized and reference nodes for SL and Uu positioning are selected separately. For Uu positioning, a reference TRP (e.g. TRP 1) is selected as reference node, Uu RSTD measurement is obtained by calculating relative timing difference between the anchor TRP and a reference TRP. For SL positioning, a reference UE (e.g. anchor UE 1) is selected as reference node, SL measurement RSTD is obtained by calculating relative timing difference between the anchor UE and the SL reference node. For UE-assisted positioning, DL measurement and SL measurement can be reported to LMF jointly. In order for the LMF to know the time difference or time synchronization error between Uu session and SL session, a special RSTD measurement can be introduced and reported from target UE to LMF. This special RSTD is used to specify the measurement of the relative timing difference between SL reference node and Uu reference node.

Proposal 11: Support DL-TDOA-like joint SL & Uu positioning:

- For a target UE, Support reporting a RSTD for the timing difference between SL reference node and Uu reference node.

- Conclusion

In this contribution, we propose the following observation and proposals:

Anchor/server UE selection

Proposal 1: For anchor UE selection, support target UE to consider RSRP, LOS/NLOS, UE capability and known location as RAN2 criteria. For server UE selection, support target UE to consider RSRP, LOS/NLOS and UE capability as RAN2 criteria. FFS on QoS.

Proposal 2: The selection criteria of UE roles, PLMN, in coverage or not, known location or not can be contained in discovery message(both Mode A announcement and Mode B Solicitation and Response), and selection criteria of UE capability can be contained in SLPP capability transfer message. FFS on QoS.

Proposal 3: Reply LS to SA2 that anchor/server UE selection takes place when finishing discovery message and SLPP capability transfer message.

Assistance information to UE

Proposal 4: reply SA2 with a LS indicating that, for UE based Ranging/SL Positioning, when absolute positioning and SL-TDOA is used, the UE should be provided with RTD between anchor UEs as assistance information. For other cases in Rel-18, no assistance information is needed.

Stage-2 procedure

Proposal 5: Capture at least the following scenarios in the RAN2 stage-2 specification:

  - Scenario 1: at least one of a UE has NAS connection(take TS23.273 section 6.20.1, step 10~20 as baseline)

    - Scenario 1-1: target UE has NAS connection;

    - Scenario 1-2: target UE does not have NAS connection, while one of the anchor UE has NAS connection;

  - Scenario 2: none of the UE has NAS connection(take TS23.586 section 6.8, step 4~8 as baseline)

SLPP specification design

Observation 1: Delta signaling (including all the need code and toaddmodlist/toreleaselist) is useful for forward compatibility.

Proposal 6: For SLPP specification design, support joint ASN.1 signaling of broadcast/groupcast and unicast, all the need code(need M, need R, need S, need N) can be applied, need M should be clarified that only applied to unicast mode.

Proposal 7: ToAddModList/ToReleaseList should not be introduced in the joint signaling.

Proposal 8: The term ‘sender’ and ‘receiver’ can be used to indicate signaling direction, and signali8ngs should distinguish UE/LMF when it is necessary.

RAN1-related design for SL positioning

- Observation 2: It is feasible from the format design perspective to have this empty PSSCH.

- Proposal 9: The MAC specification should have another bullet to indicate when a SL-PRS is available, even if there is zero MAC SDUs and zero MAC CEs, the MAC entity can also generate a MAC PDU.

- Proposal 10: Server UE can send the calculated location information of the target UE to the target UE. Other cases are not allowed.

Joint SL and Uu positioning methods and measurements

Proposal 11: Support DL-TDOA-like joint SL & Uu positioning:

- For a target UE, Support reporting a RSTD for the timing difference between SL reference node and Uu reference node.

- References

- RP-2301460, Revised WID on Expanded and Improved NR Positioning, 2023-06.

- Report of 3GPP TSG RAN WG2 meeting #122, Inchon, France, 2023-05.

- TS23.586, Architectural Enhancements to support Ranging based services and Sidelink Positioning, 2023-06.

- S2-2305735, Reply LS to LS to SA2 on Sidelink positioning procedure, SA2, 2023-05.

- S2-2307553, LS on assistance information provided to UE, SA2, 2023-05.

- Report of 3GPP TSG RAN WG1 meeting #113, Inchon,France, 2023-05.

- TS23.273, 5G System (5GS) Location Services (LCS), Stage-2, 2023-06.