3GPP TSG RAN WG1 #123			R2-2308176

Toulouse, France, August 21st – May 25th, 2023

Agenda Item: 	 7.16.2.1

Source:		Sony

Title:			On Other Aspects of AI/ML for positioning accuracy enhancement Document for:	Discussion & Decision

# Introduction

A new study item on Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RANP#94e [1]. Some of the objectives of the study item is shown below:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

At RAN2#122 [2}, the following agreements were made

Intention is to cover functional arch in general, e.g. covering both be model based and/or functionality based LCM

“Model Storage” in the figure is only intended as a reference point (if any) for protocol terminations etc for model transfer/delivery etc. It is not intended to limit where models are actually stored. Add a note for this.

Remove “Model” in Model Managemt and Model Inference and for the actions/the arrow form Management to Inference (to reduce the risk for misunderstanding).

Management may be model based management, or functionality based management. Add a mote for this.

With the modifications above Figure 2 from R2-2305327 is agreed

P1a: For the LS to RAN1 on data collection requirement, inform RAN1 that the reply should be per use case and per LCM purpose (i.e., Model training, inference and monitoring), and LCM sidedness should also be considered.

RAN 2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive):

- - For model inference of UE-sided model, input data for model inference is available inside the UE.

- - For UE-side (real time) monitoring of UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW.

P2a: LS to ask RAN1 to provide the required data content per use case and per LCM purpose, when available, and to what extent said data would / should be specified (in detail).

P2b: LS to ask RAN1 about the reporting type (e.g., periodic, event triggered, other) of the identified data content.

P3: LS to ask RAN1 about the typical size (value or value range) of the identified data content.

P4a: For the latency requirement of data collection, RAN2 assumes:

- - for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection

- - for model inference, when required data comes from other entities, there is a latency requirement for data collection

- - for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection.

P4b: LS to RAN1 to confirm the WA (in P4a) on the latency requirement, and ask RAN1 about the typical latency requirement (value or value range) to transfer the identified data content.

P6a: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement on the non-connected state can be revisited when needed.

P6b: LS to RAN1 to confirm the WA (in P6a) on RRC state of data collection.

P5a: For the data generation entity and termination entity deployed at different entities, RAN2 assumes:

- For CSI enhancement and beam management use cases:

- - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server.

- - For NW-sided model inference, input data can be generated by UE and terminated at gNB.

- - For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE.

- - For model monitoring at NW side, performance metrics can be generated by UE and terminated at gNB.

- For positioning enhancement use case:

- - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server.

- - For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB.

- - For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE.

- - For model monitoring at NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

P5b: LS to RAN1 to confirm the WA (in P5a) on the generation entity and termination entity of the identified data content and ask for supplement, if any.

This contribution provides our views on AI/ML for positioning, particularly on AI/ML assisted positioning to improve positioning accuracy.

# Discussions on AI/ML Assisted Positioning

## AI/ML model and procedure

In principle, the network (specifically, LMF) can collect a huge amount of positioning measurement data which could be from different UEs/gNBs. It enables LMF to further create and train an AI/ML model using the set of collected data.  This method can be particularly used for NLOS mitigation and can achieve an improved detection accuracy.

The usage of AI/ML can be briefly divided into three phases:

- Data generation/collection phase

- Model training and updating phase

- Model deployment phase (Inference)

These three phases and the involved communication units are illustrated in Figure 2. In the data generation/collection phase, channel quality and multipath information are generated and collected for further analysis. This procedure is performed by UE or gNB, particularly for the UE with a fixed-location that can provide an exact location. The sources of the data can differ in DL and UL cases. In DL, UE receives the reference signal and processes the data, while in UL, the gNB performs this procedure. After the measurement is performed, LMF collects the results, such as the path information, quality matrix and possible its own location. It should be noted that in this phase, LMF also needs to process and validate the data to further transfer it to a training set as an input to the model training.

Once LMF collects a certain amount of data, LMF can perform ML model training. Model training step may include model selection, training, validation and testing. Lastly, at the output from the training phase, an inference model will be deployed in UE or gNB or LMF to handle specific inference operation, e.g., NLOS detection.

Data Collection with processing and validation

(LMF or reference UE/gNB)

Model Training and Updating

(LMF)

Model deployment

(UE/gNB/LMF)

Figure 2: Procedure of Machine learning utilization for positioning.

The procedure of AI/ML for positioning can be at least divided in three phases:

- Data generation/collection with data processing and validation,

- Model Training and updating,

- Model deployment.

Observation 1: The basic concept of AI/ML procedure has been agreed.

In one of the examples of AI/ML assisted, the network (e,g., LMF) trains and create AI/ML model or here, we call it prediction models (PM) given the information from the UE/gNB. Note, we consider the details of the PM creation using AI/ML is up to the implementation. The second aspect is about signalling the prediction model in the model deployment phase (AI/ML model inference). After the model training, the LMF can utilize the model and apply it on the new measurements. But the model may not necessarily be used only in LMF. The UE can also perform the PM deployment or also known as inference. For example, LMF can also provide the prediction model to UE and allow them/it? to perform ML inference. This can avoid heavy computation in LMF.

Observation 2: Support AI/ML Positioning with model training at LMF and model inference at the UE side has been agreed at last RAN2 meeting [3].

This process require the transmission of PM to the UE. Hence, we should also consider the inference model structure and size. It is beneficial to provide the inference model in smaller size (without compromising the performance), particularly to the UEs given the environment to the UE operates in. The inference model from LMF to the UE can consider the following aspects:

- The inference model can be partially or fully provided.

- The time validity of the inference model.

- The structure (i.e., number and types of stages, interconnections, etc.) of the predictive neural network as part of the inference model.

- The measurement that the UE should perform associated with the provided inference model.

Observation 3: Support for AI/ML model indication, and definition of the inference model (e.g., contents, structure, size) to be provided from LMF to UE/gNB has been agreed at last RAN2 meeting.

## Multipath reporting

One of examples in AI/ML assisted positioning can be facilitated by utilizing multipath reporting that was introduced in Rel-17, particularly the multiple paths reporting mechanism. For DL-TDOA and Multi-RTT positioning, the UE is capable of measuring and reporting a maximum of 8 additional paths. In the measurement report, the indices of the additional paths and the associated relative path power measurement (PRS-RSRPP) for each path could be reported. Likewise in UL case, the gNB could also report multipath measurements (i.e, UL-RSRPP) to LMF.

By allowing UE/TRP to report the received power of the multipath components, the LMF has the possibility to process and measure in the channel delay domain. Based on the outcome of positioning in Rel-17, it is up to LMF on how to utilize this information (i.e., implementation aspect of LMF). We consider the usage of AI/ML to process such information, including the collected path measurements from UE/TRP, would be beneficial in AI/ML assisted positioning sub use-case, especially to improve the positioning accuracy.

Multipath measurement and reporting open up a new opportunity for AI/ML deployment, since LMF can collect more detailed channel information from UE/TRP. For example, in the legacy, LMF would only obtain one timing measurement per measurement report. The accuracy of identifying LOS/NLOS mainly relies on the UE. But now, since Rel-17, the LMF is capable of acquiring multiple path information. Having the path information including relative signal power (PRS-RSRPP) and relative delay of multiple propagation path, LMF can make its own decision on the LOS path identification.

Proposal  1: Support multiple paths reporting from UE/TRP to LMF in order to assist network-side (e.g., LMF) to make its own decision on LOS path selection.

## Ground truth label information

In the last RAN1 meeting [3], RAN1 has made an agreement on ground truth label generation, that the UE/LMF is able to provide estimated/known location information via NR RAT-dependent or NR RAT-independent methods as ground truth label. Apart from that we also consider LOS information can be used as ground truth label in terms of LOS/NLOS identification.

To train an AI/ML model for NLOS identification, LMF needs to be aware of the true LOS/NLOS labels of the measurements in the training set. These labels can be considered as the ground truth label and it should be informed by the UE, preferably a reference UE or reference gNB which knows its own location. Furthermore, the location information itself can be considered as the ground truth label.

Proposal 2: Apart from location information, support LOS/NLOS information as the ground truth label.

# Summary

In this contribution, we have discussed our view on usage of AI/ML for positioning. Our observations and proposals are listed below:

Observation 1: The basic concept of AI/ML procedure has been agreed.

Observation 2: Support AI/ML Positioning with model training at LMF and model inference at the UE side has been agreed at last RAN2 meeting [3].

Observation 3: Support for AI/ML model indication, and definition of the inference model (e.g., contents, structure, size) to be provided from LMF to UE/gNB has been agreed at last RAN2 meeting.

Proposal 1: Support multiple paths reporting from UE/TRP to LMF could assist network-side (e.g., LMF) to make its own decision on LOS path selection.

Proposal 2: Apart from location information, support LOS/NLOS information as the ground truth label.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #94e

- Minutes RAN2#123

- Minutes RAN1#113