3GPP TSG-RAN WG2 Meeting #123	R2-2308195

Toulouse, France, 21– 25 August 2023

Agenda item:	7.16.2.1

Source:	Nokia, Nokia Shanghai Bell, T-Mobile US, Samsung

Title:	AI/ML Architecture

SID:	FS_NR_AIML_air (Rel-18)

Document for:	Discussion and Decision

# Introduction

A modified agenda item was added as of RAN3#123 for AIML architectural aspects.

(FS_NR_AIML_air; leading WG: RAN1; REL-18; WID:RP-221348) Time budget: 1 TU Tdoc Limitation: 4 tdocs Aspects of on-line/real-time training are deprioritized. NOTE that from Now also R1 parts of the TR can be / should be used as baseline for R2 discussions. ### 7.16.1 Organizational LS ins. Rapporteur input. Expect to discuss and clarify the RAN2 input to the TR. Including [Post122][059][AIML] TR text proposal (Ericsson) ### 7.16.2 AIML methods Explore AIML methods that are expected applicable to this SI and their expected or potential architecture (allocation of functionality to entities), Identification asepcts, other framework aspects, impact on RAN2. Most of LCM is in RAN2 scope. Both general aspects and use-cases specific aspects are applicable (for use cases in scope). . Please input to 7.16.2.x #### 7.16.2.1 Architecture and General Discussion on Model ID is postponed. Based on RAN1 progress, Can discuss the AIML algorithm dependency on locality (e.g. cell specific), UE-side AIML dependency on gNB configuration etc, dependency on other aspects such as UE speed, Network-side AIML dependency to be UE specific etc, and the related procedure impacts. Can discuss the expected impacts for Network Side-algorithms. UE Cap: On a high level, Identify potential impacts to RRC and LPP UE capabilities or equivalent functionality if any. Progress the logical arch (if needed). Mapping of Functionality to entities, general aspects. Including [Post122][060][AIML] Mapping of functions to physical entities (CMCC)

In this paper, we will discuss architectural aspects related to the functionality and model identification. Further we emphasize on functionality-based life cycle management (LCM) for ML-enabled features and functionality support indication as well as associated information.

# Discussion

## Two different issues to handle

As discussed in our last several papers [1, 5] and from the previous agreements in RAN1#112, RAN1#112bis-e [2, 3], we identify two different methods to identify and manage AIML-enabled features or feature groups [6]. However, during RAN2#122 [4], RAN2 agreed on the functional architecture, which reflects only the model-ID based LCM and fails to address the elements significant for functionality-based LCM. It is even difficult to have a single framework as many elements become unnecessary in functionality-based LCM. For instance, training and inference are not defined for functionality-based LCM and hence the figure captured becomes invalid for this case.

Observation 1: As agreed in RAN2#122 [4], modified functional architecture of [Figure2, 7] would require further changes to accommodate functionality based LCM.

In addition to the above observation, in model based LCM, data collection cannot be a single entity as shown in the agreed figure. Moreover, other information such as fine tuning of the arrows, which entity will control the data collection are not clear. We further continue addressing these limitations in the appropriate email discussion.

## 2.1 Frameworks for functionality-based LCM

Model-ID based LCM requires the management of a large number of physical model(s) which are expected to be deployed to enable the AIML-enabled features. The management of these models becomes particularly cumbersome in the case of the two-sided models. Seeing the complications associated with the model-ID based LCM, we first focus on studying the functionality-based LCM, which is simpler to enable.

Observation 2: Both the one-sided and two-sided model ID based LCM frameworks are significantly more complex than the functionality-based LCM.

Figure 1 provides an illustration of one-sided functionality-based LCM framework for the one-sided use cases. Furthermore, Figure 2 provides an illustration of the LCM framework for two-sided use cases, where the inference and data collection are split between the UE-side and gNB-side. The main blocks for both the frameworks are the data collection, inference, and functionality management blocks.

Figure 1: A simplified framework for one-sided functionality-based LCM.

Figure 2: A simplified framework for two-sided functionality-based LCM.

For the Release 18 use cases concerning the UE-sided models and the UE-part of the two-sided models, the functionality on UE-side includes two types of functions:

- (ML) Inference: function required to execute the functionality at the UE-side by running underlying ML algorithms in inference operating mode.

- Data collection: functions required to enable the data collection and pre-processing of inference data and collection and post-processing of monitoring data.

For the studied Release 18 use cases, the (ML) functionality management is located at the gNB/LMF-side and implements the ML related management functions required by the gNB/LMF to identify, configure, activate/deactivate/switch and monitor the functionalities executed at the UE side for a specific ML-enabled feature or feature group [6]. The management functions on the gNB/LMF-side include at least monitoring data collection functions i.e., functions required to enable the collection and post-processing of monitoring data received from the functionality at the UE-side.

For UE-side inference, the inference function will reside in the UE and interact with functionality management in the NW/gNB. The functionality management may comprise of the following functions:

- Identification: identification of the ML Model/Functionality in the LCM management module of a given ML-enabled Feature/Functionality based on conditions indicated by the UE capability. This procedure could re-use most the existing UE capabilities reporting framework [6].

- Configuration: configuration of a registered ML-enabled Feature/Functionality using the conditions (configuration parameters) indicated by the UE capability.

- Monitoring: monitoring signaling procedure(s) for configured ML-enabled Feature/Functionality using the conditions (monitoring parameters) indicated by the UE capability.

- Activation, (de)activation, switching: signaling procedure(s) needed to allow the LCM management module operational control of configured ML-enabled Feature/Functionality.

Proposal 1: RAN2 to use as working assumption that the ML functionality management functions, including the functionality identification, configuration, monitoring and activation/deactivation/switching functions, are located at the gNB/LMF side.

For the Release 18 use cases with a UE-part of a two-sided model, in addition to the ML functionality management, ML model management might also be required to achieve model alignment, model finetuning and better generalization. The ML model management functions are expected to operate at physical model level, with similar functions as for the functionality management described above. However, given the proprietary nature of the models and vendor specific implementation, RAN2 can assume as working assumption that the ML model management is located in the same entity where the corresponding ML models are implemented; in the gNB for all the gNB-side/part models and in the UE for all  the UE-side/part models

Proposal 2: RAN2 to use as working assumption that ML model management functions can be located in the same entity where the corresponding ML models are implemented; at the gNB for all the gNB-side/part models; at the LMF for all the LMF-side models; and at the UE for all the UE-side/part models.

There is clearly need for interaction between the functionality management functions and the corresponding ML model management functions. Some of these interactions are likely to be implementation specific. Nevertheless, the RAN2 related details of this interaction can be addressed after role the ML functionality management and ML model management functions are clarified.

Observation 3: The RAN2 related details of this interaction between the functionality management functions and the corresponding ML model management functions can only be addressed after their roles are clarified.

Proposal 3: RAN2 to study the interaction between functionality-based and model-based LCM.

In most of the use cases, the data collection functions need to have both a UE-side and gNB/LMF-side implementation to enable sending the collected data from one entity to another e.g., functionality monitoring data from UE to gNB. The actual signalling channel and protocol for this can be the existing LPP/NAS/RRC or MAC signalling.

Furthermore, data collection needs to be enabled for both UE-side functionality and gNB/LMF-side functionality, to enable higher configuration flexibility of the functionality-based LCM purposes e.g., for cases where one gNB/LMF-side functionality is interacting with UE-side functionalities located in different UEs.

The ML inference function and data collection functions located in the same entity, gNB/LMF or the UE, are expected to communicate with each other using implementation specific solutions i.e., there is no need for RAN2 to study these internal protocols.

Observation 4: The functionality inference function and functionality data collection functions located in the same node NW/gNB or UE node, communicate with each other using implementation specific solutions i.e., do not require any RAN2 specifications.

Proposal 4: RAN2 to consider Figure 1 as a baseline for one-sided functionality-based LCM framework.

Proposal 5: RAN2 to consider Figure 2 as a baseline for two-sided functionality-based LCM framework.

Proposal 6: RAN2 to study the protocol specifications required to enable the communication between the functionality management located at the gNB/LMF-side and the UE-side ML functionality to ensure interoperability, at least for identification, configuration, monitoring and activation, deactivation and switching of Functionality at UE-side.

## 2.4 UE capabilities reporting

The RRC UE capability exchange procedure will indicate support for conditions that enable AIML features such as beam management, CSI compression, CSI prediction, and positioning. A specific configuration of an AIML-enabled feature is captured by a functionality, which supports a set of conditions. Obtaining a deeper understanding on these additional conditions together with the type of LCM purposes that are applicable to logical models is FFS and waits for RAN1 progress on the matter.

Proposal 7: Functionalities will be signalled through the RRC capability exchange procedure for the beam management, CSI compression, and CSI prediction use cases.

Proposal 8: Functionalities will be signalled through the LPP capability exchange procedure for the positioning use-case.

The functionality identification procedure [6] would allow us to understand the specific interaction required between the NW and UE. In the following table we describe an example of how the general applicable conditions and additional conditions could be signaled by the UE as part of the UE capabilities.

Table 2.4-1: General conditions on different configurations.

| Conditions on configurations | Description |
| --- | --- |
| Functionality configuration | - indicates the max number of configured/activated functionalities, - delays in activating/switching of functionalities, and Generalization condition of functionalities for the same ML-enabled Feature/FG, - configuration parameters for the procedure activation/deactivation/switching/fallback procedures, - including corresponding delay budgets and including optional indication of any non-ML operations/algorithms involved in the model functionality e.g., as an indication of the expected delay budget for such operations |
| Performance monitoring | indicates UE support for NW-sided functionality monitoring, and conditions on related configuration options for functionality performance monitoring, including system and intermediary KPIs to be used for functionality-based LCM monitoring purposes |
| Inference input(s) | input data type/source and preparation/pre-processing including an indication of any delay-sensitive ML-specific data processing to be performed e.g., as an indication of the expected delay budget for such operation |
| Inference output(s) | output data and post-processing (optional) including an indication of any delay-sensitive ML-specific output post-processing is performed e.g., as an indication of the expected delay budget for such operation |
| Functionality validation procedure | configuration parameters for the procedure required to validate the performance of the Functionality e.g., after ML Model update or switch (NW-initiated, UE-triggered) |
| Supporting fallback | indicates UE support for one or more fallback features (triggers/events, delays, etc.) |
| Context information, if applicable | indicates UE support for monitoring and reporting conditions for UE-side inference context e.g., radio KPIs not explicitly linked to the ML Functionality, and/or non-radio metrics (position, movement, temperature, etc.) |

Observation 5: Once RAN2 has defined the functional framework, general conditions on different configurations for enabling AIML-based functionalities should be discussed in RAN2.

# Conclusion

In this contribution we discussed architectural aspects related to the functionality and model identification. Further we emphasized on functionality-based life cycle management (LCM) for ML-enabled features and functionality support indication as well as associated information.

Based on the discussion, the following proposals are made:

Proposal 1: RAN2 to use as working assumption that the ML functionality management functions, including the functionality identification, configuration, monitoring and activation/deactivation/switching functions, are located at the gNB/LMF side.

Proposal 2: RAN2 to use as working assumption that ML model management functions can be located in the same entity where the corresponding ML models are implemented; at the gNB for all the gNB-side/part models; at the LMF for all the LMF-side models; and at the UE for all the UE-side/part models.

Proposal 3: RAN2 to study the interaction between functionality-based and model-based LCM.

Proposal 4: RAN2 to consider Figure 1 as a baseline for one-sided functionality-based LCM framework.

Proposal 5: RAN2 to consider Figure 2 as a baseline for two-sided functionality-based LCM framework.

Proposal 6: RAN2 to study the protocol specifications required to enable the communication between the functionality management located at the gNB/LMF-side and the UE-side ML functionality to ensure interoperability, at least for identification, configuration, monitoring and activation, deactivation and switching of Functionality at UE-side.

Proposal 7: Functionalities will be signalled through the RRC capability exchange procedure for the beam management, CSI compression, and CSI prediction use cases.

Proposal 8: Functionalities will be signalled through the LPP capability exchange procedure for the positioning use-case.

The following observations are made:

Observation 1: As agreed in RAN2#122 [4], modified functional architecture of [Figure2, 7] would require further changes to accommodate functionality based LCM.

Observation 2: Both the one-sided and two-sided model ID based LCM frameworks are significantly more complex than the functionality-based LCM.

Observation 3: The RAN2 related details of this interaction between the functionality management functions and the corresponding ML model management functions can only be addressed after their roles are clarified. Proposal: RAN2 to study the interaction between functionality-based and model-based LCM.

Observation 4: The functionality inference function and functionality data collection functions located in the same node NW/gNB or UE node, communicate with each other using implementation specific solutions i.e., do not require any RAN2 specifications.

Observation 5: Once RAN2 has defined the functional framework, general conditions on different configurations for enabling AIML-based functionalities should be discussed in RAN2.

# 4	References

[1] R1-2304680, “Further discussion on the general aspects of ML for Air-interface”, Nokia, Nokia Shanghai Bell.

[2]	R1-2300603. “Further discussion on the general aspects of ML for Air-interface”, 3GPP RAN WG1 #112, Athens, Greece, February 27th – March 3rd, 2023

[3]	R1-2302627, “Further discussion on the general aspects of ML for Air-interface”, TSG RAN WG1 #112bis-e, Online, April 17th – April 26th, 2023.

[4] “RAN2 Chair’s Notes”, 3GPP TSG RAN WG2 #122, May 22th – May 26th, 2023

[5] R2-2305145, “AI/ML Architecture”, Nokia, Nokia Shanghai Bell.

[6] R1-2306052, “Final summary of General Aspects of AI/ML Framework” 3GPP RAN WG1 #113, Incheon, May 22-26, 2023

[7] R2-2305327, “Discussion on architecture general”, Vivo.