3GPP TSG RAN WG2#123	R2-2308273

Toulouse, France, 21st - 25th August 2023

3GPP TSG RAN WG1 #114	R1-2308396

Toulouse, France, August 21st – August 25th, 2023

Title:	LS to RAN2 on CBSR for Rel-18 MIMO

Response to:

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Eko Onggosanusi

E-mail Address:	eko.o@samsung.com

1. Overall Description:

RAN1 would like to inform RAN2 about the following agreements and conclusions pertaining to CBSR (codebook subset restriction):

Conclusion:

On the Type-II codebook refinement for CJT mTRP, regarding CBSR for NTRP>1, there is no consensus in supporting the additional optional soft amplitude restriction. Therefore, only hard amplitude restriction (per CSI-RS resource, based on the legacy design) is supported.

Conclusion:

On the Type-II codebook refinement for CJT mTRP, regarding CBSR for NTRP=1, there is no consensus in supporting the additional optional soft amplitude restriction. Therefore, only hard amplitude restriction (per CSI-RS resource, based on the legacy design) is supported.

Conclusion: On the Type-II codebook refinement for high/medium velocity, regarding CBSR, there is no consensus in supporting the additional optional soft amplitude restriction. Therefore, only hard amplitude restriction (based on the legacy design) is supported.

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding CBSR, whether to use only 1 bit or 2 bits per beam in a beam-group restriction is up to RAN2

- Note: RAN1 has previously agreed to support only 2 hypotheses per beam in a beam-group restriction for Rel-18 Type-II CJT codebook

- Send an LS to RAN2 regarding this conclusion

2. Actions:

To: RAN4

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account and inform RAN1 if additional description is needed for CBSR.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #114bis	9th – 13th October, 2023	Xiamen, China