3GPP TSG RAN2 #123										R2-2308644

Toulouse, France, 21 – 25 August, 2023

Agenda Item:		6.5.2

Source:			Huawei, HiSilicon

Title:		Discussion on NB-IoT location in RLF report

Document for:		Discussion and Decision

1 Introduction

At RAN2#122 meeting, NB-IoT UE location issue was discussed, and it was postponed. The discussion is only for LTE part, i.e. TS 36.331.

R2-2306034 NB-IoT UE location Info in RLF report Qualcomm Incorporated discussion Rel-17 => Postponed to next meeting

In the past, NB-IoT related SON topics were mainly discussed in NB-IoT sessions, and the topics were there were rarely discussed in SON/MDT sessions. In this paper, we show background, technical analysis on issues and solutions.

2 Discussion

For RLF report for NB-IoT UEs, locationInfo-r16 has been defined. However, according to the current specification, the obtainLocation is configured by the eNB only if includeLocationInfo is configured for one or more measurements. Since neither includeLocationInfo nor obtainLocation can be configured for NB-IoT UEs, how NB-IoT UEs handle the locationInfo-r16 in RLF-Report-NB-r16 is unclear.

RLF-Report-NB-r16 ::=                 SEQUENCE {

failedPCellId-r16                     CellGlobalIdEUTRA,

reestablishmentCellId-r16              CellGlobalIdEUTRA                 OPTIONAL,

locationInfo-r16                      LocationInfo-r10                  OPTIONAL,

measResultLastServCell-r16             SEQUENCE {

nrsrpResult-r16                       NRSRP-Range-NB-r14,

nrsrqResult-r16                       NRSRQ-Range-NB-r14             OPTIONAL

},

timeSinceFailure-r16                  TimeSinceFailure-r11               OPTIONAL

}

Observation 1: For locationInfo in SON reports, the UE can only include it (if available) when obtainLocation/includeLocationInfo are indicated by NW.

Observation 2: In ASN.1, RLF-Report-NB-r16 can optionally include the locationInfo-r16 IE, but how NB-IoT Ues handle the IE is unclear.

For this ambiguity/issue, we think there can be two options to solve it:

Option 1: The UE is not allowed to report the locationInfo-r16 in the RLF-Report-NB-r16 IE since Rel-16. It would need a small change to TS 36.331, which can be found in section 5

Option 2: The UE can report the locationInfo-r16, if the UE gets an indication from the network. In [2], some details were provided, and some proposals are also listed as below. The key idea is “the network can use a (new) flag to indicate NB-IoT Ues to report location information in the RLF report”

Proposal 1: Similar to the normal UE behavior, NB-IoT UEs should not report the location information in any report (i.e., RLF report) if it is not configured to include location information. Observation 5: According to the current procedure, the NB-IoT UEs do not report the availability of the RLF report to the network, and the network may request NB-IoT UE to report the RLF report without explicit indication in the UEInformationRequest-NB by configuring rlf-ReportReq. Proposal 2: If the network is interested in obtaining the location information in the RLF report from NB-IoT UEs, it should configure a flag in the UEInformationRequest-NB to indicate NB-IOT UEs to report location information in the RLF report. Proposal 3: If the flag (to indicate NB-IOT UEs to report location information in the RLF report) is not configured, then NB-IoT UE does not report location information in the RLF report. Proposal 4: If the flag (to indicate NB-IOT UEs to report location information in the RLF report) is configured and UE has available location information, UE should report location information in the RLF report. Proposal 5: RAN2 is requested to approve CR in R2-2303717 [3].

For option 2, we think it is an enhancement, and we have the following comments:

Firstly, RAN2#122 made the following agreement. From network point of view, user consent before activating the MDT functionality is required because of privacy and legal obligations, and user consent procedures have been defined in TS 32.422 [3]. We observe that for legacy SON/MDT features, user consent should be checked before the network decides to send location related flags to UE.

R2-2306096	Discussion on location configuration for SON and MDT features	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

Agreements:

1	RAN2 to confirm understanding#1 for both LTE SON/MDT and NR SON/MDT. If agreeable, there are no specification impacts.

Understanding#1: For measurement reports and other reports (i.e. RLF report/SCG failure report), the NR can configure IE (2) obtainCommonLocation only if NR configures IE (1) includeCommonLocationInfo

However, TS 32.422 has not defined anything related to NB-IoT Ues, thus if the above new flag is needed, we think it is better to contact SA5 for their opinion.

Secondly, as mentioned in [2], P5 have some changes to TS 36.331, and we think it is an enhancement so it should be started from Rel-18 if proposals are agreeable. For Rel-16 and Rel-17, we still need a solution, e.g. Option 1.

Thirdly, in the curren TS 36.306, an optional UE capability bit has been defined for RLF Report for NB-IoT. If the new flag is introduced, a new UE capaiblity may be needed. If not, such UE behaviours will be coupled with the exsiting “Radio Link Failure Report for NB-IoT”.

6.10.2 Radio Link Failure Report for NB-IoT It is optional for UE to support the storage of RLF-Report and the reporting in UEInformationResponse message as specified in TS 36.331 [5] when connected to EPC. This feature is only applicable if the UE supports any ue-Category-NB.

For UE capability aspect, there are two options if new features are to be introduced:

- If the UE supports the existing “Radio Link Failure Report for NB-IoT”, the UE shall support the new feature

- Introduce a new UE capability bit , and if the UE supports it, the UE shall support the new feature

Observation 3: Option 2 has some important issues to be solved.

Proposal 1: The UE is not allowed to report the locationInfo-r16 in the RLF-Report-NB-r16 IE since Rel-16.

3 Conclusion

In this paper, we discuss the NB-IoT UE location in RLF report, and we have the following observations and proposal:

Observation 1: For locationInfo in SON reports, the UE can only include it (if available) when obtainLocation/includeLocationInfo are indicated by NW.

Observation 2: In ASN.1, RLF-Report-NB-r16 can optionally include the locationInfo-r16 IE, but how NB-IoT Ues handle the IE is unclear.

Observation 3: Option 2 has some important issues to be solved.

Proposal 1: The UE is not allowed to report the locationInfo-r16 in the RLF-Report-NB-r16 IE since Rel-16.

For proposal 1, we have prepared a draft CR in section 5.

4 References

[1]  Draft_R2_122_meeting_report_v2

[2]  R2-2306034, NB-IoT UE location Info in RLF report, Qualcomm Incorporated

[3]  TS 32.422

5 An example for changes to TS 36.331 v16.12.0 (for Proposal 1)

6.7.2	NB-IoT Message definitions

<Partially omitted>

–	UEInformationResponse-NB

The UEInformationResponse-NB message is used by the UE to transfer the information requested by the E-UTRAN.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UEInformationResponse-NB message

-- ASN1START

UEInformationResponse-NB-r16		::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

ueInformationResponse-r16			UEInformationResponse-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationResponse-NB-r16-IEs ::=	SEQUENCE {

rach-Report-r16							RACH-Report-NB-r16					OPTIONAL,

rlf-Report-r16							RLF-Report-NB-r16					OPTIONAL,

anr-MeasReport-r16						ANR-MeasReport-NB-r16				OPTIONAL,

lateNonCriticalExtension				OCTET STRING						OPTIONAL,

nonCriticalExtension					SEQUENCE {}							OPTIONAL

}

RACH-Report-NB-r16 ::=					SEQUENCE {

numberOfPreamblesSent-r16				INTEGER (1..64),

contentionDetected-r16					BOOLEAN,

initialNRSRP-Level-r16					INTEGER (0..2),

edt-Fallback-r16						BOOLEAN

}

RLF-Report-NB-r16 ::=					SEQUENCE {

failedPCellId-r16						CellGlobalIdEUTRA,

reestablishmentCellId-r16				CellGlobalIdEUTRA					OPTIONAL,

locationInfo-r16						LocationInfo-r10					OPTIONAL,

measResultLastServCell-r16				SEQUENCE {

nrsrpResult-r16							NRSRP-Range-NB-r14,

nrsrqResult-r16							NRSRQ-Range-NB-r14				OPTIONAL

},

timeSinceFailure-r16					TimeSinceFailure-r11				OPTIONAL

}

-- ASN1STOP

UEInformationResponse-NB field descriptions

anr-MeasReport Indicates the ANR measurement information.

contentionDetected Value TRUE indicates that contention was detected for at least one of the transmitted preambles, see TS 36.321 [6].

edt-Fallback Value TRUE indicates that EDT fallback indication was received from the lower layers, see TS 36.321 [6].

failedPCellId Indicates the PCell in which RLF is detected.

initialNRSRP-Level Indicates the NRSRP level of the NPRACH resource selected for the first preamble transmission.

locationInfo In this release of the specification, the UE does not include this field.

measResultLastServCell Refers to the last measurement results taken in the PCell, where radio link failure happened.

numberOfPreamblesSent Indicates the number of RACH preambles that were transmitted. Corresponds to parameter PREAMBLE_TRANSMISSION_COUNTER in TS 36.321 [6].

reestablishmentCellId Indicates the cell in which the re-establishment attempt was made after connection failure.

timeSinceFailure Indicates the time that elapsed since the connection failure. Value in seconds. The maximum value 172800 means 172800s or longer.