3GPP TSG-RAN WG2 Meeting #123	DRAFT_R2-2308967

Toulouse, France, August 21-25, 2023

Source: 	Session chair (Huawei)

Title:	Report from MBS breakout session

# Offline discussions

Kicked-off together with a meeting start:

[AT123][600] Organizational - MBS session

Scope:

    - Share plans and list of ongoing email discussions for MBS sessions

    - Share meeting notes and agreements for review and endorsement

## 2.4	Instructions

Rel-18 CR Handling

- 	Current Plan: Rel-18 R2 Functional Freeze is Q4 2023, i.e. Rel-18 TSes need to be created at latest at this point in time.

-	CRs for all Rel-18 WIs to be agreed at RAN2#124 (November 2023). Running Draft CRs need to be updated to be real CRs.

-	Previously in-principle-agreed Rel-18 CRs (e.g. for TEI18 or WIs ending before November 2023) need to be updated towards the latest TS version and submitted for final CR agreement at RAN2#124 (CR editor / proponent need to do this).

-	Such CRs do not need to be resubmitted to intermediate meetings before RAN2#124.

-	Such CR may be superseded by revision due to correction, which is in-principle agreed (see bullet below). CR editor / proponent should be ready to handle such revisions.

-	For WG meetings until functional freeze (including this) it is possible to maintain and revise Rel-18 CRs, also in-principle-agreed Rel-18 CRs, also for WIs with no TU budget (they are kept in the agenda for this purpose). It is better to fix issues now rather than wait for ASN.1 review.

-	For revision proposals for Rel-18 CRs/DraftCRs, use TPs attached to discussion documents or DraftCRs (Includes current running Rel18 CRs or update of in-principle agreed Rel-18 CRs)

-	CR editors / Rapporteurs are requested to continue even after close of their respective WIs to support maintenance related to their respective CR / WI.

Rel-18 RRC parameters and MAC CEs

-	RRC parameters, including those requested by other groups, e.g. RAN1, are covered by WI-specific RRC CRs.

-	MAC CE parameters, including those requested by other groups, e.g. RAN1, are covered by WI-specific MAC CRs

-	For information see also R2-2306732, LS on Signalling alternatives, from R2#122.

Rel-18 UE capabilites

-	Handling in RAN2 is expected similar to Rel-17.

-	For information see also R2-2306810 Further Guidelines on UE capability definitions LS out, from R2#122.

Expected Outcomes

-	EUTRA UE capabilities are covered in WI-specific CRs.

-	NR UE capabilities are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 are covered in CR for the Positioning WI.

During the work on NR UE caps:

-	In a Common Rel-18 Agenda Item (AI): RAN1 and RAN4 features are handled jointly under a common AI, with some explicit exceptions. Running UE cap MegaCRs are maintained for the parts handled in the common AI.

-	In WI-specific Rel-18 Agenda Items: RAN2 features are handled per WI. Case-by-case, for selected WIs, RAN1 and RAN4 features handled specifically per WI. The outcomes are covered in WI-specific Running CRs (draft CRs). It is expected that WI-specific UE cap running CRs will be merged with the Running Mega CRs only at/after RAN2#124.

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	WI rapporteurs input for WI planning etc,

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

## 7.11	Enhancements of NR Multicast and Broadcast Services

(NR_MBS_enh-Core; leading WG: RAN2; REL-18; WID: RP-221458)

Time budget: 0.75 TU

Tdoc Limitation: 3 tdocs

### 7.11.1	Organizational

LS in, rapporteur input, running CRs etc.

MBS UE capabilities CRs rapporteur is requested to provide an initial analysis of the required UE capabilities and identification of the related discussion points.

R2-2307015	Reply LS on multicast reception in RRC_INACTIVE (R1-2306243; contact: Apple)	RAN1	LS in	Rel-18	NR_MBS_enh-Core	To:RAN2

R2-2307112	Initial Consideration on UE Capability of eMBS	vivo	discussion	Rel-18	NR_MBS_enh-Core

R2-2307492	RRC running CR for eMBS	Huawei, HiSilicon	draftCR	Rel-18	38.331	17.5.0	B	NR_MBS_enh-Core

### 7.11.2 Multicast reception in RRC_INACTIVE

Objective: Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3], PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]. Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3].

Papers should not be submitted to 7.11.2, please use 7.11.2.1 or 7.11.2.2 instead.

#### 7.11.2.1 Control plane

Including aspects such as:

- PTM configuration structure (exact parameters etc.)

- details of multicast MCCH configuration and MCCH handling by the UE

- service continuity during mobility and state transitions (e.g. access control for connection resume due to MBS, resume due to bad reception quality etc.)

- details of notifications/group paging enhancements due to session activation/deactivation/temporary no data

- details of frequency prioritization and multicast NCL

- UE capabilities

Resume due to bad quality and new resume cause(s)

R2-2308558	Connection resumption triggering for more reliable MBS reception	InterDigital Inc.	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1:	RAN2 to discuss the following two options to trigger a connection resumption while an RRC_INACTIVE UE has an active MBS session:

a) MCH BLER falls below a configured threshold

b) serving cell RSRP/RSRQ falls below a configured threshold

Proposal 2:	Define a RRC resume cause value for indicating the UE is triggering the connection resumption due to reliability issues of MBS reception in RRC_INACTIVE.

R2-2308200	PTM configuration and session deactivation	LG Electronics Inc.	discussion	Rel-18

Proposal 1	If SIBx is not scheduled in SIB1, or if the PTM configuration is not available in serving cell, UE sets the resume cause to a new resume cause, such as multicast configuration.

Proposal 2	If the reception quality of the multicast is below the configured threshold, UE sets the resume cause to another new resume cause, such as multicast quality.

Proposal 3	If the RRC connection resume is initiated for multicast continuity (in three cases above), UE sets Access category to ‘2’ or ‘8’, depending on whether or not an emergency service is on-going.

Proposal 4	For an UE receiving multicast in RRC_INACTIVE, the UE resumes the RRC connection when the measured RSRP or RSRQ of the serving cell becomes lower than the threshold configured by network.

Proposal 5	The threshold can be configured in PTM configuration, i.e., via RRCRelease or multicast MCCH message.

Frequency prioritization

R2-2307594	CP aspects for Multicast reception in RRC_INACTIVE	Samsung R&D Institute India	discussion

Proposal 17: Dedicated frequency prioritization information for multicast reception in the RRC_INACTIVE is provided to the UE through the RRCRelease with SuspendConfig.

Proposal 18: UE can be provided with the de-prioritization request for multicast in RRCRelease with suspendConfig to control RAN overload on specific frequency.

R2-2308109	Control plane aspects on multicast reception in RRC INACTIVE 	Kyocera 	discussion	Rel-18	R2-2306147

Proposal 8	RAN2 should agree that the frequency information may be broadcasted by the gNB.

Session deactivation

R2-2308013	Control plane aspects of Multicast reception in RRC_INACTIVE	Lenovo	discussion	Rel-18

Proposal 1	For a deactivated session, the PTM configuration is optionally provided in RRCRelease message. A new deactivated state indication is provided in the PTM configuration to notify UE not to monitor the corresponding G-RNTI.

Proposal 2	For notification of deactivation of a MC session, either MRB release or MRB setup/modify can be used in MCCH. If MRB setup/modify is used, a new deactivated state indication is provided in the PTM configuration to notify UE stop monitoring the corresponding G-RNTI.

R2-2307263	Discussion on Control Plane for Multicast Reception in RRC_INACTIVE	CATT, CBN	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1: Multicast MCCH should be present for a cell providing multicast reception in RRC_INACTIVE.

Proposal 7a: UE determines the MBS session as active upon receiving the PTM configuration in RRCRelease if it has been configured to receive multicast in RRC_CONNECTED state.

Proposal 7b: If the MBS session is not activated when receives PTM configuration in RRCRelease, UE does not perform MBS multicast reception in RRC_INACTIVE immediately.

Proposal 7c: If the MBS session is not activated when UE receives PTM configuration in RRCRelease, it is assumed the PTM configuration does not change until session activation.

Proposal 7d: If the MBS session is not activated when UE receives PTM configuration in RRCRelease, UE directly uses the stored PTM configuration for multicast reception and starts to monitor MCCH DCI for change notification upon receiving group paging that indicates to allow the inactive multicast reception.

Proposal 7e: If UE did not receive PTM configuration in RRCRelease due to session deactivation, UE reads MCCH to acquire the PTM configuration upon receiving group paging that indicates to allow the inactive multicast reception.

Proposal 8: The session deactivation is indicated in the content of multicast MCCH.

MBS + SDT

R2-2308343	Multicast reception in RRC_INACTIVE	ZTE, Sanechips	discussion	Rel-18	NR_MBS_enh-Core

Proposal 8	It is possible a UE are both configured with SDT and MBS, no specific optimization shall be made in Rel-18.

Proposal 9	RRC_INACTIVE UE monitors paging, regardless of whether SDT procedure is ongoing.

MBS + eDRX/MICO

R2-2308552	MBS multicast and UE power saving	Ericsson	discussion	Rel-18	NR_MBS_enh-Core	R2-2305917

Proposal 1: Clarify in 38.304 that when the UE is configured with eDRX or MICO mode, the UE monitors paging at the scheduled activation time as if eDRX or MICO mode was not configured.

Proposal 2: The UE monitors paging at the scheduled activation time for the duration of the configured CN PTW (when configured with eDRX) or Active Time (when configured with MICO mode).

R2-2307084	Control plane for multicast reception in RRC_INACTIVE state	TD Tech, Chengdu TD Tech	discussion	Rel-18

R2-2307085	MCCH change notification for multicast sessions in RRC_INACTIVE state	TD Tech, Chengdu TD Tech	discussion	Rel-18

R2-2307109	Discussion on eMBS from the CP Perspective	vivo	discussion	Rel-18	NR_MBS_enh-Core

R2-2307135	Control plane discussion for multicast reception in RRC INACTIVE	MediaTek inc.	discussion	Rel-18	NR_MBS_enh-Core

R2-2307155	Discussion on security issue with multicast MCCH	CANON Research Centre France	discussion	Rel-18	NR_MBS_enh-Core

R2-2307412	Consideration on the control plane issue for multicast reception in RRC_INACTIVE	Beijing Xiaomi Software Tech	discussion

R2-2307459	Discussion on control plane for Multicast reception in RRC_INACTIVE 	NEC Corporation	discussion	NR_MBS_enh-Core

R2-2307493	CP issues for multicast reception for RRC INACTIVE UE	Huawei, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core

R2-2307638	Service continuity, RRC state transitions and notifications	Qualcomm Incorporated	discussion	Rel-18	NR_MBS_enh-Core

R2-2307768	Control plane details for multicast reception in RRC_INACTIVE state	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MBS_enh-Core

R2-2307779	RRC Resume for Multicast in RRC_INACTIVE	SHARP Corporation	discussion	R2-2306049

R2-2307843	Control plane aspects for multicast reception in RRC INACTIVE	Apple	discussion	Rel-18	DUMMY

R2-2307895	Discussion on SDT and MBS multicast reception in RRC_INACTIVE	ITRI	discussion	NR_MBS_enh-Core

R2-2308133	Discussion on Service Continuity and RRC state transitions	Spreadtrum Communications	discussion	Rel-18

R2-2308201	Multicast servic continuity	LG Electronics Inc.	discussion	Rel-18

R2-2308304	Discussion on multicast reception in RRC_INACTIVE CP issues	CMCC	discussion	Rel-18	NR_MBS_enh-Core

R2-2308568	Ensuring desired level of reliability for an MBS session in RRC_INACTIVE	Interdigital Inc.	discussion	Rel-18	NR_MBS_enh-Core	Withdrawn

R2-2308649	MCCH Monitoring and Configuration of UE with Multicast reception in RRC_INACTIVE	SHARP Corporation	discussion

R2-2308652	Support of SDT and Multicast in RRC_INACTIVE configured together	SHARP Corporation	discussion

R2-2308850	PTM configuration for eMBS	Shanghai Jiao Tong University, NERCDTV	discussion

R2-2308889	Multicast reception in RRC_INACTIVE	Ericsson	discussion	Rel-18	NR_MBS_enh-Core

#### 7.11.2.2 User plane

Including aspects such as:

- CFR configuration

- MAC operation (e.g. DRX, scheduling)

- L2 operation during state transitions and mobility (e.g. MRBs establishment/release etc.)

- further discussion on PHY layer impacts (considering the LS in from RAN1 in R1-2306243) etc.

Connected mode MRBs handling

R2-2307110	Discussion on eMBS from the UP Perspective	vivo	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1	NW indicates explicitly which on-going multicast service, e.g. in term of TMGI, can be received in INACTIVE in suspendConfig of RRC Release.

Proposal 2	If P1 is agreeable, UE behaviour is not to suspend corresponding multicast MRBs and to keep current CONNECTED MRB L2 configurations except for some MAC configuration, e.g. HARQ feedback, to continue this multicast service reception.

R2-2307264	Discussion on User Plane for Multicast reception in RRC_INACTIVE	CATT, CBN	discussion	Rel-18	NR_MBS_enh-Core

Proposal 4: When entering RRC_INACTIVE state, UE suspends the existing MRBs used in CONNECTED state.

PDCP COUNT handling

R2-2308853	Discussion and draft TP on the PDCP operation for the support of multicast reception in RRC_INACTIVE state	Beijing Xiaomi Software Tech	discussion	Rel-18

Observation 2: PDCP count re-initialisation may cause data loss as the PDCP SDUs in the reception buffer will be discarded.

Proposal 3: Upon cell reselection, UE re-initializes the PDCP count of the MRB configured for the multicast reception in RRC_INACTIVE state via the PDCP entity re-establishment.

Observation 3: It introduces extra signalling overhead and complexity to re-initialize the PDCP count via the RRC configuration.

Proposal 4: Upon cell reselection, UE sets the initial PDCP count of the MRB for the multicast reception in RRC_INACTIVE state based on the same mechanism as R17 MBS broadcast.

R2-2307494	UP issues for multicast reception for RRC INACTIVE UE	Huawei, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core

Proposal 6: One cell can indicate "synchronized", if by implementation, it follows a common QoS flow to MRB mapping rule and at the same time PDCP COUNT is set according to the MBS QoS Flow SN.

Proposal 7: UE can regard two cells as synchronized if both indicate "synchronized". Otherwise, they are not synchronized.

SPS support

R2-2308594	Discussion on UP issues for Multicast in RRC Inactive	LG Electronics Inc.	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1. Support MBS SPS without HARQ feedback in RRC_INACTIVE state.

Proposal 2. Support L1 SPS activation without HARQ feedback for MBS SPS in RRC_INACTIVE.

Proposal 3. Support L1 SPS deactivation without HARQ feedback for MBS SPS in RRC_INACTIVE.

Proposal 4. Introduce group paging for SPS release in RRC_INACTIVE.

R2-2308305	Discussion on multicast reception in RRC_INACTIVE UP issues	CMCC	discussion	Rel-18	NR_MBS_enh-Core

Proposal 3: SPS is not supported for multicast reception in RRC_INACTIVE.

DRX handling

R2-2307984	User plane aspects of multicast reception in RRC_INACTIVE state	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MBS_enh-Core

Proposal 3: RAN2 enables RRC_INACTIVE UE receiving multicast to also receive possible PTM retransmissions initiated by UEs receiving multicast in RRC_CONNECTED.

Proposal 4: DRX for MBS multicast in RRC_INACTIVE should be based on DRX for MBS multicast in RRC_CONNECTED.

Proposal 5: Allow configuration of drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM for INACTIVE UEs (38.331).

Proposal 8: UE receiving MBS multicast in RRC_INACTIVE should start drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM as specified in 38.321 when reception of the transport block has not been successful, but need not start drx-HARQ-RTT-TimerDL or drx-RetransmissionTimerDL.

R2-2307146	User plane aspects for eMBS	NEC	discussion	NR_MBS_enh-Core

Proposal-4: the UE does not start drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM for multicast reception during RRC_INACTIVE.

CFR restrictions

R2-2307844	User plane aspects for multicast reception in RRC_INACTIVE	Apple	discussion	Rel-18	DUMMY

Proposal 1: Confirm that there is no restriction on the multicast CFR in RRC_INACTIVE and the broadcast CFR configuration in the same cell.

Proposal 2: Confirm that the multicast CFR configuration in RRC_CONNECTED and in RRC_INACTIVE state can be same or different, and it’s up to network implementation.

R2-2307639	Further views on multicast CFR configuration aspects	Qualcomm Incorporated	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1.	When Multicast CFR for RRC_INACTIVE and broadcast CFR are configured differently, one of the two CFRs is fully contained (or overlapping) with the other CFR.

Proposal 2.	If multicast CFR for RRC_INACTIVE is not configured, the default is same as CORESET#0.

R2-2307086	User plane for multicast reception in RRC_INCTIVE stat	TD Tech, Chengdu TD Tech	discussion	Rel-18	Withdrawn

R2-2307136	L2 operation during state transitions and mobility for R18 multicast	MediaTek inc.	discussion	Rel-18	NR_MBS_enh-Core

R2-2307148	User plane for multicast reception in RRC_INCTIVE state	TD Tech, Chengdu TD Tech	discussion	Rel-18

R2-2307758	UP Aspects for Multicast Reception in RRC_INACTIVE	Samsung	discussion	Rel-18	NR_MBS_enh-Core

R2-2308014	User plane aspects of Multicast reception in RRC_INACTIVE	Lenovo	discussion	Rel-18

R2-2308344	CFR design for Multicast reception in RRC_INACTIVE	ZTE, Sanechips	discussion	Rel-18	NR_MBS_enh-Core	R2-2305663

R2-2308535	MBS remaining issues on DRX	Ericsson	discussion	Rel-18	NR_MBS_enh-Core

### 7.11.3 Shared processing for MBS broadcast and Unicast reception

Objective: Specify Uu signalling enhancements to allow a UE to use shared processing for MBS broadcast and unicast reception, i.e., ‎including UE capability and related assistance information reporting regarding simultaneous unicast reception in RRC_CONNECTED and MBS broadcast reception from the same or different operators [RAN2]

Including aspects such as:

- what exact parameters should be reported

- whether/how to address the case where additional information cannot be read by the UE from the non-serving cell

- whether any special handling is needed when the non-serving cell updates the configuration which is relevant for MII

R2-2307640	Shared processing for MBS broadcast and Unicast reception	Qualcomm Incorporated	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1.	In case additional information (SCS, bandwidth) is not available at the time of sending the MII to the unicast serving cell (e.g. the UE is not able to read SIB1 from the non-serving cell), the UE reports whatever is available information at that time (i.e. at least the frequency, and optionally SCS and/or BW as available). UE reports updated MII after acquiring additional information from the non-serving cell.

Proposal 2.	In case the non-serving cell updates its configurations relevant to the MII already sent by UE to the unicast serving cell, UE reports updated MII after acquiring the updated information from the non-serving cell. No additional special handling is needed.

R2-2307111	Further Discussion on Shared Processing in eMBS	vivo	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1	The SCS in the MII is set to the SCS of the initial BWP for the MBS broadcast cell.

Proposal 2	Frequency domain location and bandwidth information in the MII can be set based on configuration in CFR-ConfigMCCH-MTCH of the MBS broadcast cell’s SIB20 or the separate CFR for RedCap UE, as well as some additional information to derive the absolute value, e.g. absoluteFrequencyPointA and offsetToCarrier.

R2-2307265	Remaining Issues on Shared Processing	CATT, CBN	discussion	Rel-18	NR_MBS_enh-Core

R2-2307460	Discussion on shared process for MBS broadcast and unicast 	NEC Corporation	discussion	NR_MBS_enh-Core

R2-2307495	Discussion on shared processing for MBS broadcast and unicast reception	Huawei, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core

R2-2307596	Rel-18 MII Enhancements	Samsung R&D Institute India	discussion

R2-2307675	Discussion on the reporting signaling for shared MBS capability	Xiaomi	discussion	Rel-18	NR_MBS_enh-Core

R2-2308306	Discussion on shared processing	CMCC	discussion	Rel-18	NR_MBS_enh-Core

R2-2308345	Non-serving cell configuration update in case of shared processing	ZTE, Sanechips	discussion	Rel-18	NR_MBS_enh-Core

R2-2308744	Additional scenarios for shared processing	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MBS_enh-Core