3GPP TSG-RAN WG2 Meeting #123	R2-23xxxxx

Toulouse, France, 21-25 August 2023

Source: 	Session Chair (MediaTek)

Title:	Report from session on positioning and sidelink relay

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.4	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

This Agenda Item will be handled by email.

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 8 tdocs in total for all sub agenda items.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treatee together), the sub-AIs below this

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

### 5.3.1	General and Stage 2 corrections

Including incoming LSs if any, Including impact to 36.305 and 38.305. Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

Incoming LS

R2-2308268	LS on SSR orbit and clock correction reference for BDS in 3GPP LPP (contact: Ericsson)	RTCM SC 104	LS in	Rel-16	NR_pos-Core	To:RAN2

CRs

R2-2308476	GNSS SSR BDS orbit emphemeris reference clarification to align with RTCM	Ericsson	CR	Rel-16	37.355	16.11.0	0460	-	F	NR_pos-Core

R2-2308477	GNSS SSR BDS orbit emphemeris reference clarification to align with RTCM	Ericsson	CR	Rel-17	37.355	17.5.0	0461	-	A	NR_pos-Core

Withdrawn/Not available

R2-2307357	Correction to 38.305 on E-CID	Huawei, HiSilicon	CR	Rel-16	38.305	16.9.0	0137	-	F	NR_pos-Core	Withdrawn

R2-2307358	Correction to 38.305 on E-CID	Huawei, HiSilicon	CR	Rel-17	38.305	17.5.0	0138	-	A	NR_pos-Core	Withdrawn

### 5.3.2	RRC corrections

Including impact to 36.331, 38.331, and 38.306.

### 5.3.3	LPP corrections

R2-2308474	Correcting GNSS Ionospheric and Troposperic Delay Correction quality representation	Ericsson	CR	Rel-16	37.355	16.11.0	0458	-	F	NR_pos-Core

R2-2308475	Correcting GNSS Ionospheric and Troposperic Delay Correction quality representation	Ericsson	CR	Rel-17	37.355	17.5.0	0459	-	A	NR_pos-Core

R2-2308688	Addition of missing field description for nr-DL-PRS-ResourceID/nr-DL-PRS-ResourceSetID	Samsung	CR	Rel-16	37.355	16.11.0	0462	-	F	NR_pos-Core

R2-2308689	Addition of missing field description for nr-DL-PRS-ResourceID/nr-DL-PRS-ResourceSetID	Samsung	CR	Rel-17	37.355	17.5.0	0463	-	A	NR_pos-Core

### 5.3.4	MAC corrections

# 6	NR Rel-17

## 6.2	NR Sidelink relay

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

Tdoc Limitation: 2 tdocs

### 6.2.1	Control plane and Stage-2 corrections

A single CR with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to the CR rapporteur.  Larger open issues can be discussed with contributions (limited time).

Rapporteur summary

R2-2308953	[Pre123][401][Relay] Summary of AI 6.2.1 on Rel-17 relay control plane (Huawei)	Huawei, HiSilicon	discussion	Rel-17

Other contributions

R2-2307194	38.331_CR_Corrections to processing of paging information received via Relay UE	Samsung Electronics Co., Ltd	CR	Rel-17	38.331	17.5.0	4177	-	F	NR_SL_relay-Core

R2-2307239	Correction of RemoteUEInformationSidelink transmission condition	OPPO	CR	Rel-17	38.331	17.5.0	4180	-	F	NR_SL_relay-Core

R2-2307727	Conditions for RRC connection establishment and resume for NR sidelink discovery	Samsung, Huawei, HiSilicon	CR	Rel-17	38.331	17.5.0	4209	-	F	NR_SL_relay-Core, NR_SL_enh-Core

R2-2307755	Correction on NR Sidelink Relay RRC	Philips International B.V.	CR	Rel-17	38.331	17.5.0	4212	-	F	NR_SL_relay-Core

R2-2307852	Corrections on SRAP related configurations for SL relay	Apple	CR	Rel-17	38.331	17.5.0	4215	-	F	NR_SL_relay-Core

R2-2307853	Corrections on the reporting of L2 ID for L2 U2N relay operation	Apple	CR	Rel-17	38.331	17.5.0	4216	-	F	NR_SL_relay-Core

R2-2307955	Correction on CHO and Path Switching of Remote UE	NEC Corporation	CR	Rel-17	38.300	17.5.0	0695	-	F	NR_SL_relay-Core

R2-2308210	Miscellaneous corrections for SL relay	Huawei, HiSilicon	CR	Rel-17	38.331	17.5.0	4235	-	F	NR_SL_relay-Core

R2-2308271	Corrections to TS 38.331 on SL relay (re)selection	ZTE, CAICT, Sanechips	CR	Rel-17	38.331	17.5.0	4241	-	F	NR_SL_relay-Core

R2-2308272	Corrections to TS38.300 on SL relay (re)selection	ZTE, CAICT, Sanechips	CR	Rel-17	38.300	17.5.0	0698	-	F	NR_SL_relay-Core

R2-2308275	Correction to 38.331 on U2N relay (re)selection	vivo	CR	Rel-17	38.331	17.5.0	4240	-	F	NR_SL_relay-Core

R2-2308550	Miscellaneous Corrections for SL Relays	Ericsson España S.A.	CR	Rel-17	38.331	17.5.0	4261	-	D	NR_SL_relay-Core

R2-2308553	Miscellaneous Correction for SL Relays	Ericsson	CR	Rel-17	38.300	17.5.0	0703	-	D	NR_SL_relay-Core

R2-2308714	Corrections on U2N Relay	ASUSTeK	CR	Rel-17	38.331	17.5.0	4281	-	F	NR_SL_relay-Core

### 6.2.2	User plane corrections

A single CR with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to the CR rapporteur for the corresponding spec.  Larger open issues can be discussed with contributions (limited time).

R2-2307238	Correction of IE name sl-SRAP-ConfigRemote	OPPO	CR	Rel-17	38.351	17.5.0	0023	-	F	NR_SL_relay-Core

R2-2307756	Correction on SRAP for sidelink relay	Philips International B.V.	CR	Rel-17	38.351	17.5.0	0024	-	F	NR_SL_relay-Core

R2-2308211	Clarification on the BEARER ID in SRAP data PDU	Huawei, HiSilicon	CR	Rel-17	38.351	17.5.0	0025	-	F	NR_SL_relay-Core

## 6.4	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

Tdoc Limitation: 2 tdocs

### 6.4.1	Stage 3 corrections

A single CR per TS (RRC, LPP, MAC, UEcap 306) with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to the CR rapporteur.  Larger open issues can be discussed with contributions (limited time).

R2-2307359	Correction to Multi-RTT	Huawei, HiSilicon	CR	Rel-17	37.355	17.5.0	0455	-	F	NR_pos_enh-Core

R2-2307360	Correction to UE capability for batch reporitng	Huawei, HiSilicon	CR	Rel-17	37.355	17.5.0	0456	-	F	NR_pos_enh-Core

R2-2307504	Missing error cause code for DL PRS Measurements	Fraunhofer IIS, Ericsson	CR	Rel-17	37.355	17.5.0	0457	-	F	NR_pos_enh-Core

R2-2308478	Missing finer periodicities than 1s	Ericsson	CR	Rel-17	37.355	17.5.0	0450	1	F	NR_pos_enh-Core	R2-2306026

R2-2308479	Missing LPP support for sub 1s location information reporting periodicity	Ericsson	discussion	Rel-17

R2-2308690	Addition of missing values for dl-prs-ResourceSetPeriodicityReq-r17	Samsung	CR	Rel-17	37.355	17.5.0	0464	-	F	NR_pos_enh-Core

### 6.4.2	Stage 2 corrections

A single CR with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to the CR rapporteur.  This agenda item will be handled at lower priority.

R2-2308759	Correction of PRU overview description	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.305	17.5.0	0139	-	F	NR_pos_enh-Core

# 7	Rel-18

## 7.2	Expanded and improved NR positioning

(NR_pos_enh2; leading WG: RAN1; REL-18; WID: RP-231460)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 7.2.1	Organizational

Including incoming LSs and rapporteur inputs.

Incoming LSs with RAN2 in Cc:

R2-2307007	Reply LS on Sidelink positioning procedure (R1-2306208; contact: Xiaomi)	RAN1	LS in	Rel-18	Ranging_SL	To:SA2	Cc:RAN2

R2-2307031	Reply LS on Authorization and Provisioning for Ranging/SL positioning service (R3-233424; contact: Xiaomi)	RAN3	LS in	Rel-18	Ranging_SL, NR_pos_enh2	To:SA2	Cc:RAN1, RAN2

R2-2307052	Reply LS on the requirement on low power or high accuracy positioning (S1-231370; contact: Huawei)	SA1	LS in	Rel-18	5G_eLCS_Ph3	To:SA2	Cc:RAN1, RAN2

Incoming LS with “take into account” action

R2-2307004	LS reply on the RAT-dependent positioning integrity (R1-2306157; contact: InterDigital)	RAN1	LS in	Rel-18	NR_pos_enh2	To:RAN2	Cc:RAN3

Other incoming LSs and draft replies

R2-2307010	LS to RAN2 on SRS bandwidth aggregation for positioning (R1-2306214; contact: ZTE)	RAN1	LS in	Rel-18	NR_pos_enh2	To:RAN2

R2-2308139	[draft]Reply LS to RAN1 on SRS bandwidth aggregation for positioning	ZTE Corporation	LS out	Rel-18	NR_pos_enh2	To:RAN1

[AT123][402][POS] LS to RAN1 on SRS bandwidth aggregation (ZTE)

Scope: Draft a reply to R2-2307010, taking online discussion into account.

Intended outcome: Approved LS (without CB if possible)

Deadline: Wednesday 2023-08-23 2000 UTC

R2-2307032	Reply LS on SRS Configuration Request (R2-2302278; contact: Huawei)	RAN3	LS in	Rel-18	NR_pos_enh2	To:RAN2	Cc:RAN1

R2-2307042	LS on reporting granularity for timing related positioning measurements (R4-2310166; contact: Huawei)	RAN4	LS in	Rel-18	NR_pos_enh2	To:RAN2, RAN3	Cc:RAN1

R2-2307126	Draft reply LS on timing measurement reporting granularity	Huawei, HiSilicon	LS out	Rel-18	NR_pos_enh2	To:RAN4	Cc:RAN1, RAN3

R2-2307127	Discussion on measurement reporting granularity	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

[AT123][403][POS] LS to RAN4 on timing measurement reporting granularity (Huawei)

Scope: Draft a reply to R2-2307042, taking online discussion into account.

Intended outcome: Approved LS (without CB if possible)

Deadline: Wednesday 2023-08-23 2000 UTC

R2-2307054	Reply LS to LS to SA2 on Sidelink positioning procedure (S2-2305735; contact: Xiaomi)	SA2	LS in	Rel-18	Ranging_SL	To:RAN2, RAN1	Cc:SA3

R2-2307056	LS on assistance information provided to UE (S2-2307553; contact: Xiaomi)	SA2	LS in	Rel-18	Ranging_SL	To:RAN2

[AT123][404][POS] LS(s) to SA2 on sidelink positioning (Xiaomi)

Scope: Draft a reply (or separate replies) to R2-2307054 and R2-2307056, taking online discussion into account.

Intended outcome: Approvable LS(s)

Deadline: Wednesday 2023-08-23 2000 UTC

LS-related documents from non-contact companies

R2-2308053	Discussion on the reply LSs to SA2 on SL Positioning	OPPO	discussion	Rel-18	NR_pos_enh2

Draft TS

R2-2307663	TS 38.355 v0.0.4	Intel Corporation	draft TS	Rel-18	38.355	0.0.4	NR_pos_enh2

[AT123][409][POS] TS 38.355 (Intel)

Scope: Collect comments on the draft TS in R2-2307663 and produce an endorsable version.

Intended outcome: Endorsable draft TS

Deadline: Thursday 2023-08-24 2000 UTC

Running CRs

R2-2307124	Running MAC CR for LPHAP	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2307125	Running MAC CR for Sidelink Positioning	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

[AT123][405][POS] Rel-18 positioning MAC CRs (Huawei)

Scope: Collect comments on the CRs in R2-2307124 and R2-2307125 and produce endorsable versions.

Intended outcome: Endorsable CRs

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307391	LPP running CR for RAT-dependent integrity	CATT	draftCR	Rel-18	37.355	17.5.0	NR_pos_enh2

[AT123][406][POS] Rel-18 LPP CR on RAT-dependent integrity (CATT)

Scope: Collect comments on the CR in R2-2307391 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308385	Running Stage 2 CR for 'Expanded and improved NR positioning'	Qualcomm Incorporated	draftCR	Rel-18	38.305	17.5.0	B	NR_pos_enh2

R2-2308386	Stage 2 TP for SL-MO-LR/SL-MT-LR	Qualcomm Incorporated	discussion

R2-2308387	Stage 2 TP for SLPP Transport between UE and LMF	Qualcomm Incorporated	discussion

R2-2308395	Stage 2 TP for SLPP Transport between UEs	Qualcomm Incorporated	discussion

[AT123][407][POS] Rel-18 positioning stage 2 CR and TPs (Qualcomm)

Scope: Collect comments on the CR in R2-2308385 and related TPs in R2-2308386 / R2-2308387 / R2-2308395, and produce an endorsable version of the CR.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308484	Rapporteur CR for Positioning RRC Changes	Ericsson	draftCR	Rel-18	38.331	17.5.0	B	NR_pos_enh2

[AT123][408][POS] Rel-18 positioning RRC CR (Ericsson)

Scope: Collect comments on the CR in R2-2308484 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

Rapporteur inputs on spec handling

R2-2307662	Further considerations on SLPP specification	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308259	Discussion on R18 positioning UE capabilities	Xiaomi	discussion

[AT123][410][POS] Rel-18 positioning capabilities (Xiaomi)

Scope: Discuss the proposals in R2-2308259 and conclude on handling of the Rel-18 capabilities.

Intended outcome: Report to CB session

Deadline: Wednesday 2023-08-23 2000 UTC

Other

### 7.2.2	Sidelink positioning

Positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning.  Including measurements to enable RTT-based positioning, SL-AoA, and SL-TDOA; signalling and associated UE behaviour for support of unicast, groupcast (not including many-to-one) and broadcast of SL-PRS transmissions; reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios; and signalling to NG-RAN for SL positioning and service authorization as needed.

Including report of [Post122][402][POS] SLPP session handling (Intel)

Email discussion summary

R2-2307660	Report of [ 402] SLPP session handling	Intel Corporation	discussion	Rel-18	NR_pos_enh2	Late

Agenda item summary (excluding items related to the email discussion)

R2-23xxxxx	Summary of AI 7.2.2 Sidelink positioning	CATT	discussion	Rel-18	NR_pos_enh2

The following documents will not be individually treated

R2-2307122	Discussion on higher layer aspects for Sidelink Positioning	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2307123	Discussion on lower layer aspects for Sidelink Positioning	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2307185	UE Positioning using Sidelink in OoC	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2307187	Preconfigured Assistance Data for UE Positioning in Hybrid Uu and PC5 scenarios	Fraunhofer IIS, Fraunhofer HHI, Ericsson	discussion

R2-2307232	Discussion of SLPP / LPP signalling procedures 	Nokia Netherlands	discussion	Rel-18

R2-2307241	Discussion of session-less and session-based positioning	Nokia Netherlands	discussion	Rel-18

R2-2307340	SLPP signalling in UE-only sidelink positioning/ranging procedure	MediaTek Inc.	discussion	Rel-18	NR_pos_enh2

R2-2307341	Pathological cases of network-based operation for sidelink positioning	MediaTek Inc.	discussion	Rel-18	NR_pos_enh2	Revised

R2-2307392	Discussion on sidelink positioning	CATT	discussion	Rel-18	NR_pos_enh2

R2-2307426	Discussion on sidelink positioning	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2307507	Discussion on SL positioning	Xiaomi	discussion	Rel-18

R2-2307661	Further considerations on sidelink positioning	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2307778	SLPP design for session aspects 	Samsung Electronics Romania	discussion

R2-2307823	SL positioning procedures	Apple	discussion	NR_pos_enh2

R2-2308052	Further discussion on sidelink positioning	OPPO	discussion	Rel-18	NR_pos_enh2

R2-2308125	Discussion on sidelink positioning	Spreadtrum Communications	discussion	Rel-18

R2-2308138	Discussion on sidelink positioning	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308152	Considerations on sidelink positioning resources	Sony	discussion	Rel-18	FS_NR_pos_enh2

R2-2308276	Discussion on SL Positioning	Lenovo	discussion	Rel-18

R2-2308284	Discussion on sidelink positioning	ROBERT BOSCH GmbH	discussion	Rel-18

R2-2308316	Considerations on Sidelink positioning	CMCC	discussion	Rel-18	NR_pos_enh2

R2-2308384	Discussion on sidelink positioning	InterDigital, Inc.	discussion	Rel-18	NR_pos_enh2

R2-2308396	Sidelink Positioning Protocol (SLPP) Signaling and Procedures	Qualcomm Incorporated	discussion

R2-2308416	Pathological cases of network-based operation for sidelink positioning	MediaTek Inc., CATT	discussion	Rel-18	NR_pos_enh2	R2-2307341

R2-2308480	Sidelink positioning	Ericsson	discussion	Rel-18

R2-2308557	Discussion of resource allocation aspects	Nokia Netherlands	discussion

R2-2308595	Discussion on higher layer aspects for sidelink positioning	LG Electronics Inc.	discussion	Rel-18

R2-2308600	Discussion on lower layer aspects for sidelink positioning	LG Electronics Inc.	discussion	Rel-18

R2-2308657	Discussion on priority value for SL-PRS	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_pos_enh2

R2-2308800	 Further Discussions on Sidelink Positioning & Ranging	CEWiT	discussion

R2-2308884	Discussion on Anchor UE discovery and selection in sidelink positioning	KT Corp.	discussion	Rel-18	NR_pos_enh2

R2-2308908	On the selection of Anchor UEs for Sidelink Positioning	Philips International B.V.	discussion	Rel-18	NR_pos_enh2

R2-2308935	On the support of UE-only SL positioning in in-coverage and partial coverage scenarios 	Philips International B.V.	discussion	Rel-18	NR_pos_enh2

### 7.2.3	RAT-dependent integrity

Error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods.

R2-2308397	Integrity of NR Positioning Technologies	Qualcomm Incorporated	discussion

R2-2307393	Discussion on RAT-Dependent integrity	CATT	discussion	Rel-18	NR_pos_enh2

R2-2307427	Remaining issues of RAT-dependent integrity	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2307664	Further considerations on RAT dependent integrity	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2307999	Discussion on RAT-dependent  integrity	Lenovo	discussion	Rel-18

R2-2308050	Consideration on RAT-dependent positioning integrity	OPPO	discussion	Rel-18	NR_pos_enh2

R2-2308136	Discussion on RAT-dependent methods positioning integrity	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308260	Discussion on RAT-dependent positioning integrity	Xiaomi	discussion

R2-2308482	On RAT-dependent positioning Integrity	Ericsson	discussion	Rel-18

R2-2308616	Discussion on RAT dependent integrity	InterDigital, Inc.	discussion	Rel-18

### 7.2.4	LPHAP

Enhancements for enabling LPHAP use case 6 (TS 22.104), including extending eDRX cycle (coordinated with RedCap WI); SRS configuration enhancements based on validity area for UEs in RRC_INACTIVE; DL-PRS measurements in RRC_IDLE and reporting in RRC_CONNECTED; and alignment between eDRX and PRS configurations.

Including report of [Post122][401][POS] SRS configuration and activation in LPHAP (CATT)

Email discussion summary

R2-2308812	Report of [Post122][401][POS] SRS configuration and activation in LPHAP (CATT)	CATT	discussion	Rel-18	NR_pos_enh2

Agenda item summary (excluding items related to the email discussion)

R2-23xxxxx	Summary for 7.2.4 LPHAP excluding SRS configuration & activation part	OPPO	discussion	Rel-18	NR_pos_enh2

The following documents will not be individually treated

R2-2307121	Discussion on LPHAP	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2307186	Enhancements for supporting LPHAP	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2307394	Discussion on SRS configuration with validity area and alignment between PRS and (e)DRX	CATT	discussion	Rel-18	NR_pos_enh2

R2-2307428	Discussion on solution of LPHAP	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2307665	Further considerations on LPHAP	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2307824	Alignment between DRX and PRS	Apple	discussion	NR_pos_enh2

R2-2308000	Discussion on low power high accuracy positioning	Lenovo	discussion	Rel-18

R2-2308051	Discussion on LPHAP enhancement	OPPO	discussion	Rel-18	NR_pos_enh2

R2-2308126	Discussion on LPHAP	Spreadtrum Communications	discussion	Rel-18

R2-2308135	Discussion on LPHAP	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308153	Considerations on Low Power High Accuracy Positioning	Sony	discussion	Rel-18	FS_NR_pos_enh2

R2-2308261	Discussion on LPHA positioning	Xiaomi	discussion

R2-2308317	Further considerations on LPHAP	CMCC	discussion	Rel-18	NR_pos_enh2

R2-2308398	Enhancements for LPHAP	Qualcomm Incorporated	discussion

R2-2308481	Discussion on Low Power High Accuracy Positioning	Ericsson	discussion	Rel-18

R2-2308618	Discussion on LPHAP	InterDigital, Inc.	discussion	Rel-18

R2-2308693	Discussion on alignment between (e)DRX and PRS	Samsung	discussion	Rel-18	NR_pos_enh2

R2-2308694	Discussion on SRS configuration in RRC_INACTIVE	Samsung	discussion	Rel-18	NR_pos_enh2

### 7.2.5	RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning

RAN1 led objectives that may require progress in RAN1 before RAN2 can take decisions.

R2-2308001	Discussion on RedCap positioning, carrier phase positioning and PRS/SRS bandwidth aggregation	Lenovo	discussion	Rel-18

R2-2307395	Discussion on carrier phase positioning, bandwidth aggregation for positioning and Redcap positioning	CATT	discussion	Rel-18	NR_pos_enh2

R2-2307429	RAN2-related issues about bandwidth aggregation	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2307455	Discussion on RAN1 led positioning topics	Huawei, HiSilicon	discussion

R2-2307666	Considerations on other RAN1 led items	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2307827	On-demand PRS with bandwidth aggregation	Apple	discussion	NR_pos_enh2

R2-2308137	Discussion on BW aggregation and RedCap positioning	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308174	Discussion on Frequency hopping for Positioning for RedCap Ues	Sony	discussion	Rel-18	FS_NR_pos_enh2

R2-2308262	Discussion on RedCap positioning, carrier phase positioning and bandwidth aggregation for positioning	Xiaomi	discussion

R2-2308399	Configuration Enhancements for DL-PRS Aggregation	Qualcomm Incorporated	discussion

R2-2308483	Discussion based upon RAN1 agreements on CPP, RedCap, Bandwidth aggregation	Ericsson	discussion	Rel-18

R2-2308619	Discussion on positioning for RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning	InterDigital, Inc.	discussion	Rel-18

R2-2308761	Assessment of impact of carrier phase positioning on higher layer protocols	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_pos_enh2-Core]

## 7.9	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

Time budget: 1.5 TU

Tdoc Limitation: 4 tdocs

### 7.9.1	Organizational

Including incoming LSs and rapporteur inputs.

Incoming LSs with “take into account” action

R2-2307057	Reply LS to SA2 on authorization for multi-path Scenario 2 (S2-2307707; contact: LGE)	SA2	LS in	Rel-18	NR_SL_relay_enh, 5G_ProSe_Ph2	To:RAN2	Cc:RAN3

R2-2307072	Reply LS on security for L2 UE-to-UE relay (S3-233323; contact: Lenovo)	SA3	LS in	Rel-18	NR_SL_relay_enh, FS_5G_ProSe_Ph2	To:RAN2

Other incoming LS

R2-2307055	Reply LS on 5G ProSe Layer-2 UE-to-UE Relay QoS enforcement (S2-2305915; contact: Qualcomm)	SA2	LS in	Rel-18	5G_ProSe_Ph2	To:RAN2

Running CRs and related documents

R2-2307235	Running CR of TS 38.351 for SL Relay enhancement	OPPO	draftCR	Rel-18	38.351	17.5.0	B	NR_SL_relay_enh-Core

[AT123][411][Relay] Rel-18 SRAP CR (OPPO)

Scope: Collect comments on the CR in R2-2307235 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307546	Introduction of NR sidelink U2U relay	vivo	draftCR	Rel-18	38.331	17.5.0	B	NR_SL_relay_enh-Core

[AT123][412][Relay] Rel-18 RRC CR on U2U relay (vivo)

Scope: Collect comments on the CR in R2-2307546 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307720	38.322 running CR for enhanced NR sidelink relay	Xiaomi	draftCR	Rel-18	38.322	17.3.0	B	NR_SL_relay_enh-Core

[AT123][413][Relay] Rel-18 relay RLC CR (Xiaomi)

Scope: Collect comments on the CR in R2-2307720 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307854	Draft Running CR 38.321	Apple	draftCR	Rel-18	38.321	17.5.0	B	NR_SL_relay_enh-Core

[AT123][414][Relay] Rel-18 relay MAC CR (Apple)

Scope: Collect comments on the CR in R2-2307854 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307920	Draft running CR 38.300	LG Electronics Inc.	draftCR	Rel-18	38.300	17.5.0	B	NR_SL_relay_enh-Core

[AT123][415][Relay] Rel-18 relay stage 2 CR (LG)

Scope: Collect comments on the CR in R2-2307920 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308203	RRC running CR for Rel-18 multi-path support	Huawei, HiSilicon	draftCR	Rel-18	38.331	17.5.0	B	NR_SL_relay_enh-Core

R2-2308204	Considerations on Multi-path RRC running CR	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

[AT123][416][Relay] Rel-18 RRC CR on multi-path relay (Huawei)

Scope: Collect comments on the CR in R2-2308203 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308559	Introduction of Rel-18 support for SL Relay Enhancements	Ericsson España S.A.	draftCR	Rel-18	38.304	17.5.0	B	NR_SL_relay_enh

[AT123][417][Relay] Rel-18 relay idle mode CR (Ericsson)

Scope: Collect comments on the CR in R2-2308559 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308687	Introduction of Rel-18 SL relay service continuity	MediaTek, Inc	CR	Rel-18	38.331	17.5.0	4277	-	B	NR_SL_relay_enh-Core

[AT123][418][Relay] Rel-18 RRC CR on relay service continuity (MediaTek)

Scope: Collect comments on the CR in R2-2308687 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

### 7.9.2	UE-to-UE relay

Single-hop Layer-2 and Layer-3 UE-to-UE relay for unicast.  Including common L2/L3 functionality comprising relay discovery and (re)selection and L2-specific functionality including adaptation layer design, control plane procedures, and QoS handling if needed.

Agenda item summary

R2-2308956	Summary of UE-to-UE relay	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

[AT123][401][Relay] Summary proposals on UE-to-UE relay (Qualcomm)

Scope: Discuss the proposals in R2-2308956 and progress towards agreements.

Intended outcome: Report to Wednesday online session in R2-2309101

Deadline: Tuesday 2023-08-22 2000 UTC

R2-2309101	(Report of [AT123][4101])	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

The following documents will not be individually treated

R2-2307233	Discussion on U2U relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307386	Discussion on remaining issue of U2U relay	NEC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307402	Discussion on the adaptation layer	Fujitsu	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307446	Discussion on U2U relay	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307547	Remaining issues on U2U discovery and relay (re)selection	vivo	discussion

R2-2307548	Discussion on the remaining issues of L2 U2U relaying	vivo	discussion

R2-2307551	Discussion on U2U Relay	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307641	U2U Relay selection reselection, SRAP design	Beijing Xiaomi Mobile Software	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307655	Discussion on using short ID in U2U relaying	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-18	NR_SL_relay_enh

R2-2307716	Discussion on U2U relay	TCL	discussion

R2-2307732	QoS and bearer configuration for L2 U2U relaying	Samsung	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307742	Common part and Layer-2 specific part on U2U Relay	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2307743	gNB involvement and capability on U2U relay	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2307750	Considerations for U2U L2 relay operations 	Kyocera	discussion	Rel-18

R2-2307855	Discussion on remaining issues on UE-to-UE Relay	Apple	discussion	Rel-18

R2-2307932	Control plane procedure for U2U relay	LG Electronics Inc.	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307944	Further discussion on L2 U2U relay	China Telecom	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307989	Discussion on L2 U2U relay	Lenovo	discussion	Rel-18

R2-2308100	Discussion on U2U Relay discovery and (re)selection	ZTE, Sanechips	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308101	Discussion on U2U relay L2-specific functionality	ZTE, Sanechips	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308104	SRAP design for U2U Sidelink Relay	Samsung	discussion

R2-2308119	Discussion on UE-to-UE Relay	Spreadtrum Communications	discussion	Rel-18

R2-2308160	UE-to-UE relay (re)selection	Sony	discussion	Rel-18	NR_SL_relay_enh

R2-2308161	Discussion on DRX for Sidelink UE to UE Relay	Sony	discussion	Rel-18	NR_SL_relay_enh

R2-2308205	Discussion on UE-to-UE relay	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308220	Remaining issues for UE-to-UE relay	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308321	Discussion on U2U relay	CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308368	Considerations on U2U relay (re)selection and Local ID assignment	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_SL_relay_enh-Core	R2-2305590

R2-2308380	Open Issues on Discovery, Relay Selection, and SRAP for UE to UE Relays	InterDigital	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308381	QoS and Configuration for L2 UE-to-UE Relays	InterDigital	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308469	Discussion on Relay (re)selection and Discovery	Ericsson España S.A.	discussion	Rel-18

R2-2308470	Control Plane Procedures for Layer 2 UE-to-UE Relays	Ericsson España S.A.	discussion	Rel-18

R2-2308611	Discussion on Adaptation Layer for L2 U2U Relay	ETRI	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308721	Discussion on E2E PC5-RRC procedures	ASUSTeK	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308722	Discussion on AS layer configuration for L2 U2U Relay	ASUSTeK	discussion	Rel-18	NR_SL_relay_enh-Core

### 7.9.3	Service continuity enhancements for L2 UE-to-network relay

Inter-gNB direct/indirect path switching; intra-gNB indirect/indirect path switching; and inter-gNB indirect/indirect path switching, to be supported by reuse of solutions for the other scenarios.

R2-2307945	Discussion on the procedure for intra-gNB indirect to indirect path switch	China Telecom	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307226	Discussion on service continuity enhancement	Xiaomi	discussion

R2-2307281	SL Relay service continuity considerations	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307549	Remaining issues on service continuity enhancement for L2 U2N relay	vivo	discussion

R2-2307552	Further Consideration on Service Continuity Enhancements	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307733	Discussion on measurement quantity configuration	Samsung	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307744	Proposal on additional enhancements for service continuity	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2307856	Discussion on path switching to IDLE/INACTIVE relay	Apple	discussion	Rel-18

R2-2307940	Discussion on Remaining Issues of Service Continuity	NEC Corporation	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307990	Discussion on enhanced path switching	Lenovo	discussion	Rel-18

R2-2308102	Further discussion on service continuity for SL relay	ZTE, Sanechips	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308162	Service continuity enhancements for UE sidelink relay	Sony	discussion	Rel-18	NR_SL_relay_enh

R2-2308221	Remaining issues for U2N path switching	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308322	Discussion on service continuity	CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308471	Discussion on Inter-gNB Service Continuity	Ericsson España S.A.	discussion	Rel-18

R2-2308584	Discussion on Service Continuity	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

### 7.9.4	Multi-path relaying

Mechanisms to support multi-path scenarios where a UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal).  This agenda item will include a rapporteur contribution summarising open issues from RAN2#121 (invited contribution not counted against the tdoc limit).

Including report of [Post122][403][Relay] Procedures for multi-path relay (LG)

Email discussion summary

R2-2307973	Report of [AT121bis-e][419][Relay] Remaining high-priority proposals on multi-path (LG)	LG Electronics France	report	Rel-18	NR_SL_relay_enh

- Revised in R2-2308950 (title correction)

R2-2308950	Report of [Post122][403][Relay] Procedures for multi-path relay (LG)	LG Electronics France	report	Rel-18	NR_SL_relay_enh

Agenda item summary (excluding items related to the email discussion)

R2-2308949	Offline 402 on A.I 7.9.4 Multi-path relaying	Nokia	discussion	NR_SL_relay_enh-Core

The following documents will not be individually treated

R2-2307093	Discussion on multi-path SL relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307182	Discussion on Multi-path relaying	Lenovo	discussion	NR_SL_relay_enh-Core

R2-2307227	Discussion on multi-path	Xiaomi	discussion

R2-2307363	Discussion on non-split SRB	OPPO, Samsung, China Telecom, Huawei, HiSilicon, Ericsson, vivo, CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307387	Discussion on remaining issue of multi-path relay	NEC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307403	Discussions on multi-path	Fujitsu	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307550	Remaining Issues for Multi-path Scenario 1 2	vivo	discussion

R2-2307553	Discussion on Multi-path	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307656	Throughput Enhancement in U2N Relaying	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-18	NR_SL_relay_enh

R2-2307719	Discussion on multi-path scenario 1	III	discussion	NR_SL_relay_enh-Core

R2-2307745	Open issues on multi-path relay for scenario 1 and scenario 2	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2307751	Considerations for multipath relay operations for Scenario 1 	Kyocera	discussion	Rel-18

R2-2307857	Discussion on Multi-path Relay	Apple	discussion	Rel-18

R2-2307941	Discussion on UP Issues of Multi-path relay	NEC Corporation	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307946	Discussion on remaining issues of multi-path relaying in Scenario 1	China Telecom	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307947	Discussion on remaining issues of multi-path relaying in Scenario 2	China Telecom	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307991	Procedure for second path addition	Lenovo	discussion	Rel-18

R2-2308103	Further discussion on the support of multi-path relaying	ZTE, Sanechips	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308120	Discussion on multi-path relaying	Spreadtrum Communications	discussion	Rel-18

R2-2308163	Multi-path relaying discussion	Sony	discussion	Rel-18	NR_SL_relay_enh

R2-2308206	Remaining issues on multi-path operation	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308222	Remaining issues for multi-path relay	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308224	Discussion on remaining issues on multiple path for sidelink relay	Samsung	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308323	Discussion on multi-path scenario 1	CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308324	Considerations on multi-path scenario 2	CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308382	User Plane Aspects for Multipath	InterDigital	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308383	Control Plane Aspects for Multipath	InterDigital	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308472	Discussion on Multipath Relays	Ericsson España S.A.	discussion	Rel-18

R2-2308723	BSR reporting for Multi-path Scenario 2	ASUSTeK	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308724	Discussion on duplicate PDCP PDU discarding for Multi-path transmission Scenario 1	ASUSTeK	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308749	On Remaining issues on multipath SL relay	Nokia, Nokia Shanghai Bell	discussion	NR_SL_relay_enh-Core

### 7.9.5	DRX

Study the gains and, if needed, specify signalling between gNB and relay UE in sidelink mode 2 to assist the determination of the sidelink DRX configuration used for remote UE.  This agenda item will be handled at lower priority.

R2-2307228	Discussion on SL DRX in U2N relay	Xiaomi	discussion

R2-2307234	Discussion on DRX for L2 U2N relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307554	Discussion on DRX for L2 U2N Relay	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307858	Discussion on SL DRX for L2 UE-to-NW relay	Apple	discussion	Rel-18	R2-2305065

R2-2308207	Discussion on sidelink DRX for L2 U2N relay	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308369	Considerations on DRX and paging for sidelink relay	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_SL_relay_enh-Core	R2-2305592

## 7.24	NR TEI18

Specific items may be allocated to a breakout session for treatment.

Time budget: 1 TU

### 7.24.1	TEI proposals by Other Groups

Items initiated by other groups that is/has been communicated by LS, where the other group indicate this is TEI18. (Specific other-group-WIs should use the R18 Other Agenda Item below).

R2-2307009	LS on 1-symbol PRS (R1-2306212; contact: ZTE)	RAN1	LS in	Rel-18	TEI18	To:RAN2	Cc:RAN3, RAN4

R2-2308140	Introduction of 1-symbol PRS in 38.331[1symbol_PRS]	ZTE Corporation	CR	Rel-18	38.331	17.5.0	4014	3	B	TEI18	R2-2306793

R2-2308141	Introduction of 1-symbol PRS in 37.355[1symbol_PRS]	ZTE Corporation	CR	Rel-18	37.355	17.5.0	0437	3	B	TEI18	R2-2306794

R2-2308142	Introduction of UE capability of 1-symbol PRS in 37.355[1symbol_PRS]	ZTE Corporation	CR	Rel-18	37.355	17.5.0	0453	2	B	TEI18	R2-2306795

R2-2308143	Introduction of UE capability of 1-symbol PRS in 38.331[1symbol_PRS]	ZTE Corporation	CR	Rel-18	38.331	17.5.0	4128	2	B	TEI18	R2-2306796

R2-2308144	Introduction of UE capability of 1-symbol PRS in 38.306[1symbol_PRS]	ZTE Corporation	CR	Rel-18	38.306	17.5.0	0923	2	B	TEI18	R2-2306797

### 7.24.2	TEI proposals by RAN2

Items initiated in RAN2.

Tdoc limitation: 1 tdoc, limitation only applicable for non-previously-agreed-to-be-considered TEI proposals.

Relay: paging cause forwarding [previously seen]

R2-2307176	Paging Cause forwarding	Samsung Electronics Co., Ltd	discussion	Rel-18	TEI18

R2-2307694	Discussion on MUSIM paging cause forwarding	vivo	discussion	Rel-18

Relay: emergency cause value [previously seen]

R2-2307237	Discussion on emergency cause value for SL Relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core, TEI18

Relay: voice/video support [previously seen]

R2-2308932	Considerations on voice and video support for Relays	Philips International B.V., FirstNet, InterDigital, KPN, TNO, 	discussion	Rel-18	R2-2306516

Positioning: multiple QoS [previously seen]

R2-2307342	Multiple QoS for positioning	MediaTek Inc.	discussion	Rel-18	TEI18

R2-2308830	Introduction of ‘multiple QoS’ class in positioning 	Samsung Electronics Romania	discussion

Positioning: SSR PCV residuals [previously seen]

R2-2307757	Support for SSR Satellite PCV Residuals	Swift Navigation	discussion

Positioning: NavIC enhancements [new]

R2-2308193	NavIC L5 A-GNSS support updates to RRC protocol specification	Reliance Jio	CR	Rel-18	38.331	17.5.0	4234	-	F	TEI18

Positioning/relay: positioning for remote UEs [previously seen]

R2-2308485	Relay based Positioning posSIB forwarding	Ericsson, Deutsche Telekom, AT&T	discussion	Rel-18

R2-2308486	Information on posSIBs relaying to remote UE [PosL2RemoteUE]	Ericsson, Deutsche Telekom, AT&T	CR	Rel-18	38.331	17.5.0	4254	-	B	TEI18

R2-2308487	Information on posSIBs relaying to remote UE	Ericsson, Deutsche Telekom, AT&T	draftCR	Rel-18	38.455	17.5.0	B	TEI18

R2-2308695	Discussion on positioning support for L2 U2N remote UE	Samsung	discussion	Rel-18	TEI18

Positioning: BT AoA/AoD [new]

R2-2308489	Adding support for Bluetooth AoA/AoD	Ericsson, AT&T, Polaris Wireless, u-blox	discussion	Rel-18

- Revised in R2-2308955

R2-2308955	Adding support for Bluetooth AoA/AoD	Ericsson, AT&T, Polaris Wireless, u-blox, T-Mobile	discussion	Rel-18

## 7.25	R18 Other

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-18 specific WIs/SIs that has no RAN WI.

Time budget: 2 TU

Tdoc Limitation: -

### 7.25.3	Other

RAN3, SA2, SA3, CT1 led items and others, e.g. eNPN, Slicing.

R2-2308400	On Positioning Reference Unit support in LPP	Qualcomm Incorporated	discussion

R2-2308488	On the Positioning Reference Units aspects	Ericsson, vivo	discussion	Rel-18

[AT123][419][POS] Location information type for PRUs (Ericsson)

Scope: Discuss the proposals in R2-2308400 and R2-2308488, gather company views, and work towards a conclusion.

Intended outcome: Report to CB session

Deadline: Wednesday 2023-08-23 2000 UTC