3GPP TSG-RAN WG2 Meeting #123	R2-23xxxxx

Toulouse, France, 21-25 August 2023

Source: 	Session Chair (MediaTek)

Title:	Report from session on positioning and sidelink relay

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.4	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

This Agenda Item will be handled by email.

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 8 tdocs in total for all sub agenda items.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treatee together), the sub-AIs below this

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

### 5.3.1	General and Stage 2 corrections

Including incoming LSs if any, Including impact to 36.305 and 38.305. Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

Incoming LS

R2-2308268	LS on SSR orbit and clock correction reference for BDS in 3GPP LPP (contact: Ericsson)	RTCM SC 104	LS in	Rel-16	NR_pos-Core	To:RAN2

- Noted

CRs

R2-2308476	GNSS SSR BDS orbit emphemeris reference clarification to align with RTCM	Ericsson	CR	Rel-16	37.355	16.11.0	0460	-	F	NR_pos-Core

Discussion:

CATT indicate that B3I is not enabled in Rel-16, so there is a problem with this version of the CR: It should only refer to B1I.

Qualcomm note that there is an editorial issue with the quote marks (should be straight, not “curly”).  Can be fixed in update.

- Agreed with these changes as R2-2309102

R2-2308477	GNSS SSR BDS orbit emphemeris reference clarification to align with RTCM	Ericsson	CR	Rel-17	37.355	17.5.0	0461	-	A	NR_pos-Core

- Agreed as R2-2309103 (changed to cat F)

Withdrawn/Not available

R2-2307357	Correction to 38.305 on E-CID	Huawei, HiSilicon	CR	Rel-16	38.305	16.9.0	0137	-	F	NR_pos-Core	Withdrawn

R2-2307358	Correction to 38.305 on E-CID	Huawei, HiSilicon	CR	Rel-17	38.305	17.5.0	0138	-	A	NR_pos-Core	Withdrawn

### 5.3.2	RRC corrections

Including impact to 36.331, 38.331, and 38.306.

### 5.3.3	LPP corrections

R2-2308474	Correcting GNSS Ionospheric and Troposperic Delay Correction quality representation	Ericsson	CR	Rel-16	37.355	16.11.0	0458	-	F	NR_pos-Core

- Not pursued

R2-2308475	Correcting GNSS Ionospheric and Troposperic Delay Correction quality representation	Ericsson	CR	Rel-17	37.355	17.5.0	0459	-	A	NR_pos-Core

- Not pursued

Discussion:

CATT understand that the definition of the correction was copied from the RTCM message, and this would introduce a conflict.  Ericsson indicate it was copied from CLAS; Qualcomm understand that this is the same as the RTCM CSSR.

Qualcomm do not see this as a correction; they agree the grids can be large, but this is also the case in CLAS.  Nokia have the same view and think it is an enhancement rather than a correction.

Swift are generally supportive but have a few questions for clarification.  They understand the interpretation is that you must receive the precorrection as well as the residual, and they think the model being used for extrapolation also needs to be included.

Ericsson think it is a practical issue from Rel-16.

Qualcomm think nothing is broken in the existing specs.

Apple agree that this is an enhancement, not a correction.

Swift think some clarification is needed to the existing interpretation of the tiles.  Qualcomm think this would be a separate issue and a separate CR.

Ericsson indicate that a large grid will create quality differences across the grid if we do not have corrections per grid point.  Qualcomm think we do not provide assistance data for grids of a size that would make it critical (e.g. notification area, not the whole of Europe).  Ericsson think this depends on implementation, and the specification allows very large grid areas.

Nokia think it is clearly an enhancement and could be considered as a TEI18 proposal.

R2-2308688	Addition of missing field description for nr-DL-PRS-ResourceID/nr-DL-PRS-ResourceSetID	Samsung	CR	Rel-16	37.355	16.11.0	0462	-	F	NR_pos-Core

- Not pursued

R2-2308689	Addition of missing field description for nr-DL-PRS-ResourceID/nr-DL-PRS-ResourceSetID	Samsung	CR	Rel-17	37.355	17.5.0	0463	-	A	NR_pos-Core

- Not pursued

Discussion:

Nokia do not see it as an essential correction; they think the description of the corresponding IE is sufficient.  CATT also do not think it is an essential correction, and for DL-TDOA the wording is not correct.

Ericsson have the same view as Nokia.

vivo agree with Ericsson and Nokia and think the spec is currently clear enough.  Samsung are OK to follow the majority view, but they indicate that in the Provide Location Information message, the resource is used for each measurement element, and the intention was to capture this in the field description.

### 5.3.4	MAC corrections

# 6	NR Rel-17

## 6.2	NR Sidelink relay

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

Tdoc Limitation: 2 tdocs

### 6.2.1	Control plane and Stage-2 corrections

A single CR with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to the CR rapporteur.  Larger open issues can be discussed with contributions (limited time).

Rapporteur summary

R2-2308953	[Pre123][401][Relay] Summary of AI 6.2.1 on Rel-17 relay control plane (Huawei)	Huawei, HiSilicon	discussion	Rel-17

38.300 corrections

Proposal 1: The stage 2 CRs in R2-2308272 and R2-2308553 are not essential, thus not pursued.

- R2-2308272 and R2-2308553 are not pursued

38.331 corrections

Proposal 2: The following RRC changes are agreeable. Can further discuss whether to merge into one Miscellaneous CR or have separate CRs.

1)	the change in R2-2307194: Updated the text to clarify that UuMessageTransferSidelink message includes only one PagingRecord.

2)	the change in R2-2307239: Adding trigger condition “or if the information carried by the sl-PagingInfo-RemoteUE has changed since the last transmission of the RemoteUEInformationSidelink message”.

3)	the changes in R2-2307727: Change #1: In 5.3.3.1a, added the conditions to check SIB12 whether the network supports L2 U2N relay discovery or L3 U2N relay discovery or non-relay discovery. Change #2: In 5.3.13.a1, added the conditions to check SIB12 whether the network supports L2 U2N relay discovery or L3 U2N relay discovery or non-relay discovery.

4)	the change in R2-2307755: Correct the “ue-TimersAndConstantsRemoteUE” in the specification text to “sl-TimersAndConstantsRemoteUE”.

5)	the changes in or related to R2-2307852: In 5.3.8.3 and 5.3.11, clarify that relay UE can reestablish SL-RLC0/SL-RLC1/SRAP entity after release or not release SL-RLC0/SL-RLC1 /SRAP entity upon going to idle/inactive state. In 9.2.5, added “RRCResumeRequest1” and changed “Identity” to “identity”.

6)	the change in R2-2307853: In 5.8.3.3, moved the inclusion of “sl-SourceIdentityRemoteUE“ to one level up (from level-4 to level-3), so that it can be decided independently from the level-3 Rx Discovery conditions.

7)	the changes in R2-2308210: Change #1: In clause 5.8.15.2, add “PCell” before “camping cell”, to cover connected state, i.e. when RSRP of Pcell is evaluated, Remote UE should take Relay UE’s serving cell as Pcell. Change #2: In clause 6.3.2, add discovery case into the IE description of ReportConfigNR-SL, and related field description. Change #3: In clause 6.3.5, add discovery case into the IE description of SL-BWP-Config, and SL-ConfigDedicatedNR. Change #4: In 5.3.3.7, 5.3.3.8, 5.3.5.5.2, 5.3.13.5, 5.3.15.2, “Notification message” is replaced with “NotificationMessageSidelink” in the sentence “sends Notification message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.”. Change #5: In 5.3.7.7, fix typo “receiption”.

8)	the change related to R2-2308271: For a UE capable of L2 U2N Remote UE, it can perform relay selection when cell selection is triggered, which can be added as a unified condition of relay selection in 5.8.15.3.

9)	the changes in R2-2308275: In subclause 5.5.3.2, add the “for U2N Relay (re)selection evaluation” entry to apply Layer 3 filtering, and remove “L2” to cover both L2 and L3 U2N Relay UEs (if applicable).

10)	the change in R2-2308550: in Section 6.3.5, remove “, e.g. SRAP-Config” from the IE description of SL-L2RelayUE-Config.

11)	the change in R2-2308714: In clause 5.2.2.2.1, “clause 5.8.9.8.3” in the corresponding statement is modified as “clause 5.8.9.9.3”.

Discussion:

Xiaomi understand that point 2 is not in line with past decisions.  OPPO indicate that the current text does not explain the conditions clearly.  Apple agree with Xiaomi that the current sentence indicates that any information change in RemoteUEInformationSidelink will trigger this message.

Xiaomi think if we take this change we should reconsider R2-2207179.

Apple wonder why we need the initial “upon change of any information” in this section if we do not retransmit upon any change.

Huawei recall that when the spec was drafted, we had a general principle that if any information was changed the UE could send this message, and the additional conditions were added later, creating some apparent overlap.  The contribution referred to by Xiaomi is about change of interest in the SIB, and they understand that companies felt this was already clear.  They also agree with Apple that we could generalise the text and avoid including more details in the conditions.

Xiaomi think the earlier proposal and this change are quite similar in their effects.,

Ericsson want to make sure that a generic solution would not include any specific scenarios.

To be included in offline.

Apple understand the intention of point 3, but they think it is only for the discovery process and may not apply to the L2 relay case; they understand that the condition will never be evaluated in this case.  They also are not sure why the UE wants to go to connected state in this scenario.  Ericsson have a similar understanding; upon reception of SIB12, the UE knows whether it can use discovery.  They think this was discussed in the last meeting.

Samsung think this change is needed to align with previous meetings’ discussion of how to handle SIB12.  For Apple’s concern, they understand that we had some general discussion on the upper layer interpretation, but relay selection will be triggered from upper layers and the AS layer will deliver the AS container to the upper layer; they see that it applies for L2/L3 relay.

Huawei indicate that last meeting, we said the UE checks SIB12 and calls the discovery procedure if discovery is supported, and there is currently no mention of how the UE obtains the resource pool configuration; there is a procedure that triggers the UE to move into RRC_CONNECTED if there is no resource pool in SIB12.  So they understand that the change is needed.  To Apple’s comment, they agree that a UE implementation may have this kind of inter-layer interaction, but it has not been specified.

Ericsson think this is just about whether the network supports relay discovery, and they understand that we captured in a NOTE that it is up to network implementation to guarantee that the UE has the needed resource pools.

NEC agree with the intention of the CR, but when they checked the IE structure of SIB12, they found that the wording is not quite accurate and it should be about whether the gNB can support the corresponding operation.

Samsung agree that NEC’s comment is correct, but the CR tries to follow the existing wording in SIB12.  NEC think it is not accurate as written.

Ericsson understand that the concern is that the UE gets into connected state and the network does not support discovery, and this allows the UE to request a dedicated configuration.  They do not see the point of such a scenario, so they think the first case is not valid.  Huawei understand the intention is to avoid the UE moving to RRC_CONNECTED when it cannot get a configuration because the network does not support the feature.  Ericsson think the UE will not request a relay configuration without knowing if the network supports it.  Samsung indicate that this is directed to the case where SIB12 indicates support but there is no Tx discovery pool.

Apple think the change may not be necessary because the condition will never trigger, and maybe a different change could be considered to capture an indication to upper layers to prevent them from trying to transmit inappropriately.

- Intention is to avoid the UE going to connected mode to request a relay configuration that the network cannot provide.

- Offline to conclude on how best to realise this.

Nokia think on point 5, the intention is to add some clarification on whether the UE releases or re-establishes the RLC channels; they are not sure if something is wrong with the current spec.

Apple think the current spec has a problem because it forces the RLC channels to be released, and the relay UE will not be able to receive on them.

Huawei indicate that upon going to idle/inactive, the relay UE will release all resources including RLC channels, but the remote UE will re-establish SL-RLC0/1, which should only be needed at unicast link establishment.  So they understand that the current spec forces the remote UE to perform unicast link release and establishment, which is not absolutely broken but not ideal, and they think it makes sense to allow re-establishment of the entities instead of release.,

Nokia agree that nothing is broken.  Apple think the current normative text has the remote and relay keep the PC5 unicast link, which gives the impression that the link will be reused, but the reuse can never be successful because the RLC channels have been released.

OPPO wonder if this is NBC.  Huawei understand that if we required that the relay shall not release the channels, this would impact existing implementations, but if we add a NOTE that the relay is allowed to maintain them, this would be BC.  Nokia agree it does not break existing implementations.

Apple indicate the intention is to allow the relay to keep receiving messages from the remote UE.  They think the NOTE suggested by Huawei would be acceptable.

- Capture a NOTE that the relay may re-establish the RLC channels.

Ericsson wonder if point 8 is already covered in 38.304.  Huawei indicate that in the relay reselection subclause there is no condition for triggering based on cell selection.

Xiaomi think it is already specified that the UE can perform relay selection or cell selection.

OPPO understand that there are conditions for the UE handling in different states for this case, and they think all cases are clearly captured.

Huawei agree that relay reselection triggered by cell selection during RRC re-establishment/release has already been captured, but for this clause there are a lot of detailed triggers listed, and the cell reselection case has not been included.

Nokia think the current text covers the case under “if the UE has no serving cell”.

NEC think the intention is that it is up to UE implementation to perform relay selection or cell selection, and they think the NOTE we have is safer than changing the normative text.

ZTE can accept having the existing NOTE.

- Point 8 is not included.

Apple note that L2 has been removed in the first change of point 9, and they think it is a little strange that we would take the first change without the second.  Huawei think the changes to the second clause are not needed because relay reselection does not need to invoke the second clause, but they can accept keeping “L2” in the first clause (i.e., keep the last two changes in the first clause).

vivo think the procedure is intended to cover both L2 and L3, so we should delete “L2” in both.

Huawei indicate the relay reselection clause reuses the filtering procedure, so the filtering procedure is applicable to both L2 and L3, but the measurement derivation is only for L2.

Apple can accept taking the last two changes in the first section.

- Do not delete “L2” in the first requirement of section 5.5.3.2

Apple think one big CR would be OK.

[AT123][427][Relay] Rel-17 relay RRC CR (Huawei)

Scope: Merge agreed changes for 38.331 to a single general CR.

Intended outcome: Agreeable CR in R2-2309109

Deadline: Thursday 2023-08-24 2000 UTC

Other contributions

R2-2307194	38.331_CR_Corrections to processing of paging information received via Relay UE	Samsung Electronics Co., Ltd	CR	Rel-17	38.331	17.5.0	4177	-	F	NR_SL_relay-Core

R2-2307239	Correction of RemoteUEInformationSidelink transmission condition	OPPO	CR	Rel-17	38.331	17.5.0	4180	-	F	NR_SL_relay-Core

R2-2307727	Conditions for RRC connection establishment and resume for NR sidelink discovery	Samsung, Huawei, HiSilicon	CR	Rel-17	38.331	17.5.0	4209	-	F	NR_SL_relay-Core, NR_SL_enh-Core

R2-2307755	Correction on NR Sidelink Relay RRC	Philips International B.V.	CR	Rel-17	38.331	17.5.0	4212	-	F	NR_SL_relay-Core

R2-2307852	Corrections on SRAP related configurations for SL relay	Apple	CR	Rel-17	38.331	17.5.0	4215	-	F	NR_SL_relay-Core

R2-2307853	Corrections on the reporting of L2 ID for L2 U2N relay operation	Apple	CR	Rel-17	38.331	17.5.0	4216	-	F	NR_SL_relay-Core

R2-2307955	Correction on CHO and Path Switching of Remote UE	NEC Corporation	CR	Rel-17	38.300	17.5.0	0695	-	F	NR_SL_relay-Core

R2-2308210	Miscellaneous corrections for SL relay	Huawei, HiSilicon	CR	Rel-17	38.331	17.5.0	4235	-	F	NR_SL_relay-Core

R2-2308271	Corrections to TS 38.331 on SL relay (re)selection	ZTE, CAICT, Sanechips	CR	Rel-17	38.331	17.5.0	4241	-	F	NR_SL_relay-Core

R2-2308272	Corrections to TS38.300 on SL relay (re)selection	ZTE, CAICT, Sanechips	CR	Rel-17	38.300	17.5.0	0698	-	F	NR_SL_relay-Core

R2-2308275	Correction to 38.331 on U2N relay (re)selection	vivo	CR	Rel-17	38.331	17.5.0	4240	-	F	NR_SL_relay-Core

R2-2308550	Miscellaneous Corrections for SL Relays	Ericsson España S.A.	CR	Rel-17	38.331	17.5.0	4261	-	D	NR_SL_relay-Core

R2-2308553	Miscellaneous Correction for SL Relays	Ericsson	CR	Rel-17	38.300	17.5.0	0703	-	D	NR_SL_relay-Core

R2-2308714	Corrections on U2N Relay	ASUSTeK	CR	Rel-17	38.331	17.5.0	4281	-	F	NR_SL_relay-Core

### 6.2.2	User plane corrections

A single CR with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to the CR rapporteur for the corresponding spec.  Larger open issues can be discussed with contributions (limited time).

R2-2307238	Correction of IE name sl-SRAP-ConfigRemote	OPPO	CR	Rel-17	38.351	17.5.0	0023	-	F	NR_SL_relay-Core

- Agreed

Discussion:

Huawei agree with the intention, but they think the current description is a bit unclear.

Samsung think Huawei’s concern is not related to the change in the name.

R2-2307756	Correction on SRAP for sidelink relay	Philips International B.V.	CR	Rel-17	38.351	17.5.0	0024	-	F	NR_SL_relay-Core

Discussion:

Samsung wonder if the clarification is needed.  Philips understand we have the parallel clarification in the uplink direction.

Huawei agree that we should align uplink and downlink.

Apple think the current description of BEARER ID is OK, and if the procedure text covers all cases, we do not need to update the field description.

OPPO would prefer to agree to the Philips CR, and then the Huawei CR in R2-2308211 is not needed.

Huawei are OK to take the Philips CR, but they think a general description of the field is also useful.

Samsung agree that the change from Philips is needed, and they are concerned that we will have redundant text if we also take the change from Huawei.  Huawei indicate that there is a bracket in the procedural text explaining the interpretation, but they do not think it is redundant wrt the text in the field definition.

Nokia suggest aligning the text in 5.2.3.  Samsung think this could be looked at offline.

Apple think the RAN box should not be checked.

[AT123][428][Relay] BEARER ID correction in SRAP (Philips)

Scope: Check the CR in R2-2307756 and determine if a parallel change is needed in section 5.2.3.

Intended outcome: Agreeable CR in R2-2309111

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308211	Clarification on the BEARER ID in SRAP data PDU	Huawei, HiSilicon	CR	Rel-17	38.351	17.5.0	0025	-	F	NR_SL_relay-Core

Discussion:

Samsung note a typo (srb-Identity should be drb-Identity the second time).

OPPO indicate that “info” should be “information”.

Chair notes additional typos: “se” for “set” and “idenfity” for “identify”.

- Agreed with typos fixed as R2-2309110

## 6.4	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

Tdoc Limitation: 2 tdocs

### 6.4.1	Stage 3 corrections

A single CR per TS (RRC, LPP, MAC, UEcap 306) with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to the CR rapporteur.  Larger open issues can be discussed with contributions (limited time).

R2-2307359	Correction to Multi-RTT	Huawei, HiSilicon	CR	Rel-17	37.355	17.5.0	0455	-	F	NR_pos_enh-Core

- Agreed as R2-2309104, with ASN.1 alignment fixed

Discussion:

Ericsson think we wanted to avoid having conditions in the uplink.  They think we could add something in the field description, but not a condition indicator.

Qualcomm think the CR is fine because we already use the same description for DL-TDOA and DL-AoD, and they disagree with Ericsson’s comment because this is different from a need code as such; they see it as an oversight.

CATT do not think it is essential, because the request already indicates whether this field should be reported.

vivo agree with Qualcomm and think we have the condition for DL-TDOA and DL-AoD.

Intel agree with Qualcomm.

Nokia wonder why the CR does not apply to other methods, and they ask if we cannot report one instance with the existing field.

Samsung share the view of Qualcomm and vivo.

Huawei clarify that the intention is not to have both fields present at the same time, and if the request uses the legacy format, the response will use the legacy field.

Intel agree that this is alignment with the downlink methods.

OPPO agree with the CR.

Qualcomm think there are editorial details with the ASN.1 alignment that should be fixed to save effort in CR implementation.

CATT indicate that the reason for the difference from the DL methods is that there is no UE-based multi-RTT.  Qualcomm think it is still valid for UE-assisted and this was just a copying oversight.

R2-2307360	Correction to UE capability for batch reporitng	Huawei, HiSilicon	CR	Rel-17	37.355	17.5.0	0456	-	F	NR_pos_enh-Core

- Postponed

Discussion:

Lenovo wonder what the reason was for the addition in the Provide Location Information.  Huawei indicate that this is the question they want to address: The LMF can request differently for different methods, but the UE only has a single capability.

vivo think the issue is valid but would prefer to fix the field description (Huawei’s approach 1), because RAN1 indicated only a single capability for this feature.

Intel agree that the capability came from RAN1, and they understand the guidance was that the UE supports it as one capability.  They do not think it should be changed in RAN2.

Nokia wonder whether UE-based was also intended by RAN1.

Ericsson agree with Intel and think that RAN1 intended to capture a single functionality for multiple measurements or multiple locations.

Apple think it would be good to have separate capabilities.  Qualcomm also think it makes sense, and they suggest that we could ask RAN1.  Intel agree.

ZTE agree with sending an LS, and they wonder if we should include the additional option of changing the field description.  Intel think how we implement it would be a RAN2 decision, but we can ask RAN1 if it is needed to have a separate capability.

[AT123][420][POS] LS to RAN1 on batch reporting capability (ZTE)

Scope: Draft an LS to RAN1 inquiring about the need for multiple capabilities for batch reporting, as proposed in R2-2307360.

Intended outcome: Approved LS (without CB if possible) in R2-2309105

Deadline: Wednesday 2023-08-23 2000 UTC

R2-2307504	Missing error cause code for DL PRS Measurements	Fraunhofer IIS, Ericsson	CR	Rel-17	37.355	17.5.0	0457	-	F	NR_pos_enh-Core

- Not pursued

Discussion:

Qualcomm wonder what the new cause code really means, and if we need to start having cause codes at such fine granularity.  There could be many reasons for an “unable to measure” condition.

Lenovo agree with Qualcomm and think the priority setting is quite dynamic, so they are not sure if the new code helps the LMF.

vivo agree with Qualcomm and think it is too detailed; the network is already aware of the PPW configuration, and if this situation occurs, they think the UE should request a measurement gap.

Ericsson agree that it is granular, but they think the inability to measure in certain PPWs is an error case that should be reflected in the error report.  They see that the LMF would benefit from gathering statistics on this sort of event.

R2-2308478	Missing finer periodicities than 1s	Ericsson	CR	Rel-17	37.355	17.5.0	0450	1	F	NR_pos_enh-Core	R2-2306026

- Not pursued

R2-2308479	Missing LPP support for sub 1s location information reporting periodicity	Ericsson	discussion	Rel-17

- Noted

Discussion:

Huawei are generally fine with the proposal, and consider that the AS layer granularity should generally be finer than the service layer granularity.  They do wonder in which release we should introduce it.

Qualcomm think this is not a correction; they understand that we implemented the RAN1 feature list correctly, and this should be considered as an enhancement.

CATT also think it is not a correction, and they think we need to determine which use case needs it to infer which release it should be introduced in.

vivo note that the minimum value in NRPPa is 640 ms, which is not drastically off from 1 s, so they do not see a big gain from alignment.

Apple think it is an enhancement, not a correction.  They think such an enhancement should normally come from lower layers.

Ericsson indicate that the proposal did come from lower layers, and we considered finer periodicities during Rel-17 discussions.  They see that the network specs have sub-second periodicities in Rel-17 and LPP is the missing piece, so they consider this an alignment correction rather than an enhancement.  They think that the Rel-17 latency requirements force us to have the ability to schedule the reporting with short intervals, so the device can report measurements/position estimates close in time.

Qualcomm think periodic reporting is not really a low-latency feature.  They think if we change periodic reporting it should apply to all positioning methods, which again suggests that this is more of an enhancement.

Huawei understand that the LPP periodic reporting is intended to align with the periodicity requests from the service layer, and this just brings LPP into line with CT4 specs.

Ericsson indicate that the CT4 agreements apply to all positioning methods.

vivo think if CT4 want a change, there should be an LS to guide us.

Qualcomm see no connection between LPP periodic reporting and the CT4 specs, which are for deferred MT-LR, whereas periodic reporting in LPP is between UE and LMF,  They do not see that the specs are broken.

CATT wonder what the CT4 spec defines for periodic reporting between LMF and AMF.  Ericsson indicate that the CT4 specs for reporting between LMF/AMF/GMLC/MAP have all been updated with sub-second periodicities.

CATT think CT4 should send an LS.

OPPO agree there should be an LS.

Qualcomm would like to see how it works end-to-end.

R2-2308690	Addition of missing values for dl-prs-ResourceSetPeriodicityReq-r17	Samsung	CR	Rel-17	37.355	17.5.0	0464	-	F	NR_pos_enh-Core

Discussion:

Lenovo support the CR.

vivo think it is not needed, because the periodicities are indicated in the SCS of the PCell.

Qualcomm think these are periodicities supported for the PRS, but not supported in the on-demand request.  They think this is a real correction.

CATT ask if the proponent checked the RAN1 parameter list; if RAN1 did not provide these periodicities, they think maybe we should check with RAN1.  Samsung indicate that they did not find the periodicities in the RAN1 list, but we have a misalignment between the ASN.1 and the field description; they would be OK to send an LS to check.

Lenovo agree that there is a mismatch.

Nokia think it is an essential correction, but they are OK to confirm with RAN1.

CATT would like time to check the RRC parameters.

[AT123][421][POS] dl-prs-ResourceSetPeriodicityReq-r17 range check (Samsung)

Scope: Evaluate the change proposed in R2-2308690 in light of the RAN1 parameter list.

Intended outcome: Agreeable CR if necessary and report in R2-2309106

Deadline: Wednesday 2023-08-23 2000 UTC

### 6.4.2	Stage 2 corrections

A single CR with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to the CR rapporteur.  This agenda item will be handled at lower priority.

R2-2308759	Correction of PRU overview description	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.305	17.5.0	0139	-	F	NR_pos_enh-Core

- Not pursued

Discussion:

Qualcomm think the existing text is correct and the change would be wrong.  Their understanding is that you know the PRU location and what it is supposed to measure, and the point is to compare the measurements taken by the PRU at a known location to the reported measurements from a UE.

vivo agree with Qualcomm and think the comparison is about the measurements.

CATT have the same view, and they checked the LS from RAN1 and found that it did not mention how to use the measurements.  They think an alternative change to replace “compared” with “used” could be valid.

Intel wonder if we need to do anything for PRUs in Rel-17.

Nokia can accept majority view and keep the existing text.

# 7	Rel-18

## 7.2	Expanded and improved NR positioning

(NR_pos_enh2; leading WG: RAN1; REL-18; WID: RP-231460)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 7.2.1	Organizational

Including incoming LSs and rapporteur inputs.

Incoming LSs with RAN2 in Cc:

R2-2307007	Reply LS on Sidelink positioning procedure (R1-2306208; contact: Xiaomi)	RAN1	LS in	Rel-18	Ranging_SL	To:SA2	Cc:RAN2

- Noted

R2-2307031	Reply LS on Authorization and Provisioning for Ranging/SL positioning service (R3-233424; contact: Xiaomi)	RAN3	LS in	Rel-18	Ranging_SL, NR_pos_enh2	To:SA2	Cc:RAN1, RAN2

- Noted

R2-2307052	Reply LS on the requirement on low power or high accuracy positioning (S1-231370; contact: Huawei)	SA1	LS in	Rel-18	5G_eLCS_Ph3	To:SA2	Cc:RAN1, RAN2

- Noted

Incoming LS with “take into account” action

R2-2307004	LS reply on the RAT-dependent positioning integrity (R1-2306157; contact: InterDigital)	RAN1	LS in	Rel-18	NR_pos_enh2	To:RAN2	Cc:RAN3

- Noted

Other incoming LSs and draft replies

R2-2307010	LS to RAN2 on SRS bandwidth aggregation for positioning (R1-2306214; contact: ZTE)	RAN1	LS in	Rel-18	NR_pos_enh2	To:RAN2

R2-2308139	[draft]Reply LS to RAN1 on SRS bandwidth aggregation for positioning	ZTE Corporation	LS out	Rel-18	NR_pos_enh2	To:RAN1

[AT123][402][POS] LS to RAN1 on SRS bandwidth aggregation (ZTE)

Scope: Draft a reply to R2-2307010, taking online discussion into account.

Intended outcome: Approved LS (without CB if possible)

Deadline: Wednesday 2023-08-23 2000 UTC

R2-2307032	Reply LS on SRS Configuration Request (R2-2302278; contact: Huawei)	RAN3	LS in	Rel-18	NR_pos_enh2	To:RAN2	Cc:RAN1

- To be concluded after LPHAP discussion

R2-2307042	LS on reporting granularity for timing related positioning measurements (R4-2310166; contact: Huawei)	RAN4	LS in	Rel-18	NR_pos_enh2	To:RAN2, RAN3	Cc:RAN1

R2-2307126	Draft reply LS on timing measurement reporting granularity	Huawei, HiSilicon	LS out	Rel-18	NR_pos_enh2	To:RAN4	Cc:RAN1, RAN3

R2-2307127	Discussion on measurement reporting granularity	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

[AT123][403][POS] LS to RAN4 on timing measurement reporting granularity (Huawei)

Scope: Draft a reply to R2-2307042, taking online discussion into account.

Intended outcome: Approved LS (without CB if possible)

Deadline: Wednesday 2023-08-23 2000 UTC

R2-2307054	Reply LS to LS to SA2 on Sidelink positioning procedure (S2-2305735; contact: Xiaomi)	SA2	LS in	Rel-18	Ranging_SL	To:RAN2, RAN1	Cc:SA3

R2-2307056	LS on assistance information provided to UE (S2-2307553; contact: Xiaomi)	SA2	LS in	Rel-18	Ranging_SL	To:RAN2

[AT123][404][POS] LS(s) to SA2 on sidelink positioning (Xiaomi)

Scope: Draft a reply (or separate replies) to R2-2307054 and R2-2307056, taking online discussion into account.

Intended outcome: Approvable LS(s)

Deadline: Wednesday 2023-08-23 2000 UTC

LS-related documents from non-contact companies

R2-2308053	Discussion on the reply LSs to SA2 on SL Positioning	OPPO	discussion	Rel-18	NR_pos_enh2

Draft TS

R2-2307663	TS 38.355 v0.0.4	Intel Corp]oration	draft TS	Rel-18	38.355	0.0.4	NR_pos_enh2

Discussion:

Lenovo think this TS may not be stable enough to submit to plenary from this meeting.

Intel think we should follow the WI schedule and send the TS for information, but they agree it is not very developed yet.

Nokia think there should be post-meeting review time for the TS and the running CRs generally.  Huawei agree that we will need post-meeting discussions to implement the agreements of this meeting.

[AT123][409][POS] TS 38.355 (Intel)

Scope: Collect comments on the draft TS in R2-2307663 and produce an endorsable version, taking into account also discussion of the proposals in R2-2307662.

Intended outcome: Endorsable draft TS

Deadline: Thursday 2023-08-24 2000 UTC

Running CRs

R2-2307124	Running MAC CR for LPHAP	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2307125	Running MAC CR for Sidelink Positioning	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

[AT123][405][POS] Rel-18 positioning MAC CRs (Huawei)

Scope: Collect comments on the CRs in R2-2307124 and R2-2307125 and produce endorsable versions.

Intended outcome: Endorsable CRs

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307391	LPP running CR for RAT-dependent integrity	CATT	draftCR	Rel-18	37.355	17.5.0	NR_pos_enh2

[AT123][406][POS] Rel-18 LPP CR on RAT-dependent integrity (CATT)

Scope: Collect comments on the CR in R2-2307391 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308385	Running Stage 2 CR for 'Expanded and improved NR positioning'	Qualcomm Incorporated	draftCR	Rel-18	38.305	17.5.0	B	NR_pos_enh2

R2-2308386	Stage 2 TP for SL-MO-LR/SL-MT-LR	Qualcomm Incorporated	discussion

R2-2308387	Stage 2 TP for SLPP Transport between UE and LMF	Qualcomm Incorporated	discussion

R2-2308395	Stage 2 TP for SLPP Transport between UEs	Qualcomm Incorporated	discussion

[AT123][407][POS] Rel-18 positioning stage 2 CR and TPs (Qualcomm)

Scope: Collect comments on the CR in R2-2308385 and related TPs in R2-2308386 / R2-2308387 / R2-2308395, and produce an endorsable version of the CR.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308484	Rapporteur CR for Positioning RRC Changes	Ericsson	draftCR	Rel-18	38.331	17.5.0	B	NR_pos_enh2

[AT123][408][POS] Rel-18 positioning RRC CR (Ericsson)

Scope: Collect comments on the CR in R2-2308484 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

Rapporteur inputs on spec handling

R2-2307662	Further considerations on SLPP specification	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308259	Discussion on R18 positioning UE capabilities	Xiaomi	discussion

[AT123][410][POS] Rel-18 positioning capabilities (Xiaomi)

Scope: Discuss the proposals in R2-2308259 and conclude on handling of the Rel-18 capabilities.

Intended outcome: Report to CB session

Deadline: Wednesday 2023-08-23 2000 UTC

Other

### 7.2.2	Sidelink positioning

Positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning.  Including measurements to enable RTT-based positioning, SL-AoA, and SL-TDOA; signalling and associated UE behaviour for support of unicast, groupcast (not including many-to-one) and broadcast of SL-PRS transmissions; reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios; and signalling to NG-RAN for SL positioning and service authorization as needed.

Including report of [Post122][402][POS] SLPP session handling (Intel)

Email discussion summary

R2-2307660	Report of [ 402] SLPP session handling	Intel Corporation	discussion	Rel-18	NR_pos_enh2	Late

Proposal 1: For LMF involved SL based positioning, follow SA2 on how to handle LMF involved SL based positioning between UE (who has connection with network), LMF and AMF. FFS on how to handle session for UEs involved in the same LMF involved SL based positioning and the relationship between routing ID/correlation ID and session ID.

Discussion:

Ericsson think following SA2 is fine, but the FFSs may be possible to resolve.  They understand that when the LMF is initiating the session it can assign the session ID.  Huawei think this may not be possible for the SLPP session ID, but the routing ID and correlation ID can be assigned by the LMF as usual.  Huawei think we could follow the UE-only case.

Intel suggest postponing the discussion on the FFS aspects until we handle the summary below and determine if we have conclusions.

Lenovo are OK in principle with the proposal, but they wonder if SA2 will do anything further without prompting from us; they should be into the maintenance phase.  They think we may need to update SA2 about our status.

Qualcomm do not see SA2 impact from these aspects.  They understand the LMF can only talk to one UE, and the question is whether the LMF also needs to see a second SLPP session ID between the UEs; this is a RAN2 question.  Huawei agree with Qualcomm and think the SLPP session ID only needs to be used between UEs; it should have no SA2 impact.

Ericsson also agree that there is no SA2 impact.  They think that at least for the SL MT-LR case, the LMF needs to know what session it needs to get measurement results from; they agree that some more discussion is needed.

Intel think we should focus on the proposal instead of expanding the discussion.  They have not identified SA2 impact and think we can continue discussion, with the understanding that we will notify SA2 if something comes up.

CATT think on Ericsson’s comment, the handling depends on the solution for the session between the LMF and the UE.  They agree with Intel that we can follow the overall SA2 procedure, and they do not think an SLPP session ID is required between the UE and the LMF.

vivo think SA2 may have some impact from RAN2 decisions; the anchor UE can distinguish different sessions by the correlation ID or routing ID, but if there are different correlation IDs for different SLPP sessions, then the SLPP session ID is not needed between LMF and anchor UE.

Proposal 2: RAN2 should focus on single target scenario and will continue the discussion on multiple target UEs, and broadcast/groupcast once the basic functionality has been defined.

Discussion:

OPPO think SA2 have already covered multiple targets with SL MT-LR, and they do not see the RAN2 impact of supporting it.  Huawei agree; from RAN2 perspective, there may be only stage 2 description for multiple targets and for broadcast/groupcast.

Huawei point out that SA3 are still working on the security aspects, and this might be the bottleneck for groupcast.

Xiaomi see some misunderstanding between RAN2 and SA2: SA2 have specified one target UE and multiple reference UEs, and for multiple target UEs they have not specified any detailed procedures, although they may address it this meeting cycle.  They think we can at least discuss groupcast/broadcast.

Intel see different understandings from companies about SA2 status, and the intention of the proposal is that we focus our discussion on the basic procedure with one target and unicast first.

Nokia agree with Intel: The proposal does not preclude multiple target UEs, it just prioritises where we focus our time.  Ericsson agree.

Qualcomm think there is some support in SA2 for multiple target UEs, although it is not complete through all specs.  They do not consider single target/unicast to be the basic functionality, because SL positioning always involves multiple UEs, and unicast is just a transport option that may not affect SLPP.  If such a prioritisation is needed, they would like to delete the second half of the proposal.

Xiaomi understand that there is a preference to focus on the stage 3 impact.

Proposal 3: For UE-only operation, introduce explicit field “sessionID” in SLPP, and put it under message header of SLPP message. FFS how session ID is defined

Discussion:

OPPO think LPP embeds the session ID in the NAS layer, and for SLPP maybe we should follow the legacy behaviour and use the V2X/ProSe layer below SLPP.  Intel indicate that the reason they propose to include it in SLPP is to avoid relying on other groups to specify the ID and inter-layer interaction to use the session ID at the UE.

Huawei think we have already supported UP transport without using PC5-S as a carrier.

Apple support the proposal and think it would be convenient to have the ID in our specs.

vivo think the proposal is OK, and the suggestion from OPPO does not work because we do not have the V2X/ProSe layer.  OPPO understand there is a layer to map the service data to QoS flows, and they think there is also such a layer here; they are OK to check the protocol layers and decide later.

Ericsson agree with the proposal and think it will simplify things to have the ID in SLPP; they think there was a preference for a unified solution between network-based and UE-only operation.

CATT support the proposal; they understand that the LMF has a session ID, but there is no legacy session ID between UE and LMF, and in the SLPP case we need to identify the sessions between UEs.

Qualcomm also support the proposal and think SLPP should be self-contained as much as possible.

CEWiT see the benefit of having the session ID in the message.

ZTE also support the proposal; regarding network-based operation, they think there is still a case where UEs need to exchange SLPP, and the session ID will be needed there.

Proposal 4: (10 vs 8) For UE-only operation, RAN2 will continue the discussion on whether “sessionID” is applied for groupcast/broadcast once the basic functionality has been defined.

Proposal 5: (13 vs 4 ) from SLPP perspective, the UE who receives the] LCS request at least needs to:

-	Initiate the SLPP procedure;

-	Assign the sessionID, and include it in the SLPP messages (Rx side should use the received sessionID for messages in the same positioning session);

Discussion:

Huawei think there should be no session ID for broadcast.  Intel think we can continue discussion on broadcast, but we do not have time to fully discuss it online now.

vivo think we need to scope this as “at least for UE-only operation”. Intel think this was the intention.

Huawei would like to understand if there is spec impact.  Intel think we should capture the assignment of the session ID.  CATT agree with Intel and think we need to reach a common understanding about the procedure; this proposal aligns with SA2.

Huawei think there is no SA2 guidance on which end initiates the SLPP procedure.

Nokia think this does not clarify the scope of uniqueness of the ID, and maybe we could capture that this aspect will be further looked at.

vivo think it is not clear what happens if the target and server UEs are different; they see that in this case two UEs could receive the LCS request.  They would prefer to clarify that the server UE performs the allocation.  Intel indicate there was no consensus on this point.

Qualcomm think we are repeating a discussion, and the proposal has nothing wrong with it; if it later turns out that it must be a server UE, we can clarify that.

CATT agree with Intel and Qualcomm; we are talking about UEs here irrespective of different roles.  They see the proposal as a good way forward.

OPPO agree with the proposal and think the server UE definition does not imply such a function.

Proposal 6: (15 vs 1 ) if the UE who receives the LCS request can act as the SL Positioning Server UE, then the UE shall trigger following procedures with each of UEs (UE2-UEn in the figure) in the SLPP session:

-	SL Positioning Capability Transfer procedure,

-	SL Location Information Transfer (FFS on who decide positioning method) and

-	SL Positioning Assistance Data exchange (depends on RAN1 discussion on how to select the PRS resources)

Proposal 7: (11 vs 2 ) In stage 3 specification, use "Tx Endpoint" and "Rx Endpoint(s) to describe the procedure instead of target UE, anchor UE and server UE concept, e.g. [figure omitted]

Discussion:

Lenovo wonder why we need “Tx” and “Rx”.  Qualcomm clarify it came from 38.300, and they think it is helpful to show which side initiates the procedure without tying it to UE roles.

Apple would prefer not to have the Tx and Rx language.

CATT think we should clarify SLPP Tx/Rx vs. SL-PRS Tx/Rx.

Xiaomi note that the transmitter of one operation may be the receiver of another, so the Tx/Rx terminology may not be clear.

ZTE think in stage 3, we always have the SLPP Tx/Rx, not the SL-PRS Tx/Rx.  So they think Tx/Rx can be kept.

Proposal 8: Reuse LPP session management as a start, and we may revisit it if any issue is found.

Discussion:

Intel clarify the intention is to avoid introducing new session management procedures.

Qualcomm do not think we need an agreement in this direction and there is no clear definition of LPP session management.  They see benefit from having explicit session start/end.

Lenovo agree with Qualcomm’s comment and think it is related to session release at the UE.  We have not discussed how many sessions a UE can support.

Xiaomi agree with Qualcomm and think we could have an FFS on the session release.

CATT wonder what the stage 3 impact for session start/release is.

Nokia think the proposal does not have a big impact.

Agreements:

For LMF involved SL based positioning, follow SA2 on how to handle LMF involved SL based positioning between UE (who has connection with network), LMF and AMF. FFS on how to handle session for UEs involved in the same LMF involved SL based positioning and the relationship between routing ID/correlation ID and session ID.

At least for UE-only operation, introduce explicit field “sessionID” in SLPP, and put it under message header of SLPP message. FFS how session ID is defined.

At least for UE-only operation, the UE who receives the LCS request at least needs to:

-	Initiate the first SLPP procedure;

-	Assign the sessionID, and include it in the SLPP messages (Rx side should use the received sessionID for messages in the same positioning session).

FFS within what scope the session ID is unique.

At least for UE-only operation, if the UE who receives the LCS request can act as the SL Positioning Server UE, then the UE shall trigger following procedures with each of UEs (UE2-UEn in the figure) in the SLPP session:

-	SL Positioning Capability Transfer procedure,

-	SL Location Information Transfer (FFS on who decide positioning method) and

-	SL Positioning Assistance Data exchange (depends on RAN1 discussion on how to select the SL-PRS resources)

In stage 3 specification, use "Endpoint A" and "Endpoint B” to describe the procedure instead of target UE, anchor UE and server UE concept, e.g. [figure omitted]

Agenda item summary (excluding items related to the email discussion)

R2-2308973	Summary of AI 7.2.2 Sidelink positioning	CATT	discussion	Rel-18	NR_pos_enh2

[Scenario aspect]

Proposal 3: RAN2 to apply terms of “UE-only Operation” and “Network-based Operation” defined in TS 23.586 by SA2 for SLPP procedures.

Discussion:

Lenovo wonder why network-assisted operation is not included.  CATT indicate that the terms are not intended to be exclusive, and they understand that SA2’s “network-assisted operation” just means UE-based positioning in our terms; they see it as not relevant to the scenario issues.

Huawei agree with Lenovo and think we should capture the network-assisted term.

OPPO checked the SA2 spec and found that UE-only operation only applies when the network is not involved, so this is not related to UE-based/UE-assisted.

Xiaomi think we should follow the SA2 terminology.

Proposal 4: RAN2 to discuss how to capture the cases of “UE-only Operation” and “Network-based Operation” in stage-2 specification. The following definition can be as baseline:

Network-based operation:

-	Case 1: at least one of a UE has NAS connection, and 5GC supports SL positioning.

•	Case 1-1: both target UE and all anchor UEs have NAS connection; (in-coverage)

•	Case 1-2: target UE has NAS connection, and at least one of anchor UE does not have NAS connection; (partial-coverage)

•	Case 1-3: target UE does not have NAS connection, while at least one of the anchor UEs has NAS connection; (partial-coverage)

UE-only operation:

-	Case 2: none of the UE has NAS connection; (out-of-coverage)

-	Case 3: target UE has NAS connection, but SL-MO-LR request is rejected by the network. (in-coverage)

Discussion:

Lenovo think per SA2 specs, there is a missing case 4: UE is configured by NW to allow UE-only operation.

Intel wonder if we need to care about these cases in RAN2; we need to specify SLPP, and the scenarios are more in SA2 scope.  Qualcomm agree with both Intel and Lenovo; they think it is not complete and does not impact our work.  Nokia agree as well.

ZTE think P4 is useful for progressing the stage 2, but we should not have the brackets with the coverage scenarios.

Xiaomi understand the motivation for capturing something in stage 2, but they think there are additional SA2 criteria that are not captured here.

Apple are not sure what the purpose of the scenarios is in light of the next proposal; they also have some concerns with the language.

CATT indicate the intention is just to get a common understanding of the coverage scenarios.

Nokia think we are repeating SA2 discussions; the LMF is used if a UE can reach the LMF.

Proposal 5: RAN2 can prioritise to discuss in-coverage and out-of-coverage sidelink positioning cases. The additional functionality for partial-coverage cases can be revisited if time allows.

Discussion:

InterDigital think partial coverage should be considered.

Xiaomi think we should look at the stage 3 impacts, and if we can support partial coverage without extra work it is OK.  They do not see differences so far.

Intel think it is related to forwarding messages from other UEs.

Apple support P5 and think we should look at what is realistic to support.

Nokia think we should not divide the work into subcases but try for a unified design that covers seamlessly.

Huawei understand SA2 have already supported the partial coverage scenario, and they do not identify RAN2 impact.  They wonder why RAN2 should veto another group’s decisions.  They understand that it is only related to MO-LR.

OPPO think whether to treat partial coverage depends on how big the delta is.  They see the delta as identifying the relay point for the UEs out of coverage and potentially using sidelink relay to forward the messages; they are not sure how big the impact is as a whole.

Qualcomm agree with Huawei; from a procedural point of view, they think partial coverage is the same as out of coverage.

Intel support the proposal and agree that we would need to identify the impact; they see that there is a concern with the amount of time left and we need to prioritise.  MediaTek agree with Intel.

CATT clarify that the intention is not to exclude partial coverage but to prioritise.

vivo agree with Intel and think this is related to forwarding of messages.  We would have to design a forwarding mechanism for out of coverage UEs.

Ericsson think we could use UE-to-network relay.  Huawei think this could be used as transport for the out-of-coverage UEs and it would be transparent from the positioning pov.  To vivo’s comment, they think we need to identify what the required changes are.

InterDigital think the anchor UE can forward messages for an out-of-coverage target UE, and if we reuse the UE-to-network relay there would be no extra impact.  Chair wonders if we would assume that all SL positioning UEs support relaying.

Xiaomi point out that even for in-coverage, forwarding may be needed, because some of the anchor UEs may not be served by the same LMF, and in this case one UE will forward the information from those anchor UEs.

Nokia also agree that forwarding is needed.

Qualcomm are not prepared to agree to the forwarding capability; they see it as a corner case.  Their understanding of the SA2 specs is that the LMF talks to one UE and that UE instigates the SLPP sessions.

CATT found there are two candidate solutions: sidelink relay and SLPP message forwarding.  Their concern is finalising the procedures for the in-coverage and out-of-coverage procedures.

Huawei point out that SA2 are not considering sidelink relay but the case where an “OOC” UE does not have a NAS connection.

Apple agree with Qualcomm.

Ericsson agree with Huawei and think we do not need a new mechanism for forwarding.

Qualcomm think the LMF in any case will only talk to a single UE.  They think we could take the original proposal, and they understood the intention was to exclude SLPP PDU forwarding, but they think forwarding of information from UEs to the LMF is required by the SA2 design.

Qualcomm think PDU forwarding between the LMF and a non-connected UE is not feasible.

Lenovo think the “forwarding” involves additional information added to the forwarded message.

Apple think the forwarding is not completely clear, e.g., in what happens with the session IDs.

Agreement:

RAN2 to apply terms of “UE-only Operation” and “Network-based Operation” defined in TS 23.586 by SA2 for SLPP procedures.

[AT123][429][POS] UEs without connection to LMF and SLPP forwarding (Apple)

Scope: Discuss the need for support of communication with UEs that do not have a direct connection to the LMF (e.g., no NAS connection) and potential need for a forwarding mechanism in SLPP.  Sidelink relay is not considered in this discussion.

Intended outcome: Report to CB session

Deadline: Thursday 2023-08-24 2000 UTC

[Discovery related including information and procedures]

Proposal 8: RAN2 to discuss whether define the individual metafield structures separately for different discovery messages (Announcement message, Solicitaion message and Response message).

Proposal 9: RAN2 to discuss following parameters can be included in the metadata in the discovery message:

1)	Supported sidelink positioning methods; (CATT,vivo, Xiaomi, Intel, OPPO, CMCC)

2)	In coverage or not; (CATT, Xiaomi, Philips)

3)	Location; (CATT,vivo, Xiaomi, Spreadtrum, CMCC)

4)	PLMN; (CATT，Xiaomi)

5)	Stationary or movable; (CATT,vivo, Xiaomi)

6)	Location accuracy; (Xiaomi，Philips)

7)	Filter condition: e.g. Requested SL positioning methods, Low Mobility required, In coverage required, LOS path required, Location accuracy requirement, PLMN, required QoS requirement; (Nokia, vivo, Xiaomi, OPPO, Ericsson)

8)	Supported positioning QoS requirement(s); (OPPO, Ericsson)

9)		SLPP support; (Lenovo, CMCC)

[Anchor UE selection]

Proposal 10:	RAN2 to discuss who performs anchor UE selection: LMF/server UE or target UE.

Proposal 11:	Anchor UE selection bases on information from discovery procedure and the positioning capability exchange procedure.

[Server UE selection]

Proposal 12: The SL positioning server UE can be either co-located in a target UE/anchor UE, or operated by a separate UE.

Proposal 13: The following parameters can be considered for SL positioning server UE selection:

1)	Supported roles of UE (SL positioning server UE)

2)	Supported sidelink positioning methods

3)	RSRP

4)	Stationary or movable

[AT123][430][POS] Discovery and selection for sidelink positioning (CATT)

Scope: Discuss further on P8/P9/P10/P11/P12/P13 of R2-2308973 and progress toward agreements.

Intended outcome: Report to CB session

Deadline: Thursday 2023-08-24 2000 UTC

[SLPP Reliable transport]

Proposal 21: SLPP over PC5-U supports reliable transport for groupcast.The same principles of LPP reliable transport (including duplicate detection, acknowledgement and message retransmissions) shall be used for SLPP.

[SL-PRS resources allocation and request]

Proposal 23-1: RAN2 to discuss UE how to request sidelink resource in Scheme 1 for SL-PRS transmission, including allocation from gNB by CG and DG:

- When SL-PRS transmission is triggered for the UE configured with Scheme1 SL-PRS resource allocation by configured grant, the UE sends an RRC message to the gNB for SL-PRS transmission. The UE sends a CG confirmation MAC CE when the DCI for CG type 2 activation/deactivation is successfully received.

- When SL-PRS transmission is triggered for the UE configured with Scheme1 SL-PRS resource allocation by dynamic grant, the UE sends a MAC CE to the gNB for SL-PRS resource request. SR can be triggered for SL-PRS resource allocation request MAC CE when there is no UL-SCH resources to accommodate the MAC CE.

Send LS to RAN1 to confirm the mechanism and detailed content in the MAC CE if required.

Proposal 23-2: RAN2 to discuss how to select a dedicated SL resource pool or a shared SL resource pool and agree the below rules to align with RAN1 agreement:

-	When there are both sidelink data and SL-PRS pending for transmission, select shared resource pool.

-	When there are only SL-PRS pending for transmission while there is no data, prioritize dedicated resource pool

-	For shared resource pool, all the legacy conditions for resource selection/reselection can be reused.

Proposal 23-3: RAN2 to support CBR measurement on both shared and dedicated resource pool for SL-PRS transmission, FFS how to report CBR in line with RAN1 progress.

[SL-PRS priority]

Proposal 22:	RAN2 determines an SL-PRS priority value for SL-PRS, given defined SL positioning QoS (5/8). UE’s own higher layer provides this value of SL-PRS priority to its physical layer. There is no RAN2 impact when the SL-PRS priority value is provided by the SCI from the peer UE triggering the SL-PRS transmission. RAN2 to send the agreement to RAN1 for confirm and inform SA2.

[MAC issues]

Proposal 25: RAN2 discuss if there is a MAC specification impact for the case that only SL-PRS transmission in a shared resource pool when the UE may not have data available for transmission.

Proposal 26: RAN2 further discuss the MAC issues when SL-PRS transmission in share resource pool, including the destination selection and how to handle the SL-PRS priority and SL-SCH priority, in line with RAN1.

Proposal 27: RAN2 further discuss the MAC issues of P12, P13, P14, P15, P17 in R2-2307123.

[AT123][431][POS] Sidelink positioning MAC issues (Huawei)

Scope: Progress discussion on P22/P23-x of R2-2308973 (possible extension to post-meeting discussion), prioritising topics related to SL resource allocation.

Intended outcome: Report to CB session

Deadline: Thursday 2023-08-24 2000 UTC

The following documents will not be individually treated

R2-2307122	Discussion on higher layer aspects for Sidelink Positioning	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2307123	Discussion on lower layer aspects for Sidelink Positioning	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2307185	UE Positioning using Sidelink in OoC	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2307187	Preconfigured Assistance Data for UE Positioning in Hybrid Uu and PC5 scenarios	Fraunhofer IIS, Fraunhofer HHI, Ericsson	discussion

R2-2307232	Discussion of SLPP / LPP signalling procedures 	Nokia Netherlands	discussion	Rel-18

R2-2307241	Discussion of session-less and session-based positioning	Nokia Netherlands	discussion	Rel-18

R2-2307340	SLPP signalling in UE-only sidelink positioning/ranging procedure	MediaTek Inc.	discussion	Rel-18	NR_pos_enh2

R2-2307341	Pathological cases of network-based operation for sidelink positioning	MediaTek Inc.	discussion	Rel-18	NR_pos_enh2	Revised

R2-2307392	Discussion on sidelink positioning	CATT	discussion	Rel-18	NR_pos_enh2

R2-2307426	Discussion on sidelink positioning	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2307507	Discussion on SL positioning	Xiaomi	discussion	Rel-18

R2-2307661	Further considerations on sidelink positioning	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2307778	SLPP design for session aspects 	Samsung Electronics Romania	discussion

R2-2307823	SL positioning procedures	Apple	discussion	NR_pos_enh2

R2-2308052	Further discussion on sidelink positioning	OPPO	discussion	Rel-18	NR_pos_enh2

R2-2308125	Discussion on sidelink positioning	Spreadtrum Communications	discussion	Rel-18

R2-2308138	Discussion on sidelink positioning	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308152	Considerations on sidelink positioning resources	Sony	discussion	Rel-18	FS_NR_pos_enh2

R2-2308276	Discussion on SL Positioning	Lenovo	discussion	Rel-18

R2-2308284	Discussion on sidelink positioning	ROBERT BOSCH GmbH	discussion	Rel-18

R2-2308316	Considerations on Sidelink positioning	CMCC	discussion	Rel-18	NR_pos_enh2

R2-2308384	Discussion on sidelink positioning	InterDigital, Inc.	discussion	Rel-18	NR_pos_enh2

R2-2308396	Sidelink Positioning Protocol (SLPP) Signaling and Procedures	Qualcomm Incorporated	discussion

R2-2308416	Pathological cases of network-based operation for sidelink positioning	MediaTek Inc., CATT	discussion	Rel-18	NR_pos_enh2	R2-2307341

R2-2308480	Sidelink positioning	Ericsson	discussion	Rel-18

R2-2308557	Discussion of resource allocation aspects	Nokia Netherlands	discussion

R2-2308595	Discussion on higher layer aspects for sidelink positioning	LG Electronics Inc.	discussion	Rel-18

R2-2308600	Discussion on lower layer aspects for sidelink positioning	LG Electronics Inc.	discussion	Rel-18

R2-2308657	Discussion on priority value for SL-PRS	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_pos_enh2

R2-2308800	 Further Discussions on Sidelink Positioning & Ranging	CEWiT	discussion

R2-2308884	Discussion on Anchor UE discovery and selection in sidelink positioning	KT Corp.	discussion	Rel-18	NR_pos_enh2

R2-2308908	On the selection of Anchor UEs for Sidelink Positioning	Philips International B.V.	discussion	Rel-18	NR_pos_enh2

R2-2308935	On the support of UE-only SL positioning in in-coverage and partial coverage scenarios 	Philips International B.V.	discussion	Rel-18	NR_pos_enh2

### 7.2.3	RAT-dependent integrity

Error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods.

R2-2308397	Integrity of NR Positioning Technologies	Qualcomm Incorporated	discussion

Proposal 1:	The DNU flags are provided per TRP and per error contribution (e.g., TRP location, RTD, beam information, etc.) in a new IE NR-Integrity-ServiceAlert.

Proposal 2:	DNU flags for TRP/UE positioning measurements are not needed.

Proposal 3:	Analogous to GNSS, the mean and standard deviation of rate error bounds can optionally also be provided for the NR positioning error sources.

Proposal 4:	The 'Integrity Correlation Times', defining the minimum time interval beyond which two sets of assistance data parameters for a given error can be considered to be independent from one another, can optionally be provided for the integrity assistance data.

Proposal 5: 	Confirm the Working Assumption: "It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN."

Proposal 6:	The beam related information (Beam Bore-Sight Direction/Beam Antenna Information) are error sources for DL-AoD positioning.

R2-2307393	Discussion on RAT-Dependent integrity	CATT	discussion	Rel-18	NR_pos_enh2

R2-2307427	Remaining issues of RAT-dependent integrity	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2307664	Further considerations on RAT dependent integrity	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2307999	Discussion on RAT-dependent  integrity	Lenovo	discussion	Rel-18

R2-2308050	Consideration on RAT-dependent positioning integrity	OPPO	discussion	Rel-18	NR_pos_enh2

R2-2308136	Discussion on RAT-dependent methods positioning integrity	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308260	Discussion on RAT-dependent positioning integrity	Xiaomi	discussion

R2-2308482	On RAT-dependent positioning Integrity	Ericsson	discussion	Rel-18

R2-2308616	Discussion on RAT dependent integrity	InterDigital, Inc.	discussion	Rel-18

### 7.2.4	LPHAP

Enhancements for enabling LPHAP use case 6 (TS 22.104), including extending eDRX cycle (coordinated with RedCap WI); SRS configuration enhancements based on validity area for UEs in RRC_INACTIVE; DL-PRS measurements in RRC_IDLE and reporting in RRC_CONNECTED; and alignment between eDRX and PRS configurations.

Including report of [Post122][401][POS] SRS configuration and activation in LPHAP (CATT)

Email discussion summary

R2-2308812	Report of [Post122][401][POS] SRS configuration and activation in LPHAP (CATT)	CATT	discussion	Rel-18	NR_pos_enh2

Easy agreed:

SRS configuration request

Proposal 1: When the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request. The SRS configuration request is sent in the RRC message RRCResumeRequest (18/18). RAN2 to confirm a new resume cause is introduced for the use case.

SRS activation/deactivation

Proposal 2-1: Periodic SRS is supported to be configured with validity area (18/18). And activation/deactivation is not required for periodic SRS (14/18).

Proposal 2-3: Aperiodic SRS is not supported to be configured with validity area (18/18). Send an LS to RAN1 to confirm this conclusion.

Preconfigured SRSs

Proposal 5: RAN2 no further consider providing pre-configured SRS via system information in Rel-18 (10/18).

Proposal 6: For the activation indication and/or request for preconfigured SRSs, RRCResumeRequest message is used (16/18). And 1 bit indication in the RRCResumeRequest is introduced for this use (12/16).

Proposal 7: The resume cause introduced for the SRS configuration request can be reused for the activation indication of the pre-configuration SRS (10/18).

Proposal 9: Sending the activation indication and/or requesting for preconfigured SRS using Msg1 is not supported (15/18).

Further discussed:

Configured UE-specific SRS

Proposal 2-2: RAN2 to further discuss whether semi-persistent SRS is supported to be configured with validity area (11/18), with considering the power consumption and whether legacy activation/deactivation mechanism can be reused for semi-persistent SRS.

Proposal 3: If semi-persistent SRS is supported to be configured with validity area, RAN2 agree to reuse legacy mechanism to deactivate the SP SRS (12/15).

Proposal 4: RAN2 postpone the discussion if there is an issue of blind monitoring by the network when UE sends periodic SRS.

Preconfigured SRSs

Proposal 8: RAN2 to clarify the concept of preconfigured SRS, including

-	For the concept of pre-configured SRS, what SRS type can be supported? e.g., periodic SRS and SP SRS.

-	Whether there should be only one SRS configuration for one validity area?

-	Whether there is a need to support multiple validity areas for the same cell?

Agenda item summary (excluding items related to the email discussion)

R2-2308959	Summary for 7.2.4 LPHAP excluding SRS configuration & activation part	OPPO	discussion	Rel-18	NR_pos_enh2

The following proposals are to be agreed (potentially long-time discussion is needed for proposal 2):

Proposal 1: RAN2 to agree that alignment of PRS to fixed (e)DRX should be adopted to ensure the alignment between PRS and (e)DRX.

Proposal 2: RAN2 to agree that both of UE-initialized and LMF-initialized on-demand PRS request procedure are adopted for the alignment from the PRS configuration to the (e)DRX configuration.

Proposal 6: RAN2 to agree that the PRS periodicity is extended to be larger than 10240ms to suit the eDRX cycle value for eDRX paging cycle in RRC_INACTIVE and/or RRC_IDLE. LS to RAN1 for confirmation of the feasibility.

Proposal 8: RAN2 to agree to support UE to utilize the positioning assistance data through posSIB or pre-configured assistance data in RRC_CONNECTED when UE is to perform positioning in RRC_IDLE.

Proposal 12: RAN2 to agree that the following criteria needs to be defined for the start/re-start of the area-specific TA timer:

	Reception of RRCRlease message containing the SRS configuration

Proposal 13: RAN2 to agree that following criteria needs to be defined for the stop of the area-specific TA timer (FFS other conditions):

	Reception of RRCResume message

	Reception of RRCSetup message

The following proposal is to be discussed:

Proposal 3: RAN2 to discuss whether following IEs are needed to be included or enhanced in the UE-initiated on-demand request message:

	to include the demanded PRS time offset associated with each requested PRS periodicity, to align with PO location>

	to include the demanded PRS time duration associated with each requested PRS periodicity, to align with PO location.

	to include more than one of the demanded PRS periodicities per PFL, to align with PO locations within  and outside the PTW, respectively.

	to include requested DL-PRS activation periodicity/start offset/duration, to align with periodic PTW location.

	to include UE-related (e)DRX information and LPHAP indication

Proposal 4: RAN2 to discuss whether a LS needs to be sent to RAN3 to trigger them for discussion of how to align the PRS with the DRX of serving cell and/or neighbour cell.

Proposal 5: RAN2 to discuss whether it is needed to align the LPHAP UEs waking up time and if the answer is yes, FFS how.

Proposal 7: RAN2 to discuss that whether or not the UE shall only enter the sleep phase after the UE has received some form of feedback that the NW has obtained the UE position with the required accuracy.  FFS: How the feedback is informed, via LPP or MAC-CE.

Proposal 9: RAN2 to discuss whether or not triggered event(s) should be defined for the positioning in the RRC_IDLE state for the UE to transit to RRC_Connected state for the measurement reporting. FFS what kind of triggered event should be introduced.

Proposal 10: RAN2 to discuss whether to introduce following two UE capabilities in the LPP and RRC spec, after more conclusions are made for SRS with validity area and SRS pre-configuration:

	supporting SRS with validity area in RRC_INACTIVE

	supporting SRS pre-configuration in RRC_INACTIVE

Proposal 11: RAN2 to discuss which option(s) to be adopted as the criteria to release the pre-configured SRS configuration:

	Network-initiated message:

	New TA timer to be introduced controlling how long the SRS resource is reserved for the UE within the validity area

	Area-specific TA timer expiration

	Reselection to other cell out of the SRS validity area

The following documents will not be individually treated

R2-2307121	Discussion on LPHAP	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2307186	Enhancements for supporting LPHAP	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2307394	Discussion on SRS configuration with validity area and alignment between PRS and (e)DRX	CATT	discussion	Rel-18	NR_pos_enh2

R2-2307428	Discussion on solution of LPHAP	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2307665	Further considerations on LPHAP	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2307824	Alignment between DRX and PRS	Apple	discussion	NR_pos_enh2

R2-2308000	Discussion on low power high accuracy positioning	Lenovo	discussion	Rel-18

R2-2308051	Discussion on LPHAP enhancement	OPPO	discussion	Rel-18	NR_pos_enh2

R2-2308126	Discussion on LPHAP	Spreadtrum Communications	discussion	Rel-18

R2-2308135	Discussion on LPHAP	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308153	Considerations on Low Power High Accuracy Positioning	Sony	discussion	Rel-18	FS_NR_pos_enh2

R2-2308261	Discussion on LPHA positioning	Xiaomi	discussion

R2-2308317	Further considerations on LPHAP	CMCC	discussion	Rel-18	NR_pos_enh2

R2-2308398	Enhancements for LPHAP	Qualcomm Incorporated	discussion

R2-2308481	Discussion on Low Power High Accuracy Positioning	Ericsson	discussion	Rel-18

R2-2308618	Discussion on LPHAP	InterDigital, Inc.	discussion	Rel-18

R2-2308693	Discussion on alignment between (e)DRX and PRS	Samsung	discussion	Rel-18	NR_pos_enh2

R2-2308694	Discussion on SRS configuration in RRC_INACTIVE	Samsung	discussion	Rel-18	NR_pos_enh2

### 7.2.5	RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning

RAN1 led objectives that may require progress in RAN1 before RAN2 can take decisions.

R2-2308001	Discussion on RedCap positioning, carrier phase positioning and PRS/SRS bandwidth aggregation	Lenovo	discussion	Rel-18

Redcap Positioning:

Proposal 1: LMF may request gNB to provide PRS FH configuration and then indicates the determined PRS FH configuration to UE by LPP ProvideAssistanceData message.

Proposal 2: LMF may request serving gNB to provide the SRS FH configuration with SRS configuration to UE and feedback the SRS FH configuration to LMF.

Proposal 3: UE performs hop switch or BWP switch autonomously according to the configuration from network.

Carrier phase positioning:

Proposal 4: UE indicates the support of carrier phase positioning in LPP ProvideCapabilities message, and further includes the support of RSCP or RSCPD measurement.

Proposal 5: The legacy LPP RequestLocationInformation and ProvideLocationInformation message for time-based positioning are enhanced to carry the CPP measurement configuration and CPP measurement reporting.

Proposal 6: For double differential CPP measurement, LMF indicates the time period of SRS transmission to the serving gNB of UE including target UE and PRU by enhancing current requested UL-SRS transmission characteristics information for UL positioning.

Proposal 7: For double differential CPP measurement, LMF indicates the time period for PRS measurement to target UE and PRU by LPP provide assistance data message.

PRS/SRS bandwidth aggregation:

Proposal 8: For DL PRS bandwidth aggregation across PFLs, RAN2 to consider following signalling enhancements:

−	Include the joint measurement indication and the aggregated PRSs resource PRS sets IDs across PFLs in LPP RequestLocationInformation message.

−	Include the PFL aggregation indication and the aggregated measurement results with PRS resource sets ID in LPP ProvideLocationInformation message.

Proposal 9: For UL SRS bandwidth aggregation across two or three carriers, the signalling enhancement between gNB and LMF to support the bandwidth aggregation indication should be confirmed with RAN3.

P2/P3/P4 provided for reference in discussion of the previous document (cf. P6/P7 above)

R2-2307395	Discussion on carrier phase positioning, bandwidth aggregation for positioning and Redcap positioning	CATT	discussion	Rel-18	NR_pos_enh2

Proposal 2: To enable simultaneous transmission of UL SRS and measurement of target UE and a PRU which are agreed in RAN1, there are impacts on RRC and NRPPa, including:

-RRC: gNB configure the transmission of the UL SRS resources within indicated time window(s);

-NRPPa: LMF request serving gNB of a UE to configure the transmission time window(s). LMF request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s).

Proposal 3: To enable simultaneous measurements on same DL PRS by a target UE and a PRU which are agreed in RAN1, there are impacts on LPP: LMF request the UEs to perform measurements within indicated time window(s).

Proposal 4: RAN2 to provide assistance data which is DL carrier phase measurement reported by a PRU to a target UE for UE-based carrier phase positioning which is agreed in RAN1.

R2-2307429	RAN2-related issues about bandwidth aggregation	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2307455	Discussion on RAN1 led positioning topics	Huawei, HiSilicon	discussion

R2-2307666	Considerations on other RAN1 led items	Intel Corporation	discussion	Rel-18	NR_pos_enh2

R2-2307827	On-demand PRS with bandwidth aggregation	Apple	discussion	NR_pos_enh2

R2-2308137	Discussion on BW aggregation and RedCap positioning	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2308174	Discussion on Frequency hopping for Positioning for RedCap Ues	Sony	discussion	Rel-18	FS_NR_pos_enh2

R2-2308262	Discussion on RedCap positioning, carrier phase positioning and bandwidth aggregation for positioning	Xiaomi	discussion

R2-2308399	Configuration Enhancements for DL-PRS Aggregation	Qualcomm Incorporated	discussion

R2-2308483	Discussion based upon RAN1 agreements on CPP, RedCap, Bandwidth aggregation	Ericsson	discussion	Rel-18

R2-2308619	Discussion on positioning for RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning	InterDigital, Inc.	discussion	Rel-18

R2-2308761	Assessment of impact of carrier phase positioning on higher layer protocols	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_pos_enh2-Core]

## 7.9	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

Time budget: 1.5 TU

Tdoc Limitation: 4 tdocs

### 7.9.1	Organizational

Including incoming LSs and rapporteur inputs.

Incoming LSs with “take into account” action

R2-2307057	Reply LS to SA2 on authorization for multi-path Scenario 2 (S2-2307707; contact: LGE)	SA2	LS in	Rel-18	NR_SL_relay_enh, 5G_ProSe_Ph2	To:RAN2	Cc:RAN3

R2-2307072	Reply LS on security for L2 UE-to-UE relay (S3-233323; contact: Lenovo)	SA3	LS in	Rel-18	NR_SL_relay_enh, FS_5G_ProSe_Ph2	To:RAN2

Other incoming LS

R2-2307055	Reply LS on 5G ProSe Layer-2 UE-to-UE Relay QoS enforcement (S2-2305915; contact: Qualcomm)	SA2	LS in	Rel-18	5G_ProSe_Ph2	To:RAN2

Running CRs and related documents

R2-2307235	Running CR of TS 38.351 for SL Relay enhancement	OPPO	draftCR	Rel-18	38.351	17.5.0	B	NR_SL_relay_enh-Core

[AT123][411][Relay] Rel-18 SRAP CR (OPPO)

Scope: Collect comments on the CR in R2-2307235 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307546	Introduction of NR sidelink U2U relay	vivo	draftCR	Rel-18	38.331	17.5.0	B	NR_SL_relay_enh-Core

[AT123][412][Relay] Rel-18 RRC CR on U2U relay (vivo)

Scope: Collect comments on the CR in R2-2307546 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307720	38.322 running CR for enhanced NR sidelink relay	Xiaomi	draftCR	Rel-18	38.322	17.3.0	B	NR_SL_relay_enh-Core

[AT123][413][Relay] Rel-18 relay RLC CR (Xiaomi)

Scope: Collect comments on the CR in R2-2307720 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307854	Draft Running CR 38.321	Apple	draftCR	Rel-18	38.321	17.5.0	B	NR_SL_relay_enh-Core

[AT123][414][Relay] Rel-18 relay MAC CR (Apple)

Scope: Collect comments on the CR in R2-2307854 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2307920	Draft running CR 38.300	LG Electronics Inc.	draftCR	Rel-18	38.300	17.5.0	B	NR_SL_relay_enh-Core

[AT123][415][Relay] Rel-18 relay stage 2 CR (LG)

Scope: Collect comments on the CR in R2-2307920 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308203	RRC running CR for Rel-18 multi-path support	Huawei, HiSilicon	draftCR	Rel-18	38.331	17.5.0	B	NR_SL_relay_enh-Core

R2-2308204	Considerations on Multi-path RRC running CR	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

[AT123][416][Relay] Rel-18 RRC CR on multi-path relay (Huawei)

Scope: Collect comments on the CR in R2-2308203 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308559	Introduction of Rel-18 support for SL Relay Enhancements	Ericsson España S.A.	draftCR	Rel-18	38.304	17.5.0	B	NR_SL_relay_enh

[AT123][417][Relay] Rel-18 relay idle mode CR (Ericsson)

Scope: Collect comments on the CR in R2-2308559 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

R2-2308687	Introduction of Rel-18 SL relay service continuity	MediaTek, Inc	CR	Rel-18	38.331	17.5.0	4277	-	B	NR_SL_relay_enh-Core

[AT123][418][Relay] Rel-18 RRC CR on relay service continuity (MediaTek)

Scope: Collect comments on the CR in R2-2308687 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

### 7.9.2	UE-to-UE relay

Single-hop Layer-2 and Layer-3 UE-to-UE relay for unicast.  Including common L2/L3 functionality comprising relay discovery and (re)selection and L2-specific functionality including adaptation layer design, control plane procedures, and QoS handling if needed.

Agenda item summary

R2-2308956	Summary of UE-to-UE relay	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

[AT123][401][Relay] Summary proposals on UE-to-UE relay (Qualcomm)

Scope: Discuss the proposals in R2-2308956 and progress towards agreements.

Intended outcome: Report to Wednesday online session in R2-2309101

Deadline: Tuesday 2023-08-22 2000 UTC

R2-2309101	(Report of [AT123][4101])	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

The following documents will not be individually treated

R2-2307233	Discussion on U2U relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307386	Discussion on remaining issue of U2U relay	NEC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307402	Discussion on the adaptation layer	Fujitsu	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307446	Discussion on U2U relay	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307547	Remaining issues on U2U discovery and relay (re)selection	vivo	discussion

R2-2307548	Discussion on the remaining issues of L2 U2U relaying	vivo	discussion

R2-2307551	Discussion on U2U Relay	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307641	U2U Relay selection reselection, SRAP design	Beijing Xiaomi Mobile Software	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307655	Discussion on using short ID in U2U relaying	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-18	NR_SL_relay_enh

R2-2307716	Discussion on U2U relay	TCL	discussion

R2-2307732	QoS and bearer configuration for L2 U2U relaying	Samsung	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307742	Common part and Layer-2 specific part on U2U Relay	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2307743	gNB involvement and capability on U2U relay	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2307750	Considerations for U2U L2 relay operations 	Kyocera	discussion	Rel-18

R2-2307855	Discussion on remaining issues on UE-to-UE Relay	Apple	discussion	Rel-18

R2-2307932	Control plane procedure for U2U relay	LG Electronics Inc.	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307944	Further discussion on L2 U2U relay	China Telecom	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307989	Discussion on L2 U2U relay	Lenovo	discussion	Rel-18

R2-2308100	Discussion on U2U Relay discovery and (re)selection	ZTE, Sanechips	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308101	Discussion on U2U relay L2-specific functionality	ZTE, Sanechips	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308104	SRAP design for U2U Sidelink Relay	Samsung	discussion

R2-2308119	Discussion on UE-to-UE Relay	Spreadtrum Communications	discussion	Rel-18

R2-2308160	UE-to-UE relay (re)selection	Sony	discussion	Rel-18	NR_SL_relay_enh

R2-2308161	Discussion on DRX for Sidelink UE to UE Relay	Sony	discussion	Rel-18	NR_SL_relay_enh

R2-2308205	Discussion on UE-to-UE relay	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308220	Remaining issues for UE-to-UE relay	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308321	Discussion on U2U relay	CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308368	Considerations on U2U relay (re)selection and Local ID assignment	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_SL_relay_enh-Core	R2-2305590

R2-2308380	Open Issues on Discovery, Relay Selection, and SRAP for UE to UE Relays	InterDigital	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308381	QoS and Configuration for L2 UE-to-UE Relays	InterDigital	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308469	Discussion on Relay (re)selection and Discovery	Ericsson España S.A.	discussion	Rel-18

R2-2308470	Control Plane Procedures for Layer 2 UE-to-UE Relays	Ericsson España S.A.	discussion	Rel-18

R2-2308611	Discussion on Adaptation Layer for L2 U2U Relay	ETRI	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308721	Discussion on E2E PC5-RRC procedures	ASUSTeK	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308722	Discussion on AS layer configuration for L2 U2U Relay	ASUSTeK	discussion	Rel-18	NR_SL_relay_enh-Core

### 7.9.3	Service continuity enhancements for L2 UE-to-network relay

Inter-gNB direct/indirect path switching; intra-gNB indirect/indirect path switching; and inter-gNB indirect/indirect path switching, to be supported by reuse of solutions for the other scenarios.

R2-2307945	Discussion on the procedure for intra-gNB indirect to indirect path switch	China Telecom	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307226	Discussion on service continuity enhancement	Xiaomi	discussion

R2-2307281	SL Relay service continuity considerations	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307549	Remaining issues on service continuity enhancement for L2 U2N relay	vivo	discussion

R2-2307552	Further Consideration on Service Continuity Enhancements	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307733	Discussion on measurement quantity configuration	Samsung	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307744	Proposal on additional enhancements for service continuity	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2307856	Discussion on path switching to IDLE/INACTIVE relay	Apple	discussion	Rel-18

R2-2307940	Discussion on Remaining Issues of Service Continuity	NEC Corporation	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307990	Discussion on enhanced path switching	Lenovo	discussion	Rel-18

R2-2308102	Further discussion on service continuity for SL relay	ZTE, Sanechips	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308162	Service continuity enhancements for UE sidelink relay	Sony	discussion	Rel-18	NR_SL_relay_enh

R2-2308221	Remaining issues for U2N path switching	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308322	Discussion on service continuity	CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308471	Discussion on Inter-gNB Service Continuity	Ericsson España S.A.	discussion	Rel-18

R2-2308584	Discussion on Service Continuity	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

### 7.9.4	Multi-path relaying

Mechanisms to support multi-path scenarios where a UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal).  This agenda item will include a rapporteur contribution summarising open issues from RAN2#121 (invited contribution not counted against the tdoc limit).

Including report of [Post122][403][Relay] Procedures for multi-path relay (LG)

Email discussion summary

R2-2307973	Report of [AT121bis-e][419][Relay] Remaining high-priority proposals on multi-path (LG)	LG Electronics France	report	Rel-18	NR_SL_relay_enh

- Revised in R2-2308950 (title correction)

R2-2308950	Report of [Post122][403][Relay] Procedures for multi-path relay (LG)	LG Electronics France	report	Rel-18	NR_SL_relay_enh

Proposal 1: Consider the above updated procedures (except direct/indirect path change in Scenario 2) and FFS issues as a baseline and future work.

Proposal 2: RAN2 is requested to include at least the following updated procedures in the running CR to 38.300, possibly with corrections and additional inputs.

	For Scenario 1

	Direct path addition in section 1.1

	Indirect path addition in section 1.2

	For Scenario 2

	Indirect path addition in section 2.1

Proposal 3: RAN2 is requested to discuss whether to include the following updated procedures in the running CR to 38.300, possibly with corrections and additional inputs.

	For Scenario 1

	Direct path change in section 1.3

	Indirect path change in section 1.4

	Direct path release in section 1.7

	Indirect path release in section 1.8

	For Scenario 2

	Indirect path release in section 2.6

Agenda item summary (excluding items related to the email discussion)

R2-2308949	Offline 402 on A.I 7.9.4 Multi-path relaying	Nokia	discussion	NR_SL_relay_enh-Core

Proposals for agreement

Proposal 1-1: RAN2 confirm the working assumption below:

For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path.

For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. And UE switches the primary path to the indirect path for reporting after direct path failure, and this switching is limited to the case where duplication is not configured as in legacy.

For Scenario 2, leave it to relay and remote UE implementation on how to trigger the RRC_IDLE/RRC_INACTIVE target relay UE to initiate RRC connection establishment procedure.

Proposal 2.1: For scenario 1, non-split SRB on the indirect path is not supported.

Proposal 4.1.1: T304 timer is reused for the direct path addition/change.

Proposal 4.1.2: Start/stop condition of T304 for direct path addition/change will be discussed after the signalling procedure for direct path addition/change is concluded.

Proposal 4.2.1: The T420 timer is reused for the indirect path addition/change.

Proposal 4.2.2: The T420 timer starts when RRCReconfiguration message for the indirect path addition/change procedure is received.

Proposal 6.1: Support of case G in scenario 2 is deprioritized.

Proposal 7.4: In packet duplication, the PDCP entity shall not indicate to the Uu RLC entity to discard the PDCP PDU when the PC5 RLC entity acknowledges the transmission of the PDCP PDU. FFS for the case where Uu RLC entity acknowledges the transmission of a PDCP PDU.

Proposal 8: RAN2 deprioritize the discussion on path/flow control.

Proposals for discussion

Easily agreeable:

Proposal 2.2: RAN2 discuss whether different bearer type can be configured for SRB1 and SRB2.

Proposal 2.3: RAN2 discuss whether the primary path of the split DRB can be set to either the direct path or the indirect path, or is fixed to the direct path as split SRB.

Proposal 4.3.2: Discuss whether a new T420-like timer is defined or the existing T420 timer is reused for the indirect path addition/release.

Proposal 7.3: RAN2 discuss whether LCH-to-carrier restriction is not required for packet duplication over sidelink multi-path.

Need more discussion:

Proposal 3.1: RAN2 discuss whether additional information such as measurement result, OoC indication, buffer information, can be included in MCGFailureInformation when reporting the direct path failure.

Proposal 3.2: For scenario 1 and 2, RAN2 discuss which message is used for report of indirect path failure from 1) MCGFailureInformation, 2) SidelinkUEInformationNR, and 3) a new message.

Proposal 3.3.1: RAN2 discuss whether failure detection on the existing path while additional path addition is an issue to be resolved. FFS how to resolve it if RAN2 agree to resolve it.

Proposal 3.3.2: RAN2 discuss whether the gNB can configure a condition not to report the RLF.

Proposal 4.1.3: For the expiry of the new T304-like timer, RAN2 discuss the followings:

-	In which condition the UE reports the failure of the direct path addition/change

-	In which condition the UE reverts to the prior path operation

-	In which condition the UE initiates RRC connection re-establishment

Proposal 4.2.3: For the stop of the T420 timer, RAN2 discuss the following options:

-	Option 1. Reuse T420 condition, i.e., upon successful sending of RRCReconfigurationComplete message

-	Option 2. When PC5-RRC connection establishment is completed

-	Option 3. When relay UE is successfully connected to the gNB

-	Option 4. When PC5-RRC connection establishment completes, and relay UE is successfully connected to the gNB

Proposal 4.2.4: For the expiry of the T420 timer, Ran2 discuss the followings:

-	In which condition the UE reports the failure of the indirect path addition/change

-	Whether or if yes, in which condition the UE reverts to the prior path operation

-	In which condition the UE initiates RRC connection re-establishment

-	Whether additional information needs to be reported to the gNB

Proposal 4.3: RAN2 discuss whether the existing measurement events are sufficient for path addition/change/removal in multi-path operation. If not, discuss what events need to be introduced.

Proposal 5: When the remote UE receives notification message indication relay UE’s handover, RAN2 discuss two options:

1)	to rely on network to release configuration of relay UE at remote UE before relay UE handover

2)	rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover

Proposal 6.2: RAN2 discuss whether the remote UE reports the RRC_IDLE and RRC_INACTIVE relay UE’s ID, and if so, which ID is used.

Proposal 7.1: RAN2 discuss the followings:

1)	Whether CA duplication is applied to the direct path of the remote UE. If yes, what is the maximum number of RLC entities over the direct path of the remote UE?

2)	Whether CA duplication is applied to the Uu link of the relay UE. If yes, FFS any impact on the specification.

Proposal 7.2: RAN2 discuss how the duplication is activated/deactivated to a certain RLC entity when the remote UE receives the Duplication A/D MAC CE or Duplication RLC A/D MAC CE using a single MAC entity.

Proposal 9: RAN2 discuss if any issue needs to be discussed/resolved for BSR operation by focusing on essential issues from operation perspective than enhancement.

The following documents will not be individually treated

R2-2307093	Discussion on multi-path SL relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307182	Discussion on Multi-path relaying	Lenovo	discussion	NR_SL_relay_enh-Core

R2-2307227	Discussion on multi-path	Xiaomi	discussion

R2-2307363	Discussion on non-split SRB	OPPO, Samsung, China Telecom, Huawei, HiSilicon, Ericsson, vivo, CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307387	Discussion on remaining issue of multi-path relay	NEC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307403	Discussions on multi-path	Fujitsu	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307550	Remaining Issues for Multi-path Scenario 1 2	vivo	discussion

R2-2307553	Discussion on Multi-path	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307656	Throughput Enhancement in U2N Relaying	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-18	NR_SL_relay_enh

R2-2307719	Discussion on multi-path scenario 1	III	discussion	NR_SL_relay_enh-Core

R2-2307745	Open issues on multi-path relay for scenario 1 and scenario 2	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2307751	Considerations for multipath relay operations for Scenario 1 	Kyocera	discussion	Rel-18

R2-2307857	Discussion on Multi-path Relay	Apple	discussion	Rel-18

R2-2307941	Discussion on UP Issues of Multi-path relay	NEC Corporation	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307946	Discussion on remaining issues of multi-path relaying in Scenario 1	China Telecom	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307947	Discussion on remaining issues of multi-path relaying in Scenario 2	China Telecom	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307991	Procedure for second path addition	Lenovo	discussion	Rel-18

R2-2308103	Further discussion on the support of multi-path relaying	ZTE, Sanechips	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308120	Discussion on multi-path relaying	Spreadtrum Communications	discussion	Rel-18

R2-2308163	Multi-path relaying discussion	Sony	discussion	Rel-18	NR_SL_relay_enh

R2-2308206	Remaining issues on multi-path operation	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308222	Remaining issues for multi-path relay	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308224	Discussion on remaining issues on multiple path for sidelink relay	Samsung	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308323	Discussion on multi-path scenario 1	CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308324	Considerations on multi-path scenario 2	CMCC	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308382	User Plane Aspects for Multipath	InterDigital	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308383	Control Plane Aspects for Multipath	InterDigital	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308472	Discussion on Multipath Relays	Ericsson España S.A.	discussion	Rel-18

R2-2308723	BSR reporting for Multi-path Scenario 2	ASUSTeK	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308724	Discussion on duplicate PDCP PDU discarding for Multi-path transmission Scenario 1	ASUSTeK	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308749	On Remaining issues on multipath SL relay	Nokia, Nokia Shanghai Bell	discussion	NR_SL_relay_enh-Core

### 7.9.5	DRX

Study the gains and, if needed, specify signalling between gNB and relay UE in sidelink mode 2 to assist the determination of the sidelink DRX configuration used for remote UE.  This agenda item will be handled at lower priority.

R2-2307228	Discussion on SL DRX in U2N relay	Xiaomi	discussion

R2-2307234	Discussion on DRX for L2 U2N relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307554	Discussion on DRX for L2 U2N Relay	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2307858	Discussion on SL DRX for L2 UE-to-NW relay	Apple	discussion	Rel-18	R2-2305065

R2-2308207	Discussion on sidelink DRX for L2 U2N relay	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2308369	Considerations on DRX and paging for sidelink relay	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_SL_relay_enh-Core	R2-2305592

## 7.24	NR TEI18

Specific items may be allocated to a breakout session for treatment.

Time budget: 1 TU

### 7.24.1	TEI proposals by Other Groups

Items initiated by other groups that is/has been communicated by LS, where the other group indicate this is TEI18. (Specific other-group-WIs should use the R18 Other Agenda Item below).

R2-2307009	LS on 1-symbol PRS (R1-2306212; contact: ZTE)	RAN1	LS in	Rel-18	TEI18	To:RAN2	Cc:RAN3, RAN4

R2-2308140	Introduction of 1-symbol PRS in 38.331[1symbol_PRS]	ZTE Corporation	CR	Rel-18	38.331	17.5.0	4014	3	B	TEI18	R2-2306793

- Agreed in principle with alignment of the terminology to the ASN.1 names

R2-2308141	Introduction of 1-symbol PRS in 37.355[1symbol_PRS]	ZTE Corporation	CR	Rel-18	37.355	17.5.0	0437	3	B	TEI18	R2-2306794

- Agreed in principle (to be merged into a single LPP CR)

R2-2308142	Introduction of UE capability of 1-symbol PRS in 37.355[1symbol_PRS]	ZTE Corporation	CR	Rel-18	37.355	17.5.0	0453	2	B	TEI18	R2-2306795

- Agreed in principle (to be merged into a single LPP CR)

R2-2308143	Introduction of UE capability of 1-symbol PRS in 38.331[1symbol_PRS]	ZTE Corporation	CR	Rel-18	38.331	17.5.0	4128	2	B	TEI18	R2-2306796

- Endorsed to be merged into mega CR

R2-2308144	Introduction of UE capability of 1-symbol PRS in 38.306[1symbol_PRS]	ZTE Corporation	CR	Rel-18	38.306	17.5.0	0923	2	B	TEI18	R2-2306797

- Endorsed to be merged into mega CR

Discussion:

Huawei are fine with the proposals, but they wonder if the LPP CR needs to be split.

ZTE think separate CRs for functionality and capabilities are cleaner.

Ericsson think we should have one LPP CR.  Qualcomm agree and think the CR can be self-contained.

Lenovo think a merged CR is OK.  On the RRC CR, they suggest replacing “Type C” and “Type C+D” with the ASN.1 names.

CATT think there is a coversheet issue: The CR numbers for the other specs should be included in the coversheets.  Qualcomm think there is no functional interaction between them and we do not need the cross-reference.

### 7.24.2	TEI proposals by RAN2

Items initiated in RAN2.

Tdoc limitation: 1 tdoc, limitation only applicable for non-previously-agreed-to-be-considered TEI proposals.

Relay: paging cause forwarding [previously seen]

R2-2307176	Paging Cause forwarding	Samsung Electronics Co., Ltd	discussion	Rel-18	TEI18

R2-2307694	Discussion on MUSIM paging cause forwarding	vivo	discussion	Rel-18

Relay: emergency cause value [previously seen]

R2-2307237	Discussion on emergency cause value for SL Relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core, TEI18

Relay: voice/video support [previously seen]

R2-2308932	Considerations on voice and video support for Relays	Philips International B.V., FirstNet, InterDigital, KPN, TNO, 	discussion	Rel-18	R2-2306516

Positioning: multiple QoS [previously seen]

R2-2307342	Multiple QoS for positioning	MediaTek Inc.	discussion	Rel-18	TEI18

- Noted

R2-2308830	Introduction of ‘multiple QoS’ class in positioning 	Samsung Electronics Romania	discussion

- Noted

Proposal 1. When LMF receives the service request with multipleQoS class from LCS client, multiple set of (H-/V-) accuracy values per QoS level same as LMF received from LCS client can be indicated to LPP location information request procedure.

Proposal 2. If UE receives LPP Request Location Information including multiple QoS, UE should evaluate whether the obtained location estimate fulfils the accompanied accuracy requirements for all the given QoS.

Proposal 3. Once the measured result/ location estimate fulfils any accuracy requirements among indicated ones, UE should report the measurement result/location estimate with the indication of the highest preferred accuracy values among fulfilled ones.

Proposal 4. This proposed operation is only applicable to the UE-based positioning.

Discussion:

Qualcomm indicate they checked the SA2 specs and confirmed that the multiple QoS is supported only for deferred MT-LR, which does not affect LPP, so they do not see interaction of the feature with LPP.  They understand that the network will try multiple times, typically with different positioning methods.

vivo agree with Qualcomm; the proposal indicates that it derives from SA2, but SA2 specified that the multiple QoS feature is realised through multiple location requests.

MediaTek are concerned that there could be a “lazy UE” problem where the UE only meets the loosest QoS, and agree that SA2 did not put a UE requirement.

Samsung agree that there is no explicit requirement from SA2, but they see that the proposal can reduce LPP signalling overhead.  On the “lazy UE” problem, they understand that the UE should make a best effort in any positioning operation.

Apple agree with MediaTek that the feature is not testable; they think the value is not clear.

OPPO agree with Qualcomm that multiple QoS normally means the LMF will try different positioning methods.  On the “lazy UE” problem, they agree that the problem exists, but they think the MediaTek proposal does not solve the problem.

Nokia do not see the value, and they understand that multiple QoS in SA2 has no RAN impact.

Intel also do not see RAN2 impact.

Positioning: SSR PCV residuals [previously seen]

R2-2307757	Support for SSR Satellite PCV Residuals	Swift Navigation	discussion

Proposal 1: RAN2 agrees to introduce the SSR IOD Update IE.

Proposal 2: RAN2 agrees to add the posSIBs for the SSR Satellite PCV Residuals.

Proposal 3: RAN2 agrees to adopt the proposed CRs for the SSR Satellite PCV Residuals.

Discussion:

Qualcomm understand that the proposals introduce a new IE covering the IOD (option 4 in the discussion paper), and they wonder if option 3 would be simpler without requiring a new posSIB.  They agree that the proposed approach works, but it forces the new posSIB.  Swift indicate that the reason for preferring option 4 was consistency with other bodies working on SSR messages (e.g., RTCM), as well as extensibility in the future.  They also indicate that option 3 would require resetting corrections when a new set of PCVs is issued, whereas this option allows a softer update without forcing a reset.

Qualcomm can accept the proposal, but they think RTCM have a different message structure, and we may be better positioned to extend our message formats.

Ericsson think we could go offline for the details.

[AT123][422][POS] SSR PCV residuals (Swift)

Scope: Discuss the TPs from R2-2307757 and develop a set of CRs.

Intended outcome: Agreeable (in principle) CRs

Deadline: Wednesday 2023-08-23 2000 UTC

Positioning: NavIC enhancements [new]

R2-2308193	NavIC L5 A-GNSS support updates to RRC protocol specification	Reliance Jio	CR	Rel-18	38.331	17.5.0	4234	-	F	TEI18

Discussion:

CEWiT present in the absence of Reliance Jio.

Lenovo agree with the change but wonder if we should make the correction from Rel-17, since there are already NavIC posSIBs in Rel-17.  MediaTek agree.

CATT agree that Rel-17 enabled NavIC and introduced the assistance data in LPP, but they understand that broadcast assistance data for NavIC is not fully enabled.  They are fine to take the CR in Rel-18.

Qualcomm agree that this is more of a correction.

CEWiT can convert it to a Rel-17 correction.  They would like to understand the details of CATT’s comment.

Lenovo think the coversheet can be clearer as well.

[AT123][423][POS] NavIC broadcast correction (CEWiT)

Scope: Check the CR in R2-2308193 and adapt it to a Rel-17 correction.

Intended outcome: Agreeable CR in R2-2308978

Deadline: Wednesday 2023-08-23 2000 UTC

R2-2308978	NavIC L5 A-GNSS support updates to RRC protocol specification	Reliance Jio	CR	Rel-17	38.331	17.5.0	4234	1	F	NR_pos_enh-Core

Positioning/relay: positioning for remote UEs [previously seen]

R2-2308485	Relay based Positioning posSIB forwarding	Ericsson, Deutsche Telekom, AT&T	discussion	Rel-18

R2-2308486	Information on posSIBs relaying to remote UE [PosL2RemoteUE]	Ericsson, Deutsche Telekom, AT&T	CR	Rel-18	38.331	17.5.0	4254	-	B	TEI18

R2-2308487	Information on posSIBs relaying to remote UE	Ericsson, Deutsche Telekom, AT&T	draftCR	Rel-18	38.455	17.5.0	B	TEI18

Proposal 1	The information on which posSIBs can be relayed is optionally provided by the NW to the UEs.

Proposal 2	RAN2 to agree to the RRC CR as provided in R2-2308486.

Discussion:

Apple understood we agreed last meeting that the forwarding should be transparent to the relay UE, and they wonder if the proposal contravenes this agreement.

Qualcomm do not understand the relation of the described issue to posSIB forwarding; the LMF selects what positioning method to use, based on information that can include coverage/remote status, but they do not see what the difference is if the gNB forwards additional assistance data.

Ericsson think we have dimensioned the assistance data support and expected performance based on assumptions about coverage.  They think there could be a latency cost if the first positioning operation uses a method that does not work well out of coverage.

MediaTek have the same understanding as Qualcomm.

Ericsson are concerned about an idle/inactive UE receiving broadcast assistance data.

Qualcomm do not see a connection to the RRC state; the LMF selects the positioning method and tells the UE what to measure, and the UE may or may not have assistance data via posSIBs, irrespective of the RRC state.

Ericsson understand that the posSIBs are a value-added service with encryption, which is the difference from other SIBs.  They intend to enable selection of appropriate positioning methods based on coverage/remote status.

Intel understand that the LMF would select an appropriate positioning method, and they do not see that the network can enforce anything because the method selection is transparent to the gNB.

Samsung think this can be helpful to reduce unnecessary overhead from posSIB signalling that would not be useful (e.g., the UE requests a posSIB that doesn’t make sense out of coverage).  However, they have the same understanding as other companies regarding network control.

OPPO wonder about the motivation to save signalling overhead for an out-of-coverage remote UE.  They do not think the relay UE can differentiate whether the remote UE is in coverage, so they are wondering how it works.  Qualcomm agree and think the proposal may make the signalling more excessive; they do not see how the gNB takes the decision on what can be forwarded, considering that the LMF selects the method.

MediaTek also see extra signalling if the LMF picks a positioning method, the UE tries to receive assistance data, and the network says “no”; the UE will just request the same assistance data from the LMF.

Ericsson understand that, e.g., DL-TDOA will not work well out of coverage.  MediaTek do not understand the use case, since the LMF selects the positioning method and the UE presumably requests what it needs.

Qualcomm think there is nothing broken with broadcast: The system can provide the assistance data either way, and if it wants to save broadcast overhead, it will use unicast LPP.

Apple also think the remote UE can be out of coverage or in coverage, and it is not clear why the network would block the relay UE from forwarding the posSIBs.

Ericsson think we could go offline for discussion.

[AT123][424][POS] Network control of posSIBs for remote UEs (Ericsson)

Scope: Discuss the proposal in R2-2308485 and determine if there is support for moving forward with it.

Intended outcome: Report to CB session in R2-2309107

Deadline: Wednesday 2023-08-23 2000 UTC

R2-2308695	Discussion on positioning support for L2 U2N remote UE	Samsung	discussion	Rel-18	TEI18

Proposal 1: Allow L2 U2N remote UE in RRC_CONNECTED to request an SFN-DFN from the connected L2 U2N relay UE via RemoteUEInformationSidelink message as in the TP to 38.331 in Annex.

Proposal 2: Clarify that L2 U2N remote UE can include remoteUE-Indication-r18 in ProvideCapabilities message only when it has been requested by the LMF before as in the TP to 37.355 in Annex.

Proposal 3: Introduce a coverage indication to indicate whether the target device operating as a L2 U2N remote UE is located within the coverage of the serving cell (i.e., the serving cell of the relay UE) as in the TP to 37.355 in Annex.

Proposal 4: Update the field description for primaryCellID-r15 to allow the target device operating as a L2 U2N remote UE to report the identity of the current primary cell/camping cell for the L2 U2N Relay UE as in the TP to 37.355 in Annex.

Discussion:

Apple wonder why P1 is needed, because the UE in RRC_CONNECTED sees signalling from the gNB.  They think the gNB may be able to provide the offset.

Samsung think companies may need some time to review.

[AT123][425][POS] Proposals on positioning for remote UE (Samsung)

Scope: Discuss the proposals in R2-2308695 and progress towards agreements where possible.

Intended outcome: Report to CB session

Deadline: Wednesday 2023-08-23 2000 UTC

Positioning: BT AoA/AoD [new]

R2-2308489	Adding support for Bluetooth AoA/AoD	Ericsson, AT&T, Polaris Wireless, u-blox	discussion	Rel-18

- Revised in R2-2308955

R2-2308955	Adding support for Bluetooth AoA/AoD	Ericsson, AT&T, Polaris Wireless, u-blox, T-Mobile	discussion	Rel-18

Proposal 1	Discuss and agree to introduce support for Bluetooth AoA/AoD positioning in the LPP Bluetooth positioning method

Proposal 2	Endorse the draft CR in the Appendix

Discussion:

Qualcomm are unsure of the objectives of the additional parameters in the CR.  They wonder if we can do everything in Rel-18, and they think some discussion is needed on the details.

[AT123][426][POS] BT AoA/AoD (Ericsson)

Scope: Discuss the proposal in R2-2308955; determine if the general change is agreeable, and if so, start discussion on the approach in the CR.

Intended outcome: Report to CB session in R2-2309108

Deadline: Wednesday 2023-08-23 2000 UTC

## 7.25	R18 Other

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-18 specific WIs/SIs that has no RAN WI.

Time budget: 2 TU

Tdoc Limitation: -

### 7.25.3	Other

RAN3, SA2, SA3, CT1 led items and others, e.g. eNPN, Slicing.

R2-2308400	On Positioning Reference Unit support in LPP	Qualcomm Incorporated	discussion

R2-2308488	On the Positioning Reference Units aspects	Ericsson, vivo	discussion	Rel-18

[AT123][419][POS] Location information type for PRUs (Ericsson)

Scope: Discuss the proposals in R2-2308400 and R2-2308488, gather company views, and work towards a conclusion.

Intended outcome: Report to CB session

Deadline: Wednesday 2023-08-23 2000 UTC