3GPP TSG-RAN WG2 Meeting #123	   R2-2307111

Toulouse, France, 21th – 25th August, 2023

Agenda item:		7.11.3

Source:	vivo

Title:	Further Discussion on Shared Processing in eMBS

Document for:		Discussion and Decision

1 Introduction

In the last RAN2 meeting [1], some agreements related to shared processing for MBS broadcast and unicast reception had been achieved, which are given as follows,

The granularity for capability of receiving MBS broadcast from a non-serving cell is at FeatureSetDownlinkPerCC level. This capability does not imply simultaneous reception on multiple CCs. No additional signalling is introduced to control information to be reported by the UE (on top of what we have already agreed). When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. FFS whether there are cases where this information is not available at the UE and what happens then. FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII) No additional information is added to MII on top of what has been already agreed.

In this contribution, we would like to provide our considerations on the remaining issues, including:

- What exact parameters should be reported;

- Whether/how to address the case where additional information cannot be read by the UE from the non-serving cell;

- Whether any special handling is needed when the non-serving cell updates the configuration which is relevant for MII.

2. Discussion

## 2.1 Reporting information

As in LTE, NR MBS R18 had been agreed that in additional MII reporting only the following information can be signalled: broadcast frequency, subcarrier spacing, and bandwidth and no additional information is added. It is FFS details/exact parameters.

We understand that the reported frequency, SCS and bandwidth should be exactly used by UE broadcast reception, e.g. not at cell level or other level. SIB20 of broadcast cell may include the field of CFR-ConfigMCCH-MTCH, which indicates common frequency resource used for MCCH and MTCH reception. If the field is absent, the CFR for broadcast has the same location and size as CORESET#0 and PDSCH configuration of MCCH is the same as PDSCH configuration provided in initialDownlinkBWP in SIB1. In the WCFR- ConfigMCCH-MTCH, there may be the field of locationAndBandwidthBroadcast, which can indicate the CFR for broadcast has the same location and size as the locationAndBandwidth for initial BWP configured in SIB1 or configure CFR with bandwidth that is larger than and fully contains the bandwidth for the initial DL BWP and CORESET#0 configured in SIB1.

Hence, the SCS included in the MII would be the SCS of the initial BWP (i.e. CORESET#0) for the MBS broadcast cell. And frequency domain location and bandwidth can be indicated with configuration in CFR-ConfigMCCH-MTCH of SIB20, i.e. locationAndbandwidth of initial BWP or locationAndBandwidthBroadcast. Of course, some other parameters are also needed to derive the absolute frequency information together, e.g.  absoluteFrequencyPointA and offsetToCarrier.

Furthermore, in last meeting, RAN2 had agreed to introduce a separate CFR which can be used when the configured bandwidth for the default CFR in SIB20 exceeds the bandwidth capability of bandwidth limited UEs (i.e. RedCap UE).  Hence, for a RedCap UE, separate CFR information is indicated in the MII.

- The SCS in the MII is set to the SCS of the initial BWP for the MBS broadcast cell.

- Frequency domain location and bandwidth information in the MII can be set based on configuration in CFR-ConfigMCCH-MTCH of the MBS broadcast cell’s SIB20 or the separate CFR for RedCap UE, as well as some additional information to derive the absolute value, e.g. absoluteFrequencyPointA and offsetToCarrier.

## 2.2 Report control

In last meeting, RAN2 agreed that UE reports the whole information (i.e. frequency, bandwidth and SCS) when indicated by SIB1 of its unicast serving cell. And it is FFS whether there are cases where this information is not available at the UE and what happens then.

In some cases that the current unicast serving cell and MBS frequency are not included in any supported band combination according to the UE’s capability, UE may not be able to receive both unicast and MBS services simultaneously. However, it is different from continuous MBS service reception that acquisition bandwidth and SCS information only needs a small amount of discontinuous monitoring. It may be up to UE implementation on how to acquire the bandwidth and SCS information, e.g. in measurement gap, in service interruption time or in DRX off period and so on.  Like LTE, there is no need to over optimize for the ROM (Receive Only Mode).

Hence,

- It is up to UE implementation on how to acquire the bandwidth and SCS information, i.e. no specified enhancement for not available case.

- UE reports directly the whole additional information in MII when indicated by SIB1 of its unicast serving cell.

The third issue is how to handle the MBS configuration updates after the first MII reporting. It is a natural way to trigger a new MII reporting procedure when any MBS parameter (i.e. bandwidth or SCS) has been changed since the last reporting. Since the MBS configuration update doesn’t happen frequently and the number of UEs in ROM is usually small, additional solution to reduce redundant MII update reporting from different UEs is not needed.

- Like LTE, UE triggers a new MII reporting when any MBS parameter, i.e. bandwidth or SCS, has been changed since the last reporting and no additional solution needs for redundancy avoidance from different UEs.

3. Conclusion

In this contribution, we have provided our considerations on the remaining issues about shared processing for MBS broadcast and unicast reception. All the proposals are summarized as follows,

- The SCS in the MII is set to the SCS of the initial BWP for the MBS broadcast cell.

- Frequency domain location and bandwidth information in the MII can be set based on configuration in CFR-ConfigMCCH-MTCH of the MBS broadcast cell’s SIB20 or the separate CFR for RedCap UE, as well as some additional information to derive the absolute value, e.g. absoluteFrequencyPointA and offsetToCarrier.

- The SCS in the MII is set to the SCS of the initial BWP for the MBS broadcast cell.

- Frequency domain location and bandwidth information in the MII can be set based on configuration in CFR-ConfigMCCH-MTCH of the MBS broadcast cell’s SIB20 or the separate CFR for RedCap UE, as well as some additional information to derive the absolute value, e.g. absoluteFrequencyPointA and offsetToCarrier.

- Like LTE, UE triggers another MII reporting when any MBS parameter, i.e. bandwidth or SCS, has been changed since the last reporting and no additional solution needs for redundancy avoidance from different UEs.

4 References

- R2#122, Meeting Reports.