3GPP TSG-RAN WG2 Meeting #123	   R2-2307114

Toulouse, France, August 21-25, 2023

Source:	vivo

Title:	Discussion on Timing Synchronization Status Monitoring

Agenda Item:	7.23.2

Document for:	Discussion and Decision

- Introduction

RAN2#122 meeting [1] has made some progress on 5GS Network Timing Synchronization Status and Reporting as follow

Agreements - Update of event ID is informed to UE by normal SI modification procedure. - Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED - For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE, (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation - Event ID is optional. Under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different. FFS if gNB always broadcasts event ID - If the UE is in RRC_INACTIVE, the UE can acquire clock quality information using the SDT procedure, if it supports and/is configured with SDT procedure

Based on the above agreements, we further investigate the following issues about change of timing synchronization status (TSS).

- Discussion

- 2.1	Event ID broadcasting

Recalling RAN2’s agreement on event ID:

Event ID is optional. Under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different. FFS if gNB always broadcasts event ID.

Broadcast in SIB9, the applicable scope for the event ID is multiple cells within one gNB. Based on the FFS, we made the following discussion.

In our understanding, “Event ID is optional” simply indicates that the feature of TSS and reporting is optionally supported by some entities. Moreover, if the gNB supports such feature, it should always broadcast event ID when TSS change from its perspective. The reasons are given below.

The latest version TS 23.501[2] describes how the RAN node associates the network timing synchronization status with the event ID, and how the UE decides on the TSS change, as the quoted text below.

gNBs may be pre-configured with thresholds for each timing synchronization status attribute, if supported, that is described in Table 5.27.1.12-1. When the network timing synchronization status exceeds the thresholds (i.e. status degradation), or the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB notifies the TSCTSF (either using N2 node level signalling via AMF, or via OAM) with the gNB ID, the scope of the timing synchronization status (i.e. all cells or a list of Cell IDs within a single gNB) and the corresponding network timing synchronization status (TSS) attributes as described in this clause. The gNB indicates the status change to the UEs via SIB information: - When the network timing synchronization status exceeds any of the pre-configured thresholds, the gNB includes in SIB information a reference report ID. When the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB broadcasts the reference report ID in SIB information. Either event serves as a notification for the UEs reading the SIB information that there is new TSS information available. NOTE 1: it is assumed that the gNB does not provide clock quality metrics to the UE which is better than the pre-configured thresholds. NOTE 2: It is assumed the pre-configured thresholds in the gNB(s) are sufficient to meet UE time sync performance requirement which are configured by the operator.

Firstly, from the above description, for each timing synchronization status attribute, there exists a threshold (or a range within which the network can be considered synchronized) to measure whether the gNB still synchronizes with 5GS. The TSS change can be interpreted as two events:

- the network TSS exceeds the threshold (i.e. status degradation);

- the network TSS meets the threshold again (i.e. status improvement).

In either event, the gNB broadcasts event ID to notify UE reading SIB9 that there is a change in TSS available.

Furthermore, from the design of the core network’s design in event ID, there can be two integers suggesting two TSS change events respectively at least for each TSS attribute, one is for TSS improvement and the other is for TSS degradation.

Observation 1: In TS 23.501, there is a threshold for each timing synchronization status attribute. Two events can be considered as the timing synchronization status change:

- the network timing synchronization status exceeds the threshold (i.e. status degradation);

- the network timing synchronization status meets the threshold again (i.e. status improvement).

Proposal 1: The gNB in support of TRS feature shall always broadcast event ID.

- 2.2	Time Synchronization Status Monitoring

2.2.1 Inter-layer indication

In regard to the exact layer that decides the change of timing synchronization and that initiates the reconnection procedure, RAN2 has come to the following agreements,

Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED.

Furthermore, in reference to TS 24.501[3], UE’s behavior related to RAN timing synchronization status change is captured in the NAS specification as:

#### 5.2.3.2 Detailed description of UE behaviour in state 5GMM-REGISTERED ##### 5.2.3.2.3 ATTEMPTING-REGISTRATION-UPDATE The UE in 3GPP access: ##### <skip the irrelevant part> shall initiate a registration procedure for mobility and periodic registration update if the UE supports the reconnection to the network due to RAN timing synchronization status change and receives an indication of a change in the RAN timing synchronization status. Relevant to UE in RRC_INACTIVE mode #### 5.3.1.4 5GMM-CONNECTED mode with RRC inactive indication ##### <skip the irrelevant part> If the UE supporting the reconnection to the network due to RAN timing synchronization status change has been requested to reconnect to the network upon receiving an indication of a change in the RAN timing synchronization status (see subclauses 5.4.4.2, 5.5.1.2.4, and 5.5.1.3.4) and the UE receives an indication of a change in the RAN timing synchronization status, the UE in 5GMM-CONNECTED mode with RRC inactive indication shall request the lower layers to transition to RRC_CONNECTED state.

Based on the above contributions, it can be confirmed that, the layer that decides the change of timing synchronization is the AS layer of UE; and the one that triggers the reconnection to the network is the NAS layer. We further identified one issue for interaction between NAS and AS layers: how is the AS layer of UE known that the indication of timing synchronization status change is needed?

As clarified in TS 24.501, during the registration stage, UE’s 5GMM capability may indicates the support of reconnection to the network due to RAN TSS change. Subsequently, AMF would request UE to reconnect to the network on condition that UE decides on the change of TSS.

The above transmissions are achieved via NAS messages, without AS perception. However, as stated beforehand, it is the AS layer that actually determines whether there is a change in timing synchronization status. In this sense, the AS layer needs to be notified by the NAS layer about the necessity of such determination when there is a requirement from a network function. That is, the NAS layer of UE should indicate the AS layer to enable the decision and indication about TSS change.

Observation 2: The AS layer of the UE is required by the NAS layer to provide the timing synchronization status change.

Proposal 2: The NAS layer of the UE shall indicate the AS layer to enable the function of timing synchronization status change.

Regarding the NBC issue, say, the UE moves from a supported gNB to a legacy gNB which does not support TRS feature. Notified by the NAS layer, the AS layer of the UE reads SIB9 and observes no event ID in a legacy gNB, and hence has nothing to compare with the local reference event ID.

In this situation, the AS layer of the UE provides the NAS layer with no indication of TSS change. Even if judging from the change of gNB ID, the UE could not decides that TSS change. Since the NAS layer of the UE would initiate the access procedure, while the network has no clock information to provide. Furthermore, it can be discussed that whether additional information can be provided by the AS layer to the NAS layer. For instance, when obtaining no event ID, the AS layer could indicate that no TSS information is obtained or, straightforwardly, UE is under the coverage of a TRS-disabled gNB.

Proposal 3: When the UE moves to a TRS-disabled gNB and is requested by the NAS layer about the indication of timing synchronization status change, the AS layer of the UE does not provide the indication of TSS change to the NAS layer. FFS additional information is needed.

2.2.2 Handling of local reference report ID

Report ID (i.e. gNB ID and event ID) is used by the UE in RRC_IDLE/INACTIVE state to determine whether TSS changes. When transiting from RRC_CONNECTED into RRC_IDLE/INATIVE, it is necessary for the UE to read report ID from SIB9 corresponding with the currently obtained Clock Quality Information with report ID it locally stored. In this understanding, when receiving RRC Release message, the UE have to read SIB9 to make sure it stores the latest report ID as the reference report ID. In this way, the scenario can be avoided that, the report ID for the current time instance is changed from the last time the UE read and stored as the local reference report ID.

Proposal 4: Upon released from CONNECTED to IDLE/IANCTIVE, UE attempts to acquire the current report ID.

Entering RRC_IDLE/INACTIVE state, the UE reads report ID based on the NAS’s requirement. When finding the TSS change and deciding to access and to acquire Clock Quality Information, the UE updates the local reference report ID.

Proposal 5: UE updates the locally stored reference report ID, once initiating RRC Setup/Resume for Clock Quality Information.

In regard to backward compatibility issue, the feature of networking timing synchronization status may not be supported for a legacy gNB, there is no field of event ID in SIB9. If the UE stores the local reference event ID within the coverage of such legacy gNB, it can be misleading for the UE about the determination on TSS when the UE returns the gNB where it lastly obtained event ID.

Considering that gNB-1 is the gNB that supports R18 TRS feature, while gNB-2 does not. The UE stores reference event ID-1 at gNB-1 coverage. Then it moves to gNB-2 keeping event ID-1 as the local one. There is no TSS provided in gNB-2, that is, the UE loses the TSS in gNB-2. After a while, the UE returns to gNB-1 still with event ID-1 as the local reference, but actually equips with no information about TSS. If the UE reads SIB9 at gNB-1 which suggests event ID-1, it will not consider TSS change. However, it should believe there is a TSS change because it lost TSS in gNB-2.

The main reason for such kind of misunderstanding is that event ID merely suggests a TSS change event but not the TSS attribute. To avoid the above mal-function, the UE shall flush the local reference event ID when moving into a gNB which does not support TRSfeature.

Proposal 6: UE flushes the locally stored reference report ID when moving into a TRS-disabled gNB.

- 2.3	Request for Clock Quality Information

Regarding UE’s access to the network for Clock Quality Information, SA1 has responded a LS[4]:

On RAN2’s question to CT1: RAN2 would like to know if we can use an existing access category for the access attempt to retrieve the latest available clock quality information, e.g., the one for MO signalling, or if it would require a new one since it should not block access triggered by other MO signalling. CT1 answered: Unless there is a service requirement to introduce a new access category for the access attempt to retrieve the latest available clock quality information, CT1 assumes that reusing Access Category 3 (MO_sig) is acceptable. And further raised the following question to SA1: CT1 respectfully asks SA1 to inform if there is a service requirement to use a new access category for the access attempt described above. [SA1 answer]: SA1 would like to inform that there are no stage 1 requirements to have a separate access category for TRS.

The core network WGs have foreseen no requirement to distinguish the access attempt to retrieve the latest available clock quality information. However, from RAN’s perspective, it can be useful for the network to be informed of the access attempt.

In this understanding, with no separation from access category 3 MO_sig, the UE is able to employ other methods to indicate the network about its purpose of obtaining Clock Quality Information. For instance, A new cause value for RRC setup request can be considered.

Proposal 7: RAN2 to discuss whether/how to differentiate UE accessing the network for Clock Quality Information.

- Conclusion

We make the following observations based on the above discussion,

Observation 1: In TS 23.501, there is a threshold for each timing synchronization status attribute. Two events can be considered as the timing synchronization status change:

- the network timing synchronization status exceeds the threshold (i.e. status degradation);

- the network timing synchronization status meets the threshold again (i.e. status improvement).

Observation 2: The AS layer of the UE is required by the NAS layer to provide the timing synchronization status change.

We hence propose,

Proposal 1: The gNB in support of TRS feature shall always broadcast event ID.

Proposal 2: The NAS layer of the UE shall indicate the AS layer to enable the function of timing synchronization status change.

Proposal 3: When the UE moves to a TRS-disabled gNB and is requested by the NAS layer about the indication of timing synchronization status change, the AS layer of the UE does not provide the indication of TSS change to the NAS layer. FFS additional information is needed.

Proposal 4: Upon released from CONNECTED to IDLE/IANCTIVE, UE attempts to acquire the current report ID.

Proposal 5: UE updates the locally stored reference report ID, once initiating RRC Setup/Resume for Clock Quality Information.

Proposal 6: UE flushes the locally stored reference report ID when moving into a TRS-disabled gNB.

Proposal 7: RAN2 to discuss whether/how to differentiate UE accessing the network for Clock Quality Information.

- Reference

- R2-2306543	Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning	Session Chair (InterDigital)

- TS 23.501		System architecture for the 5G System (5GS)

- TS 24.501		Non-Access-Stratum (NAS) protocol for 5G System (5GS)

- S1-231285		Response to Reply LS on Proposed method for Time Synchronization status reporting to UE(s)