3GPP TSG-RAN WG2 Meeting #123	R2-2307308

Toulouse, France, 21st – 25th Aug. 2023

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_Connected

Agenda Item:	7.22.3

Document for:	Discussion and Decision

- Introduction

In RAN #97e meeting, the SID for R18 low-power Wake-up Signal and Receiver for NR was agreed upon [1], and in RAN2#121 bis meeting, we have the following relevant agreements on LP-WUS and LP-WUR [2]:

- Confirm that we follow R1 and include RRC idle/inactive/connected.

Besides, RAN1 has the following agreements on RRC_Connected for LP-WUS in RAN1#113 meeting [3]:

- In RRC CONNECTED mode, study benefit of LP-WUS over existing Rel-15, R16, and R17 power saving techniques for following functionalities: - LP-WUS with similar functionality as R16 DCP. - LP-WUS activates/resumes PDCCH monitoring when LP-WUS is received. - interaction with legacy power saving techniques, if any - other functionalities are not precluded - for evaluation - companies to report - assumption on MR sleep state when LP-WUR is monitoring LP-WUS - deep sleep, - light sleep, - micro sleep - how to activate/deactivate LP-WUS monitoring and deactivate/activate PDCCH monitoring - LP-WUS waveform - In RRC CONNECTED mode, LP-WUS monitoring can be activated/deactivated by at least one or more of\ - by gNB RRC signaling, with or without UE assistance. - by gNB L1/L2 LP-WUS activation/deactivation signaling, with or without UE assistance. - based on pre-configured condition(s), such as timer. - LP-WUS monitoring by UE is known to gNB, study whether it could be transparent to gNB. - other options are not precluded.

In this contribution, we will discuss the detailed RRC_CONNECTED procedures and related aspects with LP-WUS/WUR considering RAN1’s progress.

- Discussion

  - General procedure

According to the SID, UE supporting LP-WUS/WUR will be equipped with an additional Rx module apart from the current Tx/Rx module. The corresponding terminologies were agreed by RAN1 as follows:

Use the following terminology for future discussion, - Main radio (MR): the Tx/Rx module operating for legacy NR signals/channels apart from signals/channel related to low-power wake-up. - LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

For legacy UE in RRC_Connected state, it should keep MR monitoring PDCCH in C-DRX active time. The PDCCH monitoring of MR is unnecessary if there is no UE schedule. However, the legacy UE can confirm that a PDCCH monitoring is unnecessary only after already monitored the PDCCH with MR and found that no PDCCH address to itself. As a result, the power consumption caused by unnecessary PDCCH monitoring was wasted.

This issue has been also discussed in Rel-16/Rel-17. Finally, DCP in Rel-16 and PDCCH monitoring adaptation in Rel-17 were introduced to reduce unnecessary PDCCH monitoring. They are both PDCCH based signaling. For a UE equipped with LR, the MR can be in a sleep power state, while the LR remains active to monitor Low-Power Wake-Up Signal (LP-WUS) in DRX or continuous manner. When LP-WUS is received by LR, it will trigger the MR to wake up to monitor PDCCH. As LR would adopt a minimalistic design, the power consumption of LR is expected to be significantly lower than legacy PDCCH based signaling in some cases. Anyway, this is related to the LR receiver architecture, which is being discussed in RAN1.

According to the evaluation results in Annex, it could be observed that more power saving gain is obtained via LP-WUS than the existing power saving schemes. Compared with UE in RRC_Idle/Inactive state, UE in RRC_Connected state is more sensitive to the latency. Therefore, we think RAN2 should study LP-WUS procedures for RRC_Connected state to reduce power consumption while guarantee the latency performance.

Proposal 1: RAN2 to study the procedure to use LP-WUS for RRC_Connected state to reduce power consumption while guarantee the latency performance.

According to the following agreements in RAN1#112bis meeting, the ultra-deep sleep state is not allowed for MR for RRC connected mode. Instead, the MR could be in micro, light, or even deep sleep depends on the detailed design and configuration.

- For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR.

Observation 1: Ultra-deep sleep power state is not allowed for MR for RRC_Connected state.

In order to save UE power consumption for PDCCH monitoring and guarantee the latency, LP-WUS could be used as similar logic of DCP in Rel-16. That is, when MR is in sleep state, LR will remain active to monitor LP-WUS. When LP-WUS is received by LR, it will trigger the MR to wake up to monitor PDCCH. But the details design for LP-WUS could be different from Rel-16 DCP, which needs further discussion.

Proposal 2: LP-WUS could be used to control the PDCCH monitoring in RRC_Connected state, e.g. UE’s MR should wake up to monitor PDCCH when LP-WUS is received to indicate the UE to wake up. Details FFS.

  - LP-WUS with legacy power saving features

RAN1 has agreed to study the relationship between LP-WUS and legacy UE power saving techniques in RRC connected as below [3].

- In RRC CONNECTED mode, study benefit of LP-WUS over existing Rel-15, R16, and R17 power saving techniques for following functionalities: - LP-WUS with similar functionality as R16 DCP. - LP-WUS activates/resumes PDCCH monitoring when LP-WUS is received. - interaction with legacy power saving techniques, if any - other functionalities are not precluded

According to the above RAN1 agreement, it could be observed that there are two way forwards for LP-WUS functions with C-DRX as follows:

- Direction 1: LP-WUS combines with C-DRX.

- Direction 2: LP-WUS doesn’t combine with C-DRX.

For direction 1, RAN1 has agreed to study LP-WUS with similar functionality as Rel-16 DCP, another alternative is to use LP-WUS to replace Rel-16 DCP, i.e., UE performs LP-WUS monitoring just before the start of the drx-onDurationTimer of Long DRX and UE’s MAC entity enters into Active Time for PDCCH monitoring in the next DRX onduration when LP-WUS is detected.

Besides, considering the following two characteristics for LP-WUS and C-DRX:

- LP-WUS activates/resumes PDCCH monitoring when LP-WUS is received as RAN1 agreement

- when C-DRX is applied, UE only monitors PDCCH during the DRX onDuraiton

we think another use case for direction 1 is for XR traffic. In current, since the XR data packets have a wide range of unpredictable jitter and UE couldn’t predict when the data will arrive exactly, the UE has to perform intensive PDCCH monitoring in the entire jitter range to receive the scheduling of XR traffic timely and avoid packet loss caused by exceeding the PDB when DRX onDuration starts. When LP-WUS combines with C-DRX and for UEs with long DRX onDuration in order to cover the jitter range, the UE starts to monitor LP-WUS instead of PDCCH from the beginning of DRX onDuration and the UE resumes PDCCH monitoring from micro or light sleep of MR once LP-WUS is detected.  The following figures show this procedure:

Figure 1.  Working procedure for LP-WUS/WUR based jitter handling

For direction 1, when the DRX timer expires or receives Rel-17 PDCCH skipping indication (if supported and configured), UE could stop the PDCCH monitoring.

For direction 2, when C-DRX is not configured, UE could activate/resume PDCCH monitoring when LP-WUS is received. Regarding how to stop the PDCCH monitoring, since there is no C-DRX, direction 2 has to be combined with the Rel-17 PDCCH skipping indication, in this case, UE could stop PDCCH monitoring when UE receives the Rel-17 PDCCH skipping indication.

With the above discussion, the LP-WUS application methods could be summarized as Table 1:

Table 1: LP-WUS application method and functions

| LP-WUS application | Details | Start PDCCH monitoring | Stop PDCCH monitoring |
| --- | --- | --- | --- |
| Direction 1: combined with DRX | Option 1: similar as Rel-16 DCP, located before drx-onDurationTimer | If LP-WUS is detected and drx-onDurationTimer is started | when DRX timer expires or receives Rel-17 PDCCH skipping indication (if supported and configured). |
|  | Option 2: XR use case, loacted after drx-onDurationTimer | If LP-WUS is detected, |  |
| Direction 2: combined without DRX | Combined with Rel-17 PDCCH skipping | If LP-WUS is detected | when receiving Rel-17 PDCCH skipping indication. |

In our view, all the above directions on how to use the LP-WUS in RRC_CONNECTED with legacy features are workable. Direction 1 has less specification impact, especially for option 1, while direction 2 could reduce additional latency in C-DRX case and is more flexible. We think all the directions should be captured in the TR and the details could be determined during WI phase.

Proposal 3: Capture all directions on how to use LP-WUS together legacy power saving features, including with/without C-DRX as below. Details can be discussed/determined during WI phase.

- Direction 1: LP-WUS could combine with C-DRX, including the following uses cases:

  - monitor LP-WUS only before DRX onDurationTimer, i.e. similar as Rel-16 DCP.

  - monitor LP-WUS after DRX onDurationTimer.

- Direction 2: LP-WUS doesn’t combine with C-DRX, e.g. using LP-WUS to control PDCCH monitoring at any time.

Besides, we think other Rel-17 power saving techniques, i.e. Rel-17 PDCCH skipping and Rel-17 SSSG switching, could be combined with the LP-WUS well without any restriction, since they have reverse functions on PDDCH monitoring and has no impants to each other.

Proposal 4: LP-WUS could be used naturally together with legacy mechanisms to stop/switch PDCCH monitoring, e.g. Rel-17 PDCCH skipping, and Rel-17 SSSG switching, unless critical problem is identified.

  - RRM measurement

In RAN1#121 bis meeting, it was agreed that RRM/RLM/BFD/CSI are performed by MR in RRC connected mode, and RAN1 will study whether additional support of RRM measurement by LP-WUR for RRC connected mode.

- For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode

In current specification, the RRM measurement mainly intends to the mobility, i.e. ensure UE works in a serving cell with good quality via handover. For UEs with LP-WUR, UE’s MR is in micro or light sleep power state and could perform RRM measurement normally, which is same as current DRX off state, hence the RRM measurement by MR is enough for mobility purpose. Regarding whether additional support of RRM measurement by LP-WUR is needed, it could be further studied if suitable use cases are discovered based on companies’ contributions.

Proposal 5: Current RRM measurement by Main Radio is enough for mobility in RRC connected mode, no additional support of RRM measurement by LP-WUR unless it is well justified.

- Conclusion

In this contribution, we discuss the RRC_CONNECTED procedures with LP-WUS/WUR considering RAN1’s progress. Based on the discussion, we have the following observation and  proposals:

General procedure

Observation 1: Ultra-deep sleep power state is not allowed for MR for RRC_Connected state.

Proposal 1: RAN2 to study the procedure to use LP-WUS for RRC_Connected state to reduce power consumption while guarantee the latency performance.

Proposal 2: LP-WUS could be used to control the PDCCH monitoring in RRC_Connected state, e.g. UE’s MR should wake up to monitor PDCCH when LP-WUS is received to indicate the UE to wake up. Details FFS.

LP-WUS with legacy power saving features

Proposal 3: Capture all directions on how to use LP-WUS together legacy power saving features, including with/without C-DRX as below. Details can be discussed/determined during WI phase.

- Direction 1: LP-WUS could combine with C-DRX, including the following uses cases:

  - monitor LP-WUS only before DRX onDurationTimer, i.e. similar as Rel-16 DCP.

  - monitor LP-WUS after DRX onDurationTimer.

- Direction 2: LP-WUS doesn’t combine with C-DRX, e.g. using LP-WUS to control PDCCH monitoring at any time.

Proposal 4: LP-WUS could be used naturally together with legacy mechanisms to stop/switch PDCCH monitoring, e.g. Rel-17 PDCCH skipping, and Rel-17 SSSG switching, unless critical problem is identified.

RRM measurement

Proposal 5: Current RRM measurement by Main Radio is enough for mobility in RRC connected mode, no additional support of RRM measurement by LP-WUR unless it is well justified.

- References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR.

- Report of 3GPP TSG RAN WG2 meeting #121bis-e

- Chairman’s Notes of RAN1 #113.

- Annex

  - Power evaluation results for RRC_Connected state

In the following evaluation for XR traffic, performance metrics including power consumption, latency, capacity are considered. The detail evaluation methodologies and assumptions can be found in TR 38.838.

- Evaluation for XR traffic model (refer to TR38.838 and the agreed jitter model in R18 XR SI)

Based on the R17 and R18 XR power evaluation methodologies and assumptions, we further evaluate power consumption and system capacity of different cases. And the detailed configurations can be founded in section5.2. Note that for capacity results of LP-WUS/WUR scheme (combined with main receiver light sleep) corresponding to the UE satisfaction metric with both 95% and 99% packet successful rate are shown. Evaluations for low load is given, corresponding to 5 UEs per cell.

Figure 2.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

According to the results shown in Figure 2, for low load case, comparing to R17 PDCCH monitoring adaptation scheme, LP-WUS/WUR schemes with main radio enters micro sleep during LP-WUS monitoring can bring {6%~13%} additional UE power saving gain with no capacity loss. In addition, when main radio enters light sleep during LP-WUS monitoring, the LP-WUS scheme can bring {10%~22%} additional UE power saving gain with 10% (UE satisfaction metric as 99% packet successful rate) or no (UE satisfaction metric as 95% packet successful rate) capacity loss, comparing to R17 PDCCH monitoring adaptation scheme.

Observation 1: Compared to the existing R17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {6%~13%} additional UE power saving gain with no capacity loss in low load case.

Observation 2: Compared to the existing R17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {10%~22%} additional UE power saving gain, with acceptable capacity loss in low load case.

  - Simulation assumptions for RRC_Connected state

The detail configurations for R17 PDCCH monitoring adaptation and LP-WUS/WUR schemes are illustrated as below.

R17 PDCCH monitoring adaptation scheme includes both PDCCH skipping indication and SSSG switching indication, and its key assumptions are listed below:

- Default/sparse SSSG: do PDCCH monitoring in every two slots;

- Dense SSSG: do PDCCH monitoring in every slot.

- Two candidate PDCCH skipping durations: 4ms and 6ms.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

For LP-WUS/WUR scheme, the key assumptions are introduced below:

- The LP-WUS monitoring is performed within DRX onduration.

- The main receiver enters into the micro sleep when doing LP-WUS monitoring.

- The total relative power for LP-WUS monitoring is assumed as 45 or 20 power units (i.e., the main radio enters micro or light sleep).

- And the latency from receiving LP-WUS to be ready for PDCCH monitoring is set to 1 slot.

- R17 PDCCH skipping indication is adopted. And three candidate PDCCH skipping durations are 2ms, 4ms and 6ms respectively.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

- System capacity is assumed as the maximum number of users per cell with at least 90% of UEs being satisfied. And a UE can be regarded as a satisfied UE if more than 99% or 95% of packets are successfully transmitted within a given air interface PDB.

Table 1. Simulation assumption for FR1 Indoor Hotspot scenario

| Parameter | value |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

Table 2. The DL video traffic models with 60 FPS adopted in R17 XR SI

| Traffic model | VR/AR | VR/AR |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |
| Mean packet size (Bytes) | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 | 16.67 |
| PDB (ms) | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |
| Jitter Mean (ms) | 0 |  |
| Jitter STD (ms) | 2 or 5 |  |
| Jitter Range (ms) | [-4, +4], [-6, +6] or [-8, +8] |  |