3GPP TSG-RAN WG2 Meeting #123                                                        R2-2307349

Toulouse, France, 21st - 25th August, 2023

Source:              CATT, China Unicom, DELL, Ericsson, Intel Corporation, OPPO

Title:	             PDCP discard timer model for PDU Set discard

Agenda Item:	7.5.4.2

Document for:	Discussion and Decision

Introduction

The RAN2 agreements so far related to the PDCP timer modeling when used for PDU Set discarding are as follows:

RAN2#121bis-e meeting [1]:

- PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

RAN2#122 meeting [2]:

- PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity.

This contribution discusses the next level of details for the PDCP discard timer modeling when handling the PDU Set based discard functionality.

Discussion

The first issue to address is how are expected to arrive the PDUs of a PDU Set: simultaneously or at different instances of time? Then, we derive the implication on the PDCP discard timer model, i.e. whether the legacy timer can be reused as it is, and if not, what is/are the least complex change(s) required to properly implement the PDU Set based PDCP discard timer.

  - Scenarios on arrival time(s) of PDUs from a PDU Set

The two assumptions regarding the arrival time(s) of PDUs from a PDU Set are (Figure 1):

- Scenario 1: All PDUs of a PDU Set arrive simultaneously.

- Scenario 2: PDUs of a PDU Set can arrive at different times.

Figure 1: Scenarios on arrival time(s) of PDUs from the same PDU Set

In SA2 stage 2 [3], it is described that:

NOTE: To enable support for PSDB, it is required that a maximum inter arrival time between the first received PDU and the last received PDU of a PDU Set complies with SLA. This maximum inter arrival time does not exceed PSDB. NG-RAN behaviour when the SLA is not fulfilled is out of scope of this specification.

Based on the above NOTE, it is obvious that PDUs within one PDU Set may arrive at different times. This observation is valid in both UL and DL directions and can be explained by, for example, the different processing times required for the video encoder to generate all data for one PDU Set.

Observation 1: UL PDUs of a PDU Set can arrive, in the UE buffer, at the same time, or at different points in time.

  - Validity of legacy PDCP discard timer model for PDU Set discard

With legacy PDCP discard timer model, the timer is started for each PDU of the PDU Set upon PDU arrival and runs up to the configured value based on PSDB (for simplicity the figure/discussion assume that it set equal to PSDB value). As illustrated in Figure 2, the issue with this model is that, as soon as the first PDU is successfully received, which stops its associated timer, the running timers from subsequent PDUs are biased, since they were not started from the first PDU. This can lead to a case where the timer of the last PDU keeps running although the PDU Set exceeded its PSDB.

Figure 2: Using legacy PDCP timer for PDU Set PSDB monitoring

Observation 2: Legacy PDCP discard timer cannot be reused “as is” for the delay budget monitoring associated with a PDU Set (i.e. PSDB).

Proposal 1: The operation of PDCP discard timer is enhanced to address PSDB monitoring when PDUs of a PDU Set arrive at different instances of time.

From the RAN2 #122 agreement captured in Section 1, the UE is RRC-configured per PDCP entity to apply PDU-based discarding (as legacy) or PDU Set based discarding (for example using a new RRC parameter “pduSetDiscarding”). Then it is obvious that this RRC parameter can also be used to indicate the UE which PDCP discard timer model to use, the legacy PDCP discard timer, or the enhanced PDCP discard timer addressing PDU Set based discard.

Proposal 2: The RRC configuration indicating whether PDCP performs PDU Set based discard also indicates PDCP to use the enhanced timer model addressing PDU Set based discard.

  - PDCP discard timer models for PDU Set discard

There are essentially two PDCP discard timer models for properly monitoring the overall transmission delay over Uu of a PDU Set which PDUs arrive at different times:

- Option 1: One PDU Set PDCP discard timer is maintained per PDCP SDU. Once a PDCP discard timer expires, UE discards all the PDUs in the associated PDU Set.

- Option 2: One PDU Set PDCP discard timer is maintained per PDU set (PDCP SDU Set). When the PDCP discard timer expires, UE discards all the PDUs in the associated PDU Set.

If Option 1 is used, in order to ensure the PSDB applies to the whole PDU Set, the actual used PDCP discard timer length for each PDU following the first PDU should be shorter than the configured PDCP discard timer (e.g., equals to the configured PDCP discard timer – time interval between this PDU and the first PDU in the same PDU Set), as shown in Figure 3.

Figure 3: PDCP discard timer maintenance (Option 1)

For Option 2, the timer maintenance is illustrated in Figure 4:

Figure 4: PDCP discard timer maintenance (Option 2)

Both timer models provide the same functionality, so we suggest selecting the least complex timer. Both TPs are provided in Annex 1 & 2.

Proposal 3: RAN2 selects amongst the below two options for the PDCP discard timer model addressing PSDB monitoring with different arrival times of the PDU Set PDUs:

- Option 1: One PDU Set PDCP discard timer is maintained per PDCP SDU. When a PDU in the PDU Set arrives at PDCP, its discard timer is set to run for a duration equal to the configured PDCP discard timer minus the time interval between this PDU and the first PDU of the same PDU Set. Once a PDCP discard timer expires, UE discards all the PDUs in the associated PDU Set.

- Option 2: One single PDCP discard timer is maintained per PDU Set, started upon the arrival of the first PDU of the PDU Set. When the PDCP discard timer expires, UE discards all the PDUs in the associated PDU Set.

Conclusion

According to the analysis in section 2, it is observed and proposed:

Observation 1: UL PDUs of a PDU Set can arrive, in the UE buffer, at the same time, or at different points in time.

Observation 2: Legacy PDCP discard timer cannot be reused “as is” for PDU Set PSDB monitoring.

Proposal 1: The operation of PDCP discard timer is enhanced to address PSDB monitoring when PDUs of a PDU Set arrive at different instances of time.

Proposal 2: The RRC configuration indicating whether PDCP performs PDU Set based discard also indicates PDCP to use the enhanced timer model addressing PDU Set based discard.

Proposal 3: RAN2 selects amongst the below two options for the PDCP discard timer model addressing PSDB monitoring with different arrival times of the PDU Set PDUs:

- Option 1: One PDU Set PDCP discard timer is maintained per PDCP SDU. When a PDU in the PDU Set arrives at PDCP, its discard timer is set to run for a duration equal to the configured PDCP discard timer minus the time interval between this PDU and the first PDU of the same PDU Set. Once a PDCP discard timer expires, UE discards all the PDUs in the associated PDU Set.

- Option 2: One single PDCP discard timer is maintained per PDU Set, started upon the arrival of the first PDU of the PDU Set. When the PDCP discard timer expires, UE discards all the PDUs in the associated PDU Set.

Reference

- R2-2306553	RAN2#121bis-e Meeting Report	MCC

- Draft Report of 3GPP TSG RAN WG2 meeting #122

- TS23.501 System architecture for the 5G System (5GS); Stage 2 (Release 18); 3GPP

- TS38.323  Packet Data Convergence Protocol (PDCP) specification (Release 17); 3GPP

- Annex 1 – 38.323 TP for Option 1

Note: we use pduSetDiscarding for naming the RRC parameter indicating whether PDCP performs PDU Set based discard, but the final name is left to RAN2 decision.

5.2	Data transfer

5.2.1	Transmit operation

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	if pduSetDiscarding is set for this PDCP entity:

-	if this PDCP SDU is the first received SDU of a PDCP SDU Set:

-	start the discardTimer associated with this PDCP SDU (if configured);

-	else:

-	start the discardTimer associated with this PDCP SDU (if configured) with an initial value equal to the elapsed time since the first PDCP SDU of the associated PDCP SDU Set was received.

-	else:

-	start the discardTimer associated with this PDCP SDU (if configured).

[…]

## 5.3	SDU discard

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, or pduSetDiscarding is set for this PDCP entity and, for a PDCP SDU from a PDU Set, the discardTimer of any PDCP SDU from the same PDCP SDU Set expires, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

- Annex 2 – 38.323 TP for Option 2

Note: we use pduSetDiscarding for naming the RRC parameter indicating whether PDCP performs PDU Set based discard, but the final name is left to RAN2 decision.

5.2	Data transfer

5.2.1	Transmit operation

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	if pduSetDiscarding is set for this PDCP entity:

-	if this PDCP SDU is the first received SDU of a PDCP SDU Set:

-	start the discardTimer associated with this PDCP SDU Set;

-	else:

-	start the discardTimer associated with this PDCP SDU (if configured).

[…]

## 5.3	SDU discard

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If pduSetDiscarding is set, when the discardTimer expires for a PDCP SDU Set, the transmitting PDCP entity shall discard all PDCP SDUs of the PDCP SDU Set, along with the corresponding PDCP Data PDUs. When discarding a PDCP Data PDU, iIf it the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

[…]

## 7.3	Timers

The transmitting PDCP entity shall maintain the following timers:

a) discardTimer

This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer or, when pduSetDiscarding is set for the PDCP entity, upon reception of the first SDU of a PDCP SDU Set.