3GPP TSG-RAN WG2 Meeting #123	R2-2307607

Toulouse, France, August 21-25, 2023

Agenda item:	7.5.2

Source:	Meta

Title:	Discussion on XR Awareness

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and Decision

# Introduction

In the RAN2#122 meeting, the following agreement was made:

- UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter. - Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side. - UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information. - RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

In this contribution, we discuss the reporting of UE Assistance Information (UAI).

# Discussions

XR UL flows have unique traffic characteristics (data rates, burstiness, periodicity, etc) and transmission requirements (latency, packet loss, power consumption, etc). The UAI of UL traffic is useful at the network to allocation resources and optimize system performance in terms of data rates, latency, power savings, reliability, etc.

Besides burst arrive time and UL jitter range, it is useful to report additional information such as periodicity, UL config grant offset preference, C-DRX periodicity preference.

Observation 1: The UAI of UL traffic is useful at the network to allocation resources and optimize system performance in terms of data rates, latency, power savings, reliability, etc.

Proposal 1: Besides burst arrive time and UL jitter range, UE reports additional information such as periodicity, UL config grant offset preference, C-DRX periodicity preference. The set of information to be included in the UAI shall be based on how network utilize it to improve the power saving and capacity performance.

# Conclusions

# In this contribution, we discuss the reporting of UE Assistance Information (UAI).

Observation 1: The UAI of UL traffic is useful at the network to allocation resources and optimize system performance in terms of data rates, latency, power savings, reliability, etc.

Proposal 1: Besides burst arrive time and UL jitter range, UE reports additional information such as periodicity, UL config grant offset preference, C-DRX periodicity preference. The set of information to be included in the UAI shall be based on how network utilize it to improve the power saving and capacity performance.

# References

[1] RAN2 Chairman Notes from RAN2#122, May 2023.

[2] R2-2304708 Discussion on XR awareness, Qualcomm Incorporated