3GPP TSG-RAN WG2 Meeting #123	R2-2307609

Toulouse, France, August 21-25, 2023

Agenda Item:	7.5.4.1

Source: 	Meta

Title:  	XR BSR and Delay Information Enhancements

Document for:	Discussion and Decision

# Introduction

In the RAN2#122 meeting, the following agreement was made:

- UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this. - When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

In this paper, we provide our viewpoints on buffer status report (BSR) and delay status report (DSR) for UL traffic.

# Discussion

To ensure optimal performance of XR services, it's crucial to enhance delay-awareness for UL traffic. Providing delay information such as remaining time can aid in achieving this goal. It's worth noting that UL traffic can have varying characteristics based on the types of applications/services. Periodic traffic, such as video, avatar, audio, and pose/control information, require timely delivery to maintain the immersive experience. On the other hand, aperiodic traffic like user inputs may not have strict timing requirements.

Observation 1: UL XR traffic can have varying characteristics based on the types of applications/services. Periodic traffic such as video/audio/avatar or pose control information require timely delivery while others such as user inputs may not have strict timing requirements.

Hence, to avoid the unnecessary signaling overhead, it’s desirable to only report the timing information for the traffic or QoS flow with strict timing requirements.

Proposal 1: UE only reports the delay status for QoS flows with stringent timing requirements.

In RAN2#122, it is FFS if UE reports one or multiple values. Because the delay status report is based on PDCP discard timer, we propose a one-to-one mapping between the PDCP discard timer and the reporting value.

Proposal 2: The delay status reporting contains one value per PDCP discard timer.

For BSR and DSR triggering conditions, on-demand delay status reporting ensures that reporting is only triggered when the low-latency traffic flow has been in the buffer for a long time and is experiencing delays in transmission. Upon receiving the delay status report, the network will respond by prioritizing the resources for the corresponding traffic flow. This will in turn improve the latency performance of the traffic flow.

Proposal 3: The delay status reporting is triggered when the low-latency traffic flow has been in the buffer for a long time. For example, when the remaining time falls under a pre-defined threshold, the delay status reporting is triggered.

# Conclusion

In this contribution, we consider the delay status report (DSR) for UL traffic.

Observation 1: UL XR traffic can have varying characteristics based on the types of applications/services. Periodic traffic such as video/audio/avatar or pose control information require timely delivery while others such as user inputs may not have strict timing requirements.

Proposal 1: UE only reports the delay status for QoS flows with stringent timing requirements.

Proposal 2: The delay status reporting contains one value per PDCP discard timer.

Proposal 3: The delay status reporting is triggered when the low-latency traffic flow has been in the buffer for a long time. For example, when the remaining time falls under a pre-defined threshold, the delay status reporting is triggered.

# References

[1] RAN2 Chairman Notes from RAN2#122, May 2023.

[2] R2-2306393 XR BSR and Delay Information Enhancements, Meta USA