3GPP TSG-RAN WG2 Meeting #123	R2-2307616

Toulouse, France, 21 - 25 August 2023

Agenda Item:	7.20.4

Source:	Huawei, HiSilicon

Title:	Intra-UE prioritization for STxMP

Document for:	Discussion and decision

# 1. Introduction

In the Rel-18 MIMO WID [1], one of the objectives with RAN2 impact is STxMP (Simultaneous Transmission across Multiple Panels), as shown in the following description.

6. Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission • The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation • For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this contribution, we discuss the potential impacts on RAN2 about intra-UE prioritization procedure of STxMP UEs.

# 2. Discussion

## 2.1 Intra-UE prioritization background

In Rel-16 Industrial IoT, intra-UE prioritization about how the UE deals with CG and CG conflict and with CG and DG conflict was specified in TS 38.321. The CG and CG conflict means that two configured grants have time domain overlap and the CG and DG conflict means that one dynamic grant and one configured grant have time domain overlap. In Rel-16, the UE is not required to use two UL grant resources simultaneously. Therefore, when the above conflicts happen, the UE will deprioritize one of the UL grant according to the priority rule. The priority value of an uplink grant is determined by the highest priority among priorities of the LCHs with data available that are multiplexed or can be multiplexed in the MAC PDU.

Observation 1: From Rel-16, intra-UE prioritization was specified based on the priority rule: when two UL grants (CG and CG or CG and DG) have time domain overlap, the UE deprioritizes one UL grant with lower priority and does not use this deprioritized UL grant resource.

## 2.2 Rel-18 STxMP

In Rel-18 MIMO, according to the WID [1] and RAN1 agreements, STxMP is supported for PUSCH and PUSCH simultaneous transmission, and PUCCH and PUCCH simultaneous transmission. Regarding PUSCH and PUSCH simultaneous transmission, from RAN2 perspective, the UE needs to be able to use two UL grants which have time domain overlap.

Observation 2: In Rel-18 STxMP, the UE is able to use two UL grants when they have time domain overlap, instead of deprioritizing one of the UL grant.

There are mDCI STxMP and sDCI STxMP. In mDCI STxMP scenario, there could be two UL grants for two TRPs. With the current spec, the UE will only use one of the UL grant for this time transmission, which is not the intention of STxMP. Therefore, for mDCI STxMP scenario, how to specify the UE behaviour in the above case needs to be discussed in RAN2. Regarding sDCI STxMP scenario, the current spec seems fine since there is only one UL grant. RAN2 can discuss mDCI STxMP scenario first.

When two overlapping UL grants are for mDCI STxMP, discuss how to specify the UE behaviour to avoid using only one UL grant.

## 2.3 Intra-UE prioritization for STxMP

The two UL grants for STxMP (e.g., CG1 and CG2) may be overlapping with a third UL grant for single TRP (e.g., DG1). More information from RAN1 is needed about whether the above case is allowed. If it is allowed, RAN2 should discuss the intra-UE prioritization rule accordingly.

Ask RAN1 whether it is allowed that the STxMP UL grants are overlapping with another single TRP or STxMP UL grant(s). If it is allowed, RAN2 should discuss the intra-UE prioritization rule for STxMP.

# 3. Conclusion

Based on the discussion in this contribution, we have the following proposals:

- When two overlapping UL grants are for mDCI STxMP scenario, discuss how to specify the UE behaviour to avoid using only one UL grant.

- Ask RAN1 whether it is allowed that the STxMP UL grants are overlapping with another single TRP or STxMP UL grant(s). If it is allowed, RAN2 should discuss the intra-UE prioritization rule for STxMP.

# 4. Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung