3GPP TSG-RAN2 Meeting #123	R2-2307637

# Toulouse, France, Aug 21-25, 2023

Agenda item:	7.8.5

Work Item:	NR_UAV-Core, LTE_UAV_enh-Core

Source: 	Qualcomm Incorporated

Title: 	Remaining aspects of PC5-based BRID and DAA support

Document for:     Discussion/Decision

# Introduction

RAN#99 approved a (revised) WI for NR UAV [1]. One of the objectives is the following:

3. Specify the support for UAV identification broadcast (BRID) in NR PC5. Support of DAA using the same framework as BRID without DAA specific enhancements can be considered [RAN2]. Note:. UAV use of NR PC5 is to be used only in designated bands as defined in regulation for BRID/DAA use.

RAN#99 also approved a parallel WI for LTE UAV [2], where one of the objectives is the following:

1. Specify the support for UAV identification broadcast (BRID) in LTE PC5. Support of DAA using the same framework as BRID without DAA specific enhancements can be considered [RAN2]. Note: UAV use of LTE PC5 is to be used only in designated bands as defined in regulation for BRID/DAA use.

Based on TR 23.700-58 section 8.2 and 8.3, SA2 conclusions proposed to progress to normative phase PC5-based solutions for the Broadcast Remote ID (BRID) and Detect And Avoid (DAA) in accordance with solution #5 in the SA2 TR.

From RAN2 point of view, to enable PC5-based BRID and DAA, at least the following would be needed to be specified by RAN2:

- Configuration of parameters for A2X communication for the required scenarios

- Procedures for A2X communication over PC5 reference point.

- UE capability signaling

RAN2#121 discussed and agreed the following:

Agreements:

-	PC5-U is used to support BRID for UAV

-	Support both in-coverage and out-of-coverage scenarios

-	Mode 2 will be supported.  FFS whether further mode 1 will be supported.

-	FFS whether separate pools are needed

-	FFS whether current configurations can support UAV requirements

Further, RAN2#121bis-e discussed and agreed the following:

Agreements:

- DAA can be supported using the same framework as used for BRID transmission over the LTE and NR PC5 interface, without any specific enhancements.

- LTE PC5 Mode-4 resource allocation is supported, and LTE PC5 Mode-3 is not supported

- NR PC5 mode-1 is not supported

- For LTE PC5, we will follow the NR PC5 framework agreements, unless explicitly identified e.g. a strong technical reason

- RAN2 assumes that BRID and DAA services will be delivered on a frequency designated by regulators

- As a baseline, we will use the existing V2X QoS framework.  FFS whether different resource pools are needed for UAV services

- No further enhancement on PC5 range for A2X broadcast services will be pursued in this release

- We will not investigate interference for BRID and DAA broadcast

RAN2#121bis-e also sent an LS to SA2, see R2-2304564.

# UE capability for A2X communication

A2X communication and Aerial UE communication is captured as following in the stage 2 running CR.

A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, e.g. BRID or DAA.

Aerial UE communication: functionality enabling Aerial UE function, as defined in 16.x.

From the above, we note the following:

- A2X includes at least BRID and DAA

- A2X uses PC5 based communication, also known as SL (sidelink) which is also basis for V2X.

## Existing SL/Discovery/Communication/V2X capabilities

Release 12/13 introduced D2D sidelink discovery and communication related UE categories as well as UE capabilities to LTE. See TS 36.306 for more information.

- New UE categories: ue-CategorySL-C-RX, ue-CategorySL-C-Tx, ue-CategorySL-D.

- New Sidelink capability parameters: commSupportedBandsPerBC-r12, commSupportedBands-r12, commSimultaneousTx-r12, discSupportedBands-r12, discScheduledResourceAlloc-r12, disc-UE-SelectedResourceAlloc-r12 disc-SLSS-r12 etc. (See 4.3.21.x in TS 36.306)

If a UE supports sidelink communication on at least one band, the UE shall support:

- sidelink communication transmission based on UE autonomous resource selection,

- eNB scheduled resource allocation,

- ProSe Per Packet Priority (PPPP) handling

- out of coverage sidelink discovery

- 16 sidelink processes (8 for V2X) for reception of SL-SCH.

TS 36.300 states the following: Support of V2X services via PC5 interface is provided by V2X sidelink communication as specified in TS 23.285 and/or NR sidelink communication as specified in TS 23.287, which are modes of communication whereby UEs can communicate with each other directly over the PC5 interface.

Rel-14/15/16 added a host of SL capabilities related to V2X, e.g. zoneBasedPoolSelection-r14 v2x-HighReception-r14 v2x-eNB-Scheduled-r14 ue-AutonomousWithFullSensing-r14 ue-AutonomousWithPartialSensing-r14 etc.

Similarly, NR sidelink capabilities are captured in TS 38.306 where there are several upper layer capabilities and dozens of PHY layer capabilities. E.g.,

- Upper layer: outOfOrderDeliverySidelink-r16, am-WithLongSN-Sidelink-r16, um-WithLongSN-Sidelink-r16, drx-OnSidelink-r17, lcp-RestrictionSidelink-r16 etc.

- PHY: p0-OLPC-Sidelink-r17, supportedBandCombinationListSidelinkEUTRA-NR-r16, supportedBandCombinationListSidelinkNR-r16, sl-Reception-r16, sl-TransmissionMode1-r16, sl-TransmissionMode2-r16, sync-Sidelink-r16 etc.

## UE capabilities for A2X

Now, it is clear that there are many capability indication parameters already available in the specification. However, several questions need to be looked into, such as:

- should support of A2X be mandatory for all UAVs?

- should support of NR SL be mandatory to support A2X?

- should support of V2X be mandatory to support A2X?

- can the V2X capabilities be reused or do we need new capabilities?

For the first question, it should be noted that support of PC5-based BRID and/or DAA may not be mandatory in all regions (i.e. it depends on regional regulations). Therefore, it should be optional for the UAV to support PC5-based BRID and/or DAA.

It is optional for UAV to support PC5-based BRID and/or DAA.

For the second question, it may initially look straightforward to say it should be conditional mandatory for A2X supporting UEs to support NR SL. However, at a closer look, we can see that supporting NR SL means mandatory support of at least the following (based on CY indication in the capability table in 38.306), some of which may or may not be relevant to A2X.

  - sl-Reception-r16:  indicate NR SL rx support. Mandatory for NR SL UE

  - sl-TransmissionMode1-r16: mandatory for NR SL UE supporting licensed spectrum (gNB operated/managed)

  - sl-TransmissionMode2-r16: mandatory for NR SL UE

  - sync-Sidelink-r16: mandatory for NR SL UE

  - congestionControlSidelink-r16: mandatory for NR SL UE

  - psfch-FormatZeroSidelink-r16: mandatory for NR SL UE

  - csi-ReportSidelink-r16: mandatory for NR SL UE

  - sl-openLoopPC-RSRP-ReportSidelink-r16: mandatory for NR SL UE

It is worthwhile to have a closer look on some of the aspects here.

For example, RAN2 agreed to support only SL Mode 2 for A2X, however, in the above we see that Mode1 is mandatory to be supported if the UE supports licensed spectrum. Imagine a law enforcement unit on the ground or while flying to only collect info on all drones around it may only need to receive all BRID information from other UEs, but may not need to transmit any A2X messages to the other UAVs. For such UE, transmitter functionalities may not be needed, and complexity/cost can be reduced. In such case, several NR SL parameters e.g. sl-TransmissionMode1-r16, sl-TransmissionMode2-r16 etc. may not need to be made conditional mandatory.

As another example, consider a use case of special A2X UE belonging to law enforcement, which may need to be capable of receiving the A2X messages e.g. BRID but not need to transmit any A2X messages (i.e., receive-only); while some other aerial UAV may only need to transmit the BRID but no need to receive any A2X messages (i.e., transit-only). It seems unnecessary to mandate the transmit-only UE to support sl-Reception-r16. Also, there is no feedback for broadcast BRID/DAA, then it seems not necessary to mandate the UE to support psfch-FormatZeroSidelink-r16. As noted above, RAN2 agreed to support BRID/DAA using SL Mode-2. According to 38.300, the SL UE in Mode 2 can support partial sensing-based resource allocation and random resource selection as power saving resource allocation methods. A SL mode 2 TX resource pool can be (pre)configured to allow full sensing only, partial sensing only, random selection only, or any combination(s) thereof. So, UEs in this mode may do random resource selection based on pre-configuration, and can avoid supporting even partial or full sensing, reducing the complexity and cost. On the other hand, some UEs which need to only support transmission functions may have capability of performing (full or partial) sensing for A2X resource selection, but no other receive functionality.

For the third and fourth questions, V2X and A2X have different use case scenarios. It may not be reasonable to always mandate UAV to support V2X features as a prerequisite of supporting A2X.

Therefore, RAN2 should discuss these potential use cases and decide how to define and specify A2X capability signalling for UAVs.

Discuss potential A2X use cases (e.g. receive-only, transmit-only) and decide how to define and specify A2X capability signalling for UAVs.

# SL resource configuration for A2X communication

RAN2 briefly discussed whether separate resource pools would be needed for PC5 broadcast of BRID. RAN2 also agreed that PC5 mode 2 will be supported.

## Separate SL resource pool for BRID/DAA

For PC5 resource allocation, 2-stage SCI is used. The second stage SCI format 2-A includes a field about ‘Cast type’. The existing SL resource allocation is only for different Cast types and predefined priority associated with PQI, but no QoS flows or traffic types with requirement of different communication range with interference coordination.

BRID/DAA message transmissions may require different QoS (e.g. different PQI/ packet delay budget/ packet error rate) compared to legacy SL use cases.

In the LS to SA2 in R2-2304564, RAN2 explained the following and asked related question:

Secondly, RAN2 briefly discussed whether enhancements are needed to support the QoS requirements for BRID and DAA.  RAN2 agreed to use the existing V2X QoS framework and captured FFS whether different resource pools are needed for A2X services and FFS whether current configurations can support A2X requirements. In order to progress on these open aspects, RAN2 would like to ask SA2 whether BRID and DAA broadcast over LTE and NR PC5 require new QoS requirements and parameters not supported within the ranges supported for V2X.

2. reply whether BRID and DAA broadcast over LTE and NR PC5 require new QoS requirements and parameters not supported within the ranges supported for V2X.

To the above question, SA2 replied in R2-2307059/ S2-2307781 as follows:

SA2 answer: SA2 discussed that defining new PQI(PC5 5QI) value(s) is beneficial for BRID and DAA over NR PC5 so that the AS layer can distinguish A2X services from V2X services if differentiated handling is needed. The related CR work will be progressed in Rel-18. For BRID and DAA over LTE PC5, it is considered that the existing QoS framework can be used.

Although different QoSs enable early indication of BRID or DAA, the resource pool sharing between BRID and DAA introduces potential interference impact. For example, imagine two nearby UAVs trying to communicate DAA which is more important than a faraway UAV providing its BRID information. The DAA carrying timely information from a nearby UAV may be blocked by the BRID message transmitted from a UAV far away. Therefore, in our view, separate SL resource pool for BRID/DAA broadcast is beneficial.

Due to different potential QoS characteristics, transmit power etc., separate SL resource pool for BRID and DAA broadcast is beneficial.

Once RAN2 agrees on the principal that separate SL resource pool can be configured for BRID, details can be discussed further on how to configure it. Therefore, we propose:

Separate SL resource pool for BRID and DAA broadcast is supported.

## Height dependent SL transmission configuration

Note that for V2X, different (pre)configurations for PSSCH transmission parameters for different speed are currently supported. For low speed and high speed, PSSCH transmission parameters including MCS, sub-channel number, retransmission number, max power can be separately (pre)configured.

SL-PSSCH-TxConfigList-r16 ::= SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r16)) OF SL-PSSCH-TxConfig-r16

SL-PSSCH-TxConfig-r16 ::=        SEQUENCE {

sl-TypeTxSync-r16                SL-TypeTxSync-r16             OPTIONAL,    -- Need R

sl-ThresUE-Speed-r16             ENUMERATED {kmph60, kmph80, kmph100, kmph120,

kmph140, kmph160, kmph180, kmph200},

sl-ParametersAboveThres-r16      SL-PSSCH-TxParameters-r16,

sl-ParametersBelowThres-r16      SL-PSSCH-TxParameters-r16,

...,

[[

sl-ParametersAboveThres-v1650    SL-MinMaxMCS-List-r16         OPTIONAL,    -- Need R

sl-ParametersBelowThres-v1650    SL-MinMaxMCS-List-r16         OPTIONAL     -- Need R

]]

}

V2X supports speed dependent SL transmission configuration.

For UAVs, different heights result in variant channel conditions and interference impact. Therefore, different configurations of PSSCH transmit parameters for different UE heights should be supported. For example, for a UAV flying in the air, the PSSCH may be restricted to transmit periodic short messages with lower TBS (e.g., BRID/DAA) using limited number of subchannels, reducing the strong LOS interference; for a UAV on the ground, it is more similar to terrestrial UEs, PSSCH is allowed to transmit contiguous messages with larger TBS (e.g., video or software updating, etc.) using more subchannels. Whether to support only one height threshold (and consequently one configuration below and one above threshold) or multiple height thresholds/ranges can be discussed further.

UAVs flying in the air and on the ground may need different restriction for SL transmission configuration.

Height-dependent SL transmission configuration for A2X is supported. Details FFS.

# Summary

In this contribution, we discussed on PC-5 based transmission of BRID and DAA and made the following observations:

Observation 1.	Due to different potential QoS characteristics, transmit power etc., separate SL resource pool for BRID and DAA broadcast is beneficial.

Observation 2.	V2X supports speed dependent SL transmission configuration.

Observation 3.	UAVs flying in the air and on the ground may need different restriction for SL transmission configuration.

Based on the discussion and observations, we propose:

Proposal 1.	It is optional for UAV to support PC5-based BRID and/or DAA.

Proposal 2.	Discuss potential A2X use cases (e.g. receive-only, transmit-only) and decide how to define and specify A2X capability signalling for UAVs.

Proposal 3.	Separate SL resource pool for BRID and DAA broadcast is supported.

Proposal 4.	Height-dependent SL transmission configuration for A2X is supported. Details FFS.

# References

[1] RP-230782, Revised WID: NR support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023

[2] RP-230783, New WID: Enhanced LTE Support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023