3GPP TSG-RAN WG2 Meeting #123	R2-2308016

Toulouse, FR, 21st – 25th August 2023

Agenda item:	 7.13.4

Source:		Lenovo

Title:		SON enhancements for SPR

Document for:		Discussion and Decision

1	Introduction

In RAN2#119bis-e meeting [1], agreements on SON enhancements for successful PSCell change report were achieved:

Agreements

1	RAN2 confirms the scenarios for SPR for NR-DC, including:

•	SN- and MN-initiated classic PSCell change / CPC

•	Intra-SN classic PSCell change / CPC

•	Classic Addition / CPA

1a	RAN2 will discuss HO with SN change later, after the basic solution for SPR is known

2	Given that PSCell addition is proposed by all companies, SPR is used as the abbreviations to use for the feature.

3	RAN2 confirm to prioritize NR-DC scenario for SPR.

4	SHR solution is taken as baseline for the SPR in terms of configuration and reporting at high level. Details of the configuration and report need to be tailored/customized/new message per use case.

5	Network configures SPR configuration IE for the UE, with at least the following triggering conditions:

•	T310 triggering condition

•	T312 triggering condition

•	T304 triggering condition

5a: Other triggering conditions are FFS

5b: Values of the triggering conditions are FFS

5c: Which node configures the triggering condition is FFS.

6	RAN2 agree to the following:

A.	SPR configuration is configured by network through otherConfig

B.	SPR is fetched via UE Information Request/Response procedure

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell.

7b: FFS on whether to include or on conditional inclusion of random access related information.

7c:	FFS on Location Information

In RAN2#120 meeting [2], more agreements were achieved:

Agreements:

3	Only MN can retrieve the SPR from the UE.

4	For Q8, RAN2 agree following options: depends on which of nodes initiates SPR, i.e.:

For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE

For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN.

T304 trigger needs to be configured by the target SN node.

Agreements:

1	UE stores both SPCR and SHR configuration (one for each type at most) if received from NW.

2	UE can send the (stored) SPR to gNB. FFS how long UE keeping SPR is FFS.

3	Only the latest successful PSCell change/addition is reported by the UE.

4	Random access related information is included in SPR at least when the SPR is triggered due to T304 exceeds the configured threshold. Other conditions are FFS.

5	UE records/reports PCell SHR and PSCell SPR separately

In RAN2#121 meeting [3], further agreements were achieved:

Agreement

1: UE includes available location information in SPR .

2: UE stores SPR at most 48 hours after the last successful PSCell addition/PSCell change report is stored at UE if not fetched.

3:  At least the following options are needed for releasing SPR report:

a. New SPR is initiated

b. Upon retrieval of SPR

c. Detach is initiated.

4:  In SPR, reuse CHO candidate cell flag to indicate whether a neighbor cell is CPAC candidate cell or not.

In RAN2#122 meeting [4], it was agreed that

Agreements:

SPR

1	 For values of triggering conditions of SPR, Percentage based threshold variables for SHR (T310/T312/T304) can be reused for SPR is applied.

Due to limited time left for R18 SON/MDT WI, RAN2#122 meeting also agreed that SPR except the critical issues will not be further enhanced from this meeting until the end of R18. But we find that RAN3#120 has sent an LS [5] to ask RAN2 about several questions, in this paper, we would mainly discuss the questions raised by RAN3.

2	Discussion

Based on RAN3#119 meeting’s agreements, in case the SPR is retrieved in a “new node” (different from the node that sent the SPR configuration to the UE i.e., “old MN”), the SPR is always sent from the “new node” to the “old MN” which then forwards to the respective node(s) which should perform the SPR optimization. It is needed for the “old MN” identify the UE context when receiving the SPR. RAN3#120 meeting discussed this issue, and sent an LS [5] to RAN2, mentioning that:

RAN3 discussed how to retrieve mobility information related to a UE in the nodes involved in the mobility procedure when Successful Handover Report (SHR) and Successful PSCell Change Report (SPR) related optimizations are to be performed in the network nodes involved in the mobility procedure. One solution discussed was whether UE can report the source C-RNTI and time since receiving HO/PSCell change/addition command and retrieving SHR/SPR to assist in the network node receiving the SHR/SPR identifying information related to a UE (e.g. UE related configuration of interest, mobility strategies).

But since the possibility to retrieve information related to a specific UE depends on gNB's implementation. e.g. whether it can store the UE related configuration of interest during the wanted time (up to 48 hours) after a successful HO or PSCell change/addition, RAN3 discussed an alternative solution to enable the retrieval of UE related information without requiring this to be stored on a per UE basis.

RAN3 is therefore studying whether another solution can be defined for newly defined scenarios since Rel-18 as well as intra-RAT SHR, whenever the configuration used by the UE needs to be retrieved in order to perform SON optimizations.

One potential solution RAN3 discussed is as follows:

- The node involved in mobility procedure (via implementation) create references to a configuration or a set of parameters used by one or a group of UEs and can send this “Configuration Information” to the UE in order to assist in the analysis of SHR or SPR or other SON reports (if needed).

- this “Configuration Information” can be optionally sent to the UE in dedicated signaling (e.g., together with the SHR/SPR configuration or in any other RRCReconfiguration). If received, UE should then store this “Configuration Information” together with the SON report(s) and UE should report it back to the gNB along with the SON report(s) (e.g., SHR/SPR).

- How to encode this “Configuration Information” is up to RAN2, taking into account RAN3 guidance related to the size of this information. One example to do this would be to encode this as an OCTET STRING (e.g., 32 bits) as is done for Mobility Information in XnAP (i.e., TS 38.423).

- This solution shall cover all existing mobility scenarios, including dual-connectivity scenarios.

We can find that based on RAN3’s LS, there are two options on table,

- Option 1: the UE includes C-RNTI of the PCell which sent the SPR configuration to the UE, and time since receiving PSCell change/addition command and retrieving SPR in the SPR, based on these two parameters, the old MN identifies UE context.

- Option 2: the old MN sends “Configuration Information” (e.g. a configuration or a set of parameters used by one or a group of UEs) to the UE together with the SPR configuration, then the UE include this “Configuration Information” back in SPR.

In the LS, RAN3 asked several questions to RAN2, including:

- Q1: Whether RAN2 sees any issues in defining a solution for “Configuration Information” as described above?

- Q2: For SHR/SPR, is there any issue to include this “Configuration Information” in the RRC Reconfiguration message with sync containing Handover Command or PSCell change command?

- Q3: In cases when this “Configuration Information” is not configured by the network to the UE, RAN3 discussed whether UE can include the source cell C-RNTI and the time between the event that triggered the report and the sending of the report to the network. RAN3 wants to check with RAN2 if it’s feasible in the above scenario?

Currently, last serving PCell ID, C-RNTI and time information (e.g. timeSinceFailure) are included in the RLF report, based on these information reported by the UE, network can retrieve the UE context. We can find that using C-RNTI and time related information to identify UE context is a usual way.

This principle for RLF report can be applied to SPR without any problem, i.e. in Option 1, the UE includes C-RNTI of the PCell which sent the SPR configuration to the UE, and time since receiving PSCell change/addition command and retrieving SPR in the SPR, it is a simple and straightforward solution to enable old MN to retrieve the UE context or the configuration used for the UE, e.g. by mapping the C-RNTI of the PCell to a specific UE context in the specific time period.

From RAN2 point of view, Option 2 i.e. defining a solution for “Configuration Information” seems complex and unnecessary. In Option 2, RAN2 needs to further encode and design “Configuration Information”, which means that Option 2 needs more technical discussion and specification impacts. But as we agreed in last RAN2 meeting, RAN2 would not further discuss SPR except the critical issues from this meeting until the end of R18, therefore, RAN2 would not specify “Configuration Information”.

Proposal 1: For retrieval of UE context at “old MN” which sent the SPR configuration to the UE, the UE can include C-RNTI of the PCell which sent the SPR configuration to the UE, and time between PSCell change/addition command and SPR retrieval in the SPR.

Further, RAN3 also discussed the following information to be reported in the SPR to assist in the forwarding of SPR over network interfaces:

- CGI of the PCell which sent the SPR configuration (already agreed in RAN3);

- Indication whether the PSCell change was MN-initiated or SN-initiated (WA but not agreed in RAN3). Explicit or implicit indicator can be decided by RAN2.

In the LS, RAN3 also asked Q4 to RAN2, i.e. “RAN3 kindly asks RAN2 to confirm RAN3’s understanding on CGI of the PCell and update their specifications if feasible, and also take the above information for indication on whether the PSCell change was MN-initiated or SN-initiated into account and provide feedback”.

From RAN2 point of view, CGI of the PCell which sent the SPR configuration is needed in the SPR for SPR forwarding from the receiving node to the old MN. However, since old MN knows whether the PSCell change was MN-initiated or SN-initiated when it identifies the UE context, an indication reported from the UE on whether the PSCell change was MN-initiated or SN-initiated is not needed.

Proposal 2: Include CGI of the PCell which sent the SPR configuration to the UE in the SPR.

Proposal 3: An indication on whether the PSCell change was MN-initiated or SN-initiated is not needed in the SPR.

3	Conclusion

In this contribution, SON enhancements for SPR are further discussed. We have the following proposals:

Proposal 1: For retrieval of UE context at “old MN” which sent the SPR configuration to the UE, the UE can include C-RNTI of the PCell which sent the SPR configuration to the UE, and time between PSCell change/addition command and SPR retrieval in the SPR.

Proposal 2: Include CGI of the PCell which sent the SPR configuration to the UE in the SPR.

Proposal 3: An indication on whether the PSCell change was MN-initiated or SN-initiated is not needed in the SPR.

Reference

[1] Chairman notes for RAN2#119bis-e meeting

[2] Chairman notes for RAN2#120 meeting

[3] Chairman notes for RAN2#121 meeting

[4] Chairman notes for RAN2#122 meeting

[5] R3-233380, LS on SHR and SPR, Samsung