3GPP TSG-RAN WG2 Meeting #123	R2-2308089

Toulouse, France, August 21-25, 2023

Agenda item:	7.17.2

Source: 	Intel Corporation

Title: 	Common framework for proactive and reactive approach for MUSIM

Document for:	Discussion and decision

# Introduction

RAN2 made the following agreements related to proactive signalling of capability reduction for MUSIM:

Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

RAN2 also agreed a reactive approach:

6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

A reactive approach for signalling capability restriction is also to be supported with the aim to support a common framework for both approaches.  This document discusses commonality and differences between the proactive and reactive approaches and suggests a common signalling framework.

# Discussion

## Proactive and Reactive approach

We first look at what is considered fundamentally a proactive and reactive approach:

Proactive approach: As captured in the agreements from last meeting, a proactive approach is wherein the UE can request (if allowed by the NW) capability restrictions which can be independent of current RRC configuration.

A reactive approach is where the UE can request a capability restriction on a current RRC configuration.

In a proactive approach, as agreed, the network can still control whether the UE can signal such a capability restriction.  If allowed, the UE can signal a capability restriction in Network A due to a configuration in Network B, even if the restriction in Network A is not currently configured.  This is similar to the restrictions agreed for IDC.  In IDC, the UE assistance information is used to provide the capability restriction to the network, and this can be provided for the frequencies of interest to the network independent of the current RRC configuration.  The same framework for IDC can be adopted as the baseline for MUSIM proactive approach.

Proposal #1: Adopt a solution for signalling MUSIM proactive capability restriction similar to the one agreed for IDC, where UE can provide capability restriction using UE assistance information on (including non-serving) frequencies that are requested (e.g., frequencies that are of interest) by the network.

Having a common solution for proactive and reactive will make specification and implementation simpler.  As the signalling requirement and the intention is mostly the same for both, which is to inform the network to release or not use a carrier/MIMO etc., a common solution should be considered as the baseline.

For the reactive approach, RAN2 agreed to explicitly signal the release of a serving cell (group), but the details of the signalling was FFS.

One option would be to use a MAC CE based release of the serving cell.  For MUSIM, there is no urgency to quickly release a serving cell.  And the frequency of requesting release of a serving cell is not high.  Further, the release of the serving cell/group is to be done over RRC by the CU – so providing this information over MAC CE to the DU will require additional signalling over F1 interface.  Another complication with using MAC CE to release a serving cell is that it is not possible to reuse the mechanism to indicate that the capability restriction is finished and network can set up the cell again.

Observation #1: Using MAC CE to indicate release of serving cell has no benefit and has several drawbacks.  It also cannot be used to indicate end of capability restriction to allow network to reconfigure the cell (group).

The same solution for proactive approach using UE Assistance information can also be used for reactive approach, with a couple of minor differences: reactive approach can also consider the existing UE configuration on top of the requested frequencies to provide feedback to the network.   Whether to optimize the signalling for the serving cell can be discussed if there is sufficient interest.

Proposal #2: Same signalling mechanism as for proactive approach (and IDC) using UE assistance information is adopted also for reactive approach.   MAC CE based solution is not considered.

# Summary and proposals

This document discussed the proactive and reactive approach for signalling capability restriction for MUSIM. The commonality and differences between the two approaches was also discussed.  The following proposals and observations were proposed:

Proposal #1: Adopt a solution for signalling MUSIM proactive capability restriction similar to the one agreed for IDC, where UE can provide capability restriction using UE assistance information on (including non-serving) frequencies that are requested (e.g., frequencies that are of interest) by the network.

Observation #1: Using MAC CE to indicate release of serving cell has no benefit and has several drawbacks.  It also cannot be used to indicate end of capability restriction to allow network to reconfigure the cell (group).

Proposal #2: Same signalling mechanism as for proactive approach (and IDC) using UE assistance information is adopted also for reactive approach.   MAC CE based solution is not considered.