3GPP TSG-RAN WG2 Meeting #123                                                               R2-2308103

Toulouse , FR, Aug 21st-25th, 2023

Title: 	Further discussion on the support of multi-path relaying

Source: 	ZTE, Sanechips

Agenda item:	7.9.4

Document for:	Discussion and Approval

Introduction

In this paper, we will focus on the remaining issues specific for scenario1 or common for scenario 1 and scenario 2. Our point of view are presented.

Discussion

Non-split SRBs over indirect path

In last RAN2 meeting, it is still FFS on whether non-split SRB1/2 can be configured on indirect path. And some companies have concern on if there are cases where the configuration of non-split SRBs over indirect path is useful.

From our view, following cases can be considered:

- Case1: Network implementation.  When both direct path and indirect path are available for UE to initiate initial access, according to current specification, it’s up to UE implementation to select the path used for initial access. It is possible that UE will select indirect path to perform initial access, then the SRB1/2 bearer type is indirect bearer. And after adding a direct path by gNB, gNB may do not want to switch the bearer type of SRB1 and SRB2.

- Case2: Direct path experiences bad channel condition or direct path is in congestion. For example, the RSRP of direct path is poor. In this case, although direct path is available, it is still reasonable for UE to be configured with indirect path SRB.

Furthermore, some companies think if SRB1 is configured as non split  indirect bearer, when RLF is detected on indirect path, and direct path can not be used for indirect path RLF report. It looks strange to perform RRC re-establishment since direct path is available.

From our view, UE’s behaviour is controlled by network. UE will perform RRC re-establishment when direct path is available due to SRB1 is configured as non split  indirect bearer. Based on network implementation, gNB knows the state of UE clearly, and choose to configure SRB1 as a non split indirect bearer. This means, from network perspective, it is acceptable to make UE to perform RRC re-establishment when direct path is available.

- For scenario 1, the bearer type (i.e. direct bearer, indirect bearer, or multi-path split bearer) of SRB1 and SRB2 may be configured by the gNB independently. It is not necessary to mandate the same bearer type configuration of SRB1 and SRB2.

Triggering the RRC_IDLE/RRC_INACTIVE target relay UE to initiate RRC connection establishment

In previous RAN2 meeting, during the discussion of bringing IDLE/INACTIVE relay UE to RRC connected state, legacy R17 behaviour(RLC0 or RLC1 message triggering) will be re-used when split SRB1 is configured. For other cases, i.e. the SRB1 is not configured or suspended on indirect path, PC5-RRC signaling trigger will be used.  Additionally, one FFS issue is left : if a Rel-17 relay UE is supported for use with multi-path and how the PC5-RRC signaling trigger can be supported.

From our view, whether a R17 Relay UE is supported during indirect path addition depends on following aspects:

1. whether a R17 relay UE can be discovered by remote UE during indirect path addition,

2. whether a R17 IDLE/INACTIVE relay UE can be brought to RRC connected state,

3. whether a R17 relay UE can forward indirect path traffic of MP remote UE.

For the first issue, as far as we know, for both single path and multi-path relay, the relay UE is a L2 U2N relay. The discovery message and procedure during indirect path addition re-use legacy discovery procedure, no enhancement is made. As a result, R17 relay UE can be discovered during indirect path addition procedure.

For the second issue, as we agreed in last meeting, legacy R17 behaviour is re-used for bring relay UE enter into RRC Connected, i.e. SRB1 is configured on indirect path via split bearer or indirect bearer, and transmitting the RRCReconfigurationComplete message via SRB1 and SL-RLC1.

For the third issue, after relay UE enter into RRC connected state, the relay UE will forward the remote UE’s RRCReconfigurationComplete received from SL-RLC1 to network. After receiving RRCReconfigurationComplete  message, gNB identifies the association between the remote UE and relay UE. After that, gNB configure bearer mapping to relay UE as legacy. This procedure is similar with legacy direct to indirect path switch.

In conclusion, with legacy R17 trigger behaviour, a R17 relay UE can be supported for MP relaying.

- With legacy R17 trigger behaviour, a R17 relay UE can support MP relaying.

If a IDLE/INACTIVE relay UE is discovered during indirect path addition, due to lack of release information, both remote UE and gNB do not know the release information of the relay UE. In this case, gNB can not configure remote UE to use PC5 RRC trigger mechanism arbitrarily, i.e. only direct SRB1 is configured. So,

Therefore to support PC5 RRC triggers, we may need further study how to obtain the release information or how to handle the case that R17 relay UE can not identify the PC5 RRC triggers signaling.  Corresponding study is time consuming and we do not see too much benefit to do such optimization.

- To support PC5 RRC triggers, we may need further study on how to obtain the release information or how to handle the case that R17 relay UE can not identify the PC5 RRC triggers signaling.  Corresponding study is time consuming and we do not see too much benefit to do such optimization.

Without release information, if gNB configure remote UE to use PC5 RRC triggered mechanism and the relay UE is a R17 IDLE relay UE, then we think gNB will check whether such relay UE enter into RRC connected or not via implementation method. And if gNB find that the relay UE does not enter into RRC connected, gNB can select another relay UE or configure split SRB1 to remote UE.

Therefore, considering many higher priority issues related to MP are pending on RAN2, it is suggested to revert previous RAN2 agreement to do not support PC5 RRC triggers, or at least not purse any new solution on this.

- It is suggested to revert previous RAN2 agreement and not support PC5 RRC triggers, or at least not purse any new solution on this, i.e. up to gNB implementation.

Packet split

In DC scenario, when more than one RLC entities are associated to one PDCP entity, UE can split the packet to any RLC entities to improve the data rate. Similarly, one benefit of the MP transmission is high data rate by transmitting packet on two paths simultaneously.

It is worth noting that UE can submit the packet to any RLC entities by implementations in Uu interface. The reason why leave the decision of split packet submissions to UE implementation is that UE connect with network via two direct path in DC, UE can estimate the channel condition and data rate by itself directly.

When it comes to SL MP, for direct path, UE connect with gNB directly and can also perform estimation by itself. However, for indirect path, UE connect with gNB via a relay UE. The remote UE does not know the channel condition and data rate between relay UE and gNB.

- Remote UE does not know the channel condition between relay UE and gNB.

In this case, remote UE can only perform split packet submission based on the estimation of direct Uu interface with gNB and PC5 interface with relay UE. We think such estimation is not accurate due to absent necessary transmission information between relay UE and gNB.

To solve this issue, to help remote UE performing packet split, it is suggested to make relay UE to inform some transmission assistance information to remote UE.

- Relay UE informs assistance information of indirect path to remote UE on how to split the packet.

RLF handling

In last RAN2 meeting, RAN2 agreed that when reporting direct-path failure via indirect-path, use MCGFailureInformation message.

For indirect path failure, MCGFailureInformation can also be re-used. Since remote UE only have one cell group irrespective of two paths are served by same DU or different DUs. Then, it is straightforward to re-use MCGFailureInformation to report the PC5-RLF and corresponding T316 can be re-used.

- Re-use MCGFailureInformation to report the PC5-RLF.

For direct path failure, since direct path is legacy Uu interface, MCG failure information has include Uu interface failure type as shown in following, no additional IE is needed for this.

FailureReportMCG-r16 ::= SEQUENCE { failureType-r16 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx,t312-Expiry-r16, lbt-Failure-r16, beamFailureRecoveryFailure-r16, bh-RLF-r16, spare1} OPTIONAL, measResultFreqList-r16 MeasResultList2NR OPTIONAL, measResultFreqListEUTRA-r16 MeasResultList2EUTRA OPTIONAL, measResultSCG-r16 OCTET STRING (CONTAINING MeasResultSCG-Failure) OPTIONAL, measResultSCG-EUTRA-r16 OCTET STRING OPTIONAL, measResultFreqListUTRA-FDD-r16 MeasResultList2UTRA OPTIONAL, ... }

For indirect path failure, indirect path is a PC5 interface which is not covered by legacy MCG failure information, new IE is needed to report PC5 interface failure. For the detailed type of PC5 interface failure, e.g. (T400 expiry, PC5 RLC maxnumre-tx), since all these type is considered as RLF in PC5 interface as shown in following, so no need to differentiate PC5 failure type:

1> upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or 1> upon T400 expiry for a specific destination; or 1> upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or 1> upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination: 2> consider sidelink radio link failure to be detected for this destination;

- Add a new failure type value of indirect path failure in MCF failure type, no need to differentiate detailed PC5 failure type.

UE aggregation requirement/capability

For scenario2, it is FFS whether remote-UE reports the RRC_IDLE / RRC_INACTIVE relay-UE ID for indirect path addition and which ID to report.

From our view, the report of relay UE’s ID only shows the preference and capability of MP for UE aggregation. Whether remote UE can be configured with MP is decided by gNB after receiving the report. Suppose the RRC IDLE and INACTIVE relay UE is not supported, relay UE must enter into RRC Connected before remote UE report relay UE ID to gNB. In this case, if gNB does not configure UE aggregation to remote UE immediately,relay UE needs to stay in RRC Connected state. Otherwise C-RNTI will be released upon relay UE enter into IDLE/INACTIE state. Therefore, it is suggested to support RRC IDLE and INACTIVE Relay UE, so that relay UE does not need to stay in RRC Connected state once its C-RNTI is reported to gNB.

- Suppose gNB does not configure UE aggregation to remote UE immediately, it requires the relay UE whose C-RNTI is reported by remote UE to stay in RRC Connected state.

- Support RRC IDLE and INACTIVE Relay UE when remote UE report relay UE ID to gNB.

For the detailed relay UE ID, following options can be considered:

- option1. C-RNTI

- option2. S-TMSI

- option3. SL-SourceIdentity-r17

- option4. A index assigned by the remote UE

- option5. new ID is allocated by the gNB

For option1, as we discussed above, option1 requires relay UE entered into RRC-connected, which is impossible for IDLE/INACTIVE relay UE. Option5 re-use sidelink ID which should be avoided for UE aggregation. Option4 and option5 requires remote UE and gNB to further obtain the relationship between new assigned ID/index and relay UE. For example, for option4, after relay UE enter into RRC connected, relay UE needs to further report the index assigned by remote UE to gNB so that gNB can identify which relay UE is connected with remote UE.

Option2 does not restrict the relay UE’s RRC state. And gNB can identify the relay UE via S-TMSI during relay UE’s RRC connection procedure, no extra signaling is needed.

- Support to use S-TMSI as IDLE/INACTIVE relay UE ID.

During previous RAN2 meeting, for scenario1, we have following agreement:

For a MP split bearer in scenario 1, one PDCP entity at the remote UE is configured with one direct Uu RLC channel and one indirect PC5 RLC channel. - For upstream, a PDCP entity delivers to a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side. - For downstream, a PDCP entity receives from a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side.

For scenario2, whether and how to capture the protocol entities mapping in scenario2 is FFS:

FFS if we need to take decisions on the mapping of protocol entities in scenario 2.

In our opinion, the issue is how to capture the protocol entities mapping for indirect path which is a non-3GPP link. RAN2 has agreed that no adaptation layer is placed over non-3GPP link. So we suggest to describe the mapping relation implicitly and leave the details to UE implementation.

- Actually one possible implementation is that remote UE establish internal transmission tunnels with relay UE for each remote UE’s indirect or multi-path split bearer . After PDCP PDU is generated, remote UE deliver the PDCP PDU to the corresponding tunnel. Upon receiving the PDCP PDU from a given tunnel, Relay UE may deliver the received PDCP PDU to the corresponding Uu RLC channel since 1 to 1 mapping is assumed for remote UE’s RB and relay UE’s Uu RLC channel.

- For scenario2, for upstream, the PDCP entity in remote UE side delivers PDCP PDUs to a Uu RLC entity in relay UE side via non-3GPP link based on UE implementation.

Additionally，it is also FFS on how to configure the 1:1 bearer mapping over Uu link for the indirect path.

Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. Only 1:1 bearer mapping is supported over Uu link for the indirect path. FFS how to configure the mapping.

In Uu interface, the mapping between the RB and RLC bearer supports both 1:1 and 1:N mapping, i.e. one RB can be associated with one or more RLC bearer. The mapping relation is configured by servedRadioBearer included in the RLC bearer configuration as shown in following:

RLC-BearerConfig ::= SEQUENCE { logicalChannelIdentity LogicalChannelIdentity, servedRadioBearer CHOICE { srb-Identity SRB-Identity, drb-Identity DRB-Identity } OPTIONAL, -- Cond LCH-SetupOnly reestablishRLC ENUMERATED {true} OPTIONAL, -- Need N rlc-Config RLC-Config OPTIONAL, -- Cond LCH-Setup mac-LogicalChannelConfig LogicalChannelConfig OPTIONAL, -- Cond LCH-Setup ..., [[ rlc-Config-v1610 RLC-Config-v1610 OPTIONAL -- Need R ]], [[ rlc-Config-v1700 RLC-Config-v1700 OPTIONAL, -- Need R logicalChannelIdentityExt-r17 LogicalChannelIdentityExt-r17 OPTIONAL, -- Cond LCH-SetupModMRB multicastRLC-BearerConfig-r17 MulticastRLC-BearerConfig-r17 OPTIONAL, -- Cond LCH-SetupOnlyMRB servedRadioBearerSRB4-r17 SRB-Identity-v1700 OPTIONAL -- Need N ]] }

Therefore we think 1:1 mapping for scenario2 can be achieved by re-using the servedRadioBearer.

Additionally, it has been agreed that one relay UE only serve one remote UE in scenario2. For relay UE to differentiate the received RLC bearer belongs to remote UE or it’s own, following two options can be considered:

1. Uu RLC bearer configuration(RLC-BearerConfig) is used to configure Uu channel for remote UE and relay UE’s own RLC bearer. Additionally Uu RLC bearer configuration include a remote UE indication.

2. Uu RLC channel configuration(Uu-RelayRLC-ChannelConfig) is used to configure Uu channel for remote UE. Uu RLC bearer configuration(RLC-BearerConfig) is used to set relay UE’s own RLC bearer. Additionally,  to configure 1:1 bearer mapping, Uu RLC channel configuration(Uu-RelayRLC-ChannelConfig) shall include a servedRadioBearer.

From our perspective, option2 re-use signaling designed for sidelink U2N relay, which we think may confuse the UE aggregation and SL U2N relay. So we suggest to adopt option1.

For example, for indirect path Uu RLC channel, after receiving a RLC configuration including remote UE indication and served DRB ID 1 from gNB, relay UE knows this received RLC configuration is used for forwarding the packet of remote UE’s DRB1.

- It is suggested to re-use the servedRadioBearer included in Uu RLC bearer configuration to configure the 1:1 bearer mapping for scenario2.

- For relay UE to differentiate the received RLC bearer belongs to remote UE or it’s own, Uu RLC bearer configuration(RLC-BearerConfig) shall include a remote UE indication.

Timers for path addition and change

RAN2 agree to use T304-like and T420 like timer for path addition and change, in this clause, we discuss them one by one.

T304 like timer for direct path addition and change

For the direct path addition/change, the gNB sends RRC Reconfiguration message including direct path addition/change to remote UE. Upon receiving the direct path addition/change message, remote UE start the T304-like timer and perform random access on the corresponding SpCell.

Upon successful completion of random access on the corresponding SpCell, remote UE considers the direct path addition or change is successful and the T304-like timer can be stopped.

Upon the T304-like timer expiry, due to indirect path of remote UE is still available, remote UE does not need to initiate re-establishment, it can still use the configuration prior to direct path addition message and initiate a failure information reporting procedure to report the direct path addition is failure.

From our view, corresponding change has minor impact to current T304, we can re-use T304.

- For direct path addition, upon the T304-like timer expiry, remote UE use the the configuration prior to direct path addition message and initiate a failure information reporting procedure.

- Re-use T304 for direct path addition and change.

T420 like timer for indirect path addition and change

For indirect path addition/change, the gNB sends the RRCReconfiguration including indirect path addition/change signaling message to the remote UE. The contents in the RRCReconfiguration message can include at least relay UE ID, PC5 Relay RLC channel configuration for relay traffic and the associated indirect/split bearer configuration.Upon receiving  such message, remote UE start T420-like timer and may optionally establish the PC5 link with relay UE.

For direct path to indirect path switch, the UE can only transmit the RRCReconfigurationComplete via indirect path after establishing the unicast link with relay UE successfully. Therefore, successfully sending RRCReconfigurationComplete message(PC5 RLC acknowledgement is received from target L2 U2N Relay UE) can be considered as the complete of direct to indirect path switch.

However, for indirect path addition, depending on the bearer type of SRB1, RRCReconfigurationComplete can be transmitted via direct path or indirect path or both. The stop condition, i.e. PC5 RLC acknowledgement is received from target L2 U2N Relay UE, can only be used when indirect SRB1 or split SRB1 is configured. For direct SRB1, RRCReconfigurationComplete is transmitted via only direct path, legacy condition(PC5 RLC acknowledgement is received from target L2 U2N Relay UE) can not be used again. In this case, we think the stop condition of T420-like for indirect path addition and change should be upon establishing PC5 RRC connection with relay UE.

Similar with direct path addition, upon T420-like timer expiry, the direct path of remote UE is still available. Remote UE can still use the configuration prior to indirect path addition/change message and initiate the failure information reporting. And since T420 is introduced for direct to indirect path switch, then for indirect path addition or change, we also think T420 can be re-used for indirect path addition/change.

- The T420-like stop condition for indirect path addition/change is upon establishing PC5 RRC connection with relay UE.

- Upon T420-like expiry, the remote UE can use the configuration prior to indirect path addition/change message.

- Re-use T420 for indirect path addition/change.

Conclusion

In this contribution, following proposals are given:

- With legacy R17 trigger behaviour, a R17 relay UE can support MP relaying.

- To support PC5 RRC triggers, we may need further study on how to obtain the release information or how to handle the case that R17 relay UE can not identify the PC5 RRC triggers signaling.  Corresponding study is time consuming and we do not see too much benefit to do such optimization.

- Remote UE does not know the channel condition between relay UE and gNB.

- Suppose gNB does not configure UE aggregation to remote UE immediately, it requires the relay UE whose C-RNTI is reported by remote UE to stay in RRC Connected state.

- For scenario 1, the bearer type (i.e. direct bearer, indirect bearer, or multi-path split bearer) of SRB1 and SRB2 may be configured by the gNB independently. It is not necessary to mandate the same bearer type configuration of SRB1 and SRB2.

- It is suggested to revert previous RAN2 agreement and not support PC5 RRC triggers, or at least not purse any new solution on this, i.e. up to gNB implementation.

- Relay UE informs assistance information of indirect path to remote UE on how to split the packet.

- Re-use MCGFailureInformation to report the PC5-RLF.

- Add a new failure type value of indirect path failure in MCF failure type, no need to differentiate detailed PC5 failure type.

- Support RRC IDLE and INACTIVE Relay UE when remote UE report relay UE ID to gNB.

- Support to use S-TMSI as IDLE/INACTIVE relay UE ID.

- For scenario2, for upstream, the PDCP entity in remote UE side delivers PDCP PDUs to a Uu RLC entity in relay UE side via non-3GPP link based on UE implementation.

- It is suggested to re-use the servedRadioBearer included in Uu RLC bearer configuration to configure the 1:1 bearer mapping for scenario2.

- For relay UE to differentiate the received RLC bearer belongs to remote UE or it’s own, Uu RLC bearer configuration(RLC-BearerConfig) shall include a remote UE indication.

- For direct path addition, upon the T304-like timer expiry, remote UE use the the configuration prior to direct path addition message and initiate a failure information reporting procedure.

- Re-use T304 for direct path addition and change.

- The T420-like stop condition for indirect path addition/change is upon establishing PC5 RRC connection with relay UE.

- Upon T420-like expiry, the remote UE can use the configuration prior to indirect path addition/change message.

- Re-use T420 for indirect path addition/change.